Report Concerning Space Data System Standards

Time Management

Informational Report

CCSDS XXX.0-G-1

Draft Green Book

May 2023

AUTHORITY

|  | Issue: | Green Book, Issue 1 |  |
| --- | --- | --- | --- |
|  | Date: | May 2023 |  |
|  | Location: | Not Applicable |  |

(WHEN THIS RECOMMENDED STANDARD IS FINALIZED, IT WILL CONTAIN THE FOLLOWING STATEMENT OF AUTHORITY:)

This document has been approved for publication by the Management Council of the Consultative Committee for Space Data Systems (CCSDS) and represents the consensus technical agreement of the participating CCSDS Member Agencies. The procedure for review and authorization of CCSDS documents is detailed in Organization and Processes for the Consultative Committee for Space Data Systems (CCSDS A02.1-Y-4), and the record of Agency participation in the authorization of this document can be obtained from the CCSDS Secretariat at the e-mail address below.

This document is published and maintained by:

CCSDS Secretariat

National Aeronautics and Space Administration

Washington, DC, USA

E-mail: secretariat@mailman.ccsds.org

is issued by, and represents the consensus of, the CCSDS

FOREWORD

This document is a CCSDS informational report on time management. It contains background material.

Through the process of normal evolution, it is expected that expansion, deletion, or modification of this document may occur. This Recommended Standard is therefore subject to CCSDS document management and change control procedures, which are defined in the Organization and Processes for the Consultative Committee for Space Data Systems (CCSDS A02.1-Y-4). Current versions of CCSDS documents are maintained at the CCSDS Web site:

http://www.ccsds.org/

Questions relating to the contents or status of this document should be sent to the CCSDS Secretariat at the e-mail address indicated on page i.

At time of publication, the active Member and Observer Agencies of the CCSDS were:

Member Agencies

- Agenzia Spaziale Italiana (ASI)/Italy.

- Canadian Space Agency (CSA)/Canada.

- Centre National d’Etudes Spatiales (CNES)/France.

- China National Space Administration (CNSA)/People’s Republic of China.

- Deutsches Zentrum für Luft- und Raumfahrt (DLR)/Germany.

- European Space Agency (ESA)/Europe.

- Federal Space Agency (FSA)/Russian Federation.

- Instituto Nacional de Pesquisas Espaciais (INPE)/Brazil.

- Japan Aerospace Exploration Agency (JAXA)/Japan.

- National Aeronautics and Space Administration (NASA)/USA.

- UK Space Agency/United Kingdom.

Observer Agencies

- Austrian Space Agency (ASA)/Austria.

- Belgian Federal Science Policy Office (BFSPO)/Belgium.

- Central Research Institute of Machine Building (TsNIIMash)/Russian Federation.

- China Satellite Launch and Tracking Control General, Beijing Institute of Tracking and Telecommunications Technology (CLTC/BITTT)/China.

- Chinese Academy of Sciences (CAS)/China.

- Chinese Academy of Space Technology (CAST)/China.

- Commonwealth Scientific and Industrial Research Organization (CSIRO)/Australia.

- Danish National Space Center (DNSC)/Denmark.

- Departamento de Ciência e Tecnologia Aeroespacial (DCTA)/Brazil.

- Electronics and Telecommunications Research Institute (ETRI)/Korea.

- European Organization for the Exploitation of Meteorological Satellites (EUMETSAT)/Europe.

- European Telecommunications Satellite Organization (EUTELSAT)/Europe.

- Geo-Informatics and Space Technology Development Agency (GISTDA)/Thailand.

- Hellenic National Space Committee (HNSC)/Greece.

- Indian Space Research Organization (ISRO)/India.

- Institute of Space Research (IKI)/Russian Federation.

- Korea Aerospace Research Institute (KARI)/Korea.

- Ministry of Communications (MOC)/Israel.

- Mohammed Bin Rashid Space Centre (MBRSC)/United Arab Emirates.

- National Institute of Information and Communications Technology (NICT)/Japan.

- National Oceanic and Atmospheric Administration (NOAA)/USA.

- National Space Agency of the Republic of Kazakhstan (NSARK)/Kazakhstan.

- National Space Organization (NSPO)/Chinese Taipei.

- Naval Center for Space Technology (NCST)/USA.

- Research Institute for Particle & Nuclear Physics (KFKI)/Hungary.

- Scientific and Technological Research Council of Turkey (TUBITAK)/Turkey.

- South African National Space Agency (SANSA)/Republic of South Africa.

- Space and Upper Atmosphere Research Commission (SUPARCO)/Pakistan.

- Swedish Space Corporation (SSC)/Sweden.

- United States Geological Survey (USGS)/USA.

# DOCUMENT CONTROL

| Document | Title | Date | Status |
| --- | --- | --- | --- |
| CCSDS XXX.0-G-1 | Time Management, Draft Informational Report, Issue 1 | May 2023 | Current draft |

# CONTENTS

Section	Page

DOCUMENT CONTROL	iv

CONTENTS	v

1	Introduction	1-1

1.1	Purpose	1-1

1.2	Scope	1-1

1.3	Document structure	1-1

1.4	Definitions	1-2

1.5	References	1-4

2	Overview	2-1

2.1	Concepts	2-1

2.2	The role of general relativity	2-2

2.3	Considerations for standard time services	2-5

2.4	Mission domains	2-6

3	Frequency and timing standards	3-1

3.1	Frequency standards and Clocks	3-1

3.2	Important types of frequency references	3-2

3.3	Evolution of atomic clock stability	3-4

3.4	Redefinition of the second	3-5

3.5	Time scales	3-7

3.6	Network considerations	3-9

3.7	National and international organizations	3-12

4	Global navigation satellite systems	4-15

4.1	Global Positioning System	4-16

4.2	Galileo	4-23

4.3	Global’naya Navigatsionnaya Sputnikovaya Sistema	4-26

4.4	BeiDou Navigation System	4-28

5	Clock correlation	5-1

5.1	Concept	5-1

5.2	NASA Clock correlation methods	5-2

5.3	ESA Clock correlation methods	5-15

5.4	DLR clock correlation methods	5-20

5.5	JAXA clock correlation methods	5-25

5.6	Roscosmos clock correlation methods	5-28

5.7	CAST clock correlation methods	5-30

6	Time Synchronization	6-1

7	Applications	A-1

7.1	Science activities	A-1

7.2	Ranging / Doppler	A-2

7.3	Spacecraft maneuvers	A-3

Acronyms	B-1

ANNEX A	A-5

ANNEX B	B-1

LIST OF FIGURES

Figure 2-1: Illustration of accuracy and stability of a frequency.	2-1

Figure 2-2: Mission domain types.	2-7

Figure 2-3: Aerocentric MD2.	2-8

Figure 2-4: Radiometric time transfer [48].	2-9

Figure 3-1: Principle of atomic clock operation.	3-1

Figure 3-2: Typical atomic fountain clock.	3-3

Figure 3-3: Ion trap used for ion clocks.	3-4

Figure 3-4: Optical Lattice clock: 1D dipole trap.	3-4

Figure 3-5:  Evolution of the fractional systematic uncertainty of atomic clocks over years [14].	3-5

Figure 3-6: Schematic of the clock comparison between PTB and LNE-SYRTE [16].	3-7

Figure 3-7: Detailed hierarchy of Stratum levels.	3-9

Figure 3-8: Christian algorithm principle.	3-10

Figure 4-1. A graphic detailing the different areas of GNSS coverage (from [65]).	4-19

Figure 4-2: Onboard Galileo clocks: On the right rubidium clock. On the left the passive hydrogen maser for Galileo.	4-25

Figure 4-3: Three atomic frequency generators.	4-28

Figure 5-1: Block diagram of time transfer signal and data flow.	5-3

Figure 5-2: Geometry of the ranging measurement.	5-4

Figure 5-3: Downtrack Error SCLK-SCET Error.	5-7

Figure 5-4: Flight SCLK-SCET Calibration Configuration.	5-9

Figure 5-5: Electra clock offset determination with the spacecraft clock in the Loop.	5-10

Figure 5-6: Proposed scheme to remove MRO clock and interface from Electra Clock Offset relative to UTC.	5-11

Figure 5-7: Gaia mission launch and operations requirement for the timing of Gaia.	5-15

Figure 5-8: Frequency stability of the Gaia RAFS represented by its Allan deviation from a test campaign before launch.	5-17

Figure 5-9: Schematic view of the one-way clock synchronization.	5-18

Figure 5-10: Time Transfer Model for Gaia: modelling scheme.	5-19

Figure 5-11: The fixed allocation of the onboard time report within the frame allows an alignment in time during recording on ground to provide a correlation-pair.	5-20

Figure 5-12: The Time Report Packet, TM (9, 2), can be associated with a corresponding virtual channel frame recording on ground to provide a clock correlation pair.	5-24

Figure 5-13: System overview of Time Calibration.	5-26

Figure 5-14: Latch timing.	5-27

Figure 5-15: Orbital configuration of the China Space Station.	5-31

Figure 5-16: Chang ’e-5 mission.	5-34

LIST OF TABLES

Table 2-1: Time Transfer Method Characteristics [2].	2-10

Table 3-1: Principal relativistic effects for Earth and Mars	3-12

Table 4-1: Equation (7) Variables.	4-15

Table 4-2: CNAV Message Types [33].	4-21

Table 4-3: Galileo reference orbit parameters.	4-24

Table 5-1: Final MSL SCLK-SCET table	5-8

Table 5-2: Time calibration system of Japanese planetary explorations.	5-25

Table 5-3: Equation (8) Variables	5-29

Table 5-4: Time format maintained by the core computer of the space station.	5-32

Table 7-1: Science activity timing scales.	A-1

Table 7-2: Mission survey respondents.	A-5

Table 7-3: Mission time requirements.	A-7

# Introduction

## Purpose

The purpose of this Informational Report is to document existing frequency and timing standards, Global Navigation Satellite Systems (GNSS), and clock correlation and time synchronization methods as practiced by CCSDS member agencies in their space missions. This report also identifies requirements of various space missions for time management services. This information provides an introduction and overview for implementers of these time management services, and it also provides a basis for identifying potential need for a CCSDS Blue Book describing recommended standard clock correlation and time synchronization protocols, in order that CCSDS member agencies may interoperate and cross-support each other’s missions.

In no event will CCSDS or its members be liable for any incidental, consequential, or indirect damages, including any lost profits, lost savings, or loss of data, or for any claim by another party related to errors or omissions in this report. This document is a CCSDS informational Report and is therefore not to be taken as a CCSDS Recommended Standard.

## Scope

The concepts, requirements, protocols, and data formats described herein are designed for use as part of a time management system. Such a system enables the relationship between a spacecraft clock and a ground clock – including offset, drift, etc. – to be determined (“clock correlation”), made known to both the spacecraft and the ground (“time transfer”), and aligned (“time synchronization”).

This document provides supporting and descriptive material only. It is not part of a Recommended Standard. In the event of any conflict between a CCSDS Recommended Standard and the material presented herein, the Recommended Standard shall prevail.

## Document structure

This document is divided into seven numbered sections and two annexes:

- Section 1 presents the purpose, scope, document stucture, and references;

- Section 2 provides an overview of time management concepts;

- Section 3 describes frequency and timing standards;

- Section 4 describes GNSS systems

- Section 5 describes clock correlation concepts and implementations by space agencies

- Section 6 describes the time synchronization concept;

- Section 7 describes applications;

- Annex A provides a list of mission time management requirements and characteristics; and

- Annex B defines acronyms used in this document.

## Definitions

Accuracy: The degree of conformity of a measured or calculated value to its definition, related to the offset from an ideal value. In the time and frequency community, accuracy refers to the time offset or frequency offset of a device. For example, time offset is the difference between a measured on-time pulse and an ideal on-time pulse that coincides with Coordinated Universal Time (UTC) [1].

Barycentric Dynamical Time (TDB):  a time scale defined by the International Astronomical Union (originally in 1976; named in 1979; revised in 2006) for use as an independent argument of barycentric ephemerides and equations of motion [50].

Clock: A device that generates periodic, accurately spaced signals for local timekeeping applications. A clock consists of at least three parts: an oscillator, a device that counts the oscillations and converts them to units of time interval (such as seconds, minutes, hours, and days), and a means of displaying or recording the results [1].

Clock-ensemble: A group of clocks or oscillators whose outputs are averaged to create a time scale. Typically, the relative value of each clock is weighted, so that the best clocks contribute the most to the average [1].

Coordinated Universal Time (UTC): An atomic time scale that forms the basis for the coordinated dissemination of standard frequencies and time signals. The hours, minutes, and seconds expressed by UTC represent the time-of-day at the Earth's prime meridian (0° longitude) located near Greenwich, England [1].

Ephemeris Time (ET): the time scale used prior to 1984 as the independent variable in gravitational theories of the solar system.  In 1984, ET was replaced by dynamical time [50]. See TDB and TT.

Epoch: The beginning of an era (or event) or the reference date for a system of measurements [1].

Euclidean geometry: The study of flat space. Between any two points there is a unique line segment which is the shortest curve between those two points. Line segments can be extended to lines. Euclid defined two or three dimensions but the geometry can be extended to more.

GNSS (Global Navigation Satellite Systems): A satellite system that can be used to locate a user’s receiver anywhere in the world [1]. The system simultaneously determines the four parameters of position and time.

Group delay differential: The difference in timing between the emission of two signals [30][32].

International Atomic Time (TAI): A time scale maintained internally by the Bureau International de Poids et Mesures (BIPM) calculated using data from more than 450 atomic clocks in over eighty national laboratories. TAI realizes the SI second as closely as possible, and runs at the same frequency as UTC). However, TAI differs from UTC by an integral number of seconds. This difference is related to leap seconds, and increases whenever a leap second occurs [1].

Leap second: A second added to UTC to make it agree with astronomical time to within 0.9 second. UTC is an atomic time scale, based on the performance of atomic clocks. Astronomical time is based on the rotational rate of the Earth. Since atomic clocks are more stable than the rate at which the Earth rotates, leap seconds are needed to keep the two time scales in agreement. [1]

Mission Domain (MD): The spatial domain in which a mission operates. Missions may transit multiple domains during launch, orbital transfer, orbital insertion and landing.

Navigation: The ability to determine current and desired position, and apply corrections to course, orientation, and speed to attain a desired position.

Network Time Protocol (NTP): A standard protocol used to send a time code over packet-switched networks, such as the public Internet. The NTP was created at the University of Delaware, and is defined by the RFC-1305 document. The NTP packet includes three 64-bit time stamps and contains the time in UTC seconds since January 1, 1900 with a resolution of 233 picoseconds [1].

Precision: The degree of mutual agreement among a series of individual measurements, values, or results, analogous to standard deviation or resolution. Alternately, the ability of a device to produce, repeatedly and without adjustments, the same value or result, given the same input conditions and operating in the same environment [1].

Pseudo-Riemann space: Geometries which allow for curved spaces with any number of dimensions and include Euclidean geometries as special cases.

Resolution: The degree to which a measurement can be determined. For example, if a time interval counter has a resolution of 10 ns, it can produce a reading of 3340 ns or 3350 ns, but not a reading of 3345 ns [1].

Stability: An inherent characteristic of an oscillator that determines how well it can produce the same frequency over a given time interval [1]

Stratum clock: A clock in a telecommunications system or network that is assigned a number that indicates its quality and position in the timing hierarchy. Only stratum 1 clocks may operate independently; other clocks are synchronized directly or indirectly to a stratum 1 clock [1].

Terrestrial Dynamical Time (TDT):  the time scale for apparent geocentric ephemerides defined by a 1979 IAU resolution.  In 1991, it was replaced by Terrestrial Time (TT).  Obsolete [50].

Terrestrial Time (TT): An astronomical time scale which equals TAI + 32.184 s. The uncertainty of TT is ±10 μs. It is one of the time scales that replaced the now obsolete Ephemeris Time scale in 1984 [1].

Time: The designation of an instant on a selected time scale, used in the sense of time of day [1].

Time correlation: The determination of the time offset between two continuous time scales provided. This procedure requires knowledge of clock parameters and propagation delay of the time transfer.

Time interval: The elapsed time between two events. In time and frequency metrology, time interval is usually measured in small fractions of a second, such as milliseconds, microseconds, or nanoseconds [1].

Time scale: An agreed upon system for keeping time. All time scales use a frequency source to define the length of the second, which is the standard unit of time interval. Seconds are then counted to measure longer units of time interval, such as minutes, hours, and days [1].

Time synchronization:  The process of setting two or more clocks to the same time [1].

Timestamp: A set of symbols in a pre-defined format used to encode information identifying the time a certain event occurred.

Time transfer: A measurement technique used to send a reference time or frequency from a source to a remote location [1].

Time unit: The reference time unit is a second, which is defined as the duration of 9 192 631 770 periods of the radiation corresponding to the transition between the two hyperfine levels of the ground state of the cesium 133 atom [1]. BIPM has devised a roadmap to redefine the second to fractional accuracies on the order of 10-17.

## References

The following documents are referenced in this document. At the time of publication, the editions indicated were valid. All documents are subject to revision, and users of this document are encouraged to investigate the possibility of applying the most recent editions of the documents indicated below. The CCSDS Secretariat maintains a register of currently valid publications.

- NIST Time and Frequency Services. Retrieved June 24, 2020, from  https://www.nist.gov/pml/time-and-frequency-division/popular-links/time-frequency-z/time-and-frequency-z-z-index.

- Larry Felton, Lee Pitts, and Frank VanLandingham, “NASA Architecture for Solar System Time Synchronization and Dissemination: Concept of Operations,” SpaceOps Conference, 2008. Retrieved June 24, 2020 from https://arc.aiaa.org/doi/10.2514/6.2008-3229.

- John R. Vig, “Frequency and Control and Timing Application Tutorial,” 2007. https://www.slideshare.net/kuangjou/vig-tutorial-jan2007.

- Jacques Vanier and Claude Audoin, “The Quantum Physics of Atomic Standards,” IOP publishing, 1989.

- J.D. Prestage, G. J. Dick, L. Maleki, “Linear Ion Trap Based Atomic Frequency Standard,” IEEE Trans. Instrum. Meas., vol. 40, pp. 132-136, Apr. 1991.

- Clairon, A., Salomon, C., Guellati, S., and Phillips, W. D., “Ramsey resonance in a Zacharias fountain,” Europhysics Letters, 16(2):165{170), 1991.

- Andreas Bauch, A., “The PTB primary clocks CS1 and CS2,” Metrologia, 42 S43, 2005.

- Jefferts, S. R., Heavner, T. P., Parker, T. E., and Shirley, J. H., “NIST cesium fountains - current status and future prospects,” Proceedings of 2007 SPIE Conference, 6673:9, 2007.

- H. Schatz and al, Phys. Rev. Lett., 76:18-21, Jan 1996.

- Theodore W. Hansch, Nobel lecture: Passion for precision. Rev. Mod. Phys, 78:1297-1309, Nov 2006.

- John L. Hall, Nobel Lecture: Defining and measuring optical frequencies. Rev Mod. Phys., 78:1279-1295, Nov 2006.

- C. Lisdat et al., “A clock network for geodesy and fundamental science,” Nature Communications 7, Article number: 12443, 2016.

- https://hal.sorbonne-universite.fr/hal-02183608/document.

- S. A. Klioner, Microsecond Astrometry in space: relativistic effects and reduction of observations, The Astronomical Journal, Volume 104, number 2, August 1992.

- Klioner, ‘’Numerical versus analytical accuracy of the formulae for light propagation’’, Classical and Quantum Gravity 27 (7), 27:075015, 2010.

- Klioner, ‘’High-accuracy timing for Gaia data from one-way time synchronization, in: Recent developments and prospects in ground-based and space astrometry’’, https://syrte.obspm.fr/jsr/journees2014/pdf/Klioner.pdf.

- European Cooperation for Space Standardization, “Ground systems and operations – Telemetry and telecommand packet utilization”, ECSS-E-70-41A, 30 January 2003
https://ecss.nl/standard/ecss-e-70-41a-ground-systems-and-operations-telemetry-and-telecommand-packet-utilization/.

- European Cooperation for Space Standardization, “Telemetry and telecommand packet utilization”, ECSS-E-ST-70-41C, 15 April 2016
https://ecss.nl/standard/ecss-e-st-70-41c-space-engineering-telemetry-and-telecommand-packet-utilization-15-april-2016/.

- Consultative Committee for Space Data Systems, “Time Code Formats,” 
CCSDS 301.0-B-4, November 2010, https://public.ccsds.org/Pubs/301x0b4e1.pdf.

- National Coordination Office for Space-Based Positioning, Navigation, and Timing, “Other Global Navigation Satellite Systems” November 9, 2020. Retrieved December 2, 2020 from https://www.gps.gov/systems/gnss/.

- William H.Guier and George C. Weiffenbach, “Genesis of Satellite Navigation.”  Retrieved on June 24, 2020 from https://www.jhuapl.edu/Content/techdigest/pdf/V18-N02/18-02-Guier.pdf.

- “Signal Specification GPS Civil Performance Standards” December 1993  Retrieved on June 24, 2020 from  https://www.gps.gov/technical/ps/1993-SPS-signal-specification.pdf.

- “Global Positioning System (GPS) Standard Positioning Service (SPS) Performance Standard - 5th Edition,” April 2020. Retrieved on February 28, 2023 from  https://www.gps.gov/technical/ps/2020-SPS-performance-standard.pdf.

- “Space Segment.”  Retrieved on June 24, 2020 from  https://www.gps.gov/systems/gps/space/.

- “Timing.”  Retrieved on June 24, 2020 from https://www.gps.gov/applications/timing/.

- Munther A. Hassouneh, Jason W. Mitchell, and Luke B. Winternitz, “GPS on the Moon,” Cutting Edge, Goddard’s Emerging Technologies, Volume 13, Issue 3, Spring 2019. Retrieved on June 24, 2020 from https://www.nasa.gov/sites/default/files/atoms/files/spring_2019_final_web_version.pdf.

- Katherine Schauer, “LunaNet Concept,” September 12, 2019. Retrieved on June 24, 2020 from https://esc.gsfc.nasa.gov/news/_LunaNetConcept.

- Adam Steltzner, “Challenges of Getting to Mars: Curiosity’s Seven Minutes of Terror,” June 12, 2012. Retrieved on June 24, 2020 from  https://mars.nasa.gov/resources/20049/challenges-of-getting-to-mars-curiositys-seven-minutes-of-terror/.

- “Engineering Webb Space Telescope,” Goddard Space Flight Center. Retrieved on June 24, 2020 from https://www.jwst.nasa.gov/.

- “GPS Interface Specification IS-GPS-200, Revision N,” August 2022. Retrieved on February 28, 2023 from https://www.gps.gov/technical/icwg/IS-GPS-200N.pdf.

- “2021 Federal Radionavigation Plan,” 2021. Retrieved on February 28, 2023 from https://www.navcen.uscg.gov/sites/default/files/pdf/2021_Federal_Rdionavigation_Plan.pdf.

- “GPS Interface Specification IS-GPS-705, Revision J,” August 2022. Retrieved on February 28, 2023 from https://www.gps.gov/technical/icwg/IS-GPS-705J.pdf.

- Issues in Defining a NASA Time Architecture Time Team Report; Beryl L. Brodsky (Overlook Systems Technologies, Inc.), Al Gifford (NASA Headquarters), Robert A. Nelson (Satellite Engineering Research Corporation), A. J. Oria (Overlook Systems Technologies, Inc.), Lee Pitts (Computer Sciences Corporation), Richard S. Orr (SATEL LLC), John Prestage (Jet Propulsion Laboratory), James Schier (NASA Headquarters), Frank VanLandingham (Computer Sciences Corporation).

- CCSDS BUNDLE PROTOCOL SPECIFICATION, RECOMMENDED STANDARD, CCSDS 734.2-B-1, September 2015;  https://public.ccsds.org/Pubs/734x2b1.pdf.

- IRTF RFC 5050 Bundle Protocol Specification. Retrieved on October 29, 2021 from https://datatracker.ietf.org/doc/html/rfc5050.

- IETF RFC BPBis Bundle Protocol Specification version 7, https://datatracker.ietf.org/doc/draft-ietf-dtn-bpbis.

- IOAG catalog; service catalog #2 issue 1 revision 3 - 06/02/2019

- Mills, D.L. Network Time Synchronization: the Network Time Protocol on Earth and in Space, Second Edition, CRC Press 2011.

- ITU – R, TF.2118-0 Relativistic Time Transfer, 12/2018.

- ITU Satellite Time and Frequency Transfer and Dissemination Handbook, 2010.

- Key Issues for Navigation and Time Dissemination in NASA's Space Exploration Program; 12 Ka and Broadband Communications Conference, Naples, Italy, September 2006.

- “Internet Access to Spacecraft”, Small Satellite Conference 2000, James Rash,Ron Parise, Keith Hogie, Ed Criscuolo, Jim Langston, Chris Jackson, Harold Price.

- Deep Space Atomic Clock Mission Overview, AAS-19-796, Todd A. Ely, Jill Seubert, John Prestage, Robert Tjoelker, Eric Burt, Angela Dorsey, Daphna Enzer, Randy Herrera, Da Kuang, David Murphy, David Robison, Gabrielle Seal, Jeff Stuart, Rabi Wang.

- A. Gifford, S. Pace, J. McNeff, “One-Way GPS Time Transfer 2000”, 32nd Annual Precise Time and Time Interval (PTTI) Meeting, November 2000.

- Proximity-1 Space Link Protocol-Data Link Layer, CCSDS Blue Book, July 2020. https://public.ccsds.org/Pubs/211x0b6.pdf.

- User Spacecraft Clock Calibraton System (USCCS) Users’ Guide. NASA Goddard, 531-TR-001. Greenbelt, MD: NASA Goddard, 1999.

- “Spectroscopic Methods in Mineralogy and Material Sciences”, Spectroscopy from Space, Reviews in Mineralogy and Geochemistry Vol. 78 Jan 01, 2014, Roger Clark, Gregg Swayze, Robert Carlson, Will Grundy, Keith Noll.

- Interferometry, NASA ISRO SAR Mission, Jet Propulsion Laboratory, https://nisar.jpl.nasa.gov/mission/get-to-know-sar/interferometry/.

- Magnetometry, Encyclopedia of Planetary Science, 1997, Mario Acuña.

- The Astronomical Almanac for the Year 2018, U.S. Government Publishing Office, 2017.

- “Lockheed Martin Navigation Systems Awarded $2.8 Million to Assist US Air Force In Developing Next Generation GPS Satellite Systems,” PRNewswire, King of Prussia, PA, USA, April 2002.

- W.H. Guier and G.C. Weiffenbach, “Genesis of Satellite Navigation,” Johns Hopkins APL Technical Digest, Volume 19, Number 1 (1998), Laurel, MD, USA, May 2012.

- “GPS III/IIIF: The Next Generation of Positioning, Navigation and Timing,” Lockheed Martin, Bethesda, MD, USA, December 2021.

- “Fifth GPS III Satellite Takes to the Skies,” Lockheed Martin, Bethesda, MD, USA, July 2021.

- “Selective Availability” GPS.gov, Arlington County, VA, May 2021.

- “Space Segment” GPS.gov, Arlington County, VA, May 2021.

- “U.S. Air Force Awards Lockheed Martin $395 Million Contract for GPS III Satellites 9 and 10,” PRNewswire, Denver, CO, September 2016.

- E. Almeida, “MEOSAR Space Segment (W8474-177407/A),” Public Services and Procurement Canada, Canada.

- J. LaBrecque, “Development of the GPS III Laser Retroreflector Array (LRA),” NASA HQ, Washington DC, August 2012.

- “Next Generation Operational Control System (OCX),” GPS.gov, Arlington County, VA, December 2021.

- “Global Positioning System Next Generation Operational Control System,” Raytheon, Aurora, CO.

- D. Baird, “NASA Explores Upper Limits of Global Navigation Systems for Artemis,” NASA, January 2021.

- G. Ji, K. Kwon, and J. Won, “GNSS Signal Availability Analysis in SSV for Geostationary Satellites Utilizing multi-GNSS with First Side Lobe Signal over the Korean Region,” MDPI, September 2021.

- J.J.K. Parker, “NASA Space User Update: Advancing Interoperability and Lunar PNT,” NASA, September 2020.

- Space Network Users’ Guide (450-SNUG) https://www.abbottaerospace.com/downloads/nasa-450-snug-rev-10-space-network-users-guide-snug-revision-10/.

- D. M. Boroson, J. J. Scozzafava, D. V. Murphy, B. S. Robinson and M. I. T. Lincoln, "The Lunar Laser Communications Demonstration (LLCD)," 2009 Third IEEE International Conference on Space Mission Challenges for Information Technology, 2009, pp. 23-28, doi: 10.1109/SMC-IT.2009.57.

- TM Space Data Link Protocol, CCSDS Blue Book, August 2021. https://public.ccsds.org/Pubs/132x0b3.pdf.

- R. A. Nelson and T. A. Ely, “Relativistic Transformations for Time Synchronization and Dissemination in the Solar System,” Proceedings of the 38th Annual Precise Time and Time Interval (PTTI) Systems and Applications Meeting (U. S. Naval Observatory, Washington, DC, 2006).

- “LunaNet Interoperability Specification Document,” Version 4, NASA/ESA, September 12, 2022.

- “1588-2002 - IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems,” IEEE 1588-2002, IEEE, 8 November 2002, doi:10.1109/IEEESTD.2002.94144, ISBN 978-0-7381-3369-0/

- Consultative Committee for Space Data Systems, “Space Link Extension – Return All Frames Service Specification”, CCSDS 911.1-B-4, August 2016, https://public.ccsds.org/Pubs/911x1b4.pdf.https://public.ccsds.org/Pubs/911x1b4.pdf.

- [New 1] GPS Interface Specification IS-GPS-800, Revision J - August 2022. Retrieved on February 28, 2023 from https://www.gps.gov/technical/icwg/IS-GPS-800J.pdf.

- [New 2] GPS.gov: Control Segment Retrieved on February 28, 2023 from https://www.gps.gov/systems/gps/control/.

- [New 3] GPS.gov: GPS Accuracy. Retrieved on February 28, 2023 from https://www.gps.gov/systems/gps/performance/accuracy/.

- Walker constellation design. Retrieved May 8, 2023 from  https://en.wikipedia.org/wiki/Satellite_constellation#:~:text=For%20example%2C%20the%20Galileo%20navigation,360%20degrees%20around%20the%20equator .

- IRIG Time Code Format. IRIG STANDARD 200-98, Prepared by TIMING COMMITTEE TELECOMMUNICATIONS AND TIMING GROUP RANGE COMMANDERS COUNCIL, May 1998. Retrieved May 9, 2023 from https://www.hopf.com/downloads/manuals/irig-standard_v200-98_en.pdf.

- Scanning Law. Retrieved May 9, 2023 from https://www.cosmos.esa.int/web/gaia/scanning-law.

# Overview

## Concepts

This Informational Report describes various aspects of clock correlation, time synchronization, time transfer, frequency and timing standards, Global Navigation Satellite Systems (GNSS), and CCSDS time services. These concepts, which are briefly defined in Section 1.4, are expanded upon in the remainder of this report.

### Clock, Stability, accuracy

The most basic term we define is a clock, which is a device that generates periodic, accurately spaced signals for local timekeeping applications. A clock consists of at least three parts: an oscillator, a device that counts the oscillations and converts them to units of time interval (such as seconds, minutes, hours, and days), and a means of displaying or recording the results. This device is the foundation of all other actions and measurements that relate to time.

The accuracy of a clock denotes how close the measured frequency is to the correct frequency and is usually determined by the ability to precisely evaluate systematic shifts of the clock frequency. The instability characterizes how much the frequency changes over time. Those terms are nicely visualized in Figure 2-1 [3] comparing the temporal output of an oscillator with a marksman’s sequence of bullet holes on a target.

Figure 2-1: Illustration of accuracy and stability of a frequency.

### time transfer, clock correlation, and time synchronization

Time transfer is a technique used to send a reference time or frequency from a source to a remote location. This may be done in one direction, or bi-directionally. Data is typically transmitted using an agreed-to format for expressing time, and CCSDS has standardized  time code formats for this purpose [19]. The expression of time may be explicitly signaled in a time packet or implicitly as an interpolated time in a telemetry frame transmitted with a known symbol rate.

Clock correlation is the determination of the time offset between two continuous time scales provided by a master clock and a local (spaceborne) clock. This procedure requires knowledge of clock parameters and propagation delay of the time transfer. Historically, this has also been called time correlation, but clock correlation is the more accurate and descriptive term and the one used by the Interagency Operations Advisory Group (IOAG). Clock correlation associates the stability, skew, drift, relativistic effects of one clock with that of another, accounting for the propagation delay induced by the signal path(s) during time transfer between two clocks. In a typical clock-correlation protocol, time transfer is used as a constituent step, which may involve the transmission of multiple time codes from a source to a destination.

Time synchronization is the process of setting two or more clocks to the same time. Depending on the needs of a mission, it may not be necessary to physically synchronize ground and space clocks. This might be the case, for example, for a mission in which spacecraft activities take place at the direction of ground commanding and the ground already has an accurate clock correlation file between the Earth time and spacecraft time. Time synchronization is distinguished from syntonization [37] which is the process of setting two or more oscillators to the same frequency. This is an important distinction in that a clock is a more elaborate device than an oscillator.

Several time services are defined by the IOAG in its service catalog #2 [37]. These services allow the calculation of clock correlation elements and synchronization by means of time distribution. Specifically, there are three standards identified to support this service: a space link standard for time transfer, and two ground standards: clock correlation (time correlation) and time synchronization.

## The role of general relativity

The principles of general relativity often are accounted for in time management of spacecraft. The mathematical algorithms for modeling signal propagation and relativistic effects across the solar system and the necessary protocols for exchanging time and data are widely available.

The ITU has identified algorithms and justification for Relativistic Time Transfer [39], which are beyond the scope of this document. However, a brief derivation follows to demonstrate their application.

On Earth, the purpose of time synchronization is to align clocks to a common timescale, such as UTC. This makes a fundamental assumption that clocks all run at a common rate; that is, one second on one clock is the same as one second on any other clock. “In planetary systems, things are not that simple” [38]. The duration of a second can be different in different referential frames.

Following is an example of a clock in the Earth Centered Inertial (ECI) reference frame and the effects of velocity and relativity. The invariant space-time interval under the Special Theory of Relativity (SR) is

|  | Δs2= -cΔt2+Δx2+Δy2+Δz2, | (1) |
| --- | --- | --- |

where x, y, and z are the distances in space between the two events, t is the “distance” in time between the two events and c is the speed of light. Therefore, the invariant space time interval along an axis is

| Δs2=0 | expanding wave front in an inertial reference frame | (2) |
| --- | --- | --- |
| Δs2= -cΔt2 | time of a clock at rest | (3) |

In special relativity, postulates for all physical laws can be stated as

- There is no preferred or absolute inertial system. That is, all inertial frames are equivalent for the description of all physical laws.

- The speed of light in vacuum is the same for all observers who are in uniform, rectilinear, relative motion and is independent of the motion of the source. Its free space value is the universal constant c given by Maxwell’s equations.

For factoring in space, time, and gravity in general relativity, the invariant space-time interval can be re-written such that

|  | ds2=guvdxudxv | (4) |
| --- | --- | --- |

where guv is the metric tensor for pseudo-Riemann space. General relativity gives the relationship between proper time and a coordinate time through the invariance of the space-time interval.

Proper time in general relativity is time that is observed / recorded by a clock in inertial space. A proper time is different for clocks in different gravitational potentials and in different states of motion.

Coordinate time in general relativity, by definition, has the same value everywhere for a given event and is a measure in 4-space of that event.

In the Earth-centered inertial coordinate frame, the approximate elapsed coordinate time can be determined for a satellite orbiting near Earth using terms of order 1/c2. The components of the metric tensor are g00=1-2U/c2, g0j=0, and gij=1+2U/c2δij≈1. The elapsed coordinate time in the ECI coordinate system corresponding to the elapsed proper time during the transport of a clock between points T0 and T is:

|  | Δt= T0T1+Uc2+V22c2dτ | (5) |
| --- | --- | --- |

Redshift is the second component of the integral Uc2, time dilation is the third component V22c2, and U is the Newtonian gravitational potential. Using a GNSS system as an example, the GPS is in a six MEO plane configuration with an orbital semi-major axis of 26562 km. They maintain their clocks to less than or equal to 30 ns. The two relativistic effects of time dilation and redshift have a combined effect of ~38.575 μS on a daily basis [40].

~45 μS faster per day due to gravitational red shift

~7 μS slower per day due to time dilation

As can be seen the relativistic errors dwarf the uncertainty of the spaceborne clock by a factor of 1285.83:1 and if not corrected would become a prodigious error source in a matter of minutes. These are not the only effects that may need to be considered. For instance, orbits are ellipses. In the example of GPS, they have an eccentricity of 0.02 as defined in equation (5). This equates to a sinusoidal variance of amplitude 46 nanoseconds for an orbital period. For most uses, this is negligible and may be removed post measurement since it is not cumulative.

|  | Δtr=2c2GMaesinE=2r⋅Vc2 | (6) |
| --- | --- | --- |

Where r and V are the position and velocity of the satellite and their product is a scalar [40].

A second perturbation is the Sagnac effect. The Sagnac effect must be considered for time transfer from a satellite to a ground deployed clock whether on Earth, Mars, or the Moon. For the previous example of a GPS satellite and a ground clock at the equator, the maximum correction can be as great as 133 ns [40].

Harkening back to two basic premises of special relativity, the methodology that created this result will apply in all reference frames. Specifically, to the current areas of interest. The following Mission Domains (MD) can be defined such they are applicable across the Solar system. They do not constrain the mechanisms but act as a framework to provide Positioning, Navigation, and Timing (PNT) services. Using a well-defined methodology, each mission can be handled uniquely based on supporting infrastructure.

- Terrestrial environment at least to geostationary (GEO) orbit

- Planetary domains where multiple platforms require time

- Orbital transfer activities or singleton missions not in a Mission Domain with a time infrastructure

- Lunar domain where multiple platforms require time.

The generation of the capability to provide networked PNT services is essential in future architectures, e.g., [69], and this is the type of architecture supported by Delay Tolerant Networking (DTN).

It is unreasonable to believe that as the number of deployed systems increases, there will be end-to-end connectivity to all on a routine basis or that all will have the stability of atomic clocks. This is the type of architecture supported by Delay Tolerant Networking (DTN).

## Considerations for standard time services

Time services must be internationally accessible and compatible to support mission interoperability. Every effort is to be made to maximize opportunities for interaction and insure ubiquitous access to services. General rules of the road that should be agreed upon are [2]:

- Monotonic time shall be used

  - Based on SI second in the Earth reference frame (Geoid)

  - Non-repeating (232-1 equals 136.099 years), open ended

  - Common epoch (DTN uses January 1, 2000)

- Time scale tied to TAI, UTC, and other terrestrial time scales

- Services are domain-accessible and usable in the constrained space environment

  - Every domain may not have every time service

  - A subset is supported based on the domain

    - One-way links

    - Two-way links

    - Low-bandwidth links

    - Variable Latency

    - Hop-by-hop transfers

    - Periodic discontinuities

- Application of time services to mission systems is at the discretion of the mission or set of missions to minimize discontinuities based in drift, clock rate differences, or time since last synchronization.

In order to accommodate high-precision time within such an architecture, it is necessary to define component requirements. Any reference clock must be capable of high-level accuracy and precision. To accomplish this, clocks must be maintained in a stable environment with all its components available and operating with nominal behavior. This means clock components are maintained in a thermally controlled environment with acceptable levels of hazards mitigated, system (spacecraft) functioning nominally and the ability communicate or use the clock. For primary reference clocks (PRC) in a domain, atomic clocks may be required.

As of 2020, no atomic clock has left Earth orbit though various types have been deployed in orbit. A highly accurate and precise clock available as a primary reference clock (PRC) is an enabling technology for time dissemination throughout the solar system. One such example is the NASA / JPL Deep Space Atomic lock (DSAC) launched on the US Department of Defense Space Test Program (STP)-2 mission. The DSAC demonstration unit (DU) has not incorporated all efficiencies. The DU is a small package of 17 liters volume, 16 kg mass, and 50 W power consumption. The clock within this small space-hardened package has an Allan deviation under 4×10-15 at one-day. It is anticipated that it will achieve a stability of approximately 3×10-15 at one-day [43].

A PRC with high stability enables a wide range of capabilities and is necessary for solar system wide time dissemination system. Benefits are:

- Provides more accurate measurement enabling autonomous navigation,

- Reduces time to make a correct measurement since a two-way path is not required to calculate time biases from an Earth based observer,

- Eliminates intermediate error sources,

- Infrastructure item whether deployed on ground or orbit

To be useful and efficient, timing services need to be available to spacecraft, ground platforms, flying platforms, and habitats in a broad spectrum of environments across a broad range of accuracies.

The basic components of a common solar system wide timescale will be:

- clocks (PRC and client/user),

- measurements of the proper time differences among clocks,

- relativistic transformations of the local clock,

- formation of the timescale using transformed clock observations,

- dissemination of individual clock offsets to the common timescale, and

- synchronization of individual clocks to the common timescale.

## Mission domains

Types of transforms defined in Section 2.2 are critical to the successful time services as called for by the IOAG to include correlation, transfer, and synchronization across the solar system.

The concept of Mission Domain is introduced to differentiate regions of the solar system that from a relativistic standpoint are invariant and may host sufficient assets for a "local" time infrastructure. A local infrastructure may be a composite of multinational assets. Any of the CCSDS standards may be applicable within any domain. However, the use of domains and local time infrastructure will allow common services to be used with the least overhead and reduced risk.

Within these domains, it can generally be assumed that methods and contributions are consistent as defined by reference [44]. Two domains are origin unique; terrestrial (MD1) and lunar (MD4). MD2 can be replicated where needed and MD3 devolves to the current methods of time transfer and clock correlation.

Following are definitions and discussion to address their features. Based on these domains, operational concepts can be generated to illustrate and describe methods of time transfer within and between mission domains. These domains are representative of the reference frames in which a mission operates. A time architecture developed from these concepts would utilize assets within each domain and not require extension. This requires a set of common services that are ubiquitous within a domain. The four proposed domain types are illustrated in Figure 2-2.

Figure 2-2: Mission domain types.

Within these domains, it can generally be assumed that methods and contributions are consistent as defined by reference [41]. Two domains are origin unique; terrestrial (MD1) and lunar (MD4). MD2 can be replicated where needed and MD3 devolves to the current methods of time transfer and clock correlation.

### Mission domain 1

Mission Domain 1 (MD1) comprises the terrestrial environment and is the most dynamic and commonly-used. It includes most national space agencies and national assets such as GNSSes. This includes space out to geosynchronous orbits. This is an arbitrary distinction based on activity level or population of space borne assets. MD1 is the domain with the most assets and usable operations methods, including time. They reflect a broad range of formats and capabilities and it is expected to change as more and more application are available for the terrestrial environment.

### Mission Domain 2

Mission Domain 2 (MD2), shown in Figure 2-3, comprises a minimal infrastructure that supports multiple ground and space-based missions. There may be several distinct MD2’s around different bodies, each operating separately, in coordination from Earch. The domain is managed from Earth and is based on steered clocks that act as a reference for the domain. MD2 assets act as the source of time and navigation in each MD2 domain and provide pointing, navigation, and timing.

Consumers of these services are not directly dependent on Terrestrial contact to maintain PNT knowledge. No direct connect is required for consumers of PNT services. Command and control and data delivery will be networked. (Currently, most assets establish communication directly to Earth or through an orbiting relay.) As a result, the communication infrastructure can be more fully used. There are two obvious examples where such a domain is useful; specifically, MD2 would support a planetary domain such as Mars or a free space domain such as a Lagrange point.

Figure 2-3: Aerocentric MD2.

An Areocentric (Mars-centric) MD2 could maintain replicas of a DSN-provided PRC. Replicas are maintained in the Areocentric reference space. The Martian replica clock would act as the master clock in the Areocentric domain. A secondary stable clock would be designated as well. The creation of an MD2 type domain would allow for a limited implementation of required relativistic correction [68] on a domain basis thereby eliminating the need to apply correction in each remote platform from Earth.

For an Areostationary system, a minimum of three satellites would be required to support redundancy and a required number of satellites in view. A method of implementation would be through radiometric time transfer via communication links to manage the Areostationary platforms. The following steps illustrate an example of an acceptable method to set the MD2 master clock. Figure 2-4 below illustrates the methodology. Several assumptions may need to be met to support time transfer.

- The transmission medium exhibits reciprocity

- Propagation time tp is independent of transmission direction

- The propagation delay tp is initially unknown

- The PRC or Earth-bound clock is “correct” or truth

- Its reading coincides with time in the common time scale (UTC)

- The Domain 2 clock may be “incorrect”

- Its reading is offset, with positive values meaning it is “running fast”

- Transmission time readings are error-free

- Reading is simultaneous with transmission epoch

- Reception time reading may be in error

- Reception time results from computing the arrival time of a signal. The measurement is corrupted by additive, zero-mean Gaussian noise of predictable variance.

Figure 2-4: Radiometric time transfer [45].

During contacts with Earth systems, the MD2 replica hosted terminal will initiate a change with the PRC hosting terminal. The PRC terminal will respond to the MD2 terminal with a return message containing the TPRT and TPRR clock readings. MD2 can utilize all four readings to determine clock offset from the ambient time scale. With multiple transactions and subject to the accuracy of the MD2 and the PRC, time transfer accuracy can approach 1 ns. To further isolate the systems from the hazards of an orbital platform, the MD2 replica can be on a planetary surface in a more stable environment and be used to steer the orbital clocks.

The interface/or access point for MD2 related time synchronization during normal operations (and probably most contingency operations) would be through on orbit relays that are configured as a DTN. Such an interface point would require the necessary infrastructure to support time services; including access to the high precision Martian master clock(s) with fine or precision capability. In addition, the ability to disseminate time is paramount. These timing services should equate to many of those provided in domain MD1 to include a one-way time transfer much like GNSS, a two-way network time protocol (NTP/IEEE 1588) [70], and radiometric methods supporting contingency operations.

Currently, primary methods of determining and tracking time are radiometric and correlative, respectively. This is particularly true above low Earth orbit (LEO). However, there are alternatives for spacecraft equipped to support synchronization. Both two-way and one-way synchronization techniques have been demonstrated. For spacecraft, most have been in the millisecond accuracy range which supports most command and control modes.

One of the first demonstrations of two-way synchronization was the Operating Missions as Nodes on the Internet (OMNI) [42]. This program utilized a version of NTP ported from BSD 4.4 on the UoSAT-12. Tests demonstrated that clocks could be handily synchronized in LEO orbit using NTP. Protocols such as NTP do not merely transfer time but also compensate for transfer delay as well as clock instability, within limits of the protocol. Modification of NTP can be formulated specifically for space environments to accommodate range effects such as real time latency. In its purest form, NTP requires no specialized hardware. It is a software implementation.

Another synchronization standard is IEEE-1588. 1588 has many of the same characteristics as NTP3 or NTP4. However, both NTP and IEEE-1588 are more “chatty” than traditional correlative techniques since they require a dialog. These protocols experience difficulty in interplanetary space where long baselines introduce delays that make a chatty protocol unworkable. As such, they are most supportable for users in mission domains 2 and 4.

Table 2-1-: Time Transfer Method Characteristics [2].

| Characteristic | NTP | IEEE-1588 | GPS |
| --- | --- | --- | --- |
| Spatial Extent | Wide Area Network | A Few Subnets | Wide Area Network |
| Communication | Wide Area Network / Internet | Local Area Network | Satellite |
| Target | A few milliseconds | Sub-microsecond | Sub-microsecond |
| Control Style | Client-Server or Peer-Peer | Master-Slave | Client |
| Administration | Configured | Self-Organizing | None |
| Latency Correction | Yes | Yes | Yes |
| Security | Yes | No but discussed for V2 | No |
| Hardware | No | For highest accuracy | RF receiver and processor |
| Update Interval | Varies | ~2 seconds | ~1 second |

A corollary to NTP or PTP is provided by the Proximity 1 time-service (PITS) that is available for distances of 100,000 km or less. It is detailed in CCSDS Proximity-1 space data link protocol [45], section 5.1. PITS is based on the NTP Interleaved On-Wire Protocol and is capable of being adapted and integrated into proximity links to synchronize time in the local mission domain. It is capable of higher precision due to its ability to incorporate hardware time tagging. It is an optional capability in Prox-1.

A third technique is used in GNSS systems like the GPS. It uses a one-way time transfer method for synchronization [44]. One-way transfer is perhaps the simplest method, where clock 1 sends a time signal to object 2 through a medium at a distance. For a transfer through a vacuum, the latency is approximately 3.336 μs/km.

Using this method, GPS satellites broadcast timing signals on a phase-modulated L-band carrier. The satellites also broadcast a time code referenced to the satellite clock and information enabling the user to obtain an estimated GPS system time that implies United States Naval Observatory’s (USNO’s) UTC time. The user's receiver compares the arrival time of the GPS signal to the local clock. Using estimation techniques and an approximation of the sender and receiver location allows for a fast timing solution. When the final solution is complete and, accounting for propagation delays relating to range, ionospheric and tropospheric refraction and decrease in light speed (if applicable), and hardware delays, then time can be ascertained with uncertainties measured in ns. In continuous operation and with multiple satellite in-views, time solutions are stable and reliable without requiring a USO onboard the receiving platform.

One drawback identified for one-way time transfer is that there is no feedback loop, such as is available with two-way transfer systems. However, a GNSS-like system would be an externally monitored and managed system. System feedback is provided by two mechanisms. First, a GNSS master continuously monitors all GNSS satellites. In addition, an external Earth-bound observer would monitor important indicators.

For GPS, the USNO monitors three satellite timescale references: individual satellite time, GPS ensemble time, and UTC (GPS). If USNO or the GPS master identifies an impacting failure or trend, corrective actions are taken.

Any TS supporting user systems must be intelligent enough to detect anomalies in time sources and must apply reasonable error checking. At the system level, these indications will limit the application of the specific time source. The system status should be telemetered to the applicable control center.

What is not clear is what precision is required. The basis of time synchronization requires a level of accuracy in the client commensurate with need. Therefore, even with highly accurate atomic clocks in orbit, the accuracy and precision are only as good as available clients. As stated previously, two-way time transfer is possible with a network protocol is possible.

### Mission Domain 3

Mission Domain 3 (MD3) is representative of solo mission or missions in transfer orbit. There can be as many of these as there are free-flyers with no infrastructure available. However, once they approach an infrastructure, they can join. In this environment, there is insufficient infrastructure to support a collaborative solution between multiple spacecraft. An example of this would be a spacecraft in an Earth-to-Mars transfer orbit or a spacecraft conducting a Jovian survey. For each domain, corrections for range, Doppler shift, light travel time, and relativistic effects are required on a platform basis.

### Mission Domain 4

Mission Domain 4 (MD4) ranges from Earth orbit to lunar orbit. By definition, it encompasses three zones; 1) cislunar space, 2) the lunar environs, and 3) all other regions of the lunar orbit to include the Earth-Lunar Lagrange points. It is expected that a permanent presence on the Moon requires an extensive infrastructure for MD4.

There are multiple scenarios for orbit transfer from Earth environs to the lunar orbital space. These options are mentioned here because they require consideration from a time dissemination standpoint of mission planning. It is anticipated that each type of transfer will be used for their unique features.  This is also likely in interplanetary space.

- Hohmann transfer (Apollo type transfer from Earth orbit to Moon)

- Low energy thruster transfer

- Low energy transfer orbit such as a ballistic lunar transfer (BLT)

For manned mission to the environs of the Moon in Zone 1, option 1 high energy transfers will be used. These are the most efficient for short duration transfers but are complicated by tight launch windows that are required to match orbits and two relatively high deltaV changes. With transfer time of 96 hours (345600 s) and a USO with a stability of 10-13 at 10 s, a deviation of approximately 35 ns can be expected. This is well within the in-situ clock synchronization requirement of 1 μs defined by the Lunar Architecture Team [69].

For moving large quantities of mass that has no relative “expiration date” (can’t die or be corrupted by radiation), options 2 and 3 are preferable. Option 2 uses continuous low deltaV changes via electrical propulsion and spirals the spacecraft within the orbital plane out to its rendezvous. It is basically a classic 3 body problem. Option 3 uses a ballistic shot and 4 body problem that includes a gravity assist from the sun. As a result, its trajectory extends beyond the moon. Each of these options has wide launch windows and can accommodate varied orbital insertion options making them very flexible. However, the orbital transfers can vary from weeks to months.

Time dissemination therefore becomes a larger issue for cislunar travel. The option 1 stability requirement may not be suitable. Higher stability of the onboard clock or a routine mechanism for synchronizing/correlating clocks may be required. It should be noted here, option 2 and 3 are more suited to autonomous operations. Option 2 has a slow and methodical change to its orbital characteristics and option 3 has very little maneuvering in the lunar vicinity due to its ballistic nature. A spacecraft using a properly executed ballistic transfer will “park” itself.

Note that Zone 3-time synchronization will fall within MD3 methods.

However, in Zones 3, time can be maintained via two conventional and well understood methods: radiometric and NTP. In our surveys, many users use radiometric methods to correct time to fine. For coarse precision, NTP and IEEE 1588 are sufficient throughout all zones. NTP has the additional benefit of being a software implementation that requires no additional power and, with the exception of a few specific implementations, no additional hardware. NTP, however, may be limited in the distance over which it can be usefully applied. Defining the operating limits quantitatively is beyond the scope of this document.

Zone 2 is the lunar equivalent of MD2. It is anticipated to be populated in the next several years with a large number of spacecraft and lunar ground deployed devices to support science and commercialization. As a result, a more robust architecture is required than MD2. This architecture may comprise IOAG time services of time transfer, clock correlation, and clock synchronization [37].

# Frequency and timing standards

## Frequency standards and Clocks

Frequency standards are devices producing stable and precise frequencies with a given accuracy. Frequency standards can be used as a clock if the frequency is suitably divided in a clockwork device and displayed [2].

A clock consists of an oscillator and a counter. One of the simplest clocks we can think of is the Earth oscillating by revolving around its own axis and humans counting the revolutions as days and years, which is the type of clock that led to the 1960 definition of the second. The 1967 definition of the second required well-controlled atoms in the construction of a precise clock.

The principle of an atomic clock is shown in Figure 3-1-. Coherent radiation is created by a radiation source. The radiation shines upon an atomic sample, and the atomic transition is detected. The fraction of atoms that is excited will depend on the oscillator frequency (radiation source), the intensity of the interrogation field, the duration of the pulse, and the atomic properties. From the detected transitions one can deduce how far the oscillator is from the reference frequency (the atomic transition) and a correction is subsequently applied to steer the oscillator towards the correct frequency.

Figure 3-1: Principle of atomic clock operation.

The two most commonly used figures of merit for atomic frequency standards are accuracy and stability. The frequency of an oscillator can be written as:

νosct= ν0(1+ϵ+yt)

Here, ν0 is the reference frequency, ϵ is the offset of the mean oscillator frequency with respect to ν0, caused by the environmental perturbation of the atom’s energy levels, and y(t) is the frequency noise of the oscillator. In the frequency metrology community, the uncertainty that we have on the knowledge of ϵ is called the uncertainty of the frequency standard, and y(t) is called the frequency instability, characterized using the Allan variance [5] which describes the statistical uncertainty of the frequency measurement.

Another key parameter governing the performance of any frequency discriminator is its quality factor,

Qat= ν0/Δν,

where Δν is the linewidth.

The fractional frequency instability of an oscillator locked to an atomic transition with a quality factor Qat often shows white frequency noise behavior, which we can write as

στ∝1Qosc⋅S/Nτ ,

where S/N is the signal to noise ratio of the spectroscopic signal, Qosc is the quality factor of the oscillator, and τ is the integration time.

## Important types of frequency references

### Atomic Fountain

Today most primary standard atomic clocks are fountain clocks based on the same principle as the atomic beam clocks with the atoms being laser-cooled to extend the interaction time. Since gravitational acceleration and thermal expansion of an atomic beam can limit the distance or rather the interaction time between the interaction zones, the atoms in a fountain clock are first prepared in a magneto-optical trap (MOT) or optical molasses which cools the atoms to the level of few micro-kelvin and decreases thermal expansion. The trapped atoms are released and shot up vertically through a single interaction region which then is used a second time when the atom falls down again, effectively using the Ramsey method of separated fields with a single field. The atomic state is then detected before reaching the bottom of the chamber.

Figure 3-2: Typical atomic fountain clock.

The key point of this method is to have atoms that expand much less and travel slower through the probing region while not falling out of the probing region due to gravity.

These types of clocks remain some of the best and most common primary standard clocks in the world [6][7][8].

### Ion Clocks

A conventional Paul trap for ions provides a very good technique to control atomic motion [5]. Thanks to the large confinement and trap depth attainable in an ion trap, an ion can be trapped for several days, months, or even years, providing long interrogation times. Until the last few years, optical ion clocks held the record for atomic clock accuracy. However, achievable clock stability has been limited when simultaneously using several ions and interrogating them because of the strong Coulomb interaction between ions.

Figure 3-3: Ion trap used for ion clocks.

### Optical lattice clocks

Optical lattice clocks combine the high signal-to-noise ratio of fountain clocks and the confinement of ion clocks where the motional effects are suppressed. A high number of neutral atoms are trapped in a dipole trap using a focused high-power laser. The simplest version of the dipole trap is a 1D-lattice obtained by having a focused standing wave. Around the focus point there are interference fringes (effectively potential wells) in which, if the light intensity is high enough, it is possible to trap a number of atoms in each well. The large light shift from the dipole trap that the atoms experience is cancelled to first order at the magic wavelength, where the two clock states experience exactly the same shift.

Figure 3-4: Optical Lattice clock: 1D dipole trap.

## Evolution of atomic clock stability

After more than 50 years of development and experience, microwave clocks which use a cloud of laser-cooled atoms have achieved their state-of-the-art performances with an accuracy at the 10−16 level and a frequency stability at 1.6×10-14 at 1 s limited by quantum projection noise.

The development of new standards based on optical transitions with frequencies higher than several hundreds of gigahertz was blocked by the inability to measure optical frequencies with electronic devices. Measurement techniques using multiple doublings of a coherent microwave signal, were inefficient and complicated [9]. The real revolution in the optical standards has begun with the optical frequency comb, which allows for a simple transfer of optical frequencies to the optical and microwave domains, and, more importantly, maintaining a high stability of the optical signal [10], [11]. Since that moment, a strong acceleration in the development of optical frequency standards has been observed.

Figure 3-5:  Evolution of the fractional systematic uncertainty of atomic clocks over years [14].

## Redefinition of the second

Since 1967, the microwave cesium atomic clock has been the reference of the SI second. Metrology laboratories across the world provide the outputs of their atomic clocks to the Bureau International de Poids et Mesures (BIPM) (in English, the International Bureau of Weights and Measures), which combines and weights them in order to determine International Atomic Time (TAI), which is the basis of the world time reference Coordinated Universal Time (UTC).

With the outstanding performance of optical clocks over the last decade, surpassing the cesium fountain performance, the metTrology community is more confident about the potential redefinition of the SI second in terms of an optical reference transition.

BIPM has devised a roadmap to accompany the process for the re-definition of the second. The main milestones on the roadmap towards the redefinition of the second can be summarized as the following:

Milestone 1: The existence of at least three different optical clocks using different atomic species demonstrating uncertainties about two orders of magnitude better than Cs fountains.

Milestone 2:  Multiple (at least three) independent implementations from metrology laboratories of at least one optical clock verifying Milestone 1. This comparison can be done by using a transportable clock or via advanced ground links like optical fibers.

Milestone 3: The optical clocks shall contribute regularly to TAI.

Milestone 4: Three independent measurements of an optical clock verifying Milestone 1 with three independent Cs primary standards. This measurement shall be limited essentially by the uncertainly and stability of the Cs fountains clock.

In order to cover the milestones of the BIPM roadmap, several laboratories have embarked on collaborative projects to compare potential optical clocks candidates for the redefinition of the second.

In Europe for instance, several metrology laboratories engaged in the realization of a large fiber network linking the different European entities using a dedicated dark fiber. The most established one is the link between the German metrology laboratory Physikalisch-Technische Bundesanstalt (PTB) and the French Metrology laboratory Systèmes de Référence Temps-Espace (LNE-SYRTE). See Figure 3-6-. This link of 1400 km, demonstrated the first frequency comparison between two optical clocks of its kind across national borders between fully independent clocks at the level of 10-17 fractional accuracy [12].

Figure 3-6: Schematic of the clock comparison between PTB and LNE-SYRTE [16].

## Time scales

A timescale is an agreed-to system for timekeeping using an algorithm to efficiently combine timing data from a clock ensemble. For most timescales in use, the algorithm is defined as a weighted average, where the weight determination is adapted to different needs. In the subsections below, we list different types of time scales that are commonly in use.

### International Atomic Time (TAI)

International Atomic Time (TAI) is a high-precision atomic coordinated time standard based on the notional passage of proper time on Earth’s geoid. TAI is calculated at BIPM and it’s the coordinated result of national metrology laboratories and astronomical observatories who develop and maintain primary frequency references worldwide.

TAI was formally adopted in 1971, in the perspective of providing users with reliable, accurate, and stable frequency. This realization is done by combining in the first step the data to create the timescale called “Echelle Atomique Libre’’ (EAL) translated into English as Free Atomic Scale. The requirement for accuracy in timescale is met by steering the EAL frequency using data from primary and secondary frequency standards. These standards are maintained at a few metrology laboratories that realize the second with a very high accuracy. Currently LNE-SYRTE, PTB, NIST, National Physical Laboratory (NPL), National Institute of Metrology of China (NIM), and the Italian National Institute of Metrology Research (INRiM) are providing primary and secondary frequency standard data to the BIPM. The BIPM uses these data in an algorithm to produce the steering correction for the EAL [17]. The combination of the EAL with these corrections provides the final product TAI.

Although TAI has been acknowledged as an atomic timescale, it’s not recognized as the international standard for timekeeping. That distinction is retained for UTC.

### Coordinated Universal Time (UTC)

The practical disseminated reference time scale UTC, based on TAI, is equally stable and accurate as TAI, but while TAI is continuous, UTC is affected by one-second discontinuities, known as leap seconds. The leap second is added at irregular intervals to compensate the slowing of the Earth rotation. The leap seconds are inserted as necessary to keep UTC within 0.9 second of the time standard based on Earth’s rotation.

The UTC has considerably benefitted over the years from the progress in time and frequency metrology, sciences and industry. In fact, the number of atomic clocks and their variety have dramatically increased in recent years providing more reliable and long-term frequency stability to time scale realization. The BIPM, in coordination with the world timing community, is assuring the dissemination of the UTC through a publication called Circular T, published monthly for the benefit of National Metrological Institutes, observatories and international organizations that contribute to its computation.

UTC is often referred to as Greenwich Mean Time (GMT), but since the GMT time scale is no longer used the use of GMT to designate UTC is discouraged.

### Ephemeris Time, Terrestrial Dynamical Time, and Barycentric Dynamical Time

Ephemeris time (ET) [50] was adopted as a standard for astronomy and other scientific uses in 1952 as a replacement for time scales such as UT1 that are affected by irregularities in the rotation of the Earth.

ET was replaced in 1984 by two “dynamical” time scales. Terrestrial Dynamical Time (TDT) replaced ET for applications at the surface of the Earth. Another time scale, Barycentric Dynamical Time (TDB) was defined as the time corresponding to TDT for an observer at the solar system barycenter and replaced ET for the calculation of solar system ephemerides.

In 1991, the definition of TDT was refined to make it more precise and it was renamed Terrestrial Time (TT).

That replacement of ET by two other time scales, TDT (now TT) and TDB, has led to some confusion as the name ephemeris time and abbreviation ET are still found in some literature. In general, any use of the acronym ET should include a description of which time scale it actually represents since it otherwise is not meaningful.

## Network considerations

### Network Time Protocol

Network Time Protocol (NTP) is a protocol designed to time-synchronize a network of machines. NTP is intended to synchronize all participating computers to within milliseconds of UTC. An NTP network usually gets its time from authoritative time source such as an atomic clock attached to a time server. The NTP then distributes this time across the network.

NTP uses a hierarchical structure based on layers. Each level of this hierarchy is termed a stratum and is assigned a number starting with zero. Stratum 0 comprises high precision timekeeping systems (e.g., cesium clock, Hydrogen maser, or GNSS receiver). These references generate a very accurate and stable timing signal (pulse per second signal).

Stratum 1 comprises computers with a timing system synchronized to within few microseconds of Stratum-0 devices. Stratum-2 devices are synchronized from a Stratum-1 device across a network connection. Due to network jitter and delay, Stratum-2 servers are not as accurate as Stratum-1-time servers.

Figure 3-7: Detailed hierarchy of Stratum levels.

The maximum limit for Stratum levels is 15; stratum 16 is used to indicate that a device is unsynchronized.

NTP uses a centralized clock synchronization algorithm developed by Flaviu Christian in 1989.

Figure 3-8: Christian algorithm principle.

The principle of Christian’s algorithm is:

Step1: The client machine sends a request to the Clock server at the time T0.

Step 2 The clock server listens and returns the response in form of clock server time Tserver

Step 3 The client processes the response received from the clock server at time T1 and calculates the synchronized client clock time using the following formula:

Tclient=Tserver+T1-T02

where

Tclient is the synchronized clock time,

Tserver refers to the clock time returned by the server, and

T1 refers to the time at which response was received by the client process.

When configuring an NTP, the user can specifically choose which system on the network is the authorized time source, or time server and how time is synchronized between systems on the network. To do this, one configures the router, switch, or security device to operate in one of the following modes:

- Client mode. In this mode, the local router or switch can be synchronized with the remote system, but the remote system can never be synchronized with the local router or switch.

- Symmetric active mode. In this mode, the local router or switch and the remote system can synchronize with each other. You use this mode in a network in which either the local router or switch or the remote system might be a better source of time.

- Broadcast mode. In this mode, the local router or switch sends periodic broadcast messages to a client population at the specified broadcast or multicast address. Normally, you include this statement only when the local router or switch is operating as a transmitter.

- Server mode. In this mode, the local router or switch operates as an NTP server.

### Delay/Disruption Tolerant Networking (DTN)

In the terrestrial Internet, multiple individual links connect together an end-to-end path using Internet Protocol (IP) routers. An end-end path is always available, delays are short (a few 10’s-100’s of milliseconds) and error rates are very low.

Unfortunately, the protocols used for short, reliable links do not work well when extended to the longer distances used in space. The links may not be as reliable, and the longer delays are unworkable with protocols that intend to provide once only, in order, delivery and require acknowledgments.

Delay/disruption tolerant networking is an approach to overcome these problems. The core of the DTN suite is the

Bundle Protocol (BP).

DTN works by connecting multiple individual hops into an end-to-end path with Bundle Protocol (BP) Agents that use the DTN “store and forward” router function. A single hop between BP Agents may use a variety of link layer protocols, possibly including IP, to provide the link between Bundle Agents. A continuous solar system end-to-end path is rarely available owing to disruptions and outages of individual links; delays are potentially very long (traditionally intolerable) as a consequence of Solar System geometry (minutes to days) and error rates are often high.

The BP Agents operate with a store-and-forward model in which data in the form of bundles are held at a node until the next hop becomes available. If the next hop is available and there are no pending bundles, bundles will pass through a BP router with minimal latency. In order to limit network loading from bundles existing for too long, bundle protocol has a Time To Live (TTL) feature that is defined by time with a DTN epoch of January 1, 2000. This is different from an IP network TTL that is based on router hops. The precise definition of the lifetime is, “the time at which the bundle’s payload will no longer be useful.” In addition, BP version 7 supports a Hop Count Block (HCB – type 10) that is more equivalent to the internet TTL as a safety mechanism. The HCB purpose is to remove bundles with a persistent forwarding error from the network.

The current IRTF RFC 5050, BP version 6 [35] defines TTL in SI units for seconds and nanoseconds. The CCSDS BP Standard [34] further refines it such that it be limited by the time resolution of the Command and Data Handling (C&DH) subsystem that hosts the BP Agent. This version of the bundle protocol is being deprecated and the definition of DTN time in BP version 7 [36] will be defined in SI units for seconds and milliseconds; e.g., milliseconds from start of January 1, 2000. A new concept has also been defined: bundle age extension block (BAEB – type 7). This concept is used to determine a TTL for bundles originating at nodes that lack accurate clocks. For example, if a node is in a power conservation mode or otherwise unavailable, its clock may be inaccurate. The bundle age is calculated using the BAEB rather than from the difference between current time and bundle creation time. The BAEB contains the number of milliseconds that have elapsed between the time the bundle was created and time at which it was most recently forwarded, an accumulating count.

BP makes no accommodation in the protocols for relativistic effects (see examples in Table 3-13-1-). Due to relativistic effects, clocks in different reference frames will drift because of secular and periodic terms. In Table 3-13-1-, the Earth and Mars principal relativistic effects are listed. The difference in a clock on the surface of Mars and a clock on the surface of Earth is defined in Section 2.4 on time synchronization resolves to

MT-TT=TCB-TT-(TCB-MT),

where MT is Mars Time, TT is Terrestrial Time, and TCB is Barycentric Coordinate Time (Temps-Coordonnée Barycentrique). The net secular drift between the two reference frames is 0.49 ms/day and the periodic variance can be as high as 11.4 ms.

Table 3-1-: Principal relativistic effects for Earth and Mars

| Location | Effect [33] | Value |
| --- | --- | --- |
| Earth | Geoid to geocenter Secular drift | 60.2 μs/d |
|  | Maximum amplitude of diurnal term | 2.1 μs |
|  | Geocenter to Barycenter Secular drift | 1.28 ms/d |
|  | Amplitude of principal periodic term | 1.7 ms |
| Mars | Mars surface to Mars center Secular drift | 12.1 μs/d |
|  | Maximum amplitude of diurnal term | 0.9 μs |
|  | Mars center to Barycenter Secular drift | 0.84 ms/d |
|  | Amplitude of principal periodic term | 11.4 ms |

BP uses millisecond counts for TTL and logging. As a result, secular and periodic bias will result in low levels of false logging and inaccurate TTL and arrival times of bundles.

## National and international organizations

In this section we define some national and international organizations which are playing a keys role in the Time and frequency domain.

### Bureau international des poids et mesures (BIPM)

BIPM is an intergovernmental organization, through which its 59 member states act together on measurement standards in four areas: chemistry, ionizing radiation, physical metrology, and coordinated universal time. It is based in Saint-Cloud, Paris, France.

Mains objective of BIPM :

- To represent the world-wide measurement community, aiming to maximize its uptake and impact.

- Liaise with relevant intergovernmental organizations and other international bodies in order to develop opportunities for the application of metrology to global challenges.

- To be a center for scientific and technical collaboration between member states, providing capabilities for international measurement comparisons on a shared-cost basis

- To be the coordinator of the world-wide measurement system, ensuring it gives comparable and internationally accepted measurement results.

### National Institute of Standards and Technology (NIST)

The National Institute of Standards and Technology (NIST) is a physical sciences laboratory and non-regulatory agency of the United States Department of Commerce. Its mission is to promote American innovation and industrial competitiveness. NIST's work in metrology focuses on advancing measurement science to enhance economic security and improve quality of life.

A key component of NIST’s metrology work is metrological traceability, which requires the establishment of an unbroken chain of calibrations to specified reference measurement standards: typically, national or international standards, in particular realizations of the measurement units of the International System of Units (SI).

### Consultative committee for Time and Frequency (CCTF)

The Consultative Committee for Time and Frequency (CCTF) is the new name given in 1997 to the Comité Consultatif pour la Definition de la Seconde (CCDS) established in 1956. The Consultative Committee for Time and Frequency is concerned with the definition and realization of the second, and the establishment and diffusion of International Atomic Time (TAI) and Coordinated Universal Time (UTC).

### International earth rotation and reference systems service (IERS)

The IERS was established in 1987 by the International Astronomical Union and the International Union of Geodesy and Geophysics. According to the Terms of Reference, the IERS accomplishes its mission through the following components: Technique Centres, Product Centres, Combination Centres, Analysis Coordinator, Central Bureau, Directing Board. The primary objectives of the IERS are to serve the astronomical, geodetic and geophysical communities by providing data and standards related to Earth rotation and reference frames.

### Laboratoire national de metrologie et d’essais (LNE)

The French Laboratoire national de métrologie et d'essais (National Laboratory of Metrology and Testing) is a reference laboratory responsible for carrying out measurement and testing products of all kinds for their certification for placing them on the market. It also coordinates metrology activities in France. It was previously known as the Laboratoire national d'essais (LNE), and still uses those initials. Originally established in 1901, it is now a state-owned enterprise, and operates as an établissement public à caractère industriel et commercial, a French type of state-controlled entity of an industrial or commercial nature.

### National phsyical laboratory (NPL)

The National Physical Laboratory (NPL) is the national measurement standards laboratory of the United Kingdom. It is one of the most extensive government laboratories in the UK and has a prestigious reputation for its role in setting and maintaining physical standards for British industry. Founded in 1900, it is one of the oldest metrology institutes in the world. Research and development work at NPL has contributed to the advancement of many disciplines of science, including the development early computers in the late 1940s and 1950s, construction of the first accurate atomic clock in 1955, and the invention and pioneering implementation of packet switching in the 1960s, which is today one of the fundamental technologies of the Internet.

### Physikalisch-Technische Bundesanstalt (PTB)

The Physikalisch-Technische Bundesanstalt (PTB) is the national metrology institute of the Federal Republic of Germany, with scientific and technical service tasks. It is a higher federal authority and a public-law institution directly under federal government control, without legal capacity, under the auspices of the Federal Ministry for Economic Affairs and Energy. Together with NIST in the USA, NPL in Great Britain and LNE in France, PTB ranks among the leading metrology institutes in the world. As the National Metrology Institute of Germany, PTB is Germany's highest and only authority in terms of correct and reliable measurements.

# Global navigation satellite systems

GNSSes are satellite constellations that provide timing and position data to GNSS receivers. This timing data is made available through radio frequency (RF) transmitters, accurate atomic clocks, pseudorandom noise (PRN) spread data, and Doppler effect application. A GNSS enables an institution or person to obtain the performance of atomic clock timing data for the cost of an RF receiver.

Four main GNSSes exist, those being Galileo, Compass/Beidou, the Global Positioning System (GPS), and GLObal’naya Navigatsionnaya Sputnikovaya Sistema (GLONASS). There also exist regional systems including The Indian Regional Navigational Satellite System (IRNSS), the Doppler Orbitography and Radio-positioning Integrated by Satellite (DORIS), and the Quasi-Zenith Satellite System (QZSS) [20].

The origin of GNSS technology stems from the radio transmission observations of Sputnik 1 (1957). William Guier and George Weiffenbach of Johns Hopkins University Applied Physics Laboratory (APL) found that they could track where in orbit Sputnik 1 was by observing the apparent alteration of radio frequencies emitted by the satellite (Doppler effect). Frank McClure, deputy director of APL at the time, proposed investigation of the inverse problem: determining the user’s location based on the satellite’s position [21].

Precision timing data is attained by observing the atomic clock data and factoring in a time correction, or Tc value. Tc accounts for a multitude of errors due to the signal not traveling in a vacuum environment. Physical distance, atmospheric effects, multipath errors, and ephemeris account for some of the errors that can occur between transmitter and receiver. Modern ephemeris comprises software that generates positions of bodies at virtually any time desired by the user, subject to accuracy limitations. Relativistic effects also contribute to the value of the time correction. This difference is currently accounted for in the receiver algorithm. The accuracy of the timing data is very important. To see how important, one only must look at the history of GPS specs. GPS at its inception could reliably determine position within a range of a hundred meters [22]. Since 2007, GPS can determine position within a range of a few meters [23]. Due to advancements in the understanding of how radio frequencies are affected by the aforementioned errors, drastic improvements to navigational accuracy have been made.

Position data is calculated from

|  | cTn-Tm+TcT= Xn-X2+Yn-Y2+Zn-Z2, | (7) |
| --- | --- | --- |

where the variables are described in Table 4-14-1-.

Table 4-1-: Equation (7) Variables.

| Variable | Description |
| --- | --- |
| c | Speed of light (2.99792458 × 108 m/s) |
| Tn | Time signal from satellite n was sent |
| TmT | Time signal from satellite n was acquired by receiver |
| Xn, Yn, ZnTc | Position of satellite n |
| X, Y, ZXn, Yn, Zn | Position of receiver acquiring signal from satellite nn |
| X, Y, Z | n |

The equation is solved simultaneously by the receiver with three other equations nearly identical to it, with the only difference between the equations being the locations of each satellite. This enables one to solve for the position of the receiver and the time correction. These variables are needed to solve for atomic time accuracy on a receiver, in addition to solving for position. Navigation is an extension of this calculation. Position data is needed for both the start position and destination, and the software providing the navigation capability will provide a route based on limitations of the vehicle. Both position and navigation data rely on timing data, as evidenced in Equation (1).

## Global Positioning System

The United States of America’s GNSS is the Global Positioning System (GPS) [31] which provides navigation and timing data to users via is a one-way time transfer system so that there can be an effectively infinite number of users within its service volume.

Originally, GPS had a minimum constellation of 24 satellites; however, the configuration of the GPS orbits has changed, and as of June 2011, GPS consisted of an expanded constellation of 27 satellites; today there are 31 satellites, with these extras acting as redundant backups [24]. The GPS satellites are in six Medium Earth Orbit (MEO) planes at an altitude of 20200 km above sea level and an inclination of 55 degrees.

p2

### Current applications

#### STANDARD POSITIONING SERVICE

The Standard Positioning Service (SPS) provides satellite positioning with minimum accuracy of 9 meters horizontally, 15 meters vertically, and a time transfer accuracy of 40 nanoseconds of UTC [31].  For single frequency receivers, the User Segment can usually resolve position to within 5 meters and time to within 100 billionths of a second; dual frequency receivers can improve User Segment Accuracy to within a meter [74].

##### L1 COARSE ACQUISITION SIGNAL

The L1 Coarse Acquisition (L1 C/A) [30], or otherwise known as LNAV, provides basic positioning to the User Segment and take the form of a satellite unique PRN ranging signal phase modulated with the L1 carrier.  The L1 C/A signal is comprised of three distinct data subframes: Time, Ephemeris, or Almanac.  The first subframe provides the GPS satellite clock time.  The GPS satellite’s Ephemeris subframes provide the orbital information for itself; with Ephemeris data being valid for approximately four hours.  The GPS satellite’s Almanac subframes provide coarse orbital information for the GPS constellation to facilitate the User Segment locating more GPS satellites.  The Almanac subframes also include correction data, GPS to UTC clock correlation, and satellite health and status.

##### L2 CIVILIAN NAVIGATION SIGNAL

The L2 Civilian Navigation (L2C) [30] is a satellite unique PRN ranging signal that is phase modulated with the L2 carrier. The CNAV signal denotes what data is in a particular subframe by including a message type field.  L2C is expected to be fully operational by 2023.

##### L5 AERONAVIGATION SIGNAL

The L5 Aeronavigation (L5C) [32] signal is structurally similar to the L2C signal; however, L5C is transmitted at a higher data rate than L2C.  L5C is not expected to be fully deployed until 2029.

##### L1 CIVILIAN NAVIGATION SIGNAL

The L1 Civilian Navigation (L1C) [72] signal divided into 3 three distinct subframes like L1 C/A.  The first subframe is small and only represents the changing portion of the satellite clock.  The second Subframe is largely static contains the remaining components of the satellite clock and the ephemeris data.  The third subframe usually contains sections of the Almanac but can also contain other messages such as correction parameters, satellite health and status, and interoperability parameters for other GNSS systems.

##### SPS OBSOLESCENCE

The GPS L1 C/A and L2C signals are to become obsolete once 24 satellites with L5 PRNs (1-32) are in orbital position; these signals are to be maintained for a minimum of 2 years after obsolescence to facilitate user transition [31]. They are expected to become obsolete in 2029 [31].

#### PROTECTED POSITIONING SERVICE

PPS includes GPS signals with higher data rates and signal strengths that are only available to authorized users. PPS is monitored to detect most anomalies in the service [31].

##### P(Y) CODE SIGNAL

The P(Y) Code is an encrypted GPS signal on the L1 and L2 frequencies that is available to users with a keyed receiver [31].

##### M-CODE SIGNAL

The M-Code is an enhanced, encrypted GPS signal that is spectrally separate from the SPS signals and includes improved interference resistance mechanisms [31].

##### PPS OBSOLESCENCE

The GPS L1 P(Y) and L2 P(Y) signals are to become obsolete once 24 satellites with L5 PRNs (1-32) are in orbital position; these signals are to be maintained for a minimum of 2 years afterwards to facilitate user transition [31]. They are expected to become obsolete in 2029 [31].

#### Search and Rescue

New GPS satellites have a Search and Rescue beacon repeater installed [31].

### Future Evolution

Ever increasing demands of GNSS-reliant technology require more robust systems to accommodate them. Additionally, a number of competitive GNSS implementations have been established internationally. In response to this, the US government has initiated modernization of GPS capabilities to remain competitive with our partners in GNSS technology [50]. Improvements associated with modernization include development and integration of the next generation GPS III/IIIF satellites, replacement of the current GPS Operational Control System (OCS), and incorporation of a new navigation signal that promotes interoperability between international GNSSs.

#### GPS III

The increase in age (and customer base) of the GPS necessitated upgrades to the service. With regards to the space segment, these upgrades were implemented by phasing out older satellites and introducing newer models to take their place. The newest models currently in orbit are the GPS III satellites, boasting three times greater accuracy than their predecessors (Block IIF) [52]. Anti-jamming capabilities have also been increased up to eight-fold via M-Code enablement [54]. The GPS III satellites will be procured without the Selective Availability feature, eliminating a source of PNT uncertainty for civilian GPS users [54]. Additional features include ensured compatibility with the next generation operational control system (OCX) ground control segment and L1C, which is a civilian GPS signal designed to enable interoperability between GPS and international satellite navigation systems [56]. Ten GPS III units are planned for integration into the GPS satellite constellation [57]. Once complete, a newer model of satellite, GPS IIIF, will be developed for the GPS satellite constellation. In addition to the benefits brought about by  GPS III, GPS IIIF offers new features such as the Medium Earth Orbit Search and Rescue (MEOSAR) and the Laser Retro-reflector Arrays (LRAs). MEOSAR is a search and rescue feature designed to dramatically reduce the time required to detect distress radio beacon transmissions and determine the geographic location of said beacons [58]. LRAs are passive arrays of corner-cube retroreflectors that improves GPS accuracy and ephemeris data collection [59].

The GPS ground segment is receiving an upgrade via replacement of the current GPS operational control system (OCS) with the next generation OCX [60]. This improvement is designed to support implementation and operations of the GPS III/IIIF satellites, backwards compatibility with Block IIR, IIR-M and IIF satellites, and new modernized capabilities of the Block IIR-M and IIF satellites (e.g., enhanced M-Code capabilities) [61]. The new ground segment has additionally received general improvements to its core attributes such as position accuracy, system integrity, etc. [60]. The OCX will be released in three blocks. Block 0 is the Launch and Control system (LCS), which controls launch and early orbit operations and the on-orbit checkout of all GPS III/IIIF satellites. Block 1 and 2 will provide the operational capability to control all GPS satellites and signals to an advanced degree, building off of the base Block 0 implementation [60].

A notable upgrade to the current GPS infrastructure is the introduction of a new civil signal, known as L1C. This signal enables interoperability between different GNSSs. This signal was developed alongside Europe for incorporation into both GPS and Galileo. Japan’s QZSS and China’s BeiDou system are also adopting L1C-like signals within their constellations [62]. Moving forward with this satellite signal interoperability upgrade will ensure more accurate PNT data for each user.

GPS provides users with a host of functionality related to its PNT data. However, as spacecraft systems become more reliant on computer-based operation, and strides are taken to increase terrestrial influence over the Moon and Mars, more precise PNT systems will be needed to accommodate users in the lunar and Martian orbits.

-

#### Moon

GPS provides users with a host of functionality related to its PNT data. However, as spacecraft systems become more reliant on computer-based operation, and strides are taken to increase terrestrial influence over the Moon and Mars, more precise PNT systems will be needed to accommodate users in the lunar and Martian orbits.

In the near future, the Artemis I program will endeavor to reach the Moon and establish a lunar orbiting station. Upon full realization of the project, Artemis will facilitate unique experiments due to its capability to enable repeat travel between lunar ground and lunar station. In addition to providing valuable science and exploration data of the moon, Artemis will also provide lessons learned to support the first crewed Mars mission.

NASA engineers are developing an Artemis PNT system that incorporates GNSS signals as one of its main components [26]. This system will provide a number of capabilities to the Artemis I program including precise mission timing, orbit/trajectory determination, surface navigation, and more [63].

GNSS currently provides PNT data to active receivers within the Terrestrial Service Volume (Earth’s surface to LEO) and the Space Service Volume (LEO to GEO). At altitudes beyond GNSS constellations (~20 000 to 23 000+ km), receivers will depend on spill-over from GNSS satellites on the other side of the globe for their PNT data. This is challenging due to the weaker signal caused by the distance traveled, and the dead space, or umbra, that the Earth imposes on GNSS signals. To receive PNT data, receivers must listen for GNSS signals at an angle from the GNSS satellite. Two angles have currently been identified as practical for signaling. Main-Lobe Signaling has a stronger signal at a narrower angle (~14° to 23.5°), and vice versa for Side-Lobe Signaling (~27° to 39°). These angle numbers can change slightly depending on the altitude of the specific GNSS satellite [64].

Figure 4-1. A graphic detailing the different areas of GNSS coverage (from [62]).

Side-Lobe Signaling is considered more effective for Space Service Volume (and beyond) signaling despite weaker signal strength due to the wider area of coverage. This concept has been proven out in a recent mission known as the Magnetospheric Multiscale Mission (MMS), using GPS signals to determine position at 40% lunar distance (150 000 km) [64]. A future Commercial Lunar Payload Service (CLPS) known as the Lunar GNSS Receiver Experiment (LuGRE) endeavors to be the first mission to receive a GNSS signal on the lunar surface. The experiment will receive signals from both the GPS and Galileo constellations via a dual-compatible receiver provided by ASI [64]. Receiving signals from both constellations will bolster the aggregate signals power that LuGRE will receive, counteracting the lower signal power arriving from each GNSS constellation at lunar distances.

#### Mars

Mars is far too distant to be able to make use of Earth-orbiting GNSS satellites. Therefore, new architectures will be needed to support PNT at Mars. Currently, deep space missions use the Deep Space Network (DSN) to communicate PNT data in the form of TT&C, and ad hoc time and trajectory update exchanges. Deep space missions in this context also include Martian satellites. Existing GNSS infrastructure on Earth is not powerful enough to have any influence for PNT data on Mars.

An option for having relatively current PNT data for the Martian surface would be to install a GNSS-like infrastructure in Mars’s orbit and on the surface of Mars. However, an issue inherent to building an exact architectural replica of Earth GNSS systems stems from the need for interplanetary communications. An exact architectural replica would not benefit us as much as a different approach to satellite system architecture would.

For deployment, payloads delivering MNSS satellites can be deployed in orbit, and payloads can deliver MNSS receivers to test and calibrate the system. Initially, the MNSS could be a degradation of what currently orbits the Earth. This initial phase of MNSS would still provide valuable PNT information to robotic and manned missions alike, and the existing infrastructure would facilitate even further improvements.

### GPS CLOCKS

The Master Clock for the Control Segment is an ensemble of more than 100 precise atomic clocks that are fully traceable to national and international standards for UTC timing based in Washington DC and there is a second Alternate Master Clock in Schriever AFB, Colorado, USA. [31]

The GPS satellites use atomic clocks, of either cesium or rubidium, depending on the satellite model’s implementation, that provides the time to within 100 billionths of a second.  The stability of the atomic clocks is 1×10-11 over a 3 second interval [23].

### Characteristics

GPS is divided into three segments: a space segment, a control segment, and a user segment.

#### Space Segment

The Space Segment [24] is comprised of the GPS satellite constellation has a current active configuration of 27 satellite with a total of 31 satellites currently active.  The GPS satellites sit in six Medium Earth Orbit (MEO) planes at an altitude of 20200 km above sea level, an inclination of 55 degrees, an eccentricity of 0, and an argument of perigee of 0 [23]. The orbital period is 12 hours.

GPS operates by broadcasting microwave signals that are obtained by GPS receivers. These receivers use the time and position data communicated by these signals to provide GPS services to users. There are currently six different GPS signals in operation today: L1 Coarse Acquisition, L1 and L2 P(Y), M, L2 Civilian Navigation, L5 Aeronavigation, and L1 Civilian Navigation signals.

Table 4-2: Table of Frequencies

| Frequency Band | Frequency |
| --- | --- |
| L1L1 | 1575.42 MHz |
| L2L2 | 1227.6 MHz |
| L5L5 | 1176.45 MHz |

#### Control Segment

GPS Standard Positioning Service [31] (Section 4.1.3.1) is not continuously monitored; however, the Protected Positioning Service (Section 4.1.3.2) is continuously monitored by operators at GPS Master Control Station Schriever Air Force Base Colorado USA with the Alternate Master Control Station in Vandenberg Air Force Base, California, USA.  where most anomalies are detected and corrected for.  GPS MCS is assisted by 6 Government monitor stations and 11 Non-Government Agency monitor stations. Ground antennas use S-Band to issue commands, update navigation data, and receive status telemetry [73].

#### User Segment

### The user segment is where the calculations of the user’s position and time are performed. The GPS receiver acquires the PRN ranging signal of a GPS satellite and then uses the Almanac to lock onto the PRN ranging signals of three more GPS satellites. The GPS receiver then acquires the satellite’s ephemeris data and time of each satellite. The quasi-Keplerian coordinates of the ephemeris are then converted into Earth-Centered Earth-Fixed coordinates and the user’s receiver then inserts time and position information of the four satellites into the ranging equation described in Table 4-1. The resulting four ranging equations are then used to solve for the position and time of the GPS receiver.

-

-

## Galileo

Galileo is the Europe Union’s GNSS, a highly accurate, guaranteed global positioning service under civilian control. It’s autonomous but also interoperable with existing satellite navigation systems. Galileo is not related to NASA’s Galileo mission to Jupiter.

The fully deployed Galileo system consists of 24 operational satellites plus six in-orbit spares, positioned in three circular Medium Earth Orbit (MEO) planes at 23 222 km altitude above the Earth, and at an inclination of the orbital planes of 56 degrees to the equator.

The initial services of Galileo became available on December 15, 2016.. At the time of publication, Galileo is flying 28 satellites.

### Characteristics

The Galileo system is divided into three major segments; a space segment, a ground segment and a user segment.

#### Space segment

The main functions of the Galileo space segment are to generate and transmit code and carrier phase signals with a Galileo-specific signal structure, and to store and transmit the navigation message sent by the control segment. These transmissions are controlled by highly stable atomic clocks on board the satellites, as described in Section 4.2.2.

When Galileo is fully operational, there will be 30 satellites in Medium Earth Orbit (MEO) at an altitude of 23,222 kilometers. The satellites will occupy each of three orbital planes inclined at an angle of 56° with respect to the equator. The satellites will be spread evenly around each plane and will take about 14 hours to orbit the Earth. Each orbital plane includes 8 satellites uniformly distributed within the plane. The angular shift between satellites in two adjacent planes is 15º. One satellite in each plane will be a spare, on stand-by should any operational satellite fail. The full constellation includes 6 spare satellites, resulting in a walker 24/3/1 constellation [75]. These spare satellites can be activated and allocated to a given operational slot depending on maintenance or service evolution activities. The constellation geometry repetition period corresponding to the nominal orbital parameters is 10 days (corresponding to 17 orbital revolutions). This means that for any fixed Galileo user, the local satellite geometry at a given instant is repeated every 10 sidereal days. Table 4-34-3- shows the nominal value of the different reference orbit parameters.

Table 4-3-: Galileo reference orbit parameters.

| Reference Orbit Parameter | Nominal Value |
| --- | --- |
| Orbit semi-major axis (m) | 29599801 |
| Orbit eccentricity | 10-7 |
| Orbit inclination (deg) | 56.0 |
| Argument of Perigee (deg) | 0.0 |

#### Ground segment

The Galileo ground segment is the responsible for the proper operation of the GNSS system. It comprises two control centers, a global network of transmission and receiving stations implementing monitoring and control functions and a series of service facilities which support the provision of the Galileo services.

As mentioned above, the core of the Galileo ground segment are the two Galileo Control centers situated in Oberpfaffenhofen (Germany) and Fucino (Italy). Each center manages control functions supported by a ground control segment (GCS) and mission functions, supported by a dedicated Ground Mission Segment (GMS) also located at Oberpfaffenhofen and Fucino,.

The GCS handles space craft housekeeping and constellation maintenance. The scope of this functionally includes control and monitoring of the satellites and payload, planning and automation functions that allow safe and correct operations to take place and the support of payload related operations. On the other hand, the GMS determines the navigation and timing data part of the navigation messages.

#### User segment

The Galileo user segment is composed of Galileo receivers. Their main function is to receive Galileo signals, determine pseudo-ranges and solve the navigation equation in order to obtain their coordinates and provide an accurate time.

### Galileo clocks

In the Galileo constellation, each Galileo satellite has two master passive hydrogen maser atomic clocks and two secondary rubidium atomic clocks which are independent of one other.

The Galileo satellites are configured to run one hydrogen maser clock in primary mode and a rubidium clock as hot backup. Under normal conditions, the operating hydrogen maser clock produces the reference frequency from which the navigation signal is generated. Should the hydrogen maser encounter any problem, an instantaneous switchover to the rubidium clock would be performed. In case of a failure of the primary hydrogen maser the secondary hydrogen maser could be activated by the ground segment to take over within a period of days as part of the redundant system.

Figure 4-2: Onboard Galileo clocks: On the right rubidium clock. On the left the passive hydrogen maser for Galileo.

The onboard passive hydrogen maser and rubidium clocks are very stable over a few hours. If they were left to run indefinitely, though, their timekeeping would drift, so they need to be synchronized regularly with a network of even more stable ground-based reference clocks. These include active hydrogen maser clocks and clocks based on the cesium frequency standard, which show a far better medium and long-term stability than rubidium or passive hydrogen maser clocks. These clocks on the ground are gathered together within the parallel functioning Precise Timing Facilities in the Fucino and Oberpfaffenhofen Galileo Control Centres. The ground-based clocks also generate a worldwide time reference called Galileo System Time (GST), the standard for the Galileo system and are routinely compared to the local realizations of UTC, the UTC(k) of the European frequency and time laboratories.

### Current applications

The Galileo system offers several high-performance services worldwide. The main ones are the following.

#### Open service

With positioning accurate to one meter, the freely accessible Open Service targets the mass market and is intended for motor vehicle navigation and location-based mobile telephone services. Free to the user, it provides positioning and synchronization information intended for high-volume satellite radio navigation applications.

#### High accuracy service

The High accuracy service complements the open service by providing an additional navigation signal and added value services in a different frequency band. The high accuracy service signal can be encrypted in order to control the access.

#### Public regulated service

The Public regulated service is restricted to government-authorized users, for sensitive applications which require a high level of service continuity. It will be encrypted and designed to be more robust, with anti-jamming mechanisms and reliable problem detection. This service is intended for security and strategic infrastructure (e.g., energy, telecommunications, and finance).

#### Search and rescue service

Galileo’s worldwide search and rescue service forwards distress signals to a rescue coordination center by detecting emergency signals transmitted by beacons and relaying messages to them.

### Future evolution

The evolution of the Galileo constellation is currently under technical study within the European GNSS Evolution program supported by 17 EU members states and Canada. The primary aim of the program is to define the future system architecture for the next generation of Galileo and develop technology for the future version of the system.

The Galileo Second Generation is expected to deliver improved performance in terms of reliability, maintainability, availability, continuity, accuracy, and integrity.

## Global’naya Navigatsionnaya Sputnikovaya Sistema

GLObal’naya Navigatsionnaya Sputnikovaya Sistema (Global Navigation Satellite System, or GLONASS) is the navigation system developed by Russia. It started in 1976 as a military experimental communication system and it began to finally achieve its goals in 2001.

The constellation is currently nominal with 24 satellites in three orbital planes inclined at 64.8 degrees and an orbital radius of 19 140 km. This inclination, which is higher than GPS orbits, allows more visibility of the polar regions.

Each GLONASS satellite transits a Coarse Acquisition (C/A) code for standard positioning on frequency L1, and a Precision code (P-code) for precise positioning on frequencies L1 and L2. (Note, these frequencies are not related to Lagrange points. They are frequency bands, as defined in Table 4-2.)

The purpose of the GLONASS is to provide unlimited number of air, marine, and any other type of users with all-weather three-dimensional positioning, velocity measuring and timing anywhere.

GLONASS includes three components:

- Constellation of satellites;

- Ground-based facilities;

- User equipment.

The completely deployed GLONASS constellation comprises 24 satellites in three orbital planes whose ascending nodes are 120 degrees apart. Eight satellites are equally spaced in each plane with argument of latitude displacement 45 degrees. The orbital planes have 15-degree latitude displacement relative to each other. The satellites operate in circular 19 100-km orbits at an inclination 64.8 degrees, and each satellite completes an orbit in approximately 11 hours 15 minutes. The spacing of the satellites allows providing continuous and global coverage of the terrestrial surface and the near-Earth space.

The control segment includes the System Control Center and the network of the Command and Tracking Stations that are located throughout the territory of Russia. The control segment provides monitoring of GLONASS constellation status, correction to the orbital parameters and navigation data uploading.

User equipment consists of receivers and processors receiving and processing the GLONASS navigation signals, and allows user to calculate the coordinates, velocity and time.

### Current applications

The GLONASS system offer following two services:

#### Standard Precision service

The Standard Precision (SP) Service provides signals for civilian use. Currently, GLONASS-K provides civil signal availability on the L3 band. Originally, the civil signal was transmitted only on the L1 band. In the second generation, the civil signal was transmitted on the L2 band.

#### High Precision service

The High Precision signal is broadcast in phase quadrature with the SP signal, effectively sharing the same carrier wave as the SP signal, but with a ten times higher bandwidth (5.11 Mbps) than the SP signal. GLONASS is not degraded artificially by the system operators, and there are no plans to introduce such measures in future.

#### ERA-GLONASS

The ERA-GLONASS infrastructure covers all Russian roads. Vehicles are equipped with GLONASS/GPS units. In case of a traffic accident, the unit automatically collects data concerning the location of the vehicle, accident time and severity, and automatically transmits it to an ERA-GLONASS operator. Then this information is transferred to emergency response services. The driver and passengers can also contact an ERA-GLONASS operator manually.

### Future applications

### GLONASS Clocks

The GLONASS satellites are equipped with cesium clocks (time/frequency standards) with daily instability not worse than 5×10-13 and 10-13 for the GLONASS-M satellites. An accuracy of mutual synchronization of the satellite time scales is not worse than 20 ns for the GLONASS and to 8 ns for the GLONASS-M satellites.

GLONASS time is generated on a base of GLONASS Central Synchronizer (CS) time. Daily instability of the Central Synchronizer hydrogen clocks is not worse than 2×10-15.

Figure 4-3: Three atomic frequency generators.

## BeiDou Navigation System

The BeiDou Navigation System (BDS) has been independently developed and operated by China. The BDS provides all-time, all-weather and high-accuracy positioning, navigation and timing services to global users.

The BeiDou Navigation Satellite System (BDS) has been built and developed in accordance with the “three-step” strategy:

BDS-1 construction was started in 1994 and put into use in 2000. It adopted an active positioning scheme to provide Chinese users with positioning, timing, wide-area differential and short message communication services.

BDS-2 construction was started in 2004 and put into use in 2012. Besides being technically compatible with BDS-1, BDS-2 also added a passive positioning scheme, to provide users in the Asia-Pacific region with positioning, velocity measurement, timing and short message communication services.

BDS-3 construction was started in 2009 and the constellation was fully deployed between 2015 and 2020. A total of 35 satellites were launched. This provides global users with high-quality services including navigation, positioning and timing.

BDS comprises three segments: A space segment, a ground segment and a user segment.

The space segment. The BDS space segment consists of a number of satellites located in the Geostationary Earth Orbit (GEO), Inclined Geo-Synchronous Orbit (IGSO) and Medium Earth Orbit (MEO).

The ground segment. The BDS ground segment consists of various ground stations, including master stations, time synchronization/uplink stations, monitoring stations, as well as operation and management facilities of the inter-satellite link.

The user segment. The BDS user segment consists of various kinds of the BDS products , systems, and services as well as those compatible with other navigation systems, including basic products such as chips, modules and antennae, terminal, application systems and application services.

### Current applications

BDS has two functions including navigation and positioning as well as data communication, and provides seven kinds of services. Specifically, it provides three kinds of global services including positioning, navigation and timing (RNSS), global short message communication (GSMC) and international search and rescue (SAR). Meanwhile, the satellite based augmentation system (SBAS), ground augmentation system (GAS), precise point positioning (PPP) and regional short message communication (RSMC) services are provided in China and surrounding areas.

The BDS positioning, navigation and timing service performance standards are as follows:

- System service coverage:  global;

- Positioning accuracy:  10 meters horizontally, 10 meters vertically (95%);

- Velocity measurement accuracy:  0.2 m/s (95%);

- Timing accuracy:  20 nanoseconds (95%);

- System service availability:  better than 95%.

- In the Asia-Pacific region, the positioning accuracies are 5 meters horizontally and 5 meters vertically (95%).

- According to the results of actual measurement, the BDS service capabilities have achieved and are better than above indicators in an all-round way

### Future applications

In the future, BDS will continue to improve service performance, expand service functions, and guarantee continuous and stable operation, to further improve global positioning, navigation and timing, regional short message communication, and ground augmentation service capabilities, and to provide the satellite-based augmentation, precise point positioning, global short message communication, and international search and rescue services, etc.

A more ubiquitous, integrated and intelligent, comprehensive national positioning, navigation and timing (PNT) system is scheduled to be established by 2035.

### BeiDou Navigation system clocks

The BeiDou Navigation Satellite System Time (BDT) is used as the time reference for BDS. BDT adopts the international system of unit (SI) second as the base unit, and accumulates continuously without leap seconds. The start epoch of BDT IS 00:00:00 on January 1, 2006 of Coordinated Universal Time (UTC). BDT connects with UTC via UTC(NTSC), which is maintained by National Time Service Center, Chinese Academy of Sciences. The offset between BDT and UTC is less than 50ns (modulo 1 second). The leap second information between BDT and UTC is broadcasted in the navigation message. BDS adopts the BeiDou Coordinate System (BDCS). The BDCS definition is in accordance with the International Earth Rotation Service Organization (IERS) specification and is consistent with the definition of the China Geodetic Coordinate System 2000 (CGCS2000) with identical reference ellipsoid parameters, aligned with the latest International Earth Reference Framework (ITRF), and is updated annually.

The Beidou-3 satellites use higher performance rubidium atomic clocks, which have a day stability of 10-14 and hydrogen clocks, which have a day stability of 10-15.

### Characteristics

The BDS development follows a model of developing regional service capacities, then gradually extending the services globally. This practice has blazed a path for building a satellite navigation system with Chinese characteristics and enriched the development models for navigation satellite systems worldwide.

BDS possesses the following characteristics: first, its space segment is a hybrid constellation consisting of satellites in three kinds of orbits. In comparison with other navigation satellite systems, BDS operates more satellites in high orbits to offer better anti-jamming capabilities, which is particularly observable in terms of performance in the low-latitude areas. Second, BDS provides navigation signals of multiple frequencies, and is able to improve service accuracy by using combined multi-frequency signals. Third, BDS integrates navigation and communication function, and possesses multiple service capabilities, namely, positioning, navigation and timing, short message communication, international search and rescue, satellite-based augmentation, ground augmentation and precise point positioning, etc.

# Clock correlation

## Concept

Clock correlation is the process of establishing a relationship between a spacecraft clock time and a standard time on the ground, such as UTC. For historical reasons this is also called time correlation, but clock correlation is the more accurate and descriptive term, and clock correlation is also the term used in the IOAG service catalog [37]. If spacecraft clocks never drifted with respect to Earth-based clocks, then they could be set just once, i.e., referenced to ground clocks before launch, and would keep accurate time forever. Events and data could be time-tagged accurately for science and engineering purposes, and precision in-situ navigation would also be possible. Unfortunately, the timing base for many spacecraft functions such as C&DH units are typically an imperfect oscillator which can cause a clock to drift significantly relative to UTC due to thermal effects on the spacecraft and inherent low-frequency drift. In addition, relativistic time dilation will cause even an ideal spacecraft clock to drift relative to an Earth-based clock. This creates a need to monitor and predict spacecraft clock drift with respect to UTC.

In a typical mission-specific clock-correlation protocol, the ground maintains a correlation file to associate the spacecraft clock to UTC. Correlating a clock in space with an Earth-based time standard can be accomplished using either a signal that travels one way (e.g., spacecraft to Earth) or, for more accurate correlations, a signal that travels two ways (Earth to satellite and back to Earth). The one-way or two-way systems may also be used in conjunction with a relay satellite. For example, a two-way signal may travel from Earth to a relay satellite and then to a user satellite, and then return from the user satellite to the relay and then back to Earth. These techniques can also be used between any two nodes, such as a space station and another satellite. The sections below describe specific ways these signals are used.

A one-way signal contains the value of the spacecraft clock within a telemetry frame. This value, together with the Earth-receive time of the frame and the known one-way-light time of the signal, can be used to associate UTC with the time the spacecraft clock value was latched. In a two-way signal protocol, the reception of an Earth-to-spacecraft signal by the spacecraft triggers the release of the clock-correlation signal. Using a sequence of such measurements, a clock-correlation table can be constructed that establishes the offset and drift of the spacecraft clock with respect to UTC.

When such a clock-correlation protocol is used, mission controllers can execute activities on the spacecraft at a desired UTC time by identifying the corresponding spacecraft time in the clock-correlation table and commanding the spacecraft to execute the activity at the appropriate spacecraft time. Because of this, it may not be necessary to physically adjust or synchronize the spacecraft clock to UTC, and in such cases the spacecraft counter may be allowed to run freely. Depending on the protocol used, it may be the case that only the Earth possesses the clock correlation table, and not the spacecraft.

Currently, the various space agencies do not always use a unified terminology. For example, the value of the counter of the spacecraft clock is referred to by NASA as SCLK (Spacecraft Clock), by JAXA as TI (Time Indicator), and by ESA as OBT (On-Board Time).

## NASA Clock correlation methods

NASA has identified requirements [41] for communications, navigation, and time dissemination to support IOAG services. The requirements that NASA has identified for a solar system-wide time synchronization and dissemination architecture are:

- The unit of time should be the second of the International System of Units (SI);

- The time scale should be uniform without steps (leap seconds);

- The time scale should be traceable to an internationally recognized scale of atomic time; and

- The dissemination of the time scale should include the application of appropriate mathematical algorithms that consider signal propagation delays and relativistic effects.

### Clock correlation in the user spacecraft clock correlation system

#### General

This Section describes the User Spacecraft Clock Correlation System (USCCS) used by a User Satellite communicating via a Tracking and Data Relay Satellite (TDRS).

There are several ways of setting or correlating a clock in space with an Earth-based time standard. Use of GPS for position and time is becoming common. However, it is also necessary to set clocks on near-Earth satellites that are outside the GPS constellation, do not have a GPS receiver, or are in deep space. This is commonly done by using either an RF signal that travels one way or two ways.

Sending a reference time from one point to another one at a different location is referred to as time transfer. The one-way method is referred to as the Return Data Delay (RDD) method or the Return Channel Time Delay (RCTD) method. Some missions use the uplink (forward) instead of the down (return) link. The principles are the same but the higher data rate on the down/return link allows for more accurate clock correlation. The TDRSS uses PN spread spectrum for ranging, which lends itself to an accurate two-way time transfer method called the USCCS. USCCS is outlined here and is more fully described in the USCCS User Guide [46] which also covers the RDD methods. In this discussion we do not concern ourselves with the various hardware delays that must be considered.

#### Overview

The fundamental principle of the USCCS is shown in Figure 5-1. A signal, a PN epoch, is sent from the ground to a User spacecraft via a TDRS and back to the ground. When the signal arrives at the User spacecraft, it triggers a reading of the spacecraft clock. This is called the Spacecraft Time and the reading is sent to the ground via the normal spacecraft telemetry. Knowledge of when the signal left the ground, t1, and when it returned, t3, is used to accurately calculate when it was at the spacecraft and triggered the reading of the spacecraft clock, t2. On the ground, the reading on the clock is compared to the time that the reading was triggered by the epoch. This results in a measurement (not a calculation) of the spacecraft clock error from UTC. For each transmitted PN epoch, there is a set of t1, t2, t3 but no notation is used to distinguish one set from another.

It is up to the User Mission Operations Center (MOC) to manage the spacecraft clock. Some projects adjust the oscillator frequency that drives the clock (syntonization), some reset the clock (time synchronization), and some just maintain a table of Spacecraft Time vs. UTC (clock correlation).

Figure 5-1: Block diagram of time transfer signal and data flow.

Using the USCCS requires knowledge of details of the TDRSS PN ranging system since the epochs in the PN range code are used as the timing signals. Figure 5-2 shows the geometry of the range measurement. The full range that the signal travels from the ground network element at the White Sands Complex (WSC) to the TDRS relay satellite at GEO, to the User satellite and then back to the ground network elements is of the order of 140 000 km which corresponds to about 0.47 s. The range channel PN code is 1023 × 256 = 261 888 chips long and is transmitted at approximately 3.08 Mcps, making the period of the code 261888/3.08 Mcps = 0.085 s. At the speed of light, this will cover a distance of d=ct=2.9979×105 km×0.085 s = 25 491 km. It initially appears that the range cannot be unambiguously measured with range PN patterns of this length. But, since the maximum range variation of a LEO satellite as seen by a GEO relay satellite is only a little more than the Earth’s radius of 6378 km, the required range variation measurement only needs to be about 8000 km, and the two-way variation is thus 16 000 km. The 25 000-km-length PN code is sufficient to unambiguously perform ranging to a LEO satellite. It is then also sufficient to be used unambiguously for time transfer for a LEO satellite.

#### Using range PN codes for spacecraft clock correlation

USCCS, being a two-way method, requires a coherent spread spectrum link (called DG1, modes 1 or 3) and is very similar in concept to a ranging measurement. For a ranging measurement, the difference in the time from when a PN epoch leaves the WSC, t1, and the time that the epoch returns to the WSC, t3, is recorded, accurate to a few tens of nanoseconds, i.e., 30 ns which corresponds to about 9 m. Of course, the system must also know when this difference measurement was made. Since a LEO spacecraft is only moving at about 8 km/s, it only moves about 8 m in a millisecond so the absolute UTC time of the ranging measurement need not be to the nanosecond accuracy required by the range measurement, approximately a millisecond will do. On the other hand, in order for clock correlation to be accurate to about 1 microsecond, even though it is not necessary to know the round-trip transit time accurate to tens of nanoseconds, it is necessary to know when the signal leaves the ground terminal and when it returns to the ground terminal, accurate to within a microsecond of UTC.

Figure 5-2: Geometry of the ranging measurement.

#### Time transfer messages

At the User request, the TDRSS can generate an Operations Message (OPM 66) that contains the time that the epochs left the ground station (t1s) and the time that the epochs returned to the ground station (t3s). This information is in PB4 time format [76] which is granulated to a fraction (1/16) of a microsecond. The absolute accuracy with respect to UTC, however, is only as good as the ground station’s time keeping with respect to UTC. The requirement has been that the ground station time must be within 5 microseconds of UTC but with GPS time transfer to the Ground station, it is generally kept to within 1 microsecond of UTC. Forward and Return epochs occur every 85 ms.

In order to reduce the OPM 66 message size [65], only the first epoch after the one second roll over is transmitted. The software at the User MOC that processes the time data must use the knowledge that epochs occur approximately every 85 milliseconds and interpolate from the OPM data to evaluate all of the epoch times. Variation due to Doppler from second to second is small enough that the interpolated values will be accurate to better than a microsecond.

#### User transponder and spacecraft processing

In order to make use of the USCSS service, the User transponder must be capable of coherent PN ranging. In addition, the transponder must output a pulse to the C&DH, referred to as a Time Transfer Epoch that is synchronized to the simultaneous receipt and transmission of the epoch in the PN range code that is modulated on the RF link. The Spacecraft C&DH system must use that epoch to read the spacecraft clock and the spacecraft processing system must place that clock reading in telemetry that is sent to the ground. When using CCSDS virtual channels, a Time Packet and time frame may  use a specific Application Identification (APID) within the Virtual Channels Identification (VCID) used for real time spacecraft housekeeping. Currently, the mechanisms to do this are mission specific.

Epochs occur every 85 ms and using every one over a five-minute period as an example, would result in far more data than is needed. In five minutes, 3529 epochs will occur. It is necessary for the ground processing to be able to figure out which epoch caused which clock reading. To correlate all of the data and limit the burden, several missions have used a circuit that has the reading of the spacecraft clock generally disabled, and only enables the reading of the clock at some limited interval. For example, every 16th housekeeping Virtual Channel Data Unit (VCDU) – currently called a Transfer Frame – is used by several missions. A circuit monitors the sequence count of the housekeeping VCDUs and only when the last 4 bits of the sequence count is all ones (xx…xxx1111) is the epoch circuit enabled. This is indicated in Figure 5-1 on the lower right of the User Satellite block by the telemetry (TLM) Enable.

As a frame with such a sequence count leaves the C&DH on the way to the transmitter, the t2 circuit is enabled. Upon arrival of the next Time Transfer Epoch, the spacecraft clock is read and the circuit is again disabled. Both the reading on the spacecraft clock and the sequence count of the VCDU is placed in the Time Packet and sent to the ground. At this point, there is no urgency, the time packet can arrive at the ground in a later VCDU without loss of accuracy.

#### Determine the spacecraft clock error

It is necessary to determine which epoch triggered which spacecraft clock reading. When the User spacecraft telemetry arrives at the ground (WSC) the data is time tagged with a ground received time (GRT, also called Earth Received Time, ERT). The MOC time processing software uses the GRT of the frames that enabled the spacecraft clock reading to estimate the time that the enable occurred. This involves a simple calculation similar to that used for the less accurate RDD method of clock correlation. An approximate range time to the satellite must be known in order to subtract the space propagation delay from the GRT in order to estimate when the VCDU enabled the clock reading by the Time Transfer Epoch. As complicated as it may sound, one of the nice things about the USCCS is that the Operation Message (OPM) 66 contains a list of t1 and t3 times making it almost trivial to determine the spacecraft to ground propagation time. From orbit geometry, we know that the minimum round-trip propagation time is at least 0.5 seconds.

The following steps are how the data is processed:

- Examine a time packet and find the sequence count of the frame that is associated with the clock reading in that time packet.

- Find that frame and determine the GRT for that frame [64].

- Using the list of t3, pick the one that is close to the GRT.

- Using the list of t1, find the one that is at least 0.5 sec before the chosen t3.

- Take the difference between this t1 and t3 to get the round-trip light time.

- Divide by 2 to get the one-way light time, the one-way range time.

- Subtract this from the frame GRT to get the approximate epoch enable time.

- Calculate and make a table of the possible t2 values. Do this by looking at the t1 values and then finding the first t3 that is at least 0.5 seconds later, t2=(t1+t3)/2.

- Using the table of t2 values, find the first one after the enable time found in line 7.

- t2 is the UTC time that the clock was read. By comparing this to the clock reading in the time packet used in line 1.

- Define the term Clock Error. For example, True UTC time = clock reading – Clock Error, hence Clock Error = Clock reading – True UTC time = Clock Reading – t2.

- Using several time packets, make a table of clock error values. Over about 5 minutes, the errors should be all the same to better than a microsecond accuracy. Delete any outliers that occurred due to the estimate of the epoch enable time.

#### Clock correlation conclusion

The forward range (long code) epoch UTC transmit times, t1, and the received range epoch UTC times, t3, are recorded at the WSC network element. These times are made available to the User processing center for the purpose of correlating a spacecraft clock with a ground time standard, UTC, to within about 1 microsecond. The biggest contributor to inaccuracy in the RDD clock correlation method is the inaccuracy of the spacecraft to ground propagation time which is usually based on a predicted orbit vector. When using the USCCS, the OPM 66 gives that time, measured to a fraction of a microsecond, and eliminates the inconvenience and mistakes that are made when running orbit software to determine the spacecraft to ground propagation time.

By using the received forward epoch to stimulate a reading of the spacecraft clock, and sending that reading to the operation center via spacecraft telemetry, spacecraft clock error may be determined. The calculation of the epoch arrival time at the spacecraft t2=(t1+t3)/2 is correct independent of the motion of the User spacecraft but is limited to about 1 μs accuracy due to the motion of the relay satellite and ground based network element as the Earth rotates. When microsecond or sub microsecond clock correlation is desired, and GPS is not available, the USCCS can be used but there are additional orbit geometry considerations and relativistic effects that the user must consider when performing clock correlation.

### Clock correlation in the Mars Science Laboratory during entry, descent, and landing

The Mars Science Laboratory (also known as the Curiosity Rover) used guided atmospheric entry to land within 2.4 km of the planned landing site. This process is referred to as Entry, Descent, and Landing (EDL). The Mars Science Laboratory project conducted a campaign of pre-flight and in-flight clock calibrations and validations. This effort validated the calibration process, demonstrated that there were no significant systematic errors, and provided sufficient accuracy to support the specified EDL accuracy. Spacecraft clock calibration, and validation of the clock calibration process, required an end-to-end system design. This included components of the Mars Science Laboratory (MSL) flight, test, and ground systems, as well as the Deep Space Network (DSN).

The guided entry process propagated an inertial navigation state from an initial value and epoch time provided by ground-based navigation. Any error in the ground’s knowledge of the onboard clock value contributed to the landing error. In addition, any unrecognized, systematic error would have result in a large landing error, or possible loss of the mission. The error in ground knowledge of the conversion from Spacecraft Clock (SCLK) time to Spacecraft Event Time (SCET) referred to Universal Time Coordinated (UTC) is known as the SCLK-SCET accuracy.

Figure 5-3 illustrates the effect of SCLK-SCET error on landing accuracy. Several days prior to atmosphere entry, the ground navigation team sent the spacecraft an estimate of position and velocity relative to Mars, at a time referred to as T0. Once this time was reached, the spacecraft estimated its position and velocity by inertial navigation, propagating the initial estimate, combined with gyro and accelerometer data as the spacecraft maneuvered through the Martian atmosphere.

Figure 5-3: Downtrack Error SCLK-SCET Error.

The initial time T0 was defined by the navigation time as 9 minutes before the spacecraft reached 3522.2 km from the center of Mars (approximately the top of the sensible Mars atmosphere.) This time was estimated in the Ephemeris Time (ET) frame, based on the International Atomic Time (TAI) frame. The process of computing a SCLK-SCET solution is known as a SCLK-SCET calibration.

The onboard SCLK representation consists of a 32-bit counter, with a nominal least-significant bit (LSB) resolution of 1 s, and a 16-bit subsecond counter, with a nominal LSB resolution of 2-16 s or 15.3 μs. The SCLK value is ideally defined as the time in seconds since 11:58:55.816 AM UTC, January 1, 2000. For purposes of ground operations and command generation, absolute times are represented in Spacecraft Event Time (SCET). SCET is the time in the TAI frame, at the spacecraft, when a particular event happens. SCET is normally represented in UTC (Universal Time Coordinated). This is the same epoch as ET used in navigation calculations, and prior to launch the onboard SCLK register was initialized to this value.

For consistency, a common SCLK-SCET solution is used by all ground tools. This solution is deployed as a SCLK-SCET file at a fixed location, which all tools reference. This file contains a table of SCLK-SCET conversions, as described below. A new version of the SCLK-SCET file is released when the conversion error reaches an unacceptable level (as measured by the most recent SCLK-SCET calibration.) A modified version of the final pre-EDL SCLK-SCET file in Table 5-15-1-.

Table 5-1-: Final MSL SCLK-SCET table

| SCK0 | SCET0 | DUT | SCLKRATE | SCLK rate Error |
| --- | --- | --- | --- | --- |
| 375078191.000 | 2011-324T16:22:00.389 | 66.184 | 1.000009564 | 9.564e-06 |
| 375164622.000 | 2011-325T16:22:32.165 | 66.184 | 1.000009114 | 9.114e-06 |
| 375236601.000 | 2011-326T12:22:11:809 | 66.184 | 1.000009250 | 9.250e-06 |
| 375411971.000 | 2011-328T13:05:03.432 | 66.184 | 1.000008880 | 8.880e-06 |
| 380649630.000 | 2012-024T04:00:09.801 | 66.184 | 1.000009187 | 9.187e-06 |
| 392779715.000 | 2012-164T13:30:03.146 | 66.184 | 1.000009272 | 9.272e-06 |
| 394372699.000 | 2012-183T00:00:00.917 | 66.184 | 1.000009272 | 9.272e-06 |
| 396388693.000 | 2012-206T08:00:13.449 | 66.184 | 1.000009275 | 9.275e-06 |

Each row in the table represents a linear conversion between SCLK and SCET for the time range beginning at SCET0 for that row and ending at SCET0 of the next row. The overall table represents a piecewise-linear conversion between SCLK and SCET. In the SCLK-SCET table, the SCLK0 column represents the SCLK value at the start of each line segment. The SCET0 column represents the SCET value at the start of the line segment. The SCLK Rate column represents the slope used to convert SCLK to SCET0 for this line segment. This slope is defined as (change in SCET)/(change in SCLK). The DUT column represents the leap-second Delta-UTC correction between UTC and Ephemeris Time (ET). A leap-second correction was added on day of year 183 to account for the leap-second introduced on July 1, 2012.

Accuracy:  SCLK-SCET accuracy is defined as the difference between the actual SCLK-SCET relationship and the relationship shown in the SCLK-SCET file, evaluated at some particular time. For EDL, the accuracy requirement is set in terms of the conversion between SCLK and SCET at T0 as defined by the navigation team. The SCLK-SCET accuracy requirement for EDL is the most stringent during the mission. This requirement was chosen so that the downtrack error due to errors in SCLK-SCET conversion would not significantly increase the overall navigation error.

For example, at an approach velocity of 5.6 km/sec relative to Mars, a navigation error of 3 km is roughly equivalent to a time-of-flight error of 500 ms. The requirement for SCLK-SCET accuracy was set at 40 ms. This is less than 10% of the ground-based navigation uncertainty, so it is a small contributor to landing accuracy. (The actual SCLK-SCET error turned out to be ~0.4 ms, exceeding the requirement by a factor of 100.)

The calibration process:  MSL Telemetry is controlled by the Multi-Mission Space Avionics Platform Telemetry InterFace board, or MTIF. The MTIF transmits CCSDS-standard telemetry transfer frames to the X-Band radio – the Small Deep-Space Transponder (SDST). Each transfer frame includes a Virtual Channel Frame Counter (VCFC) – a monotonically increasing counter of transfer frames transmitted. The MTIF board also contains the Spacecraft Clock (SCLK), a 48-bit counter which is counted down from an oscillator with a nominal frequency of 33 MHz. The actual frequency of this oscillator is calibrated as part of the SCLK-SCET process.

On command, the MTIF saves the value of the SCLK register when the first bit of a particular downlink frame is transmitted. The flight software assembles this SCLK time, and the corresponding VCFC counter into a clock correlation packet that is put into telemetry.

Figure 5-4: Flight SCLK-SCET Calibration Configuration.

On the Earth, the DSN station tags the ERT for the first bit of each frame received. By subtracting the One-Way Light Time (OWLT) to the spacecraft, the ground system can compute the Spacecraft Event Time when the frame was transmitted. Correlating this SCET with the SCLK value carried in a clock correlation packet establishes a SCLK-SCET pair. By fitting several of these measurements to a straight line, the ground system computes the SCLK rate. The ground system combines the SCLK-SCET offsets and rate to construct the SCLK-SCET file.

The diameter of Earth introduces a change in light-time of up to 21 ms, hence the station location must be included in the calculation of the OWLT. For the purpose of SCLK-SCET accuracy analysis, the OWLT is assumed to be perfect – i.e., no error. OWLT errors are related directly to navigation accuracy. If the entire 3 km navigation error were attributed to an error in OWLT, this would be equivalent to a clock error of ~10 μs, comparable to the SCLK LSB resolution. In practice, the OWLT is measured directly by ranging to an accuracy of meters, so the resulting error is small relative to the SCLK LSB resolution.

### Clock correlation in the Mars Reconnaissance Orbiter

The Mars Reconnaissance Orbiter (MRO) Clock Distribution System (CDS) was designed to have sufficient resolution and accuracy to capture its own clock on the order of microseconds. The DSN used more accurate ERT measurements, on the order of microseconds, and used mission-specific statistical processing function to reduce the data to a SCET/ERT correlation table. The Mars Reconnaissance Orbiter ontains the UHF radio Electra, which is a telecommunications package that acts as a communications relay and navigation aid for MRO and for future Mars spacecraft to support navigation, command, and data-return requirements.

Figure 5-5: Electra clock offset determination with the spacecraft clock in the Loop.

The interface between the MRO CDS and Electra included an edge triggered signal that was designed to capture the Electra clock within the Electra payload and capture the spacecraft clock within the CDS. These two times can then be compared. Their difference is a measure of the offset between the two clocks. Resolutions and accuracies of these clock captures are typtically on the order of microseconds.

Since the spacecraft clock offset is known, the Electra clock offset relative to UTC is just the sum of the two offsets and the error is the RSS of these two errors. If both the Electra clock and the spacecraft clock are referenced to the USO, the clock offset between Electra and the CDS should remain constant. Operators could measure the offset between the two clocks once and then retire the function. The Electra and CDS clocks would continue to drift together with respect to UTC. Alternately, one could collect multiple data points, statistically process them and drive the Electra-to-CDS clock offset measurement error towards zero, assuming all systematic errors and offsets are known and understood.

#### Removing MRO clock and MRO interface out of the loop in determining Electra clock offset relative to UTC

Electra requested that the MRO CDS send out a “Special” Type 1 Time Reference packet. The only thing special about it is that it is marked as an Electra Type 1 Time reference packet and a “special” sync marker is used. The frame number that this packet appears in is returned to Electra. As this packet passes through the SDST and out, Electra is listening at X-band, down converting, and correlating bits to sync up to the unique sync marker at the frame boundary, (or elsewhere in the frame).

Figure 5-6: Proposed scheme to remove MRO clock and interface from Electra Clock Offset relative to UTC.

When this sync marker is detected, Electra marks the time with its Clock and stores it. When the same frame is received on Earth it is marked with an Earth Receive Time, ERT. This ERT with the frame number is passed to an Electra clock correlation processing function, along with an estimate of light time from the DSN Nav function.

At the spacecraft, Electra passes its captured clock time to the CDS. A “Special” Type 2 Time Reference packet is formed which includes the “snapped” Electra clock time and the Frame number of the data frame that contained the sync marker that was “time snapped by Electra as it left the spacecraft”. Upon receipt at Earth, this packet is passed to the Electra clock correlation processor function. It matches the Electra Clock, ECLK, info with the ERT and RT light time info already captured for that data frame and it now has a single (ECLK,ERT) pair. A series of these (ECLK,ERT) pairs can be captured and statistically processed to form a ECLK/EEVT correlation table, where EEVT means Electra Event Time. This process mirrors the DSN’s Advanced Multi-Mission Operations System (AMMOS) process but takes the spacecraft clock out of the loop.

### Clock correlation in the Advanced Multi-Mission Operations System

The Multi-Mission Time Correlation (MMTC) application is being added to the AMMOS Catalog for use by all NASA missions. It is a mission-independent application that maintains on the ground an association between spacecraft onboard clock time and terrestrial time and produces several data products. Those products include a SPICE SCLK kernel, a SCLK/SCET file, and a time history file.

#### Performing clock correlation

The SPICE SCLK kernel produced by MMTC contains a set of correlations between spacecraft clock (SCLK) and terrestrial time (TT). It typically includes these correlations, in terms of time records, for the entire mission. Each time record includes an encoded form of SCLK and an estimate of the corresponding TT. It also includes an estimate of the average rate of change of TT with respect to SCLK to conform to the SPICE standard format for SCLK kernels. (The following text refers to that average rate of change as the clock change rate.)  SPICE is a ground software framework produced by the Navigation and Ancillary Information Facility (NAIF) of the Jet Propulsion Laboratory (JPL) and utilizes various kernels that customize the framework for a particular mission. The SCLK kernel is a human-readable text file that includes a table of clock correlations for an entire mission. The MMTC SCLK kernel provides, by default, an optional enhancement to the typical SPICE SCLK kernel in the way it computes the clock change rate.

Another product of the MMTC application is a cumulative time history file that contains information about the status of SCLK over time. This time history file is used for analysis by cognizant engineers or mission operations personnel. It contains information useful for assessing the health of the spacecraft clock and for diagnosing anomalies.

The MMTC application also creates an SCLK/SCET file. There is a one-to-one correspondence between the records in the SCET file and those in the SCLK Kernel. They are the same data, but in different forms.

#### The SCLK kernel and the SCLK/SCET file

As noted earlier, the SCLK kernel file complies with NAIF SPICE specifications. The MMTC application updates the mission SCLK kernel each time it runs by adding a new record consisting of an SCLK time in SPICE encoded SCLK form, a corresponding Earth time in TT form, and a clock change rate. Updating the mission SCLK kernel is a fundamental purpose of the MMTC. It provides the latest available clock correlation parameters.

MMTC also creates an SCLK/SCET file that contains an association between SCLK, a corresponding Earth time in UTC, and clock change rate. The SCLK/SCET File is updated each time it runs by adding a new record consisting of an SCLK time, a corresponding Earth time in a UTC calendar string, and a clock change rate. SCLK/SCET files, as well as SCLK kernels, are used by numerous applications.

#### The time history file

The time history file is an analysis product that is useful for assessing the health of the onboard clock and for diagnosing anomalies. MMTC updates the SCLK kernel and the SCLK/SCET file with a new clock correlation record and the time history file with a new history record with corresponding data during the same run. The time history file is most useful when each record it contains can be correlated with a record in the SCLK kernel or SCLK/SCET file.

Each run of the MMTC application appends to the time history file a new time history record that contains, at minimum, encoded SCLK, SCLK, TT, predicted clock change rate, SCLK kernel partition, one-way light time (OWLT), RF encoding form, downlink bit rate, estimated error in the SCLK kernel prediction of TT, and the temperature of the oscillator that drives the onboard SCLK clock. The time history file is used for analysis by cognizant engineers or mission operations personnel.

#### Spacecraft clock change rate

The spacecraft clock change rate is not the same as the clock drift rate. The drift rate is the number of fractional SCLK seconds gained or lost over each TT (or UTC) second. The change rate follows the JPL NAIF convention for clock rate and is the number of TT (or UTC) seconds per SCLK second. The numerical relationship between the two is SCLK drift rate = (1/ change rate) – 1. So, a clock change rate of 1 means zero SCLK drift rate. A clock change rate greater than 1 corresponds to a negative drift rate and means that the SCLK is losing time relative to TT. Conversely, a clock change rate less than 1 corresponds to a positive drift rate and means that the SCLK is gaining time relative to TT.

Whenever a new time record is added to the SCLK kernel, the predicted clock change rate is determined by either assignment or by computation from past contact data. SPICE uses the predicted clock change rate in a simple linear extrapolation to predict future mappings between SCLK and TT.

In addition, when a new time record is added to the SCLK kernel, the clock change rate in the previous time record is, by default, modified to provide a more accurate mapping between SCLK and TT for the period between the old and new time records, although that default behavior can be overridden. This new clock change rate is called the interpolated clock change rate. The interpolated clock change rate is valid only between the old and new time record. For times past the new time record, the predicted clock change rate provides a more accurate prediction of future average SCLK behavior.

#### Mission independence

As part of the AMMOS system, the MMTC application must not be tied to a specific mission or require major customizations. MMTC works with different and unrelated space mission ground systems.

There are two distinct approaches to obtaining SCLK from the spacecraft. In one, the MMTC application reads SCLK values from CCSDS transfer frame secondary headers and the Earth Received Time (ERT) of the associated reference transfer frame received from the ground station in regular downlink telemetry. This is the method of performing clock correlation described in Section 5.2.5, commonly used by APL. Note that the frame containing the SCLK will usually not be the one it is associated with.

In the other approach, MMTC determines clock correlations based upon the SCLK time contained in specified clock correlation packets and the Earth Received Time (ERT) of the associated reference transfer frame received from the ground station in regular downlink telemetry. This approach is commonly used by JPL missions.

### Clock correlation in Applied Physics Laboratory missions

NASA deep space missions developed by APL use a similar approach to what was described above for the MRO mission. There are usually two onboard time elements to these missions: (1) the spacecraft clock and (2) an onboard estimate of Terrestrial Time.

Similar to the Mars Reconnaissance Orbiter (MRO) example given above, APL missions also have a spacecraft clock that is not adjusted after launch. The primary difference between the MRO approach and the way APL missions handle the spacecraft clock is the method by which the spacecraft clock value (SCLK) is communicated to the ground.

APL missions use the CCSDS TM downlink frame format standard [67]. This standard specifies that each downlink frame includes a primary header that contains an 8-bit master channel frame count (MCFC) and an 8-bit virtual channel frame count (VCFC). Since current missions often downlink dozens of frames, or more, each second, neither the MCFC nor VCFC is sufficient for uniquely identifying the downlink frame for more than a few seconds when a high downlink bit rate is used. A packet-based approach to downlinking the SCLK value to the ground would be challenged to uniquely identify the frame for which that SCLK value applies. Instead, APL missions include the frame SCLK value in the mission-defined secondary header of every frame. Typically that SCLK value refers to a previous frame.

This alternative approach has pros and cons compared to the MRO approach of using clock correlation packets. The APL method consumes a small amount of bandwidth but no issues have been identified regarding this method on any APL missions. It is, however, simpler for Mission Operations to use as there is no need to command downlink of clock correlation packets.

APL deep space missions also include an onboard estimate of Earth time that is used to support spacecraft pointing. That Earth time estimate is used to access time-indexed onboard ephemerides so that the spacecraft knows where it is relative to other bodies and where it should point itself and onboard science instruments. That estimate is computed from SCLK using several parameters that are updated by ground command when needed to ensure the onboard estimate of Earth time is sufficiently accurate.

## ESA Clock correlation methods

### Clock correlation for space application

A precise and accurate datation of science data (the action of attributing a date to data) requires a careful design of the onboard time-distribution architecture and selection of a proper master clock. The stability and insensitivity to environment effects are the most important aspect for the selection of the onboard clocks. A special care is dedicated during the design of the payload to put the master clock in a shielded and controlled environment in order to minimize the effects of the harsh space environment conditions. This section discusses the problems of data timing and clock correlation for the ESA Deep Space mission Gaia. One aspect of the data processing for Gaia mission is that every observation must be tagged with a stable and accurate time tag.

#### Gaia mission

Gaia is the second ESA space astrometry mission launched operationally since 2014. The Gaia mission objective is to chart a three-dimensional map of our Galaxy by surveying more than a thousand million stars. This huge stellar census will provide important data to understand the origin, structure and evolution of our Galaxy. The absolute time accuracy requirement for Gaia mission is 2 microseconds, split between the ground and space segments. This requirement is met using one-way clock synchronization.

Figure 5-7: Gaia mission launch and operations requirement for the timing of Gaia.

Each observation of Gaia must be tagged with the reading of the Gaia clock. The rubidium free-running clock on board Gaia spacecraft produces time tags that are called Onboard Time (OBT). The OBT is a technical time which reflects all the imperfections of the used clock on board. OBT is a purely technical time with no apriori relation to any other time scales.

Timing requirements for Gaia mission consist of two parts. As usual in the discipline of time, we should distinguish between stability and accuracy of timing information. Some of the digital hardware components on board of Gaia, like the charged-coupled device (CCD) cameras, must be driven by some frequency standard.

Space missions often use quartz oscillators for this purpose. The stability of a state-of-the-art space quality quartz can achieve 10-9 in a thermally stable environment of Gaia, but it is not sufficient for Gaia. In fact, the scanning Law of Gaia [77] implies rotations with an angular velocity of 0.6 micro-arcsecond (µas) in 10 nanoseconds. The ultimate accuracy of centroiding for one observation is expected to be on the order of 10 µas. Systematic errors over periods of time shorter than the rotation periods are very dangerous for the quality of astrometric products of Gaia.

It was decided during the design phase that no systematic errors over the rotational period of 6 hours or 21600 sec should be introduced by the time tags of the data. This means that the timing error should be below 10 nanoseconds over the period of 21600 seconds, which implies the frequency stability of the onboard clock below 10-12. For this reason, Gaia uses an atomic (Rb) clock with the stability reaching ≤10-13 over the Gaia rotational period of 21600 seconds. More details about the onboard clock is provided in the following section.

Regarding accuracy, the official goal for the timing accuracy for Gaia was taken to be 1.7 microseconds. This accuracy represented a limit of accuracy achievable without any substantial upgrade of the timing hardware available at the ESA ground stations.

#### Onboard clock for Gaia mission

The Gaia onboard clock is a space qualify Rubidium Atomic clock Standard produced by Spectratime (nominal and redundant). This is the same kind of clock used by the Galileo GNSS program. Its specified frequency stability is better than 8.10-14 over 10000 seconds of integration time. Before launch, the clock of Gaia was tested in a laboratory environment over a long interval of time. The Figure 5-8 present the stability measurements. These clocks generate the onboard time (OBT). This OBT is used for time stamping of the event transmitted later to the ground-station where these events will be tagged in UTC upon reception.

Figure 5-8: Frequency stability of the Gaia RAFS represented by its Allan deviation from a test campaign before launch.

In-orbit experience shows that ageing is negligibly small over non-visibility periods. In addition to the standard effects like drift, ageing, and random walk, the Gaia clock is also subject to frequency jumps. These jumps, typically on the order of 5 x 10-12 and occurring less frequently than once a week, are observed for all atomic clocks in space. The origin of these jumps are not fully understood, although some jumps are correlated with Rubidium-lamp light-level changes and others may be related to stress relief.

The Rubidium clock of Gaia is a free running oscillator that should be monitored, related to and synchronized with widely-used time scales like UTC or Barycentric Coordinate Time (TCB), realization of which is based on ensemble of high-accuracy clocks on the Earth. This is done using specially organized one-way synchronization observations between ESA ground stations and the Gaia satellite. Because of the high accuracy of Gaia observations and products, the observational data of Gaia must be interpreted and modelled in a consistent relativistic framework. This is also true for the Gaia onboard clock monitoring.

#### Clock synchronization process: one-way method

Several techniques enable the synchronization of remote clocks depending on the targeted accuracy. The problem of synchronization of the Earth-based clocks using different techniques, and considering the General Relativistic framework, is documented in several articles [16].

It is well known that the two-way approach is the most accurate and allows to relax the requirements of the remote clock and also cancel out the tropospheric delay. In general, there are many technical ways to synchronize remote clocks. For a synchronization between two clocks separated by 1.5 million kilometers at the level of a microsecond, the two-way technique is the method of choice. Unfortunately, the hardware deployed in the ESA Tracking Stations is not prepared for the two-way clock synchronization, therefore for Gaia mission, ESA used a straightforward one-way clock synchronization.

Figure 5-9: Schematic view of the one-way clock synchronization.

In the case of Gaia the one-way clock synchronization can be summarized as following (illustrated also in Figure 5-9):

- Generation of the signal on board the spacecraft that contains OBT value and transmit data packet called ‘time packet’

- After some internal delay the time packet is transmitted to the ground stations

- After propagation delay the time packet arrives to the phase center of the station antenna

- After some ground-station delay the content of the time packet is recorded by the station hardware by assigning a UTC value according to the local clock and storing the time couple (OBT, UTC) in the data base.

Therefore, the raw data for clock synchronization are the time couples (OBT, UTC). These time couples are obtained at irregular intervals of time with a typical interval of 1.5 sec between the subsequent time couples during the visibility periods of Gaia which refer to the periods of time during which Gaia can communicate with a ground station. Duration of the visibility periods varies depending on the data volume that is to be transmitted to the ground.

#### Time transfer model for Gaia

The accuracy of about 1 microsecond needed for Gaia implies a rigorous relativistic model for the one-way clock synchronization data described above. The model explicitly introduces the relativistic proper time of Gaia, which is denoted as TG in Figure 5-10 and represents an ideal clock located at the Gaia center of mass.

The displacement of Gaia’s atomic clock from the Gaia center of mass is fully negligible in the current context. The deviation between OBT and TG solely reflects the errors of the Gaia clock (ignoring possible constant phase shift).

The model of the time transfer data results in a series of reduced time couples (OBT, TG) that allow construction of the relationship between OBT and TG. As it was already pointed out, this relationship reflects both the constant phase offset between OBT and TG (actual synchronization parameter that would persist even for ideal Gaia clock) and the imperfections of the Gaia clock. Since the Gaia clock is of high quality there is a simple relationship between OBT and TG. On the other hand, the OBT–TG relationship represents physical model of the Gaia clock as if the clock would be monitored in a laboratory. In this way the health status of the clock and its performance can be directly assessed.

The scheme for the overall model for the time couples (OBT, UTC) is shown on Figure 5-10. The main idea of the model is to compute the value TG (relativistic proper time of Gaia) that correspond to each OBT. On the other hand, the corresponding TG is computed from UTC in a number of steps, summarized in the following steps:

- The ground-station delay is subtracted from UTC

- The moment of reception in TBC is computed using a series of relativistic times

- The TCB moment of emission TCB is transformed into the corresponding moment of the Gaia proper time TG using the function TG(TCB) obtained as discussed above.

Figure 5-10: Time Transfer Model for Gaia: modelling scheme.

Auxiliary data, such as the coordinates of all ESTRACK stations in the International Terrestrial Reference Frame, including the Cartesian coordinates as well as the height above the geoid, is needed to complete this computation. Additionally, a good assessment of the ground-station delays and onboard delays for each telemetry mode are key information for this model. For more details about the model used are given in more details in the following references [15], [16].

The overall modelling accuracy of this algorithm is about 30 nanoseconds and is limited by the error of the distance between Gaia and the ground stations. Fortunately, since the Gaia satellite is observed by Doppler and radar techniques, the uncertainty in geocentric radial distance does not exceed 6–10 m. This accuracy is more than enough for the one-way clock synchronization with an accuracy of better than 1 microsecond.

## DLR clock correlation methods

### Overview

This section presents the time correlation and time synchronization methods at DLR (Deutsches Zentrum fur Luft und Raumfahrt) for two different examples implemented on two satellites of the European Data Relay Satellite System (EDRS). The Onboard data handling differs between the two satellites so that principles of (i) so-called “frame-based”, which means in principle fixed allocated and, periodically reported telemetry can be compared with (ii) TM source packet based methods corresponding to the ESA telemetry and telecommand packet utilization standard (PUS) [17][18].

In addition to these differences in the reporting of timing information, there are two distinct use cases for operations: one spacecraft (i) has a free running onboard clock (OBC), which ground has to correlate dynamically with UTC. The second spacecraft (ii) has an onboard clock which is required to be controlled in terms of a synchronization with GPS time; since the conversion between GPS time and UTC time on ground is known, the time correlation of GPS time and UTC is performed statically

The two examples are part of the European Data Relay Satellite System (EDRS), also called “SpaceDataHighway,” which is owned and commercially operated by Airbus whereas the hosted P/L of the first satellite and the second satellite including its P/L are technically operated by DLR. The EDRS mission is a European constellation of GEO satellites with a payload of communication links consisting of Ka-band antennas and laser communication terminals (LCT), which relay information and data between spacecraft, unpiloted aerial vehicles (UAV) in the stratosphere and ground stations. The discussion in this section follows the principles of (i) EDRS-A data handling, which has a periodic, deterministic reporting of the onboard time data, versus (ii) the EDRS-C satellite which allows reports of onboard time data transmitted via CCSDS telemetry packets according the ECSS PUS standard [17] for clock correlation.

### Example for synchronous reporting of onboard time data

The EDRS-A spacecraft sends cyclic telemetry where fixed TM allocations are reported by a single large packet embedded and transmitted within a TM transfer frame. This data structure is called “minor frame”. In total 32 of these packets containing distinct TM allocations are transported periodically (one packet per one TM transfer frame) and constitute a large-scale data structure called “major frame”. The onboard periodic TM reporting is repeated after the transmission of the individual 32 “minor frame” data structures like an image construction of a cathode-ray tube. In addition to the fixed telemetry allocations, there are also areas reserved for variable, programmable reporting mechanisms which span across the entire “major frame” data structure; like other near real-time related telemetry acquisitions, the onboard clock (OBC) reporting data also have allocations within the fixed telemetry part of the frames.

Figure 5-11: The fixed allocation of the onboard time report within the frame allows an alignment in time during recording on ground to provide a correlation-pair.

The time management of the EDRS-A onboard data handling (OBDH) has a precision of 0.1s and an end-to-end accuracy of 1 second. In addition to this platform related unit, the payload (Laser Communication Terminal, LCT) has its own data handling unit with its own clock. Whenever the difference between both clocks (platform OBDH and LCT) exceeds a threshold, the LCT clock is synchronized with the platform's clock. In following, only clock correlation methods of platform clock versus ground are discussed.

Every second, the onboard unit measures a coherent data set which is transmitted to ground in the following second. Samples of predefined TM acquisitions (i.e. technical term for the measurements of telemetry parameters) are distributed across the major frame. The onboard time is part of this data, which allows explicit timestamps for the corresponding transmitted telemetry samples on ground.

The onboard clock is a free-running counter with 1 count per ~ 1 sec and without predefined epoch. The ground clock, which follows UTC, performs a clock correlation to associate telemetry acquisitions (primary annotated with reported OBT) with UTC values. Typically, the clock correlation has a precision of at least 100 ms. On the basis of the gathered clock correlation statistics, the OBT timescale can be expressed in UTC and vice versa: e.g. to compute future events in OBT for uplink purposes, such as time-tagged commands or eclipse dates, or to timestamp downlinked onboard data in UTC for further analysis. Whenever a deviation of the onboard tick of 1 second above a given threshold is noticed, a drift correction of the oscillator can be performed. If required, the drift can be analyzed as a function of time and be adjusted accordingly.

For both clock correlation and drift correction, statistics of OBT/UTC clock correlation references (so-called clock correlation pairs) are required, which are based on pairs of periodic onboard time stamp data and ground time stamp data. Both time scales have their own representation. The OBT is defined by two telemetry values representing coarse time (the free running counter tick of ~1sec) and fine time (fraction of 1sec). The ERT is measured in UTC following UNIX time stamp representation and CCSDS definitions [19].

The OBT data reference is coupled to the so-called frame emission onboard time, which is downloaded in the first telemetry set transfer frame. As the sample is part of the fixed telemetry data pool and as its inserting into the transmitted TM frame is a deterministic process, its transmission time serves as a periodically reported alignment point for the reception time on ground.

The corresponding UTC timestamp is taken from the reception time of this data unit on ground, which is the Earth receive time (ERT) of the corresponding TM transfer frame, corrected by a given delay delta_T (i.e., the sum of spacecraft internal delays, the transmission time of space to ground, and ground related delays depending on the ground station).

Many onboard activities, such as time-tagged commanding (generation of the schedule and execution of commands) and analysis of telemetry, have to be associated with the onboard time (OBT). This is achieved by the clock correlation procedure. The onboard time Tboard is expressed as a function of the ground time Tground, which is standard Universal Time, UTC : Tboard=f(Tground).

The function f is generally not known since it depends on the onboard clock drift and the initial reset of the free running clock (a counter). However, it is assumed that on a short-term time horizon, the expression can be simplified following a linear relationship:

Tboard=D Tground+O,

where D and O are constants.

The value of D is determined by observing the drift of the onboard clock over a period of time. The value of O is determined by assessing Tboard and Tground at a given time: T0,board, T0,ground, and O=T0,board-D T0,ground on the basis of the gathered clock correlation pair statistics. This calculation is referred to as clock correlation.

During mission operations, whenever the first minor frame of a major frame is received, the transmitted reference of T0,board is stored as first value of a new clock correlation pair together with its associated T0,ground. The value of T0,board, as determined by the central onboard software, corresponds to the occurrence of the leading edge of the first bit of the attached synchronization marker of the received telemetry transfer frame (the ERT). This measurement is reported to the control center via the Space Link Extension – Return All Frames (SLE RAF) service [71] and represents after corrections T0,ground.

Delays – referred by the constant O – need to be considered as it takes time to acquire the OBT until the corresponding ERT can finally be measured. Since the EDRS-A spacecraft transmits the fixed-telemetry data pool in a pre-determined, near-real-time manner, these delays are constant and known beforehand. So, the first delay (i) is the time onboard in which the OBT acquisition is performed, copied to the allocation of the TM transfer frame and finally radiated by telemetry encoder, depending on the telemetry transmission rate. The second delay (ii) is the propagation delay of the radiated signal between the location of the vehicle in space and the antenna on ground. Its value is fixed when being in GEO but has to be considered as a function of time during the phase in which the vehicle is in the transfer orbit. If applicable, a third constant (iii) considers the reception delay before the ERT is processed, typically depending on the characteristics of the antenna’s base-band equipment, such as the duration for measuring the ASM and convolutional decoding. Post-processing delays are not considered, e.g., deciphering processes shall not influence the already measured ERT. Leap seconds are typically corrected automatically during the antenna reception process. Since jitters influence the accuracy, the end-to-end accuracy for EDRS-A is estimated with 1 second.

The control center mission control software stores the gathered clock correlation pairs for further processing. In operations, a slope is fitted, based on the given archived samples of clock correlation pairs, which allows an estimate of the gradient and offset required to obtain an expression for Tboard=f(Tground). The result is stored together with the deviation and a timestamp of the computation for future conversions of OBT / ERT values. Depending on the mission phase, the correlation is performed in the following ways: automatically after each generation of a new correlation pair, on user request, or in an event-driven mode when pre-defined thresholds for values D or O are reached. After a reset of the onboard processing unit the clock counter is also reset. In this case, the basis of clock correlation pairs also has to be refreshed for future clock correlation computations.

### Example for asynchronous reporting of onboard time Data (example ECSS packet utilization standard, EDRS-C)

The data handling principles of the second example, the EDRS-C spacecraft, follow the ECSS Packet Utilization Standard (PUS), where operation concepts and functions are subsumed by services with defined PUS service types and sub-types following the nomenclature (Type, Subtype) (see [17] and its successor [18], respectively). Besides standardized services each space project has the option to define mission-specific services. The time management service is standardized by the service number 9, where service requests and reports are grouped into the services (9,X).

The EDRS-C onboard clock of the processing unit is designed as a free running counter (1 tick = 1 s) with a sampling frequency of 1/216 per second to constitute the onboard time (OBT) data. The OBT time representation follows the TAI epoch and is to be synchronized with the GPS time. The time information is reported with the help of the ECSS-PUS Service 9 so that the mission control system of the control center is able to estimate the clock correlation on the basis of determined clock correlation pair statistics (OBT|UTC). A diverging OBT according to any detected offsets can be directly synchronized with the time on ground. The payload of the relay satellite, the Laser Communication terminal (LCT), has its own data handling processor following the ECSS PUS so that the LCT can use the platform’s clock.

In the following, the principles of time reporting and correlation are discussed according to the ECSS PUS definitions for the time management service [17]. Telemetry source packets and telecommand packets relating to the time management service are denoted by PUS, Service Type = 9. For OBT reporting, the generation rate of the OBT information can be (1, 2, 4, 8, 16, 32, 64, 128 or 256) by telecommands defined by PUS Service “TC(9,1)”. To report the OBT to the ground, a pre-defined data structure - the “Time Report TM(9,2)” - is used, which is provided by application process ID = 0. The time reporting sub-service has access to the satellite time reference. The onboard time reference is always sampled at the instant of occurrence of the leading edge of the first bit of the attached synchronization marker of the telemetry transfer frame of Virtual Channel 0 where the virtual channel frame count modulo (generation rate) equals 0. The time reporting sub-service then downlinks this satellite time reference in a spacecraft time source packet at any time before the satellite time reference is next sampled. Together with the OBT information, the generation rate can also be reported in the same source packet. The time stamp is expressed by the CCSDS Unsegmented Code (CUC) format [19].

Figure 5-12: The Time Report Packet, TM (9, 2), can be associated with a corresponding virtual channel frame recording on ground to provide a clock correlation pair.

Whenever the ground software detects the downlink of the time report source packet, the TM(9,2), it evaluates the corresponding transfer frame according to the reported rate on basis of the telemetry data archive. From the identified TM transfer frame, the associated Earth-receive time (ERT) is deduced, for which delays have to be handled. As the correlation of the OBT to the emission time of the corresponding TM transfer frame is already handled and as systematic delays are already taken into account by the onboard software, only the propagation delay of the telemetry signal between space and ground and delays on-ground (timespans during measurement of the ASM as well as for the convolution decoding) need to be considered. The gathered information is stored in the form of correlation pairs for further statistical evaluation.

During operations, a predefined set of the latest correlation pairs is continuously analyzed to provide a jitter-free estimate of the delay between the OBT and the ground time in UTC. Whenever a deviation above a given threshold is detected, the onboard clock can be synchronized on the basis of this value by a mission-specific telecommand. Depending on the mission phase, this operation can be performed manually or automated using a predefined threshold.

-

TboardTgroundTboard=f(Tground)

f

Tboard=D Tground+O,

DO

DOTboardTgroundT0,boardT0,groundO=T0,board-D T0,ground

T0,boardT0,groundT0,boardT0,ground

O

Tboard=f(Tground)DO

1/216

-

## JAXA clock correlation methods

Table 5-2-: Time calibration system of Japanese planetary explorations.

| Mission | Purpose | Period | Time management |
| --- | --- | --- | --- |
| Hayabusa (MUSES-C) | Asteroid sample return mission | Launch: May 2003 Proximity:Sep.-Nov., 2005 Return: Jun. 2010. | Create spacecraft clock and UTC mapping |
| Kaguya (SELENE) | Lunar orbiter | Launch: Sep. 2007 LOI: Oct. 2007 Nominal:Dec.2007-Oct.2008 Extend: Nov.2008-Jun.2009 | Create spacecraft clock and UTC mapping |
| Akatsuki (Planet-C) | Venus Climate Orbiter | Launch: May, 2010 VOI: Dec. 2015 (First VOI was failed in Dec. 2010) | Create spacecraft clock and UTC mapping |
| Hisaki (SPRINT-A) | Jupiter observation by EUV spectroscopy | Launch: Sep., 14, 2013 | Create spacecraft clock and UTC mapping (new common system) |
| Hayabusa2 | Asteroid sample return mission | Launch: Dec. 3, 2014 Proximity: Jun 27, 2018 to ... | Create spacecraft clock and UTC mapping (new common system) |
| Arase (ERG) | Exploration of energization and Radiation in Geospace | Launch: Dec. 20. 2016 | Create spacecraft clock and UTC mapping (new common system) |
| SLIM | Pinpoint landing on the Moon |  | Create spacecraft clock and UTC mapping (new common system) |

### System overview

Time calibration is composed of two steps:

- Creation of Time Calibration Table (UTC time indicator (UTC-TI) mapping table) using Time Calibration System

- UTC-TI mutual conversion with interpolation and extrapolation using Time Calibration Library in C language

Figure 5-13: System overview of Time Calibration.

### Time calibration system

#### Operation of Time calibration packet

Spacecraft generates time-calibration packet that corresponds the UTC to spacecraft clock. It consists of three steps: (1) Latch spacecraft clock of a transfer frame. (2) Virtual channel counter and spacecraft clock in the transfer frame are recorded into a time-calibration packet. (3) Send the time-calibration packet later.

#### Equation of the calculation of spacecraft clock latch time

Spacecraft uses a concatenation of a convolutional code as the inner code with a Reed-Solomon code as the outer code. In the convolutional code, the constraint length, K, is 7 bits following CCSDS 131.0-B-3 Page 3-1, which generates 7-bit delay to send. Figure 5-14 shows the latch timing and method to calculate from the Earth received time, ERT, recorded when the first bit of attached sync marker, ASM, is reached. Some spacecrafts latch spacecraft clock when just after sending ASM, therefore, the latch time is calculated with ERT and one-way light time, OWLT, as

UTCLatch=ERT-OWLT+ (32-7) [bits]bitrate [bps]

where bitrate is equivalent to 1/2 symbol rate. Because this equation does not include the circuit delay both of a spacecraft and the ground system, the following equation is used in the actual spacecraft operation:

UTCLatch=ERT-OWLT+Cbbitrate+Cf

where Cb is constant to correct bitrate dependent delay, and Cf is constant to correct fixed delay.

Figure 5-14: Latch timing.

#### Thinning of time calibration packets

All time calibration packets are recorded, but the packets are decimated (some are discarded)  to stabilize the increasing rate of spacecraft clock. Time calibration table is provided after decimation of time calibration packets.

#### Judgement of discontinuity

Such as when spacecraft is reset, discontinuity is determined by the following formula:

1-ΔTInΔTIstd>s106+2ϵΔt

Where s is maximum rate-change rate [ppm], delta TIstd is a standard rate, ΔTIn is a target rate, ϵ is the maximum error between TI-UTC pair. Δt is the interval between TI-UTC pairs. Roll over of TI is not judged as a discontinuity.

#### Time calibration table

Time calibration tables includes following main items:

- Spacecraft clock

- (Calibrated) UTC

- TI-UTC rate [second/tick]

- Flag of discontinuity

In addition, to calculate the values above, the source data is also recorded in the followings:

- Bitrate during operation

- Used ground antenna in operation

- Distance between spacecraft and ground antenna

- etc.

#### Time calibration library

The time calibration library is written in the C programming language and provides function to convert UTC to TI, or TI to UTC mutually. The spacecraft clock included in a secondary header in CCSDS packet is converted to UTC using this time calibration library. Both interpolation and extrapolation are supported and users can convert spacecraft clocks easily by this unified method

## Roscosmos clock correlation methods

GLONASS time is based on an atomic time scale similar to GPS. This time scale is UTC as maintained by Russia (UTC (SU)). Unlike GPS, the GLONASS time scale is not continuous and must be adjusted for periodic leap seconds. Leap seconds are applied to all UTC time references as specified by the International Earth Rotation and Reference System Service (IERS). Leap seconds are used to keep UTC close to mean solar time. Mean solar time, based on the spin of the Earth on its axis, is not uniform and its rate is gradually changing due to tidal friction and other factors such as motions of the Earth’s fluid core.

GLONASS time is maintained within 1 ms, and typically better than 1 μs, of UTC (SU) by the control segment with the remaining portion of the offset broadcast in the navigation message. As well, Moscow offsets GLONASS time from UTC (SU) by plus three hours.

The GLONASS satellites are equipped with clocks (time/frequency standards) whose daily instability is not worse than 5×10-13 and 10-13 for the GLONASS-M satellites. An accuracy of mutual synchronization of the satellite time scales is not worse than 20 ns (1 σ) for the GLONASS and 8 ns (1 σ) for the GLONASS-M satellites.

GLONASS time is generated based on GLONASS central synchronizer (CS) time. Daily instability of the central synchronizer hydrogen clocks is not worse than 2×10-15.

The time scales of the GLONASS satellites are periodically compared with the CS time scale. Corrections to each onboard time scale relative to GLONASS time and UTC (SU), are computed and uploaded to the satellites twice a day by control segment. The error of a system using the GLONASS UTC (SU) time scale should not exceed 1 μs.

The GLONASS time scale is periodically corrected to integer number of seconds simultaneously with UTC corrections that are performed according to the Bureau International des Poids et Mesures (BIPM) notification (leap second correction). Typically, these corrections (1s) are performed once a year (or 1.5 years) at midnight 00 h 00 min 00 s UTC from December 31 to January 1, 1st quarter (or from March 31 to April 1, 2nd quarter or from June 30 to July 1, 3rd quarter or from September 30 to October 1, 4th quarter) by all UTC users.

### GLONASS time generation

GLONASS Time is generated as a continuous time scale on the basis of the Main or Reserved Central Synchronizers (CS) time scale in accordance with the following equation:

|  | TGLt=ΔTMt+ΔTMphti+ΔTMfrti-ΔTCti =ΔTRt+ΔTRphtk+ΔTRfrtk+ΔTRfrtl-ΔTCti-ΔTM-R(t) | (8) |
| --- | --- | --- |

where the variables in (8) are described in Table 5-35-3-.

Table 5-3-: Equation (8) Variables

| Variable | Description |
| --- | --- |
| ΔTGL(t) | GLONASS Time offset relative to Reference Time |
| ΔTMt, ΔTRt | Main/Reserved CS time offset relative to Reference Time |
| ΔTRfrtk | Corrections for Main/Reserved CS frequency steering |
| ΔTMphti | Corrections for Main/Reserved CS phase steering |
| ΔTCti | Time correction value |
| ΔTM-R(t) | Offset between Main and Reserved CS time scales |

The Central Synchronizer that provides the best accuracy characteristics is used as Master CS, the other operates as secondary.

Central Synchronizers provide the following accuracy characteristics:

– relative frequency error Δf/f below 3×10-14;

– daily frequency instability below 2×10-15.

The backbone of CS is Frequency/Time Keeping Facility (FTKF) including four active Hydrogen Frequency Standards (HFS), a system for internal comparisons and a system for steering frequencies and phases of signals from HFS. HFS which provides the best accuracy characteristics on the results of internal comparisons becomes master standard, the others operate as secondary.

National Time Scale of Russia UTC(SU) generated by State Time/ Frequency Reference (STFR) is used as GLONASS Reference Time.

CS time scale is corrected simultaneously with the correction of coordinated time scale UTC and, as a result, there is no whole second time offset between GLONASS Time and UTC(SU). However, there is a three-hour constant offset between GLONASS Time and UTC due to GLONASS Monitoring and Control Segment operational principles.

### GLONASS time corrections

To convert from GLONASS Time to UTC(SU) SVs broadcast correction for GLONASS Time offset relative to UTC(SU) in navigation signals.

To generate data on GLONASS Time offset relative to UTC(SU) it is provided:

– the offset of CS time scale relative to STFR time scale is calculated and the results obtained are converted to the offset of GLONASS Time relative to UTC(SU);

– the values of the offsets obtained for specified observation time interval are processed together to estimate and predict the parameters of GLONASS Time drift for specified time interval;

– corrections for GLONASS Time offset relative to UTC(SU) are generated for the navigation frame to be uploaded.

The offset of CS time scale relative to STFR time scale is calculated on the base of their mutual comparisons by the signals from SVs of GLONASS and GPS in differential mode with using “all-in-view” method according to the following equation:

|  | ΔTSTFR-CSti=ΔTGL-CSti-ΔTGL-STFR(ti) | (9) |
| --- | --- | --- |

where

| ΔTSTFR-CSti | is the CS time scale offset relative to STFR time scale |
| --- | --- |
| ΔTGL-CSti | is the Main/Reserved CS time offset relative to Reference Time |
| ΔTGL-STFR(ti) | is the STFR time scale offset relative to GLONASS time. |

The parameters of GLONASS Time drift relative to UTC(SU) are estimated with using special algorithm for processing time scales offset based on linear model and LMS technique.

## CAST clock correlation methods

### Overview

Generally in CAST space missions, onboard time is set up by the spacecraft’s own clock source and corrected by ground or GNSS. Near-earth spacecraft can correct time by GNSS. For deep space exploration and other spacecraft whose time cannot be corrected by GNSS, the time difference is calculated and corrected by the data transmission from spacecraft to earth.

### China Space Station example

The China Space Station consists of three modules: core module, lab capsule I, and lab capsule II. The information system of each module works independently before docking. After the formation of the space station assembly, the information system of the core module performs the unified information management and control of the combination. The information system of the lab capsule I serves as a backup, which can also act as the unified management and control center of the combination.

Figure 5-15: Orbital configuration of the China Space Station.

When the three capsules of the space station are flying alone, the core computer periodically broadcasts the onboard time via the 1553B buses. The subsystem controller device connected to the buses updates its own time after receiving the onboard time broadcast, and triggers the broadcast to the lower layer buses. After the RT devices of the lower layer buses receive the onboard time, they update their own time as needed.

The time management of the space station in the assembly mode is performed by the kernel computer of the core module. After the docking of the three cabins is completed, the lab capsule I and the lab capsule II receive the onboard time broadcast by the core module through the docking 1553B buses. The core computers of the two lab capsules correct the onboard time of themselves based on this broadcast time.

In the assembly mode of the space station, the time synchronization of the subsystem devices of the three modules is performed by the core computers of the three cabins through the 1553B bus broadcast. Time synchronization means that the deviation of the time of all or part of the nodes in the bus network from the practical benchmark can be predicted and is less than the acceptable error range, or the node can receive the timing to synchronize the local time with the benchmark time.

#### Onboard clock for China Space Station mission

The core computer of the space station has three clock source signals:

1) 40 kHz high stability external clock signal, long-term stability is better than 10-8 (7 days);

2) 40 kHz internal clock;

3) Second pulse signal. The second pulse signal is transmitted through a standard RS-422 interface. It is a negative pulse with a width of 1 ms ± 10 us, and the falling edge of the pulse represents the starting of one second interval.

The onboard time format maintained by the core computer of the space station is as shown in Table 5-45-4-.

Table 5-4-: Time format maintained by the core computer of the space station.

| Field | Length |
| --- | --- |
| Second field | 4 bytes |
| Sub-second field | 2 bytes |

One sub-second equals 25 μs. The time zero of the relative time code is related to some key events. The length of time code supports to represent 4×109 seconds, approximately 127 years, which meets the 15-year onboard life requirement.

In general, the core computer counts the interrupts triggered by the second pulse provided by the GPS devices to generates the second field of the onboard time, and the sub-second field is obtained by the external 40 kHz high-stability clock. When the GPS second pulse is not received, the core computer uses the external 40-kHz clock signal to trigger the second interrupt and generates seconds and sub-second fields of the onboard time. When the external 40-kHz clock does not work, the internal clock is used to generate 40 kHz instead of the external 40 kHz. The above switching can be done automatically on orbit.

There are two types of onboard time maintained by the core computer of the space station: relative time and absolute time. The relative time is cleared at the start time of some key events (such as: rocket launch, rocket-station separation, etc.), while absolute time is only cleared at the rocket launch time.

#### Time difference calculation method

The formula for calculating the time difference between the onboard time and the ground time is as follows:

ΔT=Tground station time-TF-Ttelemetry time-Δt1-Δt2

where

- TF is the core module launch time judged by the ground station based on the core module’s telemetry state change.

- Ttelemetry time is the onboard relative time maintained by the core computer. When the core computer of the space station handles the interruption for the Unified S-Band (USB) channel telemetry frame output, it reads the relative time code and writes the time to a fixed position in the synchronous telemetry frame downstream of the USB channel. The ground station reads the 6-byte onboard time code from a fixed position in the synchronous telemetry frame downstream of the USB channel.

- Δt1 is the fixed delay of the station and the ground, including equipment delay from generation to modulation and transmission of the first edge in the first byte of the USB channel downlink telemetry, and the delay from reception to demodulation of the first edge in the first byte of the telemetry frame.

- Δt2 is the space transmission delay.

During the tracking of the ground USB station, the time difference between the station and the ground is calculated in real time. When the calculation result of the time difference between the station and the ground is greater than 100 ms at the ground USB station, time calibration command is generated according to the time difference, and then the time calibration is performed by the ground station.

#### Clock Synchronization method

      - The second field of onboard time is synchronized by GPS

The core computer of the space station uses the second pulse trigger provided by GPS to generate the second field of onboard time, which has good accuracy. When the GPS synchronization function works, it is generally not necessary to periodically adjust the time.

2) Centralized time calibration

The time correction value, that is, the earth-space time difference, is calculated by the ground station and transmit to the core computer. After receiving the correction command, the core computer immediately adds this correction value to the onboard time, and completes the time adjustment operation by one time, so that the onboard time is directly synchronized with the ground clock.

3) Uniform time calibration

Uniform time calibration refers to the periodical adjustment of the onboard time for a certain period of time. This operation must be carried out at the starting moment of one second, and its period is adjustable. During the ground calibration process, the core computer can receive the uniform calibration command from the ground, and performs the calibration operation periodically until it receives the uniform calibration stop command.

Through the above time calibration methods, time difference between the onboard time maintained by the core computer and the ground time can be 0~5 ms.

The time calibration of the subsystem is to receive the absolute onboard time and relative onboard time sent by the core computer through the 1553B bus, and update the subsystem computer’s own absolute onboard time and relative onboard time. Considering the influence of factors such as software processing and bus propagation delay, the time error of the multiplexer/demultiplexer (MDM) connected to the first-layer bus is between 0 and 13 ms.

The realtime (RT) devices receive the absolute onboard time and relative onboard time sent by the subsystem BC computer through the 1553B bus and update their own absolute onboard time and relative onboard time. Considering the influence of factors such as software processing, bus propagation delay of the first- and second-layer bus, the time error of RT devices connected to the second layer bus is between 0~26 ms.

In addition to the two-time calibration methods in the single aircraft mode, the lab capsule I, and lab capsule II in combined assembly mode, also have a method of time calibration through the broadcast of the docking 1553B bus from the core module. Due to the cumulative deviation of the transmission from the core module to this cabin, 13ms, has to be added to the time deviation of the subsystems in the lab capsule I, and lab capsule II, that is, their time accuracy is 0~26 ms.

### Lunar exploration example

Chang ‘e-5 is China’s first unmanned lunar probe to return samples from the lunar surface. It consists of four parts: an orbiter, a lander, an ascender and a re-lander. The mission needs a precise time system during such important processes as landing, rendezvous and docking to ensure the accuracy of system operation. The time system is supported by the spacecraft and ground systems together.

Figure 5-16: Chang ’e-5 mission.

#### Onboard clock for Chang’e-5 mission

Chang ‘e-5 is made up of four probes, and has a variety of working modes, such as single probe and multi-probe combination. Due to the limitation of system resources, the high-stability clock source is configured only on some of the probes. An oven-controlled crystal oscillator with stability better than 10-8 is used as the clock source. The onboard computer calculates and maintains the time by receiving the clock source signal for the cumulative count.

The time synchronization mechanism based on upper and lower subnets and peer subnets is adopted to realize the time distribution and synchronization among the probes, and the time accuracy is less than 5 ms.

In the upper and lower subnets, as the time maintenance device of the whole system, the bus controller in the upper-layer subnet obtains the clock correlation information from ground system and uses the high-stability clock for time keeping. The controller transfers the absolute time to the gateway. After the gateway obtains the time information and synchronizes its own time, the gateway transfers the absolute time to the lower-layer time users.

In peer subnets, the bus controller of subnet 1 is the time maintenance device of the whole system. The controller in subnet 1/2 actively transmits the absolute time to the gateway. The gateway obtains the time information of the two subnets and calculates the time difference. The controller in subnet 2 obtains the time difference calculated by the gateway to correct its own time and synchronize with the time of subnet 1.

The onboard clock adopts binary coding and forward timing mode, which is composed of coarse and fine time code segments. The coarse time code segment is 32bits, which is the second counting field. The fine time code is 16bits, which is the millisecond counting field.

#### Clock synchronization method

There are two one-way methods of time calibration designed by onboard computer, centralized time calibration and uniform time calibration. In centralized time calibration, the time difference between ground system and probe is calculated by the ground station, and transmitted upward to the probe. After receiving the command, the probe make a one-time correction to synchronize the onboard time with the ground system time. In uniform time calibration, the onboard time is periodically set up or set down a certain amount by the ground command. The period of calibration is defined in the command.

In the task, the centralized time calibration mode is used. The time code is inserted into the fixed position of the downlink telemetry frame by the onboard computer. After the downlink data is received by the ground station, the time code is extracted and the received time is recorded. When calculating the time difference, signal transmission delay, ground equipment processing delay and computer processing delay are deducted according to the time model. During missions, time correction is performed before important missions or periodically based on statistical time differences to synchronize onboard time with ground time.

# Time Synchronization

Clock correlation determines the offset between two clocks A and B:

TB=TA+ΔT.

The value of ΔT is known at the location of clock B, or can be known via a communications transmission. Clock B can be synchronized to clock A with the following simple operation:

TB←TB-ΔT.

This operation may be performed directly on the counter of clock B, or, auxiliary software or hardware can keep track of ΔT in such a way that whenever the time of clock B is requested, the software or hardware reads clock B and applies the offset before returning the value to the requestor.

# Applications

## Science activities

Science activities require varying degrees of timing accuracy for data collection. Timing accuracy specified by the mission performing a science activity informs how coherent data is received. This section includes a brief timing accuracy discussion concerning science activities and their respective timing scales.

Science activities are defined in this context as methods of observation to collect characterization data about an object of interest. The timing accuracy required for a specific science activity can vary depending on the structure of the data it collects. Data types that require multiple datums per unit of time require stricter timing accuracy. Chronological fidelity of individual datums is necessary to avoid mischaracterizations. Mission support requirements define chronological fidelity and the timing accuracy of a specific activity.

Science activity timing precision should be considered in addition to the timing accuracy. Experiments that do not consider this precision might experience variations in the timing of data measurement that could affect the result in a substantial way, and yet still fall within the accuracy that the science activity recommends. When developing and standardizing science activity timing requirements, both accuracy and precision should be considered.

The enumeration of the science activities and their respective timing accuracies in Table 7-17-1- uses information provided by the CCSDS Timing Survey. This survey includes experiment information volunteered by various mission representatives. This table is not recommended to be used as a standard, due to the small sample size that was used to create it. Also, precision is not listed within this table, as the value was not explicitly provided within the CCSDS Timing Survey. This table can be used as a potential format for a future standardization of Science Activity Timing Scales when a larger sample size of mission experiment timing requirement information is procured.

Table 7-1-: Science activity timing scales.

| Science Activity | Timing Scale | Associated Missions |
| --- | --- | --- |
| Spectroscopy A science activity that characterizes an object of interest by detecting the object’s absorption features due to specific chemical bonds or electronic transitions, with detailed analyses used to determine the abundance and physical state of the detected absorbing species [46]. The timing accuracy/resolution required for this activity falls in the millisecond (ms) range. | ms | Juno Europa Clipper Van Allen Probes |
| Interferometry Interferometry is an imaging technique in which waves are superimposed in a manner to cause interference. This interference can be used to construct 3-D topology data [47]. The timing accuracy/resolution required for this activity falls in the microsecond range or stricter. | ms – ps | SunRISE low radio frequency GRACE-FO laser |
| Magnetometry Magnetometry tools can measure the strength and direction of magnetic force fields. This technology can be used to measure the magnetic fields of celestial bodies [48]. The timing accuracy/resolution required for this activity falls in the second range. | S | Parker Solar Probe |

## Ranging / Doppler

There are various types of continuous wave radiometric ranging to include harmonic (fixed and swept tone) and non-harmonic (PRN (Pseudo Random Noise) and BINary Optimum Range-BINOR) ranging. In addition, there are pulse type ranging schemes for Radio Frequency (R/F) and laser tracking. Following is PRN ranging discussion.

For this discussion, one-way radiometric ranging is defined as the process of sending a PRN signal to a spacecraft and recording the event on the spacecraft for subsequent downlink in telemetry. The subsequent arrival time of the telemetry is noted on arrival at the G/S as a Ground Receipt Time (GRT). GRT and time of transmission of the PRN are forwarded to the Mission Control Center (MCC). A corollary capability is for a spacecraft to broadcast a PRN signal to the ground and measure the GRT directly before forwarding the measurement to an MCC.

These methods of ranging have several attractive features. First in its simplest form, little cognizance by onboard systems is required. Second, it is simple to operate and definitive in solution. Finally, it can be extended to two-way ranging through the use of a turnaround ranging system in the space borne communications hardware and return the PRN signal to a G/S. The use of these techniques can provide timing and ranging benefits from Low Earth Orbit (LEO) to deep space.

There are several liabilities as well. For long roundtrip times as required for deep space vehicles, it may be necessary to have more than one G/S support an activity. In addition, measurements embedded in telemetry may be obscured and delayed by encryption on the downlink, if required. This is further complicated since the ground architecture may not have decryption capability widely dispersed beyond the MCC. Finally, uplink of a PRN signal reduces the total power available for the carrier-tracking loop, which may only be important for locked loop operations, notably near Earth.

A number of products or measurements can be produced based on these and similar methods:

- Time of Arrival (TOA)

- Time Difference of Arrival (TDOA)

- PRN correlation

- Doppler shift, and Accumulated Delta Range (ADR)

Use of these products can approximate position and a subsequent clock correlation can be determined, assuming the requisite algorithms are utilized.

### Time transfer with optical ranging

Methods for optical time-of-flight or ranging have been developed and demonstrated (e.g.,  [65]). Among recent proposals, two-way optical ranging may be accomplished by sending an uplink data-bearing signal from Earth to a spacecraft, and returning a synchronized or unsynchronized signal from the spacecraft back to the Earth. In the synchronized case, a ground station measures the two-way time delay between transmission and reception of the optical signal. In the asynchronous case, the spacecraft measures the offset between the uplink and downlink transmissions and sends this information to Earth using the communications link. The ground then makes the two-way range computation as before, accounting for the offset.

In addition to accomplishing ranging, such signaling can also incorporate the state of the spacecraft clock. In this scenario, using the asynchronous ranging method, an additional field is present to capture the value of the spacecraft clock at the moment the uplink-downlink offset measurement is made. In this way, ranging and time-transfer can be accomplished within the same protocol.

## Spacecraft maneuvers

A spacecraft maneuver is the use of propulsion systems, atmospheric drag, and/or gravity assistance to advance a spacecraft to/from an interplanetary environment. This definition is inclusive of all spacecraft movement enabled by orbital mechanics unique to an interstellar environment. Some (not all) examples include:

- Space Launch – The first maneuver of spaceflight. The spacecraft will liftoff from its launchpad into interstellar space using some manner of propulsion. This is typically the step that will require the most resources to complete.

- Orbital Insertion – A maneuver that relies on changing a spacecraft’s momentum to achieve a stable orbit around a celestial body. This can be used to either enter a body’s orbit from space, or to transfer orbits within a body’s sphere of influence.

- Entry-Descent-Landing (EDL) – The atmospheric entry sequence for a spacecraft mission, given that it has EDL as an objective in its lifecycle. A significant amount of precision in its execution is necessary, as it involves safely slowing and landing a spacecraft that is entering the atmosphere at hypersonic speeds. Concerns for the heat generated with friction between the spacecraft and the atmosphere need to be considered as well.

- Aerobraking – A maneuver that utilizes a celestial body’s atmosphere to reduce the apoapsis of an elliptical orbit. The spacecraft flies through part of the atmosphere at its periapsis, and the resulting drag slows the spacecraft.

- Gravity Assist – A maneuver that utilizes the relative momentum and gravitational influence of a celestial body to redirect the path and increase the speed of a spacecraft, saving significant amounts of propulsion material.

Precise positioning and timing requirements are essential to successfully execute a spacecraft maneuver in context to the spacecraft’s mission, regardless of which spacecraft maneuvers the flight team decides to employ. Errors in either positioning or timing values will result in serious consequences for the mission, ranging from unnecessary consumption of delta-v budget to mission failure. This is specifically due to an error over time effect where a slightly mistimed burn in one state can result in a large position change from what was originally planned in a later state.

- Time management Mission Survey

introduction

A survey of current and future missions was conducted to gather operational information about the timing requirements of currently operating and planned missions. This information can be used to determine time architecture requirements based upon current mission experience and upon future mission requirements. Such a database thus serves as an important guide for constructing an appropriate time architecture. The mission timing survey detailed data tables summarize the data thoughtfully provided by the respective mission contacts. The limitations in the current database reflect the time and resources available for this study. Useful follow-up would entail obtaining information from more missions, talking to the responding missions to further understand their mission specifics, and working on data consistency across missions. That said, however, there is sufficient data from a reasonable mission spectrum to support this study effort.

The future time and frequency architecture is required to be an integral part of the space communication and navigation infrastructure to include networking. It must also retain the capability to support standalone dissemination systems. Specific time resolutions are:

- Coarse (1 s – 1 ms)

- Fine (1 ms – 1 s),

- Precise (1 s – 1 ns)

- Ultra (1 ns – 1 ps)

Mission survey respondents

Table 7-2-: Mission survey respondents.

| Mission | Web Site | Launch | Domain |  |
| --- | --- | --- | --- | --- |
| ACE | http://www.srl.caltech.edu/ACE/ASC/related_sites.html | 8/1997 |  | D3 |
| Aquarius | http://aquarius.nasa.gov/ | 2009 |  | D1 |
| BIOMASS | https://directory.eoportal.org/web/eoportal/satellite-missions/b/biomass | 8/2023 |  | D1 |
| Cassini | http://saturn.jpl.nasa.gov/ | 10/1997 |  | D3 |
| Chandra | http://chandra.harvard.edu | 7/1999 |  | D2 |
| CloudSat | http://cloudsat.atmos.colostate.edu/ | 4/2006 |  | D1 |
| Con. X | http://constellation.gsfc.nasa.gov/ | After 2015 |  | D3 |
| DART | https://dart.jhuapl.edu/ | 2021 |  | D3 |
| Dawn | https://solarsystem.nasa.gov/missions/dawn/overview/ | 9/2007 |  | D3 |
| Europa Clipper | https://europa.nasa.gov | 10/2024 |  | D |
| ExoMars 2022 | http://www.esa.int/Science_Exploration/Human_and_Robotic_Exploration/Exploration/ExoMars | 2022 |  | D3 |
| Gaia | http://gaia.esa.int/ | 12/2011 |  | D3 |
| GFO | http://gfo.bmpcoe.org/gfo | 2/1998 |  | D1 |
| GLAST | http://glast.gsfc.nasa.gov/ | 2007 Oct. |  | D1 |
| GPM | http://gpm.gsfc.nasa.gov | 2010 Dec. (Core) 2014 (Constellation) |  | D1 |
| Herschel Space Observatory | http://www.esa.int/science/herschel | 2008 Jul. |  | D3 |
| IMAGE | http://image.gsfc.nasa.gov/ | 2000 Mar |  | D2 |
| INTEGRAL | http://integral.esac.esa.int | 2002 Oct. |  | D2 |
| Insight | https://mars.nasa.gov/insight/ | 2018 May |  | D3 |
| JUICE | https://sci.esa.int/web/juice | 2022 |  | D3 |
| Juno | http://juno.wisc.edu/ | 2010 Aug. |  | D3 |
| LISA | http://lisa.nasa.gov | 2015 Dec. |  | D3 |
| LRO | http://lunar.gsfc.nasa.gov/ | 2008 Oct. |  | D2 |
| Lunar Flashlight |  | 2021 Nov. |  | D2 |
| Mars Phoenix | http://phoenix.lpl.arizona.edu/ | 2007 Aug. |  | D4 |
| Mars Rovers | http://marsrovers.jpl.nasa.gov | 2003 Jun. / Jul. |  | D4 |
| MESSENGER | http://messenger.jhuapl.edu/ | 2004 Aug. |  | D3 |
| MMS | http://stp.gsfc.nasa.gov/missions/mms/mms.htm | 2013 Jul. |  | D2 |
| Planck | http://www.esa.int/science/planck | 2008 Jul. |  | D3 |
| Pluto New Horizons | http://pluto.jhuapl.edu/ | 2006 Jan. |  | D3 |
| Rosetta | http://www.esa.int/SPECIALS/Rosetta/index.html | 2004 Feb. |  | D3 |
| RXTE | http://xte.gsfc.nasa.gov/docs/xte/xte_1st.html | 1995 Dec. |  | D1 |
| Sentinels Expansion Missions | https://www.esa.int/Applications/Observing_the_Earth/Copernicus/Copernicus_expansion_mission | 2020 Nov. |  | D1 |
| STEREO | http://stereo.jhuapl.edu/ | 2006 Oct. |  | D3 |
| SunRISE |  | 2024-2025 |  |  |
| Swift | http://swift.gsfc.nasa.gov | 2004 Nov. |  | D1 |
| Terra | http://terra.nasa.gov/ | 1999 Dec. |  | D1 |
| THEMIS | http://themis.ssl.berkeley.edu/index.shtml | 2007 Feb |  | D2 |
| Ulysses | http://ulysses-ops.jpl.esa.int/ http://ulysses.jpl.nasa.gov/ | 1990 Oct. |  | D3 |
| Voyager -1, -2 | http://voyager.jpl.nasa.gov | 1977 Aug. Voyager 2 1977 Sep. Voyager 1 |  | D3 |
| WISE | http://wise.ssl.berkeley.edu/ | 2009 Nov. |  | D1 |

Mission Time requirements

Table 7-3-: Mission time requirements.

| Mission | Time requirements |  |  |  |
| --- | --- | --- | --- | --- |
|  | Early Operations | Experiment/ Measurement | AD&SC/ Attitude Control | Command-ing |
| ACE |  | 100 ms |  |  |
| Aquarius | Responsibility of CONAE | N/A | 1 s | N/A |
| BIOMASS | ms | Housekeeping: few µs Science: about 60 ns | ms | ms |
| Cassini |  |  |  |  |
| Chandra |  | 100 µs; goal of 10 µs | 0.25 s | A few s |
| CloudSat | Coarse | Coarse | Coarse | Coarse |
| Con. X | 100 ms | 100 ms | Minutes | Minutes |
| DART | 20 s | N/A | 0 | 0 |
| Dawn | N/A | onboard time – 2 s; time reconstruction – 60 ms | onboard time – 2s | <1 min |
| Europa Clipper | 10 min. | 19.5 ms | 19.5 ms | S/C: 1 s Instruments: 6 s |
| ExoMars 2022 | ~1s | ~1s | ~250 ms | ~250ms |
| Gaia | ms | <60 ns | <100 ms | <1s |
| GRACE, GRACE-FO | 0 | 160 ps relative timing between the two spacecraft. | 0 |  |
| GLAST. | 120 km navigation accuracy | <10 μs | ~1 ms | 1s |
| GPM | 1 to 2.5 km | 1 ms |  |  |
| Herschel Space Observatory |  | 5 ms | 5 ms | 2s |
| IMAGE | +/- 50 km | 1 s |  |  |
| INTEGRAL |  | ~100 μs: | N/A | ~ 1s |
| Insight | 1s | <100μs | ~25μs | 0.1s |
| JUICE | 1ms | 1ms | 1ms | 1ms |
| Juno | 2-3 m | Command execution accuracy of <0.1s, reconstruction time accuracy <0.005s | 5 ms | <0.1s |
| LRO |  | 100ms |  | <100ms |
| Lunar Flashlight | N/A | Few s | N/A | Few s |
| Parker Solar Probe | 10 s | 1s 3-sigma | 5 s | 0 |
| Mars Phoenix | 1 s | 1 ms | 100 ms | 1 ms |
| Mars Rovers | N/A | sub-s to s or min. | N/A | Few s |
| MESSENGER |  | 1 ms | 0.1 to 100 s | 1 to 10 s |
| MMS. | 1 μs to 1 ms | 1 μs to 1 ms | 1 μs to 1 ms | Coarse |
| Planck |  | 5 ms | 5 ms | 2 s |
| Pluto New Horizons |  | 10 ms | 0.1 s to 100 s | 1 s to 10 s |
| Rosetta. |  | 3 ms | 3 ms | 1 s |
| RXTE |  | 1 ms | 0.25 s | 1 s |
| Sentinels Expansion missions (a.k.a. Sentinels HPCM) | Clock correlation: 1ms Time tag: 1 μs | <1 μs | 10 Hz | 10 Hz |
| STEREO |  | 0.5 s | 0.1 s to 100 s | 1 s to 10 s |
| SunRISE | Coarse | few ns (post processing), or 1 μs (onboard) | few s | Coarse |
| Swift |  | 1 ms onboard; 0.2 ms on ground |  |  |
| Terra | 50 km (Terra); 120 km (TDRS) | 100 ms |  |  |
| THEMIS | 0.4 deg. | <10 s | 8 ms | 1 s / 24 hrs |
| Ulysses | All DSN standard | 100 ms | DSN standard | DSN standard |
| Van Allen Probes | 0 | 50ms 3-sigma | 0 | 0 |
| Voyager 1, 2 |  |  |  | 48 s |
| WISE | 30km | 0.6s |  |  |

## Acronyms

INTRODUCTION

This annex lists key acronyms that are used throughout this Informational Report

to describe time management concepts.

ACRONYMS

ADR		Accumulated Delta Range

AMMOS	Advanced Multi-Mission Operations System

BAEB		Bundle Age Extension Block

BDCS	BeiDou Coordinate System

BDS	BeiDou Navigation Satellite System

BDT	BeiDou Satellite System Time

BINOR	BINary Optimum Range

BIPM	Bureau International de Poids et Mesures
(in English, the International Bureau of Weights and Measures)

BP		Bundle Protocol

C&DH		Command and Data Handling

C/A		Coarse Acquisition

CCD		Charged-Coupled Device

CCSDS	Consultative Committee on Space Data Systems

CCTF		Consultative Committee for Time and Frequency

CDS		Clock Distribution System

CGCS		China Geodetic Coordinate System

CNAV		Civilian NAVigation

CNSS		Compass Navigation Satellite System

CS		Central Synchronizer

CSNO		China Satellite Navigation Office

CSNPC	China Satellite Navigation Project Center

CUC		CCSDS Unsegmented Code format

DG		Data Group

DLR		Deutsches Zentrum fur Luft und Raumfahrt (German Aerospace Center)

DORIS	Doppler Orbitography and Radio-positioning Integrated by Satellite

DSAC		Deep Space Atomic Clock

DSN		Deep Space Network

DTN		Delay/Disruption Tolerant Networking

DU		Demonstration Unity

DUT		Delta to Universal Time (difference between ET and UTC timescales)

EAL		Echelle Atomique Libre (in English, Free Atomic Scale)

ECLK		Electra ClocK

EDL		Entry, Descent, and Landing

EDRS		European Data Relay Satellite System

ERT		Earth Received Time

ESTRACK	European Space TRACKing Network

ET		Ephemeris Time

GAS		Ground Augmentation System

GCS		Ground Control Segment

GEO		Geostationary or Geosynchronous Orbit

GLONASS	GLObal’naya Navigatsionnaya Sputnikovaya Sistema

GMS		Ground Mission Segment

GMT		Greenwich Mean Time

GNSS		Global Navigation Satellite System

GPS		Global Positioning System

GRT		Ground Received Time

GSFC		Goddard Space Flight Center

GMSC		Global Short Message Communication

GST		Galileo System Time

HGA		High Gain Antenna

IERS		International Earth Rotation and Reference Systems Service

IGSO		Inclined Geo-Synchronous Orbit

IOAG		Interagency Operations Advisory Group

IRNSS		Indian Regional Navigational Satellite System

IRTF RFC	Internet Research Task Force Request for Comment

ISS		International Space Station

INRiM		Italian National Institute of Metrology Research

ITU		International Telecommunication Union

JHU/APL	Johns Hopkins University Applied Physics Laboratory

JWST		James Webb Space Telescope

LCS		Launch and Control System

LCT		Laser Communications Terminal

LEO		Low Earth Orbit

LNE-SYRTE	Laboratoire national de métrologie et d’essais – Système de Références Temps-Espace

LNSS		Lunar Navigation Satellite System

LRA		Laser Retro-reflector Array

LuGRE	Lunar GNSS Receiver Experiment

MCC		Mission Control Center

MCFC		Master Channel Frame Counter

MD		Mission Domain

MDM		Multiplexer/demultiplexer

MEO		Medium Earth Orbit

MEOSAR	MEO Search and Rescue

MMTC	Multi-Mission Time Correlation

MNSS		Martian Navigation Satellite System

MOC		Mission Operations Center

MOT		Magneto-Optical Trap

MRO		Mars Reconnaissance Orbiter

MSFN		Manned Space Flight Network

MSL		Mars Science Laboratory

MTIF		Multi-Mission Space Avionics Platform Telemetry InterFace

NAIF		Navigation and Ancillary Information Facility

NIM		National Institute of Metrology of China

NIST		National Institute of Standards and Technology

NPL		National Physical Laboratory

NTP		Network Time Protocol

NTSC		National Time Service Center, Chinese Academy of Sciences

OBC		OnBoard Clock

OBDH		OnBoard Data Handling

OBT		OnBoard Time

OCS		Operational Control System

OCX		Next Generation Operational Control System

OMNI		Operating Missions as Nodes

OPM		Operations Message

OWLT		One-Way Light Time

PHM		Passive Hydrogen Masers

PITS		Proximity-1 Time Service

PN		Pseudo Noise

PNT		Positioning, Navigation, and Timing

PPP		Precise Point Positioning

PPS		Protected Positioning Service

PRC		Primary reference clock

PRN		Pseudo-Random Noise

PTB		Physikalisch-Technische Bundesanstalt

PUS		Packet Utilization Standard

PVT		Positioning, Velocity, Timing

QZSS		Quasi-Zenith Satellite System

RAFS		Rubidium Atomic Frequency Standard

RCTD		Return Channel Time Delay

RDD		Return Data Delay

RF		Radio Frequency

RNSS		Positioning, Navigation, and Timing

RSMC		Regional Short message communication

RT		Realtime

RT		Receive Time

SBAS		Satellite-Based Augmentation System

SCET		SpaceCraft Event Time

SCLK		Spacecraft CLocK

SDST		Small Deep Space Transponder

SI		International System of units

SP		Standard Precision

SPICE		Spacecraft Planet Instrument C-matrix Events

SPS		Standard Positioning Service

SV		Space Vehicle

TAI		International Atomic Time

TCB		Barycentric Coordinate Time

TDRSS	Tracking and Data Relay Satellite System

TDB		Barycentric Dynamical Time

TDOA		Time Difference of Arrival

TDT		Terrestrial Dynamical Time

TG		Time of Gaia

TI		Time Indicator

TLM		Telemetry

TOA		Time of Arrival

TT		Terrestrial Time

TTL		Time to Live

UAV		Unpiloted Aerial Vehicle

UHF		Ultra-High Frequency

USB		Unified S-Band

USCCS	User Spacecraft Clock Correlation System

USNO		United States Naval Observatory

USO		Ultra-Stable Oscillator

UTC		Coordinated Universal Time

VCDU		Virtual Channel Data Unit

VCFC		Virtual Channel Frame Counter

VCID		Virtual Channel Identification

WSC		White Sands Complex