D O C U M E N T

document title/titre du document

IOAG

SERVICE CATALOG #3

prepared by/préparé par		IOAG Mission Operations Systems Strategy Group (MOSSG)

issue/édition			1

revision/revision			1

date of issue/date d’édition	XX/XX/2016

status/état			Draft/Draft

Distribution/distribution		Internal IOAG

IOAG

IOAG Service Catalog Three.v1.1-

Draft doc

T A B L E	O F	C O N T E N T S

1	INTRODUCTION	3

1.1	Scope	3

1.2	Definitions	3

1.3	Services for Interoperability	4

1.4	Reference Documents	7

1.4.1	CCSDS STANDARDS	7

1.4.2	IOAG DOCUMENTS	7

1.5	Acronyms	8

2	SCOPE OF CATALOG #3	10

2.1	Overview	10

3	CATALOG #3 SERVICES	13

4	CATALOG #3 SERVICE GROUPS AND TYPES	17

4.1	Telemetry Services Group	17

4.1.1	REAL-TIME PROCESSED PARAMETER SERVICE	18

4.1.2	TELEMETRY PACKET DISTRIBUTION SERVICE	19

4.1.3	BITSTREAM DISTRIBUTION SERVICE	20

4.2	Command Services Group	21

4.2.1	REAL-TIME COMMAND SERVICE	22

4.2.2	DEFERRED-TIME COMMAND SERVICE	23

4.2.3	COMMAND HISTORY DISTRIBUTION SERVICE	24

4.3	Data Access Services Group	25

4.3.1	MISSION PARAMETERS DISTRIBUTION SERVICE	25

4.3.2	ARCHIVE RETRIEVAL SERVICE	27

4.3.3	TELEMETRY REPLAY SERVICE	28

4.3.4	PRODUCT DISTRIBUTION SERVICE	29

4.4	Planning and Scheduling Services Group	30

4.4.1	SCHEDULING SERVICE	30

4.4.2	ACTIVITY REQUEST SERVICE	31

4.5	Navigation Services Group	31

4.5.1	NAVIGATION DATA SERVICE	32

4.6	Support Services Group	34

4.6.1	ALERT AND BROADCAST MESSAGE SERVICE	34

4.6.2	DATA DEFINITIONS PROVISION SERVICE	35

4.6.3	TIME PROVISION SERVICE	36

4.7	Interoperability Framework	37

4.7.1	COMMON CORE SERVICES	37

4.7.2	IMPLEMENTATION TEMPLATE	38

5	CONCLUSION	39

# INTRODUCTION

## Scope

This document constitutes Interagency Operations Advisory Group (IOAG) Service Catalog #3, complementing IOAG Service Catalog #1 [IC1] and IOAG Service Catalog #2 [IC2]. It identifies application-level Mission Operations (MO) services that should be provided by the IOAG member agencies contributing to the global interoperability of mission operations control centers.

IOAG Service Catalog #1 services are limited to the provision of space communication and tracking capabilities for interaction between a spacecraft control center and a spacecraft directly reachable via a ground tracking asset. IOAG Service Catalog #2 defines services for in-space relay and networked cross-support scenarios. Catalog #3 addresses the services relevant to teams in operations control rooms and may be used in conjunction with or independently from Catalogs #1 and #2.

MO services are at the application level (International Organization for Standardization [ISO] stack), and directly concern operational teams on the ground and may extend to application-layer software/hardware systems onboard (and crew in manned systems). As such, the operational environment in which MO services are used is highly variable and reflects various degrees of interoperability between agencies’ systems and operations, depending on the needs of the different programs. Service Catalog #3 thus does not cover the full scope of MO services that could be defined in a single multi-agency program, let alone across multiple programs. Rather, it aims at defining a minimum common set of services, which could be used by most programs where interoperability is a key factor.

In this current version, Service Catalog #3 addresses only the MO services between ground segments of participating agencies. Space-to-ground and space-to-space services between agencies are not addressed. The definition of such services at the application level would require analysis by onboard systems architects and a better understanding of future programs such as solar system exploration missions (Moon, Mars and beyond).

IOAG Service Catalog #3 also provides guidance to the Consultative Committee for Space Data Systems (CCSDS) Mission Operations and Information Management Services (MOIMS) Area regarding service gaps and priorities to be addressed.

## Definitions

Note that the definitions of certain terms such as “service” are slightly different than those found in Service Catalog #1 and Service Catalog # 2. The primary reason for this difference is because MO services deal with the application layer and direct users of these services may involve not only other systems, but people on operations teams.

Service

A service is a provision of an exposed system interface to support actions of another system or mission operations team. A service is described by the set of operations that can be invoked and performed through the service interface. Service specifications define the external interfaces and behavior of a system, but do not define the implementation.

Service Interface

An interface is a set of interactions provided by a system for participation with another system or mission operations team for some purpose, along with constraints on how those interactions can occur. A service interface is an external interface where the behavior of an object is exposed.

Service Provider

A system that offers a service to another system or mission operations team by means of one of its interfaces is called a service provider (provider).

Service User or Consumer

A system or mission operations team that uses a service provided by another system is called a service user (user or consumer). At the application level, any given system or an actual person may be a provider of some services and a consumer of others.

## Services for Interoperability

The services described in this catalog are a set of standard service types that support inter-Agency mission operations interoperability. Individual services may apply to some missions and not others and some missions may select alternative approaches to meeting their data exchange requirements. The individual service types as defined are distinguished from one another by the functions provided, level of processing involved, and/or the type(s) of source data. A service provision interface might be exposed to the user entity in the form of an Application Programming Interface (API).

Figure 1-1 describes an example “service provider–service consumer” relationship in the service paradigm. In this ABA configuration the service interface is exposed on the ground between the service user and the service provider. The ABA configuration is typical of a payload of Agency A, using a platform of Agency B. In that case, the control center of the payload (of Agency A) is connected to the control center of Agency B (which usually owns the space-to-ground link).

Figure 1-1 ABA Configuration

Different programs use varying topologies of control/user centers belonging to the various agencies involved. Typically, most large programs will be organized around a central entity (e.g., Mars Exploration/National Aeronautics and Space Administration [NASA] Jet Propulsion Laboratory), like the ABCD configuration depicted in Figure 1-2, with multiple control or user centers connected to the central entity. However, programs with a more meshed topology also exist.

The notion of service embraces the complete task in a systems sense. It typically involves end-to-end support using a combination of software components, computing and communications hardware, personnel and the procedures they follow, as well as facilities. Furthermore, the service is also the "whole job" in the lifecycle sense. A service accepts one or more requests and returns one or more responses through a well-defined, standard service interface. Services are specified from the user's point of view, i.e., in terms of the behavior exposed at the service interface offered by the service provider. Therefore, a service is solely specified in terms of its behavior, functionality, and performance without prescribing a particular implementation.

Figure 1-2 ABCD Configuration

At the application level and between ground segment entities, the services have a number of characteristics:

- They are varied (telemetry, command, planning, navigation, product exchange, etc.) and historically many implementations exist for similar exchanges.

- They have evolved with software technology and hence, most of the time, services used have different legacies even within an individual program.

- Security is a key consideration.

- The needs of control and user centers are similar.

Some of the services invoked between control centers are best covered with information technology standards where standardization by the CCSDS is not warranted. For example, this is the case for shared information portals covering operations products or the ability to access a command and telemetry system from a remote location using RDT (remote desktop).

## Reference Documents

CCSDS documents are available at http://www.ccsds.org.

### CCSDS STANDARDS

| [ADM] | CCSDS 504.0-B-1 Attitude Data Messages. Blue Book. |
| --- | --- |
| [ASDLP] | CCSDS 732.0-B-3 AOS Space Data Link Protocol. Blue Book. |
| [CDM] | CCSDS 508.0-B-1 Conjunction Data Message. Blue Book. |
| [COM] | CCSDS 521.1-B-1 Mission Operations Common Object Model. Blue Book. |
| [DPDS] | CCSDS 522.2 Mission Operations - Mission Data Product Distribution Services—Blue Book. TBW |
| [MAL] | CCSDS 521.0-B-2 Mission Operations Message Abstraction Layer. Blue Book. |
| [MCS] | CCSDS 522.1-R-3 Mission Operations Monitor & Control Services—Magenta Book. (in review) |
| [ODM] | CCSDS 502.0-B-2 Orbit Data Messages. Blue Book. |
| [SPP] | CCSDS 133.0-B-1 Space Packet Protocol. Blue Book. |
| [TCF] | CCSDS 301.0-B-4 Time Code Formats. Blue Book. |
| [TDM] | CCSDS 503.0-B-1 Tracking Data Message. Blue Book. |
| [XNDM] | CCSDS 505.0-B-1 XML Specification for Navigation Data Messages. Blue Book. |
| [XTCE] | CCSDS 660.0-B-1 XML Telemetric and Command Exchange. Blue Book. |

### IOAG DOCUMENTS

| [IC1] | IOAG Service Catalog #1, Issue 1.4, 04 March 2010, Approved 18 June 2013. https://www.ioag.org/Public%20Documents/IOAG%20Service%20Catalog%20One.v1.4-Approved.pdf |
| --- | --- |
| [IC2] | IOAG Service Catalog #2, Issue 1.1, 18 June 2013, Approved 18 June 2013. https://www.ioag.org/Public%20Documents/IOAG%20Service%20Catalog%20Two.v1.1-Approved.pdf |

## Acronyms

| acks | Acknowledgments |
| --- | --- |
| AOS | Advanced Orbiting Systems |
| API | Application Programming Interface |
| APID | Application Process Identifier |
| CADU | Channel Access Data Unit |
| CCSDS | Consultative Committee for Space Data Systems |
| DaSS | Data Service Subsystem |
| EDSS | EGOS Data Distribution System |
| EGOS | ESOC Ground Operations Software |
| ESOC | European Space Operations Centre |
| FTP | File Transfer Protocol |
| ID | Identifier |
| IOAG | Interagency Operations Advisory Group |
| ISO | International Organization for Standardization |
| ISP-X | Information Sharing Protocol-External Interface |
| M&C | Monitor and Control |
| MAL | Message Abstraction Layer |
| MCS | Monitor & Control Services |
| MO | Mission Operations |
| MOIMS | Mission Operations and Information Management Services |
| MOSSG | Mission Operations Systems Strategy Group |
| NASA | National Aeronautics and Space Administration |
| NTP | Network Time Protocol |
| RDT | Remote desktop |
| SM&C | Spacecraft Monitor & Control |
| TBC | To be confirmed |
| TBW | To Be Written |
| TC | TeleCommand |
| TM | Telemetry |
| TM/TC | Telemetry/TeleCommand |
| VCID | Virtual channel identifier |
| VPN | Virtual private network |
| XML | Extensible Markup Language (XML) |
| XTCE | XML Telemetric and Command Exchange |

# SCOPE OF CATALOG #3

## Overview

Figure 2-1 shows the context of Catalog #3 services compared to the services described in Catalogs #1 and #2. The following should be noted:

- Catalog #3 services are not necessarily dependent on Catalog #1 and #2 services. Indeed, interactions at the application level can use other network standards, in particular between ground segment entities.

- Some of the Catalog #3 services should be compatible with Catalog #1 and #2 services (in particular Telemetry/Telecommand [TM/TC] related services).

- Interoperability at the application level may rely on more than Catalog #3 services.

- Unlike Catalog #2, which responds to a common vision of space network resources to be shared between agencies, Catalog #3 is driven by shorter-term goals and aims at defining a minimum set of services between agencies in support of interoperability.

Figure 2-1 IOAG Service Catalogs

Figure 2 2 shows the MO services in the context of Mission Operations Interoperability. The Interoperability Framework identifies the underlying exchange capabilities and services, including the very essential feature that agencies must be networked to promote interoperability.

Interoperability at the application level does not rely only on the definition of mission operations services. In fact, Figure 2-2 identifies Supporting Capabilities, in addition to Service Catalog #1 and Service Catalog #2.

Figure 2-2 Mission Operations Interoperability

Supporting Capabilities (see Table 2-1) identify a number of techniques or means essential to interoperability from an operations point of view. These Supporting Capabilities are the current practice today, but are not planned for standardization by the CCSDS MOIMS Group.

Table 2-1. Supporting Capabilities

| Supporting Capabilities | Likelihood of Use | Comments |
| --- | --- | --- |
| Remote Access | Medium | For single-user access of Agency A to Agency B facility, based on commercial standards Many commercial services available |
| Shared Data Portal | High | For flight rules, procedures, and other operations products. Not in scope of CCSDS Many commercial services available |
| Voice Loops | High | Many commercial services available |
| Video Channels | High | Many commercial services available |

In addition to the MO services covered in Service Catalog #3, a mission may take advantage of space-to-ground and mission-specific services.

Other additional services that could be the subject of future analysis include:

- Inventory management

- Medical data services

- Space situational awareness services

- Goal-oriented scheduling

- Data archive ingestion services

- Telerobotics services

# CATALOG #3 SERVICES

Figure 2-2 depicts the following IOAG service groups that are relevant to IOAG Service Catalog #3. Each service group includes several service types:

- The Telemetry Services group, which provides telemetry data of various kind to users

- The Command Services group, which allows commanding though a service provider

- The Data Access Services group, which allows access to data stored by a service provider

- The Planning and Scheduling Services group

- The Navigation Services group, which provides navigation requests and data mechanisms

- The Support Services group, which includes generic services such as Alerts and Data Definition services

Service Catalog #3 also defines an Interoperability Framework, as opposed to the Service Management Framework defined in Service Catalog #2. Although the Interoperability Framework does not provide application-level services (also called functional services) it is nevertheless essential to the proper functioning of those services. The Interoperability Framework is composed of Common Core Services such as login, directory, configuration, interaction services, and of Implementation Templates, which address the network layer, the transport layer, and language bindings (language implementations).

Catalog #3 MO services groups are largely independent of one another and are provided mostly on a peer-to-peer basis. They do not require the overall, global management of a common resource like the Solar System Internetwork in Service Catalog #2. Thus, a global Service Management Framework is not required a priori. Service management is required for MO services, however, and is supported via the Interoperability Framework.

The services defined in Catalog #3 are summarized in Table 3-1 below.

Table 3-1 Services Defined in Service Catalog #3

| Service Identifier (ID) | Service Name | Likelihood of Use | IOAG Priority |
| --- | --- | --- | --- |
| Telemetry-1 | Real-time Processed Parameter Service | High | 3 |
| Telemetry-2 | Telemetry Packet Distribution Service | Medium | 1 |
| Telemetry-3 | Bitstream Distribution Service | Medium | 3 |
| Command-1 | Real-time Command Service | High | 2 |
| Command-2 | Deferred-time Command Service | High | 1 |
| Command-3 | Command History Distribution Service | Medium | 2 |
| Data Access-1 | Mission Parameter Distribution Service | High | 2 |
| Data Access-2 | Archive Retrieval Service | High | 1 |
| Data Access-3 | Telemetry Replay Service | Medium | 2 |
| Data Access-4 | Product Distribution Service (including science data) | Medium | 2 |
| Planning-1 | Scheduling Service | High | 1 |
| Planning-2 | Activity Request Service | High | 1 |
| Navigation-1 | Navigation Data Service (flight dynamics) | High | 1 |
| Support-1 | Alert and Broadcast Message Service | High | 2 |
| Support-2 | Data Definitions Provision Service | High | 1 |
| Support-3 | Time Provision Service | High | 1 |

Each service is further characterized by a likelihood of use and a development priority.

- Likelihood of use characterizes the potential for use in multi-agency programs, given the experience of past and current programs where interoperability is a factor. Likelihood of use is ranked from High (high likelihood that the service will be used) to Low.

- IOAG Priority is related to whether that service would benefit from standardization by the CCSDS. Priorities are defined in the following way:

  - Priority 1

    - Standards or practices for core services that are currently in CCSDS development.

    - Standards or practices for core services that will lead to significant operational and/or financial benefits.

    - Standards for capabilities or services that will be committed to flight operations or tracking networks within four years with respect to the date of the IOAG meeting having agreed the corresponding priority levels.

  - Priority 2

    - Standards or practices for core services that are essential for (or enablers for) Priority 1 standards, practices, or core services.

    - Standards for capabilities or services that will be committed to flight operations or tracking networks within seven years.

  - Priority 3: All others, including standards already developed.

Each service is described using a template (see Table 3-2 below).

Table 3-2 Service Description Template

| Service ID: Group | Service Name: Group Names | Comments |
| --- | --- | --- |
| Service Description | A textual description of the service | Proposed data format or data format to be developed |
| Service Operations | Includes the tasks that can be requested by the user or the provider | E.g., - Service request - Service request status - Service request modification |
| Optional Subservices | Details the processing or added value that is provided | E.g., command routing is a subservice of a command service |
| Common Core Services | Defines the management sub- services required to enable proper functioning of the services, including the service access | E.g., - Directory service - Login service - Configuration service - Interaction service - Replay control service - Retrieval service The management services are described in paragraph 4.7.1 below as Common Core Services |
| Implementation Required by Service Consumer | Defines the minimum implementation expected from a service consumer |  |
| Desirable Service Characteristics | Defines the set of service characteristics that would be desired from the proposed service |  |
| Likelihood of Use | Reflects the likelihood of use between agencies | High, Medium, Low |
| IOAG Priority | Ranked priority of CCSDS effort towards standardization | 1, 2, or 3 |
| Standards | Defines the applicable standard or states that the standard needs to be developed |  |

# CATALOG #3 SERVICE GROUPS AND TYPES

The following sections describe in detail the following six service groups included in Catalog #3:

- Telemetry Services Group

- Command Services Group

- Data Access Services Group

- Planning and Scheduling Services Group

- Navigation Services Group

- Support Services Group

The last section in this chapter describes the Interoperability Framework.

## Telemetry Services Group

The Telemetry Services Group includes the services related to the provision in real time of telemetry data, either in the form of raw packets or in terms of calibrated and processed parameters (usually extracted from raw packets and processed).

### REAL-TIME PROCESSED PARAMETER SERVICE

| Telemetry-1 | Real-time Processed Parameter Service | Comments |
| --- | --- | --- |
| Service Description | Parameters of relevance to Agency X Telemetry (TM) system are provided in real time by Agency Y TM system | Data Format standard to be developed |
| Service Operations | - Start and stop operations - Add/suppress/update operations on requests based on parameter IDs, aggregate IDs, time - Parameter rate setting |  |
| Optional Subservices | - Statistics service on data - Parameters monitoring status delivery (out of limit, unavailability, alarm status…) |  |
| Common Core Services | - Login service (ensure authenticated session) - Directory service: automated reconnection in case of connectivity loss - Configuration service: for support to parameter ID or aggregate ID selection, to get monitoring, out of limit, and unavailability definitions |  |
| Implementation Required by Service Consumer | - Service API of Agency Y to be implemented - Provision of parameter dictionary by Agency Y |  |
| Desirable Service Characteristics | - Scalability in case of very large parameter requests - Exchange mode (complete and in sequence or expedited) |  |
| Likelihood of use | High |  |
| IOAG Priority | 3 |  |
| Standards | Monitor and Control (M&C) parameter service of CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] |  |

### TELEMETRY PACKET DISTRIBUTION SERVICE

| Telemetry-2 | Telemetry Packet Distribution Service | Comments |
| --- | --- | --- |
| Service Description | Reception of CCSDS raw telemetry packet from vehicle of Agency X by Agency Y, routing and transmission to Agency X | Telemetry packet de-commutation by one agency on behalf of another is not considered a realistic option. Data format specified in CCSDS 133.0-B-1 Space Packet Protocol [SPP] |
| Service Operations | - Start and stop operations, optionally deferred delivery - Add/suppress/update operations on requests based on Application Process Identifier (APID) and/or packet ID number - Setting of packet rate (distribute every nth packet) | Distribution can be done on a regular and automated basis or in response to a single request. Setting of packet rate is only done in case of oversampling. |
| Optional Subservices | - Packet checksum check (for validating packet) - Telemetry packet reception and transmission time tag - Storage (archiving) of telemetry packets handled | Time tags might be particularly relevant if telemetry packet was stored somewhere before reception by Agency Y. |
| Common Core Services | - Login service (ensure authenticated session) - Directory service: automated reconnection in case of connectivity loss - Configuration service: for support to APID, packet ID selection |  |
| Implementation Required by Service Consumer | Service API from Agency Y to be implemented in telemetry system of Agency X |  |
| Desirable Service Characteristics | Accountability of packet transmission, delay in routing/transmission |  |
| Likelihood of use | Medium |  |
| IOAG Priority | 1 |  |
| Standards | MOIMS Spacecraft Monitor & Control (SM&C) Working Group does not cover raw packet routing | It would not be difficult to adapt the existing standard (i.e., CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS]) to include this option. |

### BITSTREAM DISTRIBUTION SERVICE

| Telemetry-3 | Bitstream Distribution Service | Comments |
| --- | --- | --- |
| Service Description | Bitstream (e.g., Channel Access Data Unit [CADU]s) produced by an Agency X asset is received, routed, and transmitted by Agency Y to an Agency X ground facility | This service can be used for science, voice, video, telemetry data, etc. Data Format is specified in CCSDS 732.0-B-3 AOS Space Data Link Protocol [ASDLP] |
| Service Operations | - Start and stop operations - Add/suppress/update operations on requests based on virtual channel identifier (VCID) or other bitstream ID and time | Distribution can be done on a regular and automated basis or in response to a single request |
| Optional Subservices | - Deferred delivery - Storage of bitstream of Agency X by Agency Y - Reed-Solomon decoding - Accountability of bitstream reception and delivery | Replay functions are addressed under support services (see section 4.3.3) |
| Common Core Services | Login | Machine-to-machine digital certificates for authentication |
| Implementation Required by Service Consumer | - Agency Y has to perform front-end processing for bitstream extraction and VCID identification. - Service API of Agency Y to be implemented by Agency X |  |
| Desirable Service Characteristics | Scalability in case of very high rate (e.g., 600 Mbps) |  |
| Likelihood of Use | Medium |  |
| IOAG Priority | 3 |  |
| Standards | No available service standard yet |  |

## Command Services Group

The Command Services Group includes real-time services related to commanding and also a command history service (non-real-time). Although command history could be considered part of an archive service, command history provision is seen as an integral part of a commanding system, not as a function of an archive service.

### REAL-TIME COMMAND SERVICE

| Command-1 | Real-time Command Service | Comments |
| --- | --- | --- |
| Service Description | CCSDS command packet or request sent from Agency X, received by Agency Y, routed and transmitted to Agency Y’s asset | Typically for PAYLOAD of Agency X on platform of Agency Y. |
| Service Operations | - Start and stop operations - Request for sending command (raw packet or command mnemonic plus parameters) | Data Format: raw packet = CCSDS 133.0-B-1 Space Packet Protocol [SPP] mnemonic+ parameters: format to be developed |
| Optional Subservices | - Packet checksum checked by Agency Y (for validating packet) - Command identifier used by Agency X, checked by Agency Y - Time authentication of command packet - TeleCommand (TC) response/acknowledgments (acks) handling - Storage (archiving) of TC packets handled - Prioritization of commanding |  |
| Common Core Services | - Login service (ensure authenticated session of command sender) - Directory service: automated reconnection in case of connectivity loss - Interaction service: when needed, interaction service can be used to put an operator in the commanding loop - Configuration service: for support to command packet command identifier selection or checking | Digital certificate/ virtual private network (VPN) Implementation of interaction service is largely dependent on context (messages, acks, alerts, request for confirmation, etc.) |
| Implementation Required by Service Consumer | Service API from Agency Y to be implemented in TC system of Agency X | Command identifier of Agency X should be known by Agency Y |
| Desirable Service Characteristics | Accountability of packet rejection, delay in routing/transmission |  |
| Likelihood of Use | High |  |
| IOAG Priority | 2 |  |
| Standards | M&C action service of CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] Raw packet routing/transmission is not proposed in the standard |  |

### DEFERRED-TIME COMMAND SERVICE

One key assumption behind the deferred-time command service is that the final command packet is generated by an agency uplinking the command packet (at least, time insertion and checksum calculation). There are, however, many possible ways in which an Agency X can give command parameters and command information to the uplinking agency (from a mnemonic and parameter values, to command packet instances nearly fully instantiated).

| Command-2 | Deferred-time Command Service | Comments |
| --- | --- | --- |
| Service Description | CCSDS command packet sent from Agency X to Agency Y, to be uplinked by Agency Y at future time | Typically for payload of Agency X on platform of Agency Y |
| Service Operations | - Request for sending command based on command identifier, including “not before” and “before” uploading dates for each command - Update request for sending command | Data Format of request content (command identifier and parameters) is to be developed |
| Optional Subservices | - Packet checksum checked by Agency Y (for validating packet) - Command identifier used by Agency X checked by Agency Y - “Not before” and “before” uploading dates checked vs. booked passes - Time authentication of command packet - TC response/acks handling after TC uploading - Storage (archiving) of TC packets handled - Prioritization of commanding | Command queuing is a reasonable assumption in bi- or multi-lateral programs to manage commanding priorities. |
| Common Core Services | - The login service (ensure authenticated session of command sender) - The directory service: automated reconnection in case of connectivity loss - The configuration service: for support command identifier selection by agency X or checking by agency Y | Digital certificate/VPN |
| Implementation Required by Service Consumer | Service API from Agency Y to be implemented in TC system of Agency X | Command identifier of Agency X should be known by Agency Y |
| Desirable Service Characteristics | Accountability of packet rejection, delay in routing/transmission, |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | M&C action service of CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] |  |

### COMMAND HISTORY DISTRIBUTION SERVICE

| Command-3 | Command History Distribution Service | Comments |
| --- | --- | --- |
| Service Description | Agency Y provides to Agency X the command history relevant to Agency X | E.g., if Agency X and Agency Y operate on the same asset or in the case that platform commanding (of Agency Y) is relevant to Agency X’s payload Format of command history data is to be developed |
| Service Operations | Start and stop operations Add/suppress/update operations on requests based on APID and/or time range | Distribution can be done on a regular and automated basis or in response to a single request. |
| Optional Subservices | - Command status provision - Deferred delivery |  |
| Common Core Services | - Login service (ensure authenticated session) - Directory service: automated reconnection in case of connectivity loss - Configuration service: for support to APID, packet ID selection |  |
| Implementation Required by Service Consumer | It is assumed that a programmatic interface exists to handle database requests. Thus Agency X has to implement the API of Agency Y to that programmatic interface. | Another implementation is a file exchange, based on offline requests (see support services in section 4.3.2). |
| Desirable Service Characteristics | The operations nomenclature (Ops Nom) is to be associated to each command instance in command history. |  |
| Likelihood of Use | Medium |  |
| IOAG Priority | 2 |  |
| Standards | Archive service of CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] |  |

## Data Access Services Group

The Data Access Services Group includes the services related to the provision of mission data, usually including processed parameters and data aggregates (requiring post processing to aggregate data in various ways) from an archive or a system specialized in data distribution, and even whole data products (e.g., a trend analysis, a set of observation data from an instrument sessions, etc.).

In real time, there is a potential overlap with Real-time Processed Parameter Services (see section 4.1.1) and Telemetry Packet Distribution Services (see section 4.1.2) but those services assume the data provider is a Monitoring and Control system, whereas here such an assumption is not made. In other words, from a service consumer point of view, Monitor and Control (M&C) systems are no longer the only source of mission parameters in real time, because control centers have elaborated the need for other parameters (aggregates, ground system parameters, mission parameters) than those traditionally provided by M&C systems. Also, in large multi-lateral programs where various control centers interact with each others’ M&C systems, centralizing data distribution proves more efficient overall (e.g., 1 to N distribution instead of M to N). Mission examples include Information Sharing Protocol-External Interface (ISP-X) (International Space Station, NASA Johnson Space Center), Data Service Subsystem (DaSS) (International Space Station, European Space Agency), and the European Space Operations Centre (ESOC) Ground Operations Software (EGOS) Data Distribution System (EDSS).

### MISSION PARAMETERS DISTRIBUTION SERVICE

| Data Access-1 | Mission Parameters Distribution Service | Comments |
| --- | --- | --- |
| Service Description | The service allows for distribution of available parameters (telemetry, ground system status, etc.) based on a catalog (i.e., an independent broker service) | Note this is easily extendable to raw packets (TM or TC). Data Format is to be developed |
| Service Operations | - Start/stop operations - Add/suppress/update operations in response to requests based on parameters or pre-defined request IDs | Distribution can be done on a regular and automated basis or in response to a single request. |
| Optional Subservices | - Application of parameter routing restrictions - Application of parameter rates - Parameters monitoring status delivery (out of limit, unavailability, alarm status…) - Deferred delivery | Parameter out-of-limit detection is not a function of a redistribution system. |
| Common Core Services | - Login service (ensure authenticated session) - Directory service: Automated reconnection in case of connectivity loss - Configuration service : for support to parameters selection or pre-defined requests, routing restrictions (e.g., number of requests allowed by user), rates, to get monitoring, out of limit and unavailability definitions | Digital certificate Data catalog access |
| Implementation Required by Service Consumer | - Data requester: Client API implementation, use of central parameter catalog - Data source: Service API implementation to handle requests and interfaces |  |
| Desirable Service Characteristics | Scalability of system to handle very large requests and increase in number of parallel requests |  |
| Likelihood of Use | High |  |
| IOAG Priority | 2 |  |
| Standards | If mission parameters are defined as common objects, the archive service of CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] can be used |  |
|  | Services from CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] can be used in a configuration where the data distribution is both a consumer (from M&C systems) and a provider (towards final end service consumers) |  |

### ARCHIVE RETRIEVAL SERVICE

This service concerns archive retrieval, not archive ingestion (which is being worked in CCSDS areas outside of the MOIMS Group). However, the assumption is that there are mechanisms for archive ingestion implemented. Archive ingestion is not considered part of the IOAG interoperability goals.

| Data Access-2 | Archive Retrieval Service | Comments |
| --- | --- | --- |
| Service Description | Archive of Agency Y is accessed by Agency X to get data | Data format of archive is to be developed |
| Service Operations | - Start and stop operations - Add/suppress/update operations on requests based on data type (packet ID, APID or parameter ID, orbital events, logs, …) time range, stream, file, … - Automated archive gap detection | Access can be done on a regular and automated basis or in response to a single request. |
| Optional Subservices | - Archive content discovery - Deferred delivery - Automated archive gap completion - Archive accountability (knowing what should be in archive) |  |
| Common Core Services | - Login service (ensure authenticated session) - Directory service: automated reconnection in case of connectivity loss - Configuration service: for support to data selection |  |
| Implementation Required by Service Consumer | Agreement must be reached with service provider on what data is accessible. |  |
| Desirable Service Characteristics | Request accountability (logs) |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | If data are defined as common objects, the archive service of CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] can be used. |  |

### TELEMETRY REPLAY SERVICE

| Data Access-3 | Telemetry Replay Service | Comments |
| --- | --- | --- |
| Service Description | Archive of Agency Y is accessed by Agency X for telemetry data replay | For processed parameters, raw TM, bitstream. Data format of data replay assumed to be the same as the one available in real-time. |
| Service Operations | - Start and stop operations - Add/suppress/update operations on requests based on packet ID, APID or parameter ID, time range, stream, or file - Automated archive gap detection - Replay control operations (play, pause, resume, speed up/down, backward, forward …) | Note: the notion of archive replay here (i.e., generating a new data stream from an archive) and the notion of replay in SM&C (replay of a previously existing data session) are different. |
| Optional Subservices | - Archive content discovery - Deferred delivery - Automated archive gap completion - Archive accountability (knowing what should be in archive) |  |
| Common Core Services | - Login service (ensure authenticated session) - Configuration service: for support to data selection |  |
| Implementation Required by Service Consumer | Agreement must be reached with service provider on what is accessible. |  |
| Desirable Service Characteristics | Request accountability (logs) |  |
| Likelihood of Use | Medium |  |
| IOAG Priority | 2 |  |
| Standards | Replay service of CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] |  |

### PRODUCT DISTRIBUTION SERVICE

| Data Access-4 | Product Distribution Service (Including Science Data) | Comments |
| --- | --- | --- |
| Service Description | A product distribution service is when an agency provides a mechanism for other agencies to request replicated files | A product is defined as a pre-prepared set of information containing data, voice, video, etc. The product distribution service is data agnostic, which is useful when two agencies develop different applications on the same data sets. Product Data Format is not the subject of specification; however, the format of headers and metadata could be developed. |
| Service Operations | - Start and stop operations - Add/suppress/update operations on distribution requests based on list of folders to be replicated | The assumption is that the replication is fully automated. Replication can be done on a regular and automated basis or in response to a single request |
| Optional Subservices | - File checksum check for data integrity - Exchange of metadata |  |
| Common Core Services | - The directory service: automated reconnection in case of connectivity loss - The configuration service: for support to folders selection |  |
| Implementation Required by Service Consumer | File Transfer Protocol (FTP) server with automated scripts |  |
| Desirable Service Characteristics | Inclusion of metadata related to this generation and distribution |  |
| Likelihood of Use | Medium |  |
| IOAG Priority | 2 |  |
| Standards | The Mission Operations- Mission Data Product Distribution Services [DPDS], Blue Book (in development) |  |

## Planning and Scheduling Services Group

This services group includes the most basic services related to planning and scheduling, allowing multiple agencies to build shared plans and request information about planning.

### SCHEDULING SERVICE

| Planning-1 | Scheduling Service | Comments |
| --- | --- | --- |
| Service Description | A schedule service from Agency Y allows Agency X to contribute to the planning of Agency Y | Data Format is to be developed |
| Service Operations | - Get plan operation - Insert/Remove activity from Agency X in a plan operation - Plan view operation | “Get plan operation” can be done on a regular and automated basis or on single request |
| Optional Subservices | - Planning client from Agency Y to Agency X - Service portal | The notion of a planning service implies some form of machine-to-machine interaction. Similar services can be rendered through manual interaction (client or service portal). |
| Common Core Services | - Login service (ensure authenticated session of command sender) - Directory service: automated reconnection in case of connectivity loss - Interaction service : when needed, interaction service can be used to put an operator in the loop to send high level messages to the planner | Implementation of interaction service is largely dependent on context (messages, acks, alerts, request for confirmation, etc. |
| Implementation Required by Service Consumer | Service API implementation |  |
| Desirable Service Characteristics | Plan modifications accountability |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | Planning and Scheduling Services standards are in development within the Planning and Scheduling Working Group |  |

### ACTIVITY REQUEST SERVICE

| Planning-2 | Activity Request Service | Comments |
| --- | --- | --- |
| Service Description | An activity request service from Agency Y allows Agency X to contribute to the planning of Agency Y, by sending request to be take into account for the processing of the planning | The notion of a planning service implies some form of machine-to-machine interaction. Similar services can be rendered through manual interaction (client or service portal) |
| Service Operations | Add/remove/update activity request | Data Format of requests are to be developed |
| Optional Subservices |  |  |
| Common Core Services | - Login service (ensure authenticated session of command sender) - Directory service: automated reconnection in case of connectivity loss - Interaction service: when needed, interaction service can be used to put an operator in the loop to send high-level messages to the planner | Implementation of interaction service is largely dependent on context (messages, acks, alerts, request for confirmation, etc.) |
| Implementation Required by Service Consumer | Service API implementation (TBC) |  |
| Desirable Service Characteristics | Service accountability (what happened to requests) |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | Planning and Scheduling Services standards are in development within the Planning and Scheduling Working Group |  |

## Navigation Services Group

The assumption is that each agency maintains flight dynamics data on each space asset it is responsible for (e.g., ephemeris, reconstituted high precision orbit, attitude, predictions, maneuvers, collision predictions, swath data, etc.). The navigation data services group concerns the exchange of navigation data between agencies (state vectors, maneuvers, collision data, etc.). The services are restricted to the exchange of navigation data with certain characteristics (accuracy, timeliness, completeness, etc.).

### NAVIGATION DATA SERVICE

| Navigation-1 | Navigation Data Service (Flight Dynamics) | Comments |
| --- | --- | --- |
| Service Description | Agency X provides flight dynamics data (a set of files) to Agency Y on a regular and automated basis (depending on the required precision and update needs) | Data Formats: - CCSDS 502.0-B-2 Orbit Data Messages [ODM] - CCSDS 503.0-B-1 Tracking Data Message [TDM] - CCSDS 504.0-B-1Attitude Data Messages [ADM] - CCSDS 505.0-B-1 XML Specification for Navigation Data Messages [XNDM] - CCSDS 508.0-B-1 Conjunction Data Message [CDM] |
| Service Operations | - Start/stop operations - Update, upload (pull), download (push), - Add/suppress/update operations on requests based file to be exchanged - Set validity flag | Interaction with a dropbox or FTP server is assumed here. |
| Optional Subservices | - Accountability of delivered files - Detection of non-completeness of files - Status and validity of flight dynamics files provision - Level of accuracy of the data - Deferred delivery - Automated archive gap completion |  |
| Common Core Services | - Login service (ensure authenticated session of command sender) - Directory service: automated reconnection in case of connectivity loss - Configuration service: for support to file selection - Timing service | FTP servers or dropbox exchange |
| Implementation Required by Service Consumer | FTP server with automated scripts |  |
| Desirable Service Characteristics | Proposed standard minimum precision for each flight dynamics product type |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | No service standard needed – only file transfer (ftp) |  |

## Support Services Group

The support services group includes a number of functions useful in interoperability scenarios such as the broadcasting of alert messages (e.g., to warn of a contingency situation) and the exchange of data definitions (telemetry dictionaries).

### ALERT AND BROADCAST MESSAGE SERVICE

An ALERT is defined in the CCSDS Mission Operations Monitor & Control Services Magenta Book [MCS] as an asynchronous notification, such as a non-nominal event, of significance to mission operations. Alerts may be used to notify such events to operators, initiate an automatic response, or synchronize asynchronous functions. Alerts may have arguments.

| Support-1 | Alert and Broadcast Message Service | Comments |
| --- | --- | --- |
| Service Description | Agency X broadcasts to one or several agencies messages about events/modes/conditions with a priority level and a confidentiality level | Data Format is to be developed |
| Service Operations | - Send - Cancel - Acknowledge message reception - Set criticality - Set priority - Set confidentiality - Set recipients - Acknowledge message (user side) |  |
| Optional Subservices | Re-send and deferred delivery |  |
| Common Core Services | - Directory service: automated reconnection in case of connectivity loss |  |
| Implementation Required by Service Consumer | - Display of message on dedicated display or window - Agreement must be reached with service provider on what data is confidential and accessible. |  |
| Desirable Service Characteristics | Accountability regarding sending and reception of messages |  |
| Likelihood of Use | High |  |
| IOAG Priority | 2 |  |
| Standards | ALERT service in the CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] |  |

### DATA DEFINITIONS PROVISION SERVICE

| Support-2 | Data Definitions Provision Service | Comments |
| --- | --- | --- |
| Service Description | The provision of data definitions (e.g., data dictionaries). | E.g., based on CCSDS 660.0-B-1 XML Telemetric and Command Exchange [XTCE] The assumption is that data transfers in real time do not contain all of the metadata required to interpret the data. |
| Service Operations | - Get data definition operation based on a list of data definition to retrieve - Identify list of parameter names based on search criteria (e.g., metadata) |  |
| Optional Subservices | File checksum check for data integrity |  |
| Common Core Services | - Configuration service: for support to list of data definition selection |  |
| Implementation Required by Service Consumer | - FTP server with automated scripts - Folder to be accessed to get data definitions |  |
| Desirable Service Characteristics | Scalability (some missions may several hundred thousand parameter and command definitions) |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | No service standard required |  |

### TIME PROVISION SERVICE

| Support-3 | Time Provision Service | Comments |
| --- | --- | --- |
| Service Description | The service provides a sync signal and time references used by one agency to another agency. These include time references for in-orbit or deployed systems and for ground facilities. | Data Format for time code is described in CCSDS 301.0-B-4 Time Code Formats [TCF] |
| Service Operations | - Report Time - Set Time - Correlate Time |  |
| Optional Subservices |  |  |
| Common Core Services |  |  |
| Implementation Required by Service Consumer | Network Time Protocol (NTP) for sync signal |  |
| Desirable Service Characteristics |  |  |
| Likelihood of Use | High |  |
| IOAG Priority | 1 |  |
| Standards | Service standard may be considered |  |

## Interoperability Framework

The functional services described in sections 4.1 to 4.6 rely on an Interoperability Framework (see Figure 2-2). This Interoperability Framework includes a set of common core services and implementation templates.

### COMMON CORE SERVICES

| Interoperability Framework | Common Core Services | Comments |
| --- | --- | --- |
| Service Description | - Login: provides authentication mechanism and the attribution of roles and privileges to the functional service user (consumer) - Directory: publishes the address of the requested functional services and the capabilities of each functional service - Configuration: provides the information necessary to configure the functional service. - Interaction: allows user entry in the functional service |  |
| Service Operations | - Login: login, logout, handover, report roles - Directory: lookup, publish a new service, remove a service - Configuration: lookup configuration, activate configuration - Interaction: acknowledge, confirm, choice, value entry | These are the minimum set of operations |
| Desirable Service Characteristics | Logging/Accountability |  |
| Standards | See CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] |  |

### IMPLEMENTATION TEMPLATE

| Interoperability Framework | Implementation Template | Comments |
| --- | --- | --- |
| Templates | - Network: services rely on existing network connectivity (e.g., use of a VPN) and a template for such connectivity needs to be provided for each functional services. - Transport: the transport format between a provider and a consumer of a service provider needs to be defined as a template, - Bindings: the coding format of data to be exchanged between the service provider and the service consumer needs to be defined as a template for each functional service |  |
| Desirable Template Characteristics | - Network: - Transport: real-time or non-real-time characteristic of the service must drive the definition of the transport layer - Bindings: |  |
| Standards | See CCSDS 521.0-B-2 Mission Operations Message Abstraction Layer [MAL] |  |

# CONCLUSION

The services briefly described in Chapter 4 constitute the set of services at the application level that should be considered by the space agencies to promote interoperability amongst themselves. Table 5-1 below summarizes the services and priorities as well as the areas where standardization work may be required. The CCSDS MOIMS working groups are well on their way to establishing the needed service standards. Data format standardization would provide a needed complement to the Mission Operations service standards and would also improve the chance of these services standards being accepted and used.

Table 5-1. IOAG Priorities and Standards for Services in Service Catalog #3

| Service Name | IOAG Priority | Service Standard | Data Format Standard |
| --- | --- | --- | --- |
| Real-time Processed Parameter | 3 | CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] | Standard to be developed |
| Telemetry Packet Distribution | 1 | Adaptation of CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] Standard to be developed | CCSDS 133.0-B-1 Space Packet Protocol [SPP] |
| Bitstream Distribution | 3 | Standard to be developed | CCSDS 732.0-B-3 AOS Space Data Link Protocol [ASDLP] |
| Real-time Command | 2 | Adaptation of CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] Standard to be developed | Raw: CCSDS 133.0-B-1 Space Packet Protocol [SPP] Mnemonic: Standard to be developed |
| Deferred-time Command Service | 1 | CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] | Mnemonic: Standard to be developed |
| Command History Distribution | 2 | CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] | Standard to be developed |
| Mission Parameter Distribution | 2 | - CCSDS 521.1-B-1 Mission Operations Common Object Model [COM] - CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] | Standard to be developed |
| Archive Retrieval | 1 | Mission Operations Common Object Model [COM] | Standard to be developed |
| Telemetry Replay | 2 | Mission Operations Common Object Model [COM] | Data format of data replay assumed to be the same as the one available in real-time |
| Product Distribution Service (including science data) | 2 | Standard in development | Standard to be developed |
| Scheduling Service | 1 | Standard in development | Standard to be developed |
| Activity Request | 1 | Standard in development | Standard to be developed |
| Navigation Data | 1 | No standards required (dropbox/ftp) | - CCSDS 502.0-B-2 Orbit Data Messages [ODM] - CCSDS 503.0-B-1 Tracking Data Message [TDM] - CCSDS 504.0-B-1 Attitude Data Messages [ADM] - CCSDS 505.0-B-1 XML Specification for Navigation Data Messages [XNDM] - CCSDS 508.0-B-1 Conjunction Data Message [CDM] |
| Alert and Broadcast Message | 2 | ALERT service CCSDS 522.1-R-3 Mission Operations Monitor & Control Services [MCS] | Standard to be developed |
| Data Definition Provision | 1 | No service standard required | CCSDS 660.0-B-1 XML Telemetric and Command Exchange [XTCE] |
| Time Provision | 1 | Service standard may be considered | CCSDS 301.0-B-4 Time Code Formats [TCF] |