# CCSDS Reference Architecture: MOIMS Area Inputs

The CCSDS System Architecture Working Group (SAWG) is currently developing a reference architecture that addresses all aspects of CCSDS standardisation.  This builds upon the previous CCSDS Communications System Architecture, which showed how the communication layers of the SLS, SIS and CSS area standards related to each other, but did not address the application layer standardisation activities of the MOIMS and SOIS areas.

The CCSDS Reference Architecture will be a model that comprises 5 views:

- Functional:  identifying standard functions of a space mission system, and the interactions between them in terms of service interfaces and/or information exchanged at application level.

- Service:  characterisation of potential application level services in terms of the operations performed and information exchanged.

- Data: information model of the top level data objects exchanged between functions and the relationships between them.

- Protocol:  communications protocol stack view, showing how application level services can be overlaid on CCSDS communication layers.

- Deployment:  example deployment architectures identifying where key interfaces between functions are exposed to interoperability boundaries.

The work is scheduled in two phases: first to produce a set of diagrams and tables giving a “cartoon” overview of the model using [extended] CCSDS RASDS notation; second to produce a green or magenta book that presents the agreed model.

This document contains draft Functional and Service viewpoints covering the MOIMS area of standardisation.

# Functional Viewpoint

The Functional viewpoint has been represented as a set of Powerpoint diagrams using the RASDS notation (see graphical conventions below) .  As the view shows application level functions, the notation for logical links between elements has been extended to show:

- Which function is the Data or Service Provider (shown by a circle at one end of the link)

- The principal information or data items exchanged across the logical link (shown as labels in coloured boxes annotating the links).

All MOIMS functions and data are also colour coded [external functions/entities in grey], grouping functions into major functional areas as follows:

Mission Control [MCS]

Planning & Scheduling [MPS]

Navigation & Timing [NAVT]

Data Storage & Archiving

Operations Preparation [OPD]

Mission Data Products [MDP]

Common Services

A high level diagram shows these main areas and their interactions.  This is followed by lower level diagrams expanding each area.  These lower level diagrams are provided in two alternative presentations:

- The classic functional view

- A service/data oriented view using “tramlines” to show the interactions between functions that are the subject of standardisation.

## MOIMS Data and Services

CSS: Cross Support Services

CSS-M&C: CSS Monitoring & Control

CSS-SM: CSS Service Management

CSS-TS: CSS Transfer Services

## Common Services

SDIR: Service Directory

LAC: Login and Authentication Credentials

MO: Mission Operations [Services]

COM: Common Object Model

## Mission Control

M&C: Monitoring & Control

AUT: Automation

OSM: On-board Software Management

OPM: On-board Procedure Management

## Mission Control (Alternative View)

## Navigation and Timing

ADM	Attitude Data Message

CDM	Conjunction Data Message

EVM	Event Message

NHM	Network Hardware Message

ODM	Orbit Data Message

PRM	Pointing Request Message

SMM	Spacecraft Manoeuvre Message

TDM	Tracking Data Message

TIM	Time Report

TRM	Time Reception Message

TCM	Time Correlation Message

## Navigation and Timing (Alternative View)

## Mission Planning and Scheduling

PRQ: Planning Request

PLN: Plan [Distribution/Retrieval]

PLC: Planning Control

PEC: Plan Execution Control

## Mission Planning and Scheduling (Alternative View)

## Operations Preparation

SDB: Satellite Database

OSW: On-board Software

APD: Automated Procedure Definition

PDB: Planning Database [Planning Data Definitions]

XTCE: XML Telemetry and Command Exchange – TM/TC Database Exchange format (OMG/CCSDS standard)

## Operations Preparation (Alternative View)

CFG: Configuration (MO Common Service)

CFG-M: Configuration Management (Add/Remove/Activate)

CFG-D: Configuration Distribution (List/Get)

CFG-X: Configuration Exchange (Import/Export)

CAR: Common Archive (MO COM Service)

## Data Storage and Archiving

DAI: Data Archive and Ingestion working group

PAIS: Producer-Archive Interface Specification

CAIS: Consumer-Archive Interface Specification

XFDU: XML Formatted Data Unit

## Data Storage and Archiving (Alternative View)

CAR: Common Archive (MO COM Service)

FTM: File Transfer and Management (MO Service)

CAIS: Consumer Archive Interface Specification

PAIS: Producer Archive Interface Specification

# Service Viewpoint

The Service viewpoint is presented as a table that lists potential services together with the operations they provide, and the data objects exchanged or referenced by the service.  Services may be grouped together within a single standard.  The Functions column references functions identified in the functional viewpoint, either as a provider of the service [in bold] or as a consumer.  The Data column references information objects identified in the data viewpoint.  The Standards column identifies the CCSDS standards that identify or specify the service or associated key data formats.  Other columns are colour coded:

- Column A:  correlates to the functional areas identified within the Functional view:

Common Services

Mission Control

Planning & Scheduling

Navigation & Timing

Data Storage & Archiving

Operations Preparation

Mission Data Products

- Column S and D: show the current status of CCSDS specification with respect to standard CCSDS book colours for both the Service (S) and Data format (D). White indicates no specification has yet been produced; pastel green indicates a green book in production.

## COM and Common Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO COM | Archiving | Providers: Ops. Archive Consumers: <any function> | Store Update Query | <any COM obj> | Generic archive service for MO services defined in terms of the Common Object Model [COM]. | MO Common Object Model [CCSDS 521.1-B-1] |  |  |
|  |  | Activity Tracking | Providers: <any function> Consumers: <any function> | Publish/Subscribe | <any Activity> | Activities are COM objects that have a limited duration. The service provides a mechanism to report progress/status and uses the COM Event service. |  |  |  |
|  |  | Event | Providers: <any function> Consumers: <any function> | Publish/Subscribe | <any Event> | Events are COM objects that represent an occurrence at a point in time. Each service can define the Events it supports. |  |  |  |
|  | MO Common | Directory | Providers: Service Directory Consumers: <any function> | Publish Provider Withdraw Provider Lookup Provider Get Service XML | Service Descriptor | Allows Providers to publish information about the services they provide; and Consumers to query the Service Directory and retrieve Service XML descriptors. | MO Common Services [CCSDS 522.0-R-n] |  |  |
|  |  | Login | Providers: Login and Authentication Consumers: <any function> | Login Logout Report Available Roles Handover to other User | Authentication Credentials | Common login service for submission of authentication details to a deployment specific security system. Integrated with Access Control aspect of MO MAL. |  |  |  |
|  |  | Configuration | Providers: Configuration Management & Distribution Consumers: <any function> | Activate List Get Current Get XML Add Remove Store Current Store XML | Configurations XML Configurations <any Config Data> | Configurations can be hard-coded, use bespoke configuration data, or a standard COM service configuration. Service consumers can activate predefined configurations of a service provder; and list, get, add, remove and store current configurations. It also defines a standardised XML representation for configurations. |  |  |  |

## M&C Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO M&C | Action | Providers: Monitoring & Control Consumers: Automation On-board Configuration Management Navigation Interface Planning Plan Execution Operations Preparation Operations Archive User Support Mission Data Processing Spacecraft Development & Maintenance | Submit Action preCheck Action Manage Action Definitions | Action | Allows control directives (e.g. a spacecraft telecommand) to be invoked and their evolving status to be monitored. Uses COM Services for Action Tracking and Archiving. | MO Monitoring & Control Services [CCSDS 522.1-R-n] |  |  |
|  |  | Parameter |  | Monitor Value Get Value Set Value Enable Generation Manage Parameter Definitions | Parameter | Provides the capability to monitor and set parameter values. Uses COM Archiving service for Parameter Archiving. |  |  |  |
|  |  | Alert |  | Enable Generation Manage Alert Definitions | Alert | Provides a mechanism for asynchronous notification of operationally significant events or anomalies. Uses COM Event and Archiving services to publish/subscribe to Alerts and to archive them. |  |  |  |
|  |  | Check |  | Get Current Transition List Get Summary Report Enable Service Get Service Status Enable Check Trigger Check Manage Check Definitions | Parameter Check Check Link Check Transition Event | Provides on-line checking of Parameter values against defined checks [Limit, Constant, Delta] and notification of check violations. Uses COM Event service to publish/subscribe to check status transition events. |  |  |  |
|  |  | Statistics |  | Get Statistics Reset Evaluation Monitor Statistics Enable Service Get Service Status Enable Generation Add Parameter Evaluation Update Parameter Evaluation Remove Parameter Evaluation | Parameter Statistic Link Statistic Value | Provides on-line statistical evaluation of Parameter values. Uses COM Archive service. |  |  |  |
|  |  | Aggregation |  | Monitor Value Get Value Enable Generation Enable Filter Manage Aggregate Definitions | Aggregation of Parameters | Provides aggregation of separate Parameter values into coherent sets. Uses COM Archive service. |  |  |  |
|  |  | Conversion |  | none | Parameter Conversion | Provides conversion of raw Parameter values into engineering units. Uses COM Archive service |  |  |  |
|  |  | Group |  | none | Group of <any COM object>s | Provides the ability to define groupings of objects to simplify the operations of other services. Uses COM Archive service |  |  |  |
|  | MO AUT | Automation | Providers: Automation Consumers: Monitoring and Control Navigation Interface Plan Execution Operations Archive | Start Procedure Stop Procedure Suspend/Resume Procedure Manual Control Manage Procedure Definitions [TBD] | Procedure | Provides support for automation of mission operations. The service allows automated procedures or autonomous functions to be invoked, controlled, and their evolving status to be monitored. Uses COM Services for Procedure Tracking and Archiving. | MO Services Concept [CCSDS 520.0-G-3] Automation Service |  |  |
|  | MO OSM | Software Management | Providers: On-board Configuration Management Consumers: Automation Plan Execution | Load Software Image Dump Software Image Check Software Image | On-board Software Image | Supports the management of software loaded into the remote system [spacecraft]. | MO Services Concept [CCSDS 520.0-G-3] Software Management Service |  |  |
|  |  | Procedure Management | Providers: On-board Configuration Management Consumers: Automation Plan Execution | Load Procedure Definition Dump Procedure Definition Check Procedure Definition Manage Procedure Definitions [TBD] | On-board Procedure Definition | Supports the management of automated procedure definitions loaded into the remote system [spacecraft]. | Proposed extension to scope of MO OSM service. |  |  |

## Other MO Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO MDP | Data Product Distribution | Providers: Operations Archive On-board File Store Consumers: Mission Data Processing User Support Spacecraft Development & Maintenance | Subscribe Online Subscribe Batch Retrieve [TBD] | Mission Data Product | Mission Data Product Distribution Service is used for the distribution of historical archived data and on-line 'live' data It provides two delivery modes, batch mode and stream mode. | MO Services Concept [CCSDS 520.0-G-3] Data Product Management Service |  |  |
|  | MO FTM | File Transfer & Management | Providers: On-board File Store Consumers: Mission Control Planning & Scheduling Navigation and Timing | List, Rename, Move, Copy and Delete Files and Directories Add Directory Get Drive Information Upload File Download File | Directory File Drive | Supports the management of a remote [on-board] file store and the initiation of transfers of files between local and remote file stores. | MO Services Concept [CCSDS 520.0-G-3] Remote Buffer Management Service |  |  |

## Mission Planning and Scheduling Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO MPS | Planning Request [PRS] | Providers: Planning Plan Execution Consumers: User Support Operations Preparation Navigation & Timing Planning [Distributed] | Submit Request Update or cancel Requests Provide Request Status feedback Manage Request Definitions | Planning Request Plan Planning Activity Planning Event Planning Resource Planning Constraint | Asynchronous submission of planning requests, associated responses and their subsequent management and status feedback. A planning request may reference a Plan (output from an earlier planning process), in which case the provided feedback includes the status of the plan in terms of its contained activities and other items. | MO Services Concept [CCSDS 520.0-G-3] Planning Request Mission Planning and Scheduling Concept [Draft Green Book] |  |  |
|  |  | Plan Distribution & Retrieval [PLS] | Providers: Planning Plan Execution Consumers: User Support Navigation & Timing Mission Data Processing Planning [Distributed] | Retrieve Plan or Plan Status Subscribe to Plan or Plan Status | Plan Planning Activity Planning Event Planning Resource Planning Constraint | Provides distribution and access to plans generated by the planning function. | MO Services Concept [CCSDS 520.0-G-3] Scheduling [Part] Mission Planning and Scheduling Concept [Draft Green Book] |  |  |
|  |  | Planning Process Management [PMS] | Providers: Planning Consumers: Plan Execution Mission Control Navigation & Timing User Support Mission Data Processing Planning [Distributed] | Initiate, Monitor and Control Planning Processes Update Plan Status Manage Planning Definitions | Plan Planning Activity Planning Event Planning Resource Planning Constraint | Management of the planning process itself - initiation, status feedback and control. Also supports provision of plan status updates by a third party. | MO Services Concept [CCSDS 520.0-G-3] Scheduling [Part] Mission Planning and Scheduling Concept [Draft Green Book] |  |  |
|  |  | Plan Execution Management [PES] | Providers: Plan Execution Consumers: Planning Mission Control | Initiate, Monitor and Control Plan execution Edit Plan content Update Planning Events and Resources. Manage Planning Definitions | Plan Planning Activity Planning Event Planning Resource Planning Constraint | Control and management of the execution of a plan, including actions to Start/Stop and Pause/Resume execution. Update [editing] of the executing plan at activity level. Update planning events/resources. | MO Services Concept [CCSDS 520.0-G-3] Scheduling [Part] Mission Planning and Scheduling Concept [Draft Green Book] |  |  |

## Navigation and Timing Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TIM | Time | Providers: Time/Position Determination Consumers: Time Correlation | Report Time Set Time Configure rate of Time Report generation. | Time Report | Provides accurate reporting of on-board time. For an unsynchronised on-board clock this may require correlation with the system reference time | MO Services Concept [CCSDS 520.0-G-3] Time |  |  |
|  |  | Time Correlation | Providers: Time Correlation Consumers: Mission Control Mission Data Processing | Correlate Time | Time Correlation | Supports Time correlation between on-board clocks and the system reference time. | MO Services Concept [CCSDS 520.0-G-3] Time |  |  |
|  |  | Time Reception | Providers: TT&C Consumers: Time Correlation | Report Reception Time | Reception Time | Provides accurate ground reception time reporting that can be associated with a Time Report. This is currently supported by the CSS Space Link Extension Transfer Services, where Earth Receive Time is provided as an annotation parameter to the transfer data. An alternative service may be required where SLE is not used. | Space Link Extension – Return All Frames Service Specification [CCSDS 911.1-B-4] And related CSS SLE Transfer Services |  |  |
|  | NAV | Navigation Services | Providers: Navigation Functions Consumers: Mission Control Mission Planning & Scheduling Mission Data Processing Data Storage & Archiving User Support TT&C Satellite Dev. & Maintenance | Request Navigation Message Retrieve Navigation Message Subscribe to Navigation Message | Orbit Vector Attitude Tracking Data Predicted Orbital Events Spacecraft Manoeuvre Conjunction Pointing Request | Supports the provision of spacecraft positioning information such as: - Position reports (e.g., from on-board GPS) - Spacecraft ranging and range-rate measurements - Antenna tracking azimuth and elevation - Orbit vectors - Attitude vectors - Trajectory requests - Predicted orbital events (including ground station visibilities) The services will use the following data message formats defined by the CCSDS Navigation working group, but wrap these as service specifications based on the MO framework. | MO Services Concept [CCSDS 520.0-G-3] Navigation |  |  |
|  |  | Orbit Data Message [ODM] | Providers: Orbit Determination and Propagation Consumers: Manoeuvre Planning Attitude Determination Conjunction Assessment Mission Control Mission Planning & Scheduling Mission Data Processing TT&C User Support | - | Orbit Vector | The ODM contains information that defines the orbit state of a spacecraft at one or more times. | Orbit Data Messages [CCSDS 502.0-B-2] |  |  |
|  |  | Attitude Data Message [ADM] | Providers: Attitude Determination Consumers: Orbit Determination and Propagation Manoeuvre Planning Mission Control Mission Data Processing User Support | - | Attitude | The ADM contains information that defines the attitude state of a spacecraft at one or more times. | Attitude Data Messages [CCSDS 504.0-B-1] |  |  |
|  |  | Tracking Data Message [TDM] | Providers: Time/Position Determination TT&C Mission Data Processing Consumers: Orbit Determination and Propagation | - | Tracking Data | The TDM contains information that can be used to determine the orbit state of a spacecraft. | Tracking Data Messages [CCSDS 503.0-B-1] |  |  |
|  |  | Event Message [EVM] | Providers: Orbit Determination and Propagation Consumers: Attitude Determination Manoeuvre Planning Mission Planning & Scheduling Mission Control | - | Predicted Orbital Events | The EVM contains the predicted timings of orbital events, such as ground station visibilities, sensor blindings, eclipses, etc. | Proposed new Navigation message. |  |  |
|  |  | Spacecraft Manoeuvre Message [SMM] | Providers: Manoeuvre Planning Consumers: Orbit Determination and Propagation Attitude Determination Conjunction Assessment Mission Planning & Scheduling Mission Control Mission Data Processing | - | Spacecraft Manoeuvre | The SMM contains information on the transformation from one state (attitude or orbit) to another. | Navigation Data Messages Overview [CCSDS 500.2-G-1] SMM |  |  |
|  |  | Conjunction Data Message [CDM] | Providers: Conjunction Assessment Consumers: Manoeuvre Planning Mission Planning & Scheduling | - | Conjunction | The CDM contains information that defines the relationship between the orbit states of different space objects at different times. | Conjunction Data Messages [CCSDS 508.0-B-1] |  |  |
|  |  | Navigation Hardware Message [NHM] | Providers: Mission Control Consumers: Manoeuvre Planning Orbit Determination and Propagation Attitude Determination | - | Navigation Hardware Data | The NHM contains information that can be used to determine both the spacecraft attitude and orbit state. It also contains information that can be used for spacecraft manoeuvre reconstruction. | Navigation Data Messages Overview [CCSDS 500.2-G-1] NHM |  |  |
|  |  | Pointing Request Message [PRM] | Providers: User Support Mission Data Processing Consumers: Attitude Determination Manoeuvre Planning Mission Control | - | Pointing Request | The PRM contains information on the pointing of a spacecraft or instrument desired by a mission user at one or more times. | Pointing Request Message [CCSDS 509.0-R-1] |  |  |

## Operations Preparation Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OPD | Satellite DB [SDB] | Providers: Satellite DB Definition Spacecraft Development & Maintenance Consumers: Mission Control Spacecraft Development & Maintenance | - | Satellite DB | Contains definition of Telemetry Data, Telecommands and Events present in the TM/TC interface with the spacecraft and represented within the Mission Control System. XTCE provides an exchange format for TM/TC data between systems. | XML Telemetric and Command Exchange (XTCE) [CCSDS 660.0-B-1] |  |  |
|  |  | Automated Procedure Definition [APD] | Providers: Automated Procedure Definition Spacecraft Development & Maintenance Consumers: Mission Control Spacecraft Development & Maintenance | - | Automated Procedure | Definition of an operational procedure that can be automatically executed within a space system (either on-board a spacecraft, or within the mission control system). | No CCSDS standard. ECSS PLUTO [ECSS E-70-32] defines a standard model for a procedure, but not a normative representation. |  |  |
|  |  | Planning Data Definitions [PDB] | Providers: Planning DB Definition Consumers: Mission Planning & Scheduling | - | Planning Data Definitions | Definitions of Planning Data (Activities, Events, Resources and Constraints) and potentially of Planning Rules. | No CCSDS standard Mission Planning and Scheduling Concept [Draft Green Book] introduces Planning Data Model. |  |  |
|  |  | On-board Software [OSW] | Providers: On-board Software Definition Spacecraft Development & Maintenance Consumers: Mission Control Spacecraft Development & Maintenance | - | On-board Software | On-board Software Image | No CCSDS standard |  |  |

## Data Archiving Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DAI | Producer Archive Interface [PAIS] | Providers: Data Archive Consumers: Mission Data Processing | TBD | Submission Information Package (SIP) SIP Sequencing Contraint Transfer Object Collection Descriptor | The current PAIS standard provides the abstract syntax and an XML implementation of descriptions of data to be sent to an archive. It addresses how these data will be aggregated into packages for transmission and one concrete implementation for the packages based on the XML Formatted Data Unit (XFDU) standard. A service specification is proposed but has not yet been developed. | Reference Model for an Open Archival Information System (OAIS) [CCSDS 650.0-M-2] Producer-Archive Interface Specification (PAIS) [CCSDS 651.1-B-1] XML Formatted Data Unit (XFDU) Structure and Construction Rules [CCSDS 661.0-B-1] |  |  |
|  |  | Consumer Archive Interface [CAIS] | Providers: Data Archive Consumers: User Support | TBD | TBD | Delivery of digital sources from the Archive. | Reference Model for an Open Archival Information System (OAIS) [CCSDS 650.0-M-2] |  |  |