APPLICATION AND SUPPORT LAYER ARCHITECTURE – Green Book Outline

APPLICATION AND SUPPORT LAYER ARCHITECTURE – Green Book Outline	1

1.	INTRODUCTION	5

1.1	Purpose	5

1.2	Scope	6

1.3	Rationale	6

1.4	Document Structure	7

1.5	Definitions	7

1.6	References	7

2.	Application and Support Layer Architecture Concepts	9

2.1	Background	9

2.2	Role of This Architecture Description Document	9

2.3	General Description of Content	9

2.4	Application Support Layer Domains	9

2.4.1	Mission Operations and Information Management	9

2.4.2	Spacecraft Onboard Interfaces	10

2.5	Relationship with other CCSDS Architecture Documents and Standards	11

2.6	ASL Architecture: Assumptions, Goals, and Challenges	11

3.	Application and Support Layer Reference Architecture 	13

3.1	Introduction	13

3.2	Seven Views of System Architecture	14

3.2.1	Functional View	14

3.2.2	Information View	15

3.2.3	Service View	15

3.2.4	Communications View	16

3.2.5	Physical View	17

3.2.6	Deployment View	17

3.2.7	Implementation View	18

3.3	Graphical Conventions	19

3.3.1	Functional View Notation	19

3.3.2	Information View Notation	22

3.3.3	Service View Notation	24

3.3.4	Communication View Notation	25

3.3.5	Physical View Notation	26

3.3.6	Deployment View Notation	27

3.3.7	Implementation View Notation	27

3.4	Deployment Use Cases  	28

3.5	Service Agreements and Access Arrangements	28

3.6	Transitional Strategies	28

4.	Functional View	29

4.1	Overview	29

4.2	MOIMS Functions	29

4.2.1	MOIMS Area Context	30

4.2.1.1	Mission Data Processing	32

4.2.1.2	Spacecraft Maintenance and Development	32

4.2.1.3	Telemetry, Tracking and Commanding (TT&C)	33

4.2.1.4	User Support	34

4.2.2	MOIMS Functional Groups	35

4.2.2.1	Mission Control	36

4.2.2.2	Mission Planning & Scheduling	36

4.2.2.3	Navigation and Timing	37

4.2.2.4	Operations Preparation	37

4.2.2.5	Data Storage and Archiving	38

4.2.3	Common Functions and Services	39

4.2.3.1	Data Storage and Archiving	39

4.2.3.2	Login and Authentication	39

4.2.3.3	Operations Preparation	39

4.2.3.4	Service Directory	40

4.2.4	Mission Control	40

4.2.4.1	Monitoring and Control	41

4.2.4.2	On-board Configuration Management	41

4.2.4.3	Automation	41

4.2.4.4	Navigation Interface	42

4.2.5	Navigation and Timing	43

4.2.5.1	Time/Position Determination	44

4.2.5.2	Orbit Determination and Propagation	44

4.2.5.3	Attitude Determination	45

4.2.5.4	Manoeuvre Planning	45

4.2.5.5	Conjunction and Re-entry Assessment	46

4.2.5.6	Time Correlation	46

4.2.6	Mission Planning and Scheduling	47

4.2.6.1	Planning	48

4.2.6.2	Plan Execution	48

4.2.7	Operations Preparation	49

4.2.7.1	Configuration Data Definition (Editors)	50

4.2.7.2	Configuration Management and Distribution	51

4.2.8	Data Storage and Archiving	52

4.2.8.1	On-board File Store	52

4.2.8.2	Mission Operations Archive	53

4.2.8.3	Mission Data Archive	54

4.3	SOIS Functions	55

4.3.1	SOIS Area Context	55

4.3.2	SOIS Area Functions	57

4.3.3	Protocol Convergence Functions	58

4.3.4	Management Functions	59

4.4	Integrated Flight/Ground Functions (future territory, ???)	60

4.5	Security Concepts for Functional View	60

5.	Information View (Information Objects)	61

5.1	Overview	61

5.2	MOIMS Information Objects	61

5.2.1	MOIMS Information Groups	62

5.2.2	MO Common Object Model	63

5.2.2.1	Domain	63

5.2.2.2	MO COM Object	64

5.2.2.3	MO COM Event	64

5.2.2.4	MO COM Object Patterns	64

5.2.2.4.1	COM Activity Object Pattern	65

5.2.2.4.2	COM State Object Pattern	65

5.2.2.4.3	COM Static Item	65

5.2.2.4.4	COM Defined Event	66

5.2.3	MO Common Services Data	66

5.2.3.1	Service Directory (SDIR)	66

5.2.3.2	Login and Authentication Credentials (LAC)	66

5.2.4	Mission Control Data	67

5.2.4.1	Monitoring and Control Data	67

5.2.4.1.1	Action	67

5.2.4.1.2	Alert	68

5.2.4.1.3	Parameter	68

5.2.4.1.4	Aggregation	68

5.2.4.1.5	Check	68

5.2.4.1.6	Conversion	69

5.2.4.1.7	Statistic	69

5.2.4.1.8	Group	69

5.2.4.2	Other Mission Control Data	69

5.2.4.2.1	Procedure	70

5.2.4.2.2	OSM: OBSW Image	71

5.2.4.2.3	OPM: OBCP Definition	71

5.2.5	Navigation and Timing Data	71

5.2.5.1	NAV: Navigation Data	72

5.2.5.1.1	ADM: Attitude Data Message	72

5.2.5.1.2	CDM: Conjunction Data Message	72

5.2.5.1.3	NEM: Navigation Event Message	72

5.2.5.1.4	ODM: Orbit Data Message	73

5.2.5.1.5	PRM: Pointing Request Message	73

5.2.5.1.6	RDM: Re-entry Data Message	73

5.2.5.1.7	TDM: Tracking Data Message	74

5.2.5.2	TIM: Timing Data	74

5.2.5.2.1	TRP: Time Report	74

5.2.5.2.2	TRM: Time Reception Message	75

5.2.5.2.3	TCM: Time Correlation Message	75

5.2.6	Mission Planning Data	76

5.2.6.1	Plan	76

5.2.6.2	Planning Activity	77

5.2.6.3	Planning Constraint	77

5.2.6.4	Planning Event	77

5.2.6.5	Planning Resource	78

5.2.6.6	Planning Request	78

5.2.7	Mission Data Products	79

5.2.8	Operations Preparation Data	79

5.2.8.1	SDB: Spacecraft Database	80

5.2.8.2	APD: Automated Procedure Definition	80

5.2.8.3	OSW: On-board Software Image	80

5.2.8.4	PDB: Mission Planning Database	81

5.2.9	Data Storage and Archiving Data	81

5.2.9.1	DAI: Data Archive Information	81

5.2.9.2	On-board File Store	82

5.2.9.3	Operations Archive	83

5.3	SOIS Information Views	84

5.3.1	SOIS Information Model	84

5.3.2	SEDS Concepts	84

5.3.3	Dictionary of Terms	90

5.3.4	Data Exchanged across SOIS Subnetwork Functional Interfaces	91

5.3.4.1	Subnetwork Addresses	91

5.3.4.2	Flow Identifiers	92

5.3.4.3	Subnetwork Data Payloads	93

5.3.4.4	Subnetwork Metadata	93

5.4	Security Concepts for Information View	93

6.	Service View	94

6.1	Overview	94

6.2	MO Services	94

6.2.1	MO COM and Common Services	1

6.2.2	Mission Control Services	2

6.2.3	Navigation and Timing Services	5

6.2.4	Mission Planning and Scheduling Services	8

6.2.5	Operations Preparation Services	9

6.2.6	Data Archiving Services	10

6.2.7	Other MO Services	11

6.3	SOIS Services	1

6.4	Integrated Flight/Ground Services (discussion)	1

6.5	Dependence of ASL Services on SCCS Architecture 	1

6.6	Security Concepts for Service View	1

7.	Communications View (Protocol stacks)	2

7.1	Overview	2

7.2	ISO Protocol Stack and Layer Definitions	2

7.3	Specific Protocols for MOIMS Service Interface Binding	2

7.3.1	Communications Deployment Contexts	3

7.3.2	Generic Protocol Stack	5

7.3.3	Space Link Context	7

7.3.4	Ground Context	9

7.3.5	On-board Context	10

7.3.6	Context Bridging	12

7.4	Specific Protocols for SOIS Service Interface Binding	13

7.4.1	Simple Subnetwork	14

7.4.2	Software Message Bus	14

7.4.3	Multiple Processors Separating Application and Instrument	15

7.4.4	Multiple Processors Separating Applications	16

7.4.5	Connection to External Network	18

7.5	Integrated Flight/Ground Protocols	19

7.6	Remaining Challenges to Protocol Deployment	20

7.7	Security Concepts for Protocol View	20

8.	Physical (Connectivity) View	21

8.1	Overview	21

8.2	MOIMS Physical Elements	21

8.2.1	Identification of Deployment Nodes	21

8.2.2	Physical Deployment Architecture Examples	23

8.2.2.1	ABA Space Link Example	24

8.2.2.2	SSI Example	25

8.2.2.3	Hosted Payload Example	26

8.2.3	Potential Functional Deployment	27

8.3	SOIS Physical Elements	29

8.4	Security Concepts for Physical View	29

9.	[Functional] Deployment View	30

9.1	Overview	30

9.2	MO Functional Deployment	30

9.2.1	Distributed Functions	31

Proxy Functions	32

Distributed Functions	32

9.2.2	Communications Satellite (ABA) Example	32

9.2.3	Earth Observation Satellite (ABA) Example	34

9.2.4	Deep Space Mission (SSI) Example	36

9.2.5	Constellation Mission (SSI) Example	37

9.2.6	Hosted Payload Mission (SSI) Example	38

9.2.7	Manned Mission Example	40

9.3	SOIS End-to-End Deployment Views	41

9.4	Security Concepts For End-to-End Protocol View  	42

10.	Implementation View	43

10.1	Introduction	43

10.2	MOIMS Implementation	43

10.3	SOIS Implementation	43

10.3.1	SOIS EDS-Derived Device Services	44

10.3.2	Application Support Functions	45

10.3.3	SOIS System Model	45

10.3.4	Assembly of SOIS Protocol Functions	49

10.4	Security Concepts for Implementation View	50

Annex A:	ACRONYMS	51

Annex B:	BACKGROUND	53

All Contribute

RST Provide

PS Provide

RK Provide

# INTRODUCTION

## Purpose

CCSDS develops standards that support interoperable interfaces between the various elements of a space data system that may be distributed between different space agencies or other institutions.  CCSDS itself is organized into a number of Areas with responsibility for standardization of different aspects of a space data system.  While these standards have been developed over decades, and there must be potential for change as new technologies become established, there is the intention that the various standards developed by CCSDS can be deployed effectively together in actual space data systems.

CCSDS has undertaken a task to define and describe a complete reference architecture for space data systems that shows where and how all of the CCSDS standards are intended to be deployed.  This reference architecture specifically addresses both published standards and the current roadmap for development of standards.  In so doing it can be used to:

- show how CCSDS standards may be orchestrated together within actual space data systems

- identify gaps in current and planned standardization

- identify overlaps between current and planned standardization

The complete CCSDS reference architecture is being published in two parts: essentially distinguishing communications services that deliver data, and applications services that use these delivery services to carry out missions. The first step in this process defined the Space Communications Cross Support (SCCS) architecture [2],[3], which addresses the relationship between standards associated with four CCSDS areas:

- Space Link Services (SLS)

- Space Internetworking Services (SIS)

- Cross Support Services (CSS)

- System Engineering Area (SEA)

The purpose of this document is to provide the second half of this reference architecture and to cover Application and Support Layer (ASL) standards associated with the remaining CCSDS areas:

- Mission Operations and Information Management Services (MOIMS)

- Spacecraft On-board Interface Services (SOIS)

Taken together with the SCCS ADD, these documents provide an understanding of how all CCSDS services and data exchange standards work together in the context of a space data system.

## Scope

The scope of the reference architecture presented in this document covers application layer and support services between ISO upper layer functions (5-7) across interfaces that represent interoperability boundaries between agencies or systems.  It includes application layer services and protocols on ground and in flight and also addresses the syntax and semantics of data formats exchanged across such interfaces and how the standard information objects those data formats contain may be used or referenced across multiple service interfaces.  In principle application layer interactions are independent of the underlying communications architecture, but an actual deployment will need to consider the full protocol stack, including the underlying communications layers addressed in the SCCS documents, to achieve end-to-end communication.

The scope of this document is specifically those CCSDS service and data exchange standards associated with the MOIMS and SOIS areas.  In this sense it is a companion document to the SCCS ADD [2] that addresses standards associated with the SLS, SIS, CSS, and SEA areas.  All communications between application layer system elements, on the ground and in flight, will utilize these underlying link layer and/or network layer services. All are assumed to use the CSS cross support services to plan, schedule, and utilize the space communications terminals that provide access to space. Some CSS standards are referenced directly within the ASL reference architecture, as they are themselves application layer services.

With respect to the context of space link and network communications architecture, this document should be read in conjunction with the SCCS ADD [2].

The scope of this reference architecture includes (for the MOIMS and SOIS areas):

- Published CCSDS Standards

- CCSDS Standards currently under development

- Identified CCSDS Standards, forming part of the road map for future standardisation, as defined in published CCSDS Green Books or Working Group Charters.

- Functional interactions for which there is currently no identified CCSDS Standard.

The current status of standards development is clearly indicated within the reference architecture, but is also subject to change as the CCSDS Working Groups make progress on their planned development of new standards and updates to current ones.  The CCSDS website, www.ccsds.org, is the best reference for the current state of any standards referenced in this document and the plans for updating them.

It is noted that detailed definitions of the services and data exchange formats are not contained within this document, but within the referenced standards.  This reference architecture only models these to the extent required to identify the standards and the relationships between them.

## Rationale

<< To provide an understanding of how all of the CCSDS services and data exchange standards work together >>

I would move most of the current Sec 1.1 text down here, leaving just an intro paragraph or two in Sec 1.1.

## Document Structure

Following this Introduction, the document is organised into the following main sections:

- Application and Support Layer Architecture Concepts: provides a technical background and overview of the MOIMS and SOIS areas.

- Application and Support Layer Reference Architecture: describes the approach of describing the reference architecture in terms of seven modelling viewpoints and introduces the graphical notation used.

The remaining sections document each of the seven viewpoints in turn:

- Functional View

- Information View

- Service View

- Communications [Protocol] View

- Physical [Connectivity] View

- [Functional] Deployment View

- Implementation View

## Definitions

<< Reference these from MOIMS & SOIS docs and also from SCCS-ADD and RASDS, as needed.  Identify the source. >>

## References

<< Lists references from MOIMS & SOIS docs and also from SCCS-ADD and RASDS. >>

- Reference Architecture for Space Data Systems. Issue 1. Recommendation for Space Data System Practices (Magenta Book), CCSDS 311.0-M-1. Washington, D.C.: CCSDS, September 2008.

- Space Communications Cross Support – Architecture Description Document. Issue 1. Informational Report (Green Book), CCSDS 901.0-G-1. Washington, D.C.: CCSDS, November 2013.

- Space Communications Cross Support – Architecture Requirements Document. Issue 1. Recommended Practice (Magenta Book), CCSDS 901.1-M-1. Washington, D.C.: CCSDS, May 2015.

- Spacecraft Onboard Interface Services.  CCSDS 850.0-G-2. October 2011.

- Subnetwork Packet Service.  CCSDS 851.0-M-1.  December 2009.

- Subnetwork Memory Access Service.  CCSDS 852.0-M-1.  December 2009.

- Subnetwork Device Discovery Service. CCSDS 854.0-M-1.  December 2009.

- Subnetwork Synchronization Service.  CCSDS 853.0-M-1.  December 2009.

- Subnetwork Test Service.  CCSDS 855.0-M-1.  December 2009.

- XML Specification for Electronic Data Sheets.  CCSDS 876x0.r0.  not yet published.

- Dictionary of Terms.  CCSDS 876x1.r0.  not yet published.

- MOIMS ???

# Application and Support Layer Architecture Concepts

## Background

<< Motivated by existing SCCS-ADD and desire of CMC to have a more complete “CCSDS Reference Architecture”. >>

## Role of This Architecture Description Document

<< Provide an understanding of the application and support layer services and other supporting standards from MOIMS and SOIS and how they use underlying communications services and data exchange standards >>

## General Description of Content

<< A gentle intro to the content: Application and support layer functions in flight and on ground.  The information they exchange- that semantics and information content are important, not purely data formats; and that data items exposed at application level interfaces may reference other data items at the same level.  How they are assembled to define services –which includes specification of the interaction between communicating entities .  What the communications protocol stacks look like: may be a data format for file exchange, or an interactive service based on message exchange  Underlying communications protocol stack may either be as already defined in the SCCS-ADD for space links, or make use of alternative terrestrial technologies depending on the deployment context.

This document seeks to model the mission operations aspects of a space system as a set of reference function;  identify where the interactions between those functions may be at an interoperable boundary between agencies, organisations or systems; map these to existing or planned CCSDS standards; and identify any key gaps in coverage.>>

## Application Support Layer Domains

<< Introduce the scope of the document in terms of the primary domains : Mission Operations (MOIMS and SOIS The flight and ground domains are connected and secured using underlying data transport and other services provided by the other CCSDS areas: SLS, SIS, CSS, and SEA. >>)>>

### Mission Operations and Information Management

The CCSDS Mission Operations and Information Management Services (MOIMS) area is concerned with the application layer functionality required to perform mission operations and to manage mission information (both mission data products and configuration data), and specifically the interactions between its principal functions that may be exposed to interfaces between interoperating agencies, other institutions or systems.

The MOIMS area is active in standardisation of interfaces associated with the following functional domains:

- Spacecraft [and Mission] Monitoring and Control

- [Spacecraft] Navigation

- Mission Planning and Scheduling

- Data Archives

However, the MOIMS area does not address all application layer functionality of a typical space data system.  In particular it does not address the processing of mission payload or science data.

These functional domains are often distributed across multiple agencies, geographical sites or systems, exposing their interactions at interoperable interfaces.  Traditionally, these functions have been deployed primarily within the ground segment of a space data system, using specialized Telemetry, Tracking, and Command (TT&C) protocols across the space link.  Increasingly, however, MOIMS functionality is migrating on-board spacecraft, exposing the application interactions across the space link.

SLS, SIS, CSS, and SEA area standards are principally concerned with communications layer protocols that provide services for how an information “wrapper” (file or packet) is transferred to its destination and in how to architect and secure these widely distributed systems.  In contrast, MOIMS area standards are principally concerned with the format and meaning of the contained information that is exchanged between application layer functions.   Two approaches have been taken in the standardisation of MOIMS information:

- Specification of data formats that can be used for information exchange, where only the data format is standardised.

- Specification of a service that can be used for both information exchange and control.  Both the pattern of interaction between service provider and consumer and the format of exchanged information is standardised.

In the case of Navigation, data formats have been defined for the exchange of common Navigation data items, such as Orbit, Attitude and Tracking Data.

In the case of Spacecraft Monitoring and Control (SM&C), an abstract service framework has been developed that allows multiple Mission Operations services to be defined …

One of the challenges for MOIMS standardization is that there are many different kinds of mission applications software packages in use, which often have their own internal architectures for inter-process communications.  The MOIMS standards that just address data formats are more easily accepted than those that promote a whole new software architecture.  The MOIMS SM&C architecture has tackled this by defining a framework with multiple possible underlying technology mappings.  This adds complexity, but brings the promise of flexibility and adaptability as underlying technologies evolve.

### Spacecraft Onboard Interfaces

The CCSDS Spacecraft Onboard Interface Services (SOIS) provides standard development activities is to radically improve the spacecraft flight segment data systems design and development process.  It defines generic services that will simplify the way flight software interacts with flight hardware and permits interoperability and reusability.

The SOIS area is active in standardisation of interfaces associated with the following functional domains:

- Application Support Services

- Subnetwork Services

- On-board Wireless Services

The SOIS area would have a difficult task if it were only to recommend interfaces for application and subnetwork support services, due to the variety of architectures in which they should apply.  Some platforms use application programming interface (API) calls for communication with services.  Some platforms use a software message bus for the same purpose.  Some platforms are partitioned in time and space, with messages passed between partitions.

SOIS would have incomplete success in meeting the CCSDS goal of interoperability if it were to present a static recommendation, because such a recommendation does not easily accommodate the variety of real-time platform architectures typical in a resource constrained space environment.  Forcing adherence to a single static recommendation would likely be thought of as a restriction of innovation.  The SOIS-APP Working Group has embraced the need for innovation by accommodating the evolution of agency flight systems in a way that facilitates interoperability and portability across platforms.

- The application support service interfaces identified by the SOIS Green Book will be described in one or more SOIS Electronic Data Sheet (SEDS).

- Agencies can develop implementations of application support services and describe their interfaces in SEDSs.

- Each agency’s tool chain adapts an implementation to the agency’s platform through at least three mechanisms.

  - The tool chain generates device access services from device SEDSs.

  - The tool chain adapts application services to communicate in the style of the platform, which may include application programming interfaces (API) or a message bus service, such as AMS.

  - The tool chain adapts application services to mission-specific design parameters and constraints specified in EDSs.

- Subnetwork …

- Wireless …

## Relationship with other CCSDS Architecture Documents and Standards

The Application and Support Layer (ASL) architecture described in this document, uses underlying communications services and data exchange standards provided by other CCSDS Areas.

CCSDS has previously defined a space communication scross support architecture, including link and network layer data communications and cross support services, together with associated security services, that describes how the various standards defined by the CCSDS SLS, SIS, CSS and SEA areas can be deployed together in the context of a space system.  This is documented in the Space Communications Cross Support – Architecture Description Document (SCCS-ADD) [2] and Architecture Requirements Document (SCCS-ARD) [3], and in the many CCSDS standards they reference.

The ASL data exchanges and services are intended to be used over these underlying space data link and networking services, but they may be also deployed over standard terrestrial networks (ground to ground), or a variety of different, bespoke, on-board communications architectures described using CCSDS SOIS standards.

The CCSDS MOIMS and SOIS areas define standards that relate to the exchange of information or services between application layer functions.  These are referenced as elements within the Functional, Information and Service viewpoints of the ASL Reference Architecture described in this document.  How these elements relate to specific CCSDS books and standards is defined within the Service View tables in §6.

## ASL Architecture: Assumptions, Goals, and Challenges

<< Biggest assumption is that these ASL services live on top of underlying CCSDS standard data transfer services (and terrestrial ones as needed).  Other assumption, and challenge, is that the MOIMS service framework may be deployed in space as well as on the ground. >>

In this context explain how an MO compliant service may be implemented in different ways, ranging from an implementation that follows the MO layering concept – implementing a service framework – to compliance at a concrete interface level only.

<<NIH, reinvention, tailoring; DoT could be widely applied and extended for other uses; acknowledging existing SW frameworks and the need to work with them>>

# Application and Support Layer Reference Architecture

## Introduction

This chapter provides an overview of how the CCSDS Application and Support Layer (ASL) Reference Architecture is presented in the remainder of this document.

The ASL Reference Architecture has been modelled using the approach defined in the CCSDS Reference Architecture for Space Data Systems (RASDS) [RDx], modified and extended to enable a concise representation of the ASL from seven modelling viewpoints:

- Functional

- Information

- Services

- Communications (Protocol Stack)

- Physical (Deployment Nodes and Connectivity)

- Deployment

- Implementation

These viewpoints are described in §3.2 below.  The following chapters (4 to 10) of this document each contain one of these seven viewpoints of the ASL Reference Architecture in turn.  Each of these chapters contains sections addressing the following topics:

- Overview of the Viewpoint

- Mission Operations and Information Management Services (MOIMS) Aspects

- Spacecraft On-board Interface Services (SOIS) Aspects

- Security Concepts

- Additional topics specific to the viewpoint

RASDS provides a conceptual framework for describing architectures and a simple notation for the representation of Functional, Communications, Physical and Deployment viewpoints of a space domain reference architecture.  This is extended through the use of simplified UML class diagrams to represent the associated information models, and a tabular presentation of identified end-to-end services.  Specific extensions to RASDS have also been used to enable the representation of data and services in the Functional viewpoint.  A summary of the graphical notations used for the representation of the ASL Reference Architecture in this document is given in §3.3.

The ASL Reference Architecture is focussed on the representation of the Application and Support Layer, rather than on the lower layers of the communications protocol stack.  Where communications are deployed over space links, then the Communications viewpoint builds upon the RASDS models contained in the CCSDS Space Communications Cross Support Architecture Description Document (SCCS ADD) [RDx] that define the services, information models, communication protocol stacks, and deployment views for point-to-point “ABA” space links and Space System Internet (SSI) deployment cases.  The focus in these documents is on interoperable and cross supported communications, end-to-end, and the applications details are typically abstracted away.

From an ASL perspective, communications may also be deployed within a spacecraft and over terrestrial networks.  The on-board environment is often highly resource constrained and often requires specialized link layer protocols and real-time services.  More detail on these deployment use cases from a communications perspective is given in §3.4.

The kinds of service level agreements and access management arrangements that might be needed in multi-mission and multi-agency cross support and interoperability environments is discussed in §3.5.

Strategies that might be employed in the transition from ABA to SSI style deployments and from ground-only Mission Operations Services (MOS) to services that may be deployed in flight, together with the issues that may be encountered are discussed in §3.6.

## Seven Views of System Architecture

The representation of the ASL Reference Architecture from seven modelling viewpoints has already been introduced above.  This section provides a brief description of each of the views, explaining its scope and what it shows.

### Functional View

The functional scope of the model is broken down hierarchically into a set of functions corresponding to recognizable areas of functionality within space systems, which are often associated with a particular type of information.  While functions may correspond to identifiable subsystems, they may also be arbitrarily grouped within existing systems.

For the ASL Reference Architecture this corresponds to core functionality (sometimes referred to as “business logic”) at the application layer and not to supporting functions at lower communications layers.  For this reason, functions such as TT&C only appear where they are the source or sink of application layer information, and not for their role in routing telemetry and telecommands between spacecraft and ground segment facilities.

CCSDS tends to standardize the interfaces between functions, rather than the functions themselves.  The functional decomposition is only needed to a level that identifies those interfaces potentially exposed to interoperability boundaries between organizations, physical locations, various communications capabilities, or systems.

This viewpoint shows the interfaces between functions in terms of the information exchanged; and the services used to manage that information exchange, or to allow one function to control the processing performed by another.  It is concerned with application level information and not with the communications layer data formats (such as telemetry packets) that are addressed in the SCCS ADD.

The information (or data) associated with an interface between functions is identified on the interface and must correspond to an information object defined in the Information View.

Each interface may also expose a service defined in the Service View.  The representation of the interface identifies which function acts as the provider of the service, and which the consumer.  It should be noted that this is a different concept to data flow, and that the information objects associated with the interface do not necessarily flow only from provider to consumer.  This can be illustrated through the real world example of banking services:  the banks are the providers of services, the householder the consumer; deposits flow from consumer to provider, while withdrawals flow from provider to consumer (or often to other payees).

It is important to note that the interfaces between functions in the Functional View are logical interfaces corresponding to the end-to-end interactions at application level between functions, wherever they are deployed.  In an actual deployment a logical interface between two nominally “adjacent” functions may actually be routed through communications functions in multiple deployment nodes.  This is not shown in the Functional View, but is represented in the Communications, Physical and Deployment Views.

The Functional View shows interfaces between different functions, but does not show the interfaces between similar functions which may exist if a function is itself distributed, for redundancy, across several deployment nodes.  This is represented in the Deployment View.

Two formulations of the Functional View diagrams are provided.  The standard diagram is function-oriented and shows functions connected by logical interfaces.  These are contained in the body of the document.  The alternative diagrams are contained in an Annex and are service-oriented.  These show colour-coded horizontal lines corresponding to the services with vertical lines connecting to the functions that provide or consume that service.  The diagrams are equivalent, but show the interfaces from different perspectives.

### Information View

The information (data and meta-data) exchanged across interfaces between functions is the subject of the Information View, which models the principal information objects and the relationships between them, including:

- Inheritance

- Composition

- Aggregation

- Other Associations

The purpose of the information model within the ASL Reference Architecture is to identify the types of information object exchanged across individual interfaces or services that are the subject of specific CCSDS standards and not to define them in detail.  There is no attempt to expand the structure of these information objects in terms of their subordinate objects or attributes: this level of detail is contained in the individual standards that define them.

At the higher levels of the functional decomposition hierarchy, information objects may be aggregated to a more abstract representation of the information relating to a wider functional area.

The information model uses a simplified UML class diagram to show the relationships between information objects.

### Service View

The Service View identifies standard application level interfaces between functions.  A standardised service is specified in terms of the information exchanged and/or behaviour of the interface, rather than to a particular pair of interfaced functions.  The same service specification can be used to support multiple interfaces between different interfaced functions, providing they relate to the same type(s) of information and share a common interface behavior.

The information exchanged using a service corresponds to one or more information objects in the Information View.

The behaviour of a service corresponds to the pattern of interaction between functions across an interface.  This may be to support the exchange of information objects or the control of a function.  Such interactions may be supported as simple off-line transfer of data (information objects), typically as a file transfer, transfers of messages across a space link (with attendant delays), or more complex on-line interactions between service consumer and provider functions.

An interactive service specification defines the set of operations that the service consumer can invoke on the service provider and the bidirectional pattern of message exchange required to achieve this.  Bidirectional message exchanges across a space link may be particularly challenging, and must be carefully engineered, because of light-time delays. For the ASL Reference Architecture this is limited to identification of high level capabilities (groups of operations) supported by the interface.

Not all interfaces are supported by an interactive service standard.  In some cases only the format of the exchanged information object is currently standardized by CCSDS.  These information objects may be exchanged by a variety of means.

The Service View identifies the services and for each service:

- the functions that act as provider or consumer of the service

- the information objects that are exchanged across the service interface

- the behavior exposed at the interface and the operations supported by the service

- references to CCSDS standards defining the service and/or associated data formats

### Communications View

The Communications View shows how the application level services are supported by underlying communications protocol stacks, depending on their deployment context. This viewpoint should be read in conjunction with the SCCS ADD which identifies two principal deployment contexts for space links (ABA and SSI) together with their associated communications protocol stacks.

The ASL Reference Architecture builds upon the SCCS ADD by identifying three contexts for the deployment of application layer services:

- Terrestrial Link: a ground to ground communications channel

- Space Link: a space to ground, ground to space, or space to space communications channel based on the ABA and SSI deployment cases defined in the SCCS ADD

- On-board Link: a communications channel supported between functions on-board the same spacecraft.

Within each context, two principal approaches for service deployment are considered:

- Service Interaction using Message Transfer

- File Transfer

For each context and deployment approach, the Communications View provides a representative layered communications protocol stack.  In the case of the Space Link context this is by reference to the CCSDS recommendations defined in the SCCS ADD.  In the case of the Terrestrial Link context this assumes use of typical internet protocols, although alternative protocol stacks could also be used.  For the On-board Link context, this assumes the use of a CCSDS SOIS on-board architecture, as described in this document.

The same service specification at the application layer may utilize several different protocol stack deployments.

The use of communications gateway functions or protocol bridges may allow different communication protocol stacks to be used for different segments of the same end-to-end service interface.  This may be thought of as application layer bridging in a similar way that IP or DTN provide a common network layer across different underlying data link layer interfaces.

### Physical View

The Physical View identifies potential deployment nodes and the types of communication interfaces supported between them.

This document extends the limited set of deployment node classes identified in the SCCS ADD to cater to representative ground segment scenarios, by identifying a realistic set of example ground segment deployment nodes.  It is stressed that the identified ground segment nodes are purely examples for use in the presentation of representative deployment scenarios in the Deployment View.  Other deployment nodes may exist in actual mission deployments, or nodes with these same behaviors may appear with different names.

This viewpoint shows the identified set of deployment nodes together with the physical communications links between the nodes for a range of physical deployment architectures, including both ABA and SSI space links, on-board links, and terrestrial networks.  Each of the physical communications links between nodes in the diagrams representing these physical deployment architectures is classified in terms of the deployment contexts identified in the Communications View as a Space Link, an On-board Link, or a Terrestrial Link (together with their associated communications protocol stacks).

The physical deployment architectures shown are only representative examples and that other physical deployment architectures may exist in actual mission deployments.

### Deployment View

The Deployment View provides illustrative examples of how functions from the Functional View may be deployed across the set of physical deployment nodes identified in the Physical View.  This viewpoint also shows the resultant application level interactions between functions in terms of services (Service View) and information objects (Information View) exposed to the potential interoperability boundaries between deployment nodes.  Functional interactions between functions co-located on the same node are omitted for clarity.

The objective of the selected set of functional deployment scenarios is to show how the services identified in the Service View may be exposed to interoperability boundaries between organizations or systems in realistic mission deployments.

It is stressed that these are only example deployment scenarios, provided to cover a usefully broad set of options, and actual missions may have a wide variety of alternative architectures.

### Implementation View

In most of the CCSDS standards the focus is on defining a single protocol, or information structure, that may be directly implemented to provide interoperability and cross support.  Some of the standards covered in this document have those same properties, but others have specific properties that are intended more for portability, or implementation convenience.

The Implementation View provides a way to showcase these features so as to make them more accessible.  The objective is to show how some of these standards, such as the MOIMS Message Abstraction Layer (MAL) or the SOIS Electronic Data Sheets (EDS) may be used to transform specifications of interfaces and behaviors into operational code, or at least into compilable interface specifications.

The objective of the Implementation view is to describe how the ASL specifications may be extracted and transformed into executable code, or at least into code fragments that may then be used to develop application layer functionality.   This view may involve use of various tool chains to transform specifications, and adaptation rules, into code or at least interface code fragments.

## Graphical Conventions

CCSDS RASDS [RDx] defines a graphical notation for use in the modelling of space data systems.  The following figure summarises the RASDS notation and additional colour coding used within the SCCS ADD.

Figure 3-1: RASDS Graphical Notation as used in SCCS ADD

Since this document builds upon the basis of the communications architecture described in the SCCS ADD, common notation has been used, but this has been extended to enable representation of both the information model and services that are central to the Application and Support Layer.  The following sections summarize the notation used for each of the modelling views in this document.

### Functional View Notation

The functional view diagrams show functions [functional elements] and the associated logical links between them.  At the application layer, these logical links represent either simple file-based information exchange between functions or application level service interfaces which may be interactive or delayed.  The underlying communications functions and links are not shown in this viewpoint, as the diagrams are independent of deployment architecture.

The basic RASDS notation for functional viewpoint is used and has been extended to allow the representation of both services and exchanged information (or data).

Information objects exchanged across a logical interface are shown as rectangular boxes attached to the connector, annotated with their abbreviated names.  Conventions detailed in the diagram below enable differentiation between information objects that are specific or generic; and those contained within another information object.

To enable differentiation between the provider and consumer of a service, the provider end is annotated with a small circle.  Where standardization of the interface concerned is limited to the format of the exchanged information object, then the “provider” corresponds to the function that generates that information.

The current standardisation status for specific information objects and services is also encoded in the border style of the corresponding shape, as detailed in the diagram.

Figure 3-2: Graphical Notation for Functional View Diagrams

In the SCCS ADD, colour coding of functions is used to differentiate between different communications layers and associated function types, with all application layer functions having the same colour.  In the context of the Application Layer, this is not particularly useful.  For the ASL colour coding of functions and associated information objects and services is used to indicate specific functional areas or groups of related functions.

Figure 3-3: Alternative Representation for Functional View Diagrams

An alternative representation of the Functional View diagrams is also provided, which is used in an Annex of this document.  This puts the services at the centre of the picture as a set of horizontal colour-coded “tramlines”.  Functions are then attached to these with vertical lines, distinguishing between service provider and service consumer as before.  This approach makes it easier to see which functions are the providers and consumers of each service.

### Information View Notation

The Information View shows information objects and the relationships between them.  The information objects in this view correspond to those shown in the Functional View.

RASDS does not supply a recommended representation for the Information Viewpoint, so in this document a form of entity relationship diagram is used, based on simplified UML class diagram notation.  In the context of the ASL ADD, the types or classes of information object are identified, but their detailed structure is not elaborated.  Information classes can therefore be represented as a simple rounded rectangle containing the name of the information class (or an abbreviation of it), connected by standard UML relationships to other information classes.

Figure 3-4: Graphical Notation for Information View Diagrams

Standard UML notation is used for the following common relationship types:

- Inheritance:  Information Class B is derived from Information Class A, which is typically an abstract class, extending and specialising its detailed definition.

- Composition:  Information Class B forms part of Information Class A.

- Aggregation:  Information Class B is included in a collection of objects represented by Information Class A.

- Association:  a general purpose relationship between Information Classes A and B.  This is typically labelled to indicate the nature of the relationship.

Extended notation is used to represent two specialised relationship types defined by the CCSDS Mission Operations (MO) Common Object Model (COM).

- MO Source:  Information Class A is the source of Information Class B.  This is typically used to provide a link back to the Information Object responsible for the creation of another.  This may be used to represent a parent-child relationship, and can provide an audit trail of control responsibility.

- MO Related: a secondary generic relationship supported by MO COM.  This can be used to represent the relationship between an information object instance and its definition.

The status of CCSDS standardisation is shown by the border style of the information objects, using the same color and line-style coding as the functional view.

As a shorthand to represent a contained set of information classes, the contained objects can be placed within a large rounded rectangle with a dashed border, with a composition or aggregation relationship between this and the container information class.

Color coding (for functional area) and other graphical conventions (for abstract classes, non-standardised classes, etc.)  are consistent with those defined for the Functional View.

### Service View Notation

The Service View is represented as a set of tables listing the identified service interfaces or data formats for a given functional area.  These tables have the following columns:

A	Functional Area: represented by color coding consistent with that defined for the Functional View.

Group	Name or acronym for a group of related services or data formats.

Service	Name of service or data format

Functions	List of functions from the Functional View that act as Provider or Consumer of the service.  For data formats, this corresponds to the functions that output (provider) and input (consumer) the data format.

Operations	List of capabilities of the service, corresponding to high-level groups of related operations that the consumer may invoke on the provider.

Data	List of information objects from the Information View that are transferred across the service interface.

Description	Description of the purpose of the service and its dependencies on other services.

Standards	References to the CCSDS standards relevant to the service.  This may include the full service specification, other specifications it is dependent on, or informational reports that identify the service as part of the future roadmap of CCSDS standardization.

S	Status of Service Specification

D	Status of Data Format Specification

The last two columns indicate the current status of service specification by CCSDS.  It is divided into two columns to indicate service and data format specification as these may be separately defined.  The status is color-coded consistently with the Functional and Information Views to show standardization status:
    Blue	Published CCSDS Standard (Blue or Magenta Book)

Blue/Light Blue	Published CCSDS Standard providing a partial solution

Blue/White	CCSDS Standard Under Development

Grey	I CCSDS Standard (Green Book – future road map)

White	No CCSDS Standard Identified

### Communication View Notation

The Communications View shows how the application level services are supported by underlying communications protocol stacks, depending on their deployment context.  RASDS notation and colour coding consistent with the SCCS ADD (see §3.1) is used to support this viewpoint.

Figure 3-5: Graphical Notation for Communications View Diagrams

The figure above illustrates this for a generic example of protocol layering at both communication and application layers.  The protocol stacks must match on both sides of a deployed interface to enable communication at the Application Layer.  Communications Layer protocols (link and possibly network) correspond to those defined in the SCCS ADD and are shown in blue.  Application Layer protocols identified in this document are shown in pink.

As in other viewpoints, italics are used to indicate abstraction – in this case, a generic protocol that must be replaced by a specific protocol in an actual deployment.

### Physical View Notation

The Physical View identifies potential deployment nodes and the types of communication interface supported between them.  Again RASDS notation and colour coding consistent with the SCCS ADD (see §3.3 ) is used to support this viewpoint.

Figure 3-6: Graphical Notation for Physical View Diagrams

Each type of example deployment node is represented as a 3D physical node element, color coded to indicate whether it is a user node (space or ground), or a space or ground segment routing node.  Shaded colour is used where a node acts as both user and routing node.

Physical links between nodes are shown as solid lines.  As an extension to the SCCS ADD notation, these are colour-coded to indicate which Communications View link context applies:  Space, On-board, or Terrestrial Link.  A dashed blue line may be used to show use of the CCSDS Space Link Extension standard between a ground station and other ground segment facilities.

The router symbol is only used where there is a Space Link routing function.  Standard terrestrial internet routers are omitted for clarity.  Where Space Link routing is available, a logical link between nodes may be shown by a dashed magenta line running over the corresponding physical links.

### Deployment View Notation

The Deployment View shows potential functional deployment for a number of physical deployment scenarios.

Figure 3-7: Graphical Notation for Deployment View Diagrams

The diagrams combine the Physical View notation for deployment nodes and physical links, with the Functional view notation for deployed functions and the associated services and information objects that are exposed to the interfaces between deployment nodes.

Colour coding for deployment nodes and physical links is consistent with both the Physical View and the SCCS ADD (see §3.3).

Colour coding for functions, services and information objects is consistent with that of the Functional View (see §3.3.1).

To avoid clutter, typically only one function bubble is shown per higher level functional group on each node.  If only a subset of functionality is deployed, then the deployed sub-functions are listed in the text within the bubble.

For clarity, only those logical interfaces between functions exposed to physical node boundaries are shown on the diagrams.  Logical interfaces between functions deployed on the same node may also exist, consistent with the Functional View.  Where a logical interface crosses a node without terminating at a function deployed there, this indicates that the physical communication path is routed through that node, but that it is opaque to the node because the node only looks at the data link, or network layer and not the contents of those protocol data units.

### Implementation View Notation

## Deployment Use Cases

<< Summarize the SCCS nodes and deployment cases (ABA and SSI) and explain how these relate to application layer MOIMS and SOIS interactions. Identify additional fundamental deployment patterns (in addition to ABA and SSI) to represent distributed networks on ground and on board>>

<< Consider alternative deployment use cases for MO services in the context of: ground only, space to ground, and within space. Also identifying which of these are current, and which future.>>

New text to be written – use introductory diagrams from MOIMS Protocol viewpoint set.  This includes some diagrams taken from SCCS ADD [slides 4-6] and new diagrams that explain the potential deployment contexts for MOIMS functions (Space, Space-Ground, Ground-Ground) [slides 7-10].

<< Consider how we are to handle constellations & formation flying, docking, and related subjects>>

## Service Agreements and Access Arrangements

<< Discussion of the kinds of agreements and access management arrangements might be needed in multi-mission and multi-agency cross support and interoperability environments. >>

## Transitional Strategies

<< Discussion of issues and strategies that might be employed in the transition from ABA to SSI style deployments and from MOS services only on the ground to MOS services in flight >>

New text to be written.  Introduce use of proxy or gateway functions that translate to MO services on the ground, encapsulating the space link and bespoke or legacy protocols used.

# Functional View

## Overview

<< Functional view covers the groups of functions and brief descriptions of their behaviors. >>

## MOIMS Functions

This section addresses functionality within the scope of the CCSDS Mission Operations and Information Management Services (MOIMS) Area.  Although MOIMS functions have traditionally been deployed within the ground segment, increasingly they are also being considered for deployment on-board spacecraft.  The Functional View presented here is of application layer functionality, independent of its deployment location.

The MOIMS Area covers a wide range of mission operations functions and is therefore presented hierarchically with two levels of decomposition.  First the MOIMS Area is decomposed into 5 main functional groups:

- Mission Control

- Navigation and Timing

- Mission Planning and Scheduling

- Operations Preparation

- Data Storage and Archiving

Each of these is then in turn decomposed into a set of high-level functions.  The same approach is taken for the services and information objects representing the data that flows between functions over an interface.

Functional groups are differentiated by colour coding as introduced in §3.3.1. The following diagram shows the colour coding used to distinguish different functional areas within the CCSDS MOIMS Area diagrams.

Figure 4-1: Colour Coding of MOIMS Functional Areas

The remainder of the section is structured as follows:

- MOIMS Area Context: scope of the MOIMS Area

- MOIMS Area Functional Groups: top-level decomposition of MOIMS Area

- Common Functions and Services:  applicable across multiple functional groups

- <Functional Group>: a set of sub-sections, one per MOIMS Area Functional Group, providing their second level decomposition into functions.

Each of the above subsections comprises a Functional View diagram and a description of each of the functions it contains.  Descriptions for external and higher level functions introduced in earlier diagrams are not repeated.

For more detailed descriptions of the information objects and services that are also shown in the diagrams, reference should be made to the Information and Service Views respectively.

### MOIMS Area Context

Figure 4-2: MOIMS Context

The MOIMS Area covers a wide range of functions associated with the support of mission operations and mission data archiving.  This functionality is associated with mission or payload operations centres and mission data archiving facilities and includes:

- Mission Control

- Navigation and Timing

- Mission Planning and Scheduling

- Operations Preparation

- Data Storage and Archiving

The diagram above shows the external boundaries of the MOIMS Area, represented by external functions and the logical interfaces with them in terms of data (information objects) and services.

The diagram does not show the spacecraft or space segment as an external function.  This is because the functional model allows for deployment of MOIMS functions within the space segment.  Logical interfaces with the spacecraft are therefore seen as peer-to-peer application level interfaces with deployed on-board mission operations functions and are hence internal to MOIMS.  Example physical deployment scenarios showing this are provided in the Deployment View.

While most current missions do have representative mission operations functions on-board the spacecraft, their application layer interaction with the ground is either proprietary/bespoke or follows non-CCSDS standards (such as the ECSS Packet Utilisation Standard [RD.x], DEM, or other).  However, CCSDS Mission Operations (MO) standards are intentionally open to deployment across space links and the Functional View has been modelled in a way that permits this to be shown.

On-board MOIMS functions will require access to spacecraft systems and equipment to monitor status and exercise control.  This is achieved through interfaces available on-board the spacecraft, addressed in this document through CCSDS SOIS functions.  From a MOIMS perspective, integration with SOIS is modelled in the Protocol View.

At the application layer, MOIMS functions do not interact with the TT&C function [ground stations] for the acquisition of telemetry or the uplink of telecommands.  This interaction does occur at lower communications layers, but from the application layer functional viewpoint presented here, this interaction merely passes through communications layer functions located on the TT&C node, as is depicted in the later Protocol and Deployment Views.  TM, TC, and AOS are communications link layer protocols already covered in the SCCS ADD, and not application layer information.  The CCSDS Packet is often used as an application layer data transfer structure and mappings from MOIMS SM&C to that standard are available, as are others. The MOIMS application layer data (or information objects) refer to the meaningful information contained within those packets.

The application layer data exchanged between MOIMS and external functions can be grouped into types of information as follows:

- MCS:  Mission Control

- NAVT:  Navigation and Timing

- MPS:  Mission Planning and Scheduling

- OPD:  Operations Preparation Data [On-board Configuration – Software, Procedures]

- MDP:  Mission Data Products

MOIMS application layer interactions with the TT&C function are partially addressed by services standardized by the CCSDS Cross Support Services (CSS) Area.  These include:

- Negotiation of the provision of TT&C services with a TT&C network provider.  This is a specific application layer planning and scheduling service covered by the CSS Service Management (SM) standard, shown on the diagram as CSS-SM.

- Access to the space link to send commands in the forward direction is provided by SLE-F-CLTU and return telemetry data by either SLE-RAF or SLE-RCF.

- Describing space link events relating to the provision of TT&C services by a TT&C network provider is available.  The identified CSS Service Management Event Sequence (SMES) is shown on the diagram as CSS-SMES.

- Monitoring of the TT&C function itself, in terms of the cross-support services it provides and events occurring on the link is provided.  This is covered by the CSS Monitored Data Service (MDS) standard, shown on the diagram as CSTS-MD.

- Control of the TT&C function itself, in terms of the cross-support services it provides.  This is covered by the identified CSS Service Control (CS) standard, which is shown on the diagram as CSTS-SC.

- Provision of Earth reception time [Time Reception Message (TRM)]: accurate timestamping of the reception time of messages received from the spacecraft, which is needed to support on-board time correlation.  This is currently performed at telemetry frame or packet level, with the Earth Reception Time being additional information added as part of the corresponding CSS Space Link Extension Transfer Service (TS), shown in the diagram as SLE-TS TRM.

The following subsections describe each of the External Functions shown in the MOIMS Area context diagram.

#### Mission Data Processing

Function

Acquisition and processing of payload or mission data performed systematically within the mission data system.  The nature of any such processing is specific to mission type, but may include:

- Science Data Processing

- Image Data Processing (Levels 1, 2 and 3)

- Navigation System Data Processing

- Communications Network Management

Provided Interfaces

- MDP: Mission Data Products (for archiving)

- NAVT: Navigation and Timing Data (accurate spacecraft position may be derived from image data)

Required Interfaces

- MCS: Monitoring & Control Data

- MPS: Mission Planning & Scheduling Data

- NAVT: Navigation and Timing Data

- MDP: Mission Data Products (payload data may be routed via Mission Operations; archived Mission Data Products may be retrieved)

#### Spacecraft Maintenance and Development

Function

The manufacturer of a spacecraft (or payload) provides configuration data that Mission Operations requires to configure its systems.  This includes the on-board software, spacecraft database, on-board control procedures and operational procedures.  The manufacturer may be delegated responsibility for maintenance of any or all of these for the duration of the mission, or alternatively they may be provided initially and handed over to Mission Operations for maintenance.

The manufacturer may also provide analysis and support in the event of spacecraft anomalies and may perform long-term performance monitoring of the spacecraft or payload systems.  Following such analysis, updates to on-board software, spacecraft database, on-board operations procedures or operational procedures may be provided.

Required Interfaces

- OPD: Submission/Retrieval of Operations Preparation Data, including on-board software, spacecraft database and procedures

- MCS: Monitoring & Control Data (for analysis)

- NAVT: Navigation and Timing Data (for analysis)

- MDP: Mission Data Products (for analysis)

#### Telemetry, Tracking and Commanding (TT&C)

Function

The TT&C function represents a network of ground stations providing telemetry acquisition, telecommand uplink and spacecraft tracking and ranging services.  At network level it is also responsible for planning and scheduling TT&C resources based on requests for the provision of TT&C services.

As explained previously, telemetry acquisition and telecommand uplink are communications layer functions that do not directly interact at the application layer with MOIMS functions.  MOIMS logical interfaces are however carried across space links provided by these functions.

MOIMS functions do however have application layer interactions with the TT&C function:

- To negotiate the provision of TT&C services with a TT&C network provider.

- To obtain spacecraft tracking and ranging data acquired by the TT&C station.

- To provide orbit vectors or predicted orbital events to the TT&C station to enable it to acquire and track a spacecraft.

- To obtain accurate earth reception timestamps associated with messages received from the spacecraft.  This is needed to support on-board time correlation.

- To obtain monitoring data on the status of TT&C services provided.

Provided Interfaces

- CSS-SM: Service Management interface for negotiation of the provision of cross-support (TT&C) services.

- SLE-FCLTU: Space Link Extension, transfer commands (and files) to spacecraft

- SLE-RAF (and RCF): Space Link Extension, transfer telemetry (and files) from spacecraft

- CSS-SMES: Service Management Event Sequence interface for the provision of detailed timings of cross-support (TT&C) services.

- CSTS-MD: Monitored Data Service giving status of provided TT&C services

- CSTS-SC: Service Control enabling control of provided TT&C services.

- CSS-TS[TRM]: Provision of earth reception timestamps as part of the CSS SLE Transfer Service.

- CSTS-TD NAVT: Provision of Tracking and Ranging Data

Required Interfaces

- NAVT: Provision of Orbit and Event Data (really part of SM as far as CSS is concerned)

#### User Support

Function

The User Support function represents any external user of a space system, including Principal Investigators (PIs) for many science missions and those requesting specific observations in astronomy, earth observation, or planetary missions.  Two principal sub-functions are included:

- Tasking the mission to perform particular operations.  These are typically payload operations supporting scientific experiments or observations.

- Analysis of mission data products.

Tasking is primarily supported through Mission Planning and Scheduling interfaces that allow the submission and tracking of planning requests.  However this may also require supporting information, such as:  spacecraft orbit and predicted orbital events; and spacecraft or payload status.

Analysis requires the provision of Mission Data Products, but may also need data such as spacecraft orbit and attitude, and spacecraft or payload status to support interpretation.

Provided Interfaces

None

Required Interfaces

- MPS: Mission Planning & Scheduling

- MCS: Monitoring & Control Data (for analysis)

- NAVT: Navigation and Timing Data(for analysis)

- MDP: Mission Data Products(for analysis)

### MOIMS Functional Groups

Figure 4-3: MOIMS Level 1: Functional Groups

The first level decomposition of the MOIMS Area identifies five main functional groups, colour coded as defined in the introduction to this section:

- Mission Control

- Mission Planning and Scheduling

- Navigation and Timing

- Operations Preparation

- Data Storage and Archiving

In addition to these five main functional groups, there is a set of common functions and services that may be used by all MOIMS functions.  These are described in §4.2.3 below.

At this level all data (information objects) exchanged between functions can also be grouped by the principal functional group with which it is identified.  In addition Mission Data Products (MDP) are identified as a type of information that is generated externally to MOIMS .

The same acronyms are used as in the MOIMS Area Context (§4.2.1 above).  Potential service interfaces are indicated by a colour-coded circle representing the service provider.  Data formats may be separately defined (Navigation and Mission Planning data) or specified in the context of the service (MO M&C and Mission Planning).

Data Storage and Archiving differs slightly, in that it is principally concerned with the temporary storage, archiving and retrieval of the information objects generated by other functional groups.

#### Mission Control

Function

Mission Control encompasses all functions associated with the execution of mission operations, both in terms of spacecraft control and that of the wider mission system.  It includes:

- Monitoring of Spacecraft, Payload and Mission Status

- Provision of Status Displays to the operations team

- Manual Commanding

- Execution of Automated Operations

- On-board Configuration Management

Provided Interfaces

- MCS: Mission Control Data

- NAVT: Navigation and Timing Data (Navigation Hardware Message)

Required Interfaces

- OPD: Operations Preparation Data (On-board Software, Spacecraft Database and Automated Procedure Definitions)

- DSA: Data Storage and Archiving services

- CSS-MDS: Monitored Data Service giving status of provided TT&C services

- CSS-SC: Service Control enabling control of provided TT&C services.

#### Mission Planning & Scheduling

Function

Mission Planning and Scheduling encompasses both: the generation of mission plans, based on received planning requests and defined planning constraints; and the execution of those plans, using underlying Mission Control services to effect the execution of planned activities.

Provided Interfaces

- MPS: Mission Planning & Scheduling

Required Interfaces

- MCS: Mission Control Services

- NAVT: Navigation and Timing Data (including predicted orbital events)

- OPD: Operations Preparation Data (Planning Database)

- DSA: Data Storage and Archiving services

- CSS-SM: Service Management interface for negotiation of the provision of cross-support (TT&C) services.

- CSS-SMES: Service Management Event Sequence interface for the provision of detailed timings of cross-support (TT&C) services.

#### Navigation and Timing

Function

Navigation is concerned with the management of spacecraft orbital dynamics, spacecraft attitude and on-board clocks.  It includes:

- Position and/or Time Determination

- Orbit Determination and Propagation

- Attitude Determination

- Time Correlation

- Manoeuvre Planning

- Conjunction Assessment

Provided Interfaces

- NAVT: Navigation and Timing Data

  - Orbit Data

  - Attitude Data

  - Predicted Orbital Events

  - Conjunction Data (collision warnings)

  - Spacecraft Manoeuvres

  - Time Correlation Data

Required Interfaces

- NAVT: Navigation and Timing Data

  - Tracking Data

  - Attitude Data

  - Pointing Requests

  - Navigation Hardware Data

- MPS: Mission Planning & Scheduling (Manoeuvre Planning Request)

- DSA: Data Storage and Archiving services

- CSS-TS[TRM]: Earth reception timestamps as part of the CSS SLE Transfer Service

#### Operations Preparation

Function

Operations Preparation is an off-line function concerned with the preparation, maintenance, configuration management and distribution of mission operations configuration data.  This configuration data includes:

- On-board Software

- Spacecraft Databases: defining available monitoring data and commands

- Automated Procedure Definitions: scripts for both on-board procedures and those automated on ground.

- Planning Database: defining planning activities, events and resources, together with static planning constraints.

Configuration Management and Distribution functions are common to all configuration data types.  The Definition of the configuration data (and associated editing tools) are specific to the configuration data type.

Provided Interfaces

- OPD:  Operations Preparation Data

Required Interfaces

- DSA: Data Storage and Archiving services

#### Data Storage and Archiving

Function

The Data Storage and Archiving function supports the storage and archiving of mission data associated with any of the other Functional Groups or Mission Data Products.  This includes the following:

- On-board File Store

- Mission Operations Archive

- Long-Term Mission Data Archive

Provided Interfaces

- DSA: Data Storage and Archiving services

Required Interfaces

- MCS: Mission Control Data

- MDP: Mission Data Products

- MPS: Mission Planning Data

- NAVT: Navigation and Timing Data

- OPD: Operations Preparation Data

### Common Functions and Services

Figure 4-4: MOIMS Level 2: Common Functions and Services

Common Functions and Services are those that can be used by any other MOIMS function.  The associated interfaces have been omitted from the other Functional View diagrams to reduce clutter in the interests of clarity.

The associated services have been defined in the context of the Mission Operations (MO) service framework as MO Common Services or as part of the MO Common Object Model (COM).

#### Data Storage and Archiving

This corresponds to the function identified in §4.2.2 above, and specifically its Operations Archive sub-function.  The MO COM Archive service is a generic service specification for the archiving and retrieval of any data whose structure is compliant with the MO Common Object Model.

#### Login and Authentication

Login and Authentication provides a common Login service to all other functions to support user login and authentication, including access to user access rights or privileges.  The information object associated with the service is the user’s Login and Authentication Credentials (LAC).

#### Operations Preparation

This corresponds to the function identified in §4.2.2 above, and specifically its Configuration Management and Distribution sub-function. The common Configuration service can be applied to any configuration data and supports both the configuration management operations required by functions that maintain the configuration data and the configuration distribution operations required by those that use it.

#### Service Directory

The Service Directory maintains a catalogue of available services and their providers that can be interrogated by any function in order to locate a required service.

### Mission Control

Figure 4-5: MOIMS Level 2: Mission Control Functions

The Mission Control functional group is broken down into the following Level 2 functions:

- Monitoring and Control

- On-board Configuration Management

- Automation

- Navigation Interface

Similarly, MCS data comprises the following information objects:

- M&C: Monitoring and Control Data (Parameters, Actions and Alerts)

- OSM: On-board Software Management Data

- OPM: On-board Procedure Management Data

- AUT: Automation Data (Procedure level Control and Monitoring)

#### Monitoring and Control

Function

Monitoring and Control corresponds to the basic functionality required to perform manual mission operations, both in terms of spacecraft control and that of the wider mission system.  It includes:

- Monitoring of Spacecraft, Payload and Mission Status

- Provision of Status Displays to the operations team

- Manual Commanding

Provided Interfaces

- M&C: Monitoring and Control (Parameters, Actions and Alerts)

Required Interfaces

- SDB: Spacecraft Database defining available Parameters, Actions and Alerts

- TCM: On-board Time Correlation Data

- CSS-MDS: Monitored Data Service giving status of provided TT&C services

- CSS-SC: Service Control enabling control of provided TT&C services.

- DSA: Data Storage and Archiving services

#### On-board Configuration Management

Function

This function manages on-board configuration data, including on-board software and on-board procedures.  It is responsible for the uplink and activation of new versions of configuration data, as well as the selective dump to ground of on-board configuration data (memory images or procedure scripts) for comparison to configuration controlled versions on the ground.  It also maintains a record of what is currently installed on the spacecraft.

Provided Interfaces

- OSM:  On-board Software Management

- OPM:  On-board Procedure Management

Required Interfaces

- OSW: On-board Software Images

- APD: On-board Procedure Scripts

- M&C: Monitoring & Control

- DSA: Data Storage and Archiving services

#### Automation

Function

This function provides automation of pre-defined operations procedures.  It supports execution of procedures when invoked to do so via the provided Automation interface, providing feedback on execution status.

It uses the M&C service to invoke lower level Actions and monitor status through receipt of Parameter status.  It may also respond to received Alerts, and raise Alerts.  If available, it may also integrate with other services to support their automation: on-board configuration management and the iterative invocation of automated procedures (or other automated functions) via the Automation service.

It should be noted that Automation can be deployed on-board the spacecraft as well as within the ground segment, although the implementation and available service set may differ significantly.

There is currently no standard for Automated Procedure Definitions (APD) although their configuration management and distribution can make use of standard common services.

Provided Interfaces

- AUT: Automation

Required Interfaces

- APD: Automated Procedure Scripts

- M&C: Monitoring & Control (Parameters, Actions and Alerts)

- OSM: On-board Software Management

- OPM: On-board Procedure Management

- AUT: Automation (of other Procedures or Functions)

- DSA: Data Storage and Archiving services

#### Navigation Interface

Function

This function manages the translation of data and services between Mission Control and Navigation functions, in order to isolate other Mission Control functions from specific Navigation Data formats

In principle, the Navigation Interface can receive any Navigation Data format provided by the Navigation and Timing function and use Automation or M&C services to apply it.  For example a Pointing Request Message (PRM) could be used to invoke an automated procedure or discrete telecommand (M&C Action) to perform the pointing, with specific fields from the PRM being mapped to arguments of the Action or Procedure.  What mapping is performed is mission specific.

Required Interfaces

- M&C: Monitoring & Control

- AUT: Automation

- NAV: Navigation Data (ODM, ADM, NEM, PRM)

### Navigation and Timing

Figure 4-6: MOIMS Level 2: Navigation and Timing Functions

The Navigation and Timing functional group is broken down into the following Level 2 functions:

- Time/Position Determination

- Orbit Determination and Propagation

- Attitude Determination

- Manoeuvre Planning

- Conjunction and Re-entry Assessment

- Time Correlation

Similarly, Navigation and Timing Data comprises the following information objects grouped as Navigation Data and Timing Data respectively:

- Navigation Data:

  - ADM: Attitude Data Message

  - CDM: Conjunction Data Message

  - NEM: Predicted Navigation Event Message

  - ODM: Orbit Data Message

  - PRM: Pointing Request Message

  - RDM: Re-Entry Data Message

  - TDM: Tracking Data Message

Note: a Spacecraft Manoeuvre Message had been identified by CCSDS, but its requirements are instead to be satisfied by future extended capabilities within the ODM and ADM.

- Timing Data:

  - TRP: Time Report

  - TRM: Time Reception Message (Earth Reception Time)

  - TCM: Time Correlation Message

#### Time/Position Determination

Function

The function concerns the determination of a measurement of spacecraft position at a particular point in time.  There are various ways in which information relating to spacecraft position can be determined:

- Through on-board reception of GNSS satellite navigation signals

- Ground station tracking and ranging

- Processing of spacecraft observation data

Time and Position determination are combined as they are closely associated and may be determined using the same method.  However it is noted that not specific deployments of this function may only provide Position or Time measurements.

Provided Interfaces

- TDM: Tracking Data Message

- TRP: Time Report

Required Interfaces

None

#### Orbit Determination and Propagation

Function

Measurements of spacecraft position over a period of time are used to determine the characteristics of the spacecraft orbit (the orbit vector).  The orbit vector is then propagated forward in time to predict the spacecraft’s future position as a set of orbital ephemerides; and also to predict required orbital events, such as periods of ground station visibility, or sensor blindings.

Other data may be used to augment Tracking Data in determining the future orbit and/or events.  This includes the state of the spacecraft (M&C), its Attitude and notification of planned manoeuvres (using augmented Orbit Data Message).

Provided Interfaces

- ODM: Orbit Data Message

- NEM:  Predicted Navigation Event Message

Required Interfaces

- TDM: Tracking Data Message

- ADM: Attitude Data Message

- ODM: Orbit Data Message [in event of a manoeuvre]

- M&C: Monitoring & Control [to access status of Navigation Hardware]

#### Attitude Determination

Function

Measurements from on-board sensors over a period of time combined with the latest orbit vector are used to determine the characteristics of the spacecraft attitude (the attitude vector).  This is then propagated forward in time to predict the spacecraft’s future attitude.  Information on spacecraft attitude may also be derived from the processing of image data acquired by the spacecraft.

Other data may be used to augment on-board determining the future attitude.  This includes Pointing Requests and notification of planned manoeuvres (using augmented Attitude Data Message).

Provided Interfaces

- ADM: Attitude Data Message

Required Interfaces

- M&C: Monitoring & Control [to access status of Navigation Hardware]

- ODM: Orbit Data Message

- NEM: Predicted Navigation Event Message

- ADM: Attitude Data Message [in event of a manoeuvre]

- ADM: Attitude Data Message (from Mission Data Processing)

#### Manoeuvre Planning

Function

The Manoeuvre Planning function supports the derivation of the parameters for required orbital corrections, whether for station keeping or to change trajectory.  Where possible, manoeuvres are optimized to minimize fuel consumption.  The requirements for manoeuvres may be linked to the overall mission plan, result from station keeping policy, or be in response to notification of a potential collision.

Once a planned manoeuvre is approved, Mission Planning & Scheduling services can be used to request its inclusion in the mission plan.

Provided Interfaces

- ODM: Orbit Data Message [augmented to support translational manoeuvres]

- ADM: Attitude Data Message [augmented to support rotational manoeuvres]

Required Interfaces

- ODM: Orbit Data Message

- ADM: Attitude Data Message

- M&C: Monitoring & Control [to access status of Navigation Hardware]

- CDM: Conjunction Data Message (collision warning)

- RDM: Re-entry Data Message

- MPS: Mission Planning & Scheduling (manoeuvre planning request)

#### Conjunction and Re-entry Assessment

Function

Conjunction and Re-entry Assessment may be implemented as distinct functions or supported as part of a wider Space Situational Awareness (SSA) or Space Surveillance and Tracking (SST) function.

Conjunction Assessment compares the orbits of multiple spacecraft, solar system objects and space debris to identify predicted conjunctions (or potential collisions).  This requires input of the current orbit vector for each spacecraft to be considered and details of any planned manoeuvres.

Re-entry Assessment models the trajectory of a spacecraft as its orbit decays and predicts the time and position of re-entry.  Re-entry data includes remaining orbital lifetime, start and end of the re-entry and impact windows, impact location, and object physical properties.

Provided Interfaces

- CDM: Conjunction Data Message

- RDM: Re-entry Data Message

Required Interfaces

- ODM: Orbit Data Message

- ADM: Attitude Data Message

#### Time Correlation

Function

Time Correlation derives time correlation coefficients between the standard reference timeframe and on-board clocks, to enable timestamps specified in terms of the on-board clock to be accurately converted to an absolute time (and vice versa for uplinked timetags).

Each correlation measurement requires two elements:  a time report from the on-board clock and a precise timestamp in terms of the standard reference timeframe that can be associated with it.  The time report can be any timestamp in a downlinked message, although it is common to use a specific time report which gives greater resolution.  The reference time is usually provided by accurately recording the Earth Reception Time of this message, which in conjunction with knowledge of the orbit vector and location of the ground station can be used to derive the generation time of the time report.  For spacecraft in Earth orbit, the reference time may be generated on-board from received GNSS signals.

Provided Interfaces

- TCM:  Time Correlation Message

Required Interfaces

- TRP: Time Report

- TRM: Time Reception Message

- ODM: Orbit Data Message

### Mission Planning and Scheduling

Figure 4-7: MOIMS Level 2: Mission Planning and Scheduling Functions

The Mission Planning & Scheduling functional group is broken down into the following Level 2 functions:

- Planning

- Plan Execution

Planning itself may be hierarchical and distributed, while there may be multiple Plan Execution functions within the same system.

Mission Planning & Scheduling data comprises the following information objects:

- PRQ: Planning Requests

- PLN: Plans

- PPM: Planning Process Management

- PEM: Plan Execution Management

Planning Process and Plan Execution Management are concerned with the monitoring and control of the Planning and Plan Execution functions respectively.  The associated information objects and services could therefore be expressed as specialisations of generic MO Monitoring & Control.

#### Planning

Function

Planning is the function responsible for performing Mission Planning and Scheduling. Internally it may be hierarchically organized and/or distributed. Planning requests are received from multiple users of Planning services and feedback on their status provided. Other functions  may also perform high-level control of the planning processes supported by the Planning function. The output of the Planning function is plans, which may be retrieved by Planning users and distributed to Plan Execution functions. Planning may also control the execution of plans via the Plan Execution functions. Planning is itself a user of predicted orbital events; and negotiates the scheduling of ground station support via CSS Service Management.

Provided Interfaces

- PRQ: Planning Request

- PLN: Plans

- PPM: Planning Process Management

Required Interfaces

- PDB: Planning Configuration Data

- NEM: Predicted Navigation Event Message

- ODM: Orbit Data Message

- ADM: Attitude Data Message

- CDM: Conjunction Data Message (collision warning)

- RDM: Re-entry Data Message

- CSS-SM: CSS Service Management

- M&C: Monitoring & Control Data (current mission status)

And in the context of hierarchical/distributed Mission Planning:

- PRQ: Planning Request

- PLN: Plans

- PPM: Planning Process Management

#### Plan Execution

Function

Plan Execution is the function responsible for executing a plan (or part of it). There may be multiple Plan Execution functions distributed between space and ground segments. It is not a Planning function itself, but it does support a common model of the plan in its interface with Planning. It receives or retrieves distributed plans; allows external control of the Plan Execution process; and provides execution status of the plan to Planning. Plan Execution may use underlying Mission Control Services to effect the execution of planned activities

Provided Interfaces

- PRQ: Planning Request

- PLN: Plans

- PEM: Plan Execution Management

Required Interfaces

- PDB: Planning Configuration Data

- M&C: Monitoring & Control Data

- AUT: Automation

- NEM: Predicted Navigation Event Message

- PPM: Planning Process Management (to invoke next planning cycle/replanning)

- CSS-SMES: CSS Service Management Event Sequence

### Operations Preparation

Figure 4-8: MOIMS Level 2: Operations Preparation Functions

The Operations Preparation functional group is broken down at Level 2 into a Configuration Management and Distribution function plus a set of editor functions for defining each class of configuration data.

Operations Preparation Data comprises the following information objects:

- APD: Automated Procedure Definition

- OSW: On-board Software

- PDB: Planning Database (Planning Data Definitions)

- SDB: Spacecraft Database (Monitoring & Control Data Definitions)

None of these information objects are currently standardized by CCSDS, however they may be embedded in or referenced by standard services.

XTCE provides a standardized data representation for the exchange of telemetry and command definitions, which is a subset of the Spacecraft Database.

#### Configuration Data Definition (Editors)

Functions

Each class of mission operations configuration data has a dedicated function for its development and maintenance, which may be referred to as an Editor.   These include:

- Spacecraft Database Definition

- Automated Procedure Definition

- On-board Software Definition

- Planning Data Definition

All of these interact with the Configuration Management and Distribution function to place versions of their data under configuration control. The configuration management operations of the MO Common Service for Configuration can be used to support this.

The spacecraft (or payload) manufacturer typically provides at least initial versions of the Spacecraft Database, on-board Automated Procedure Definitions and On-board Software.  These may not be in a format compatible with the mission operations system and so require ingestion by the associated Operations Preparation Definition Editor.  There is currently no CCSDS standard for the exchange of data between the Spacecraft Development and Maintenance function and the Operations Preparation function, with the exception of XTCE.

Definition Editors that are responsible for configuration data that is to be deployed on-board the spacecraft (On-board Software; Automated Procedures) may use the Mission Planning Request Service (PRQ) to plan the upload of new versions of on-board configuration data.

Provided Interfaces

- SDB, APD or OSW Ingestion (not standardized)

- XTCE: Telemetry and Command Exchange (partial SDB ingestion)

Required Interfaces

- MO Common: Configuration Service (configuration management operations) applied to SDB, APD, OSW or PDB.

- PRQ: Planning Request

#### Configuration Management and Distribution

Function

The Configuration Management and Distribution function provides a configuration controlled repository of Mission Operations configuration data, including version control and history for individual configuration data items, together with their current validation status and compatibility with versions of other configuration data items.

The function is also responsible for the distribution of configuration data versions to their target functions.  For SDB, APD and OSW this is to the Mission Control function (which is itself responsible for managing on-board configuration of APD and OSW); and for PDB this is the to the Mission Planning & Scheduling function. The configuration distribution operations of the MO Common Service for Configuration can be used to support this.

The storage of [historical] configuration data versions may be delegated to an external Operations Data Archive function.

Provided Interfaces

- MO Common: Configuration Service (configuration management and distribution operations) applied to SDB, APD, OSW or PDB.

Required Interfaces

- DSA: Data Storage and Archiving services

### Data Storage and Archiving

Figure 4-9: MOIMS Level 2: Data Storage and Archiving

The Data Storage and Archiving functional group comprises three separate functions related to the storage and archiving of mission data associated with any of the other Functional Groups or Mission Data Products:

- On-board File Store

- Operations Archive

- Data Archive

The data subject to Data Storage and Archiving is that generated by other Functional Groups, but specific information objects are identified for the packaging of data associated with the Data Archive function:

- AIP: Archival Information Package

- DIP: Dissemination Information Package

- SIP: Submission Information Package

Services are identified for the submission and dissemination of data to/from the Data Archive function:

- CAIS: Consumer Archive Interface Specification

- PAIS: Producer Archive Interface Specification

#### On-board File Store

Function

The On-board File Store is a temporary on-board repository for files containing Mission Control, Mission Planning, Navigation and Operations Preparation Data or Mission Data Products, which may be generated on-board or uploaded from the ground.  There are special services for File Management and File Transfer [the latter delegated to a lower level protocol, such as CFDP or FTP], that can be used by MOIMS functions to interact with the file store.  There is an existing CCSDS File Store Model contained within the CFDP Standard [Ref xx].

The CCSDS MO Data Product Distribution service may also be provided to disseminate Mission Data Products, either directly to an external Mission Data Processing function or to the Operations Archive.

Provided Interfaces

- MO File Transfer

- MO File Management

- MO Data Product Distribution

Required Interfaces

- MCS:  Mission Control Data (files)

- NAVT:  Navigation and Timing Data (files)

- MPS:  Mission Planning & Scheduling Data (files)

- MDP: Mission Data Products (files)

#### Mission Operations Archive

Function

The Mission Operations Archive is a ground-based repository of historical mission operations data of all types.  This is used for rapid storage and retrieval by Mission Operations functions.  It may persist for the mission lifetime.

The MO Data Product Distribution Service may be used for dissemination of Mission Data Products.

Provided Interfaces

- Archiving and Retrieval of MCS, MPS, NAVT, OPD and MDP data.  The MO COM Archive service may be used for data compliant with the MO Common Object Model.

- MO DPD: Data Product Distribution

Required Interfaces

- MCS:  Mission Control Data

- MPS:  Mission Planning & Scheduling Data

- NAVT: Navigation & Timing Data

- OPD:  Operations Preparation Data

- MDP: Mission Data Products

#### Mission Data Archive

Function

The [Long-Term] Mission Data Archive is a ground-based repository of mission data products with ancillary mission operations data intended to ensure long-term preservation of data and to provide access to external users.  This function is subject to standardization by the Data Archive Ingestion (DAI) working group.

It may be further decomposed into three sub-functions:

- Data Archive Ingestion

- Data Archive Storage

- Data Archive Access

Data to be archived is submitted to the Data Archive Ingestion function as a Submission Information Package (SIP), using a service conforming to the Producer Archive Interface Specification (PAIS).  SIPs are restructured (typically collated into larger data items) as Archival Information Packages (AIP), that are stored in the Data Archive Storage function.

AIPs may be subsequently retrieved by the Data Archive Access function which restructures them for dissemination to Users as Dissemination Information Packages (DIP).  Users interact with the Data Archive Access function through a service conforming to the Consumer Archive Interface Specification (CAIS).

Provided Interfaces

- PAIS: Producer Archive Interface

- CAIS: Consumer Archive Interface

## SOIS Functions

This section addresses functionality within the scope of the CCSDS Spacecraft Onboard Interface Services (SOIS) Area.

Functional groups are differentiated by colour coding as introduced in §3.3.1. The following diagram shows the colour coding used to distinguish different functional areas within the CCSDS MOIMS Area diagrams.

Figure 4-10: Colour Coding of SOIS Functional Areas

The remainder of the section is structured as follows:

- SOIS Area Context: scope of the SOIS Area

- SOIS Area Functions: functional decomposition of SOIS Area

- Protocol Convergence Functions: future convergence functions for subnetworks

- Management Functions: future functions to expose management information blocks (MIB) as services

For descriptions of the information objects and services that are also shown in the diagrams, reference should be made to the Information and Service Views respectively.

### SOIS Area Context

Figure 18 is a summary of the architecture of SOIS.  Because SOIS is concerned both with the interoperable integration of onboard functions and with the functions themselves, the architectural diagram is necessarily a hybrid of information and functional symbols.  The diagram consists of an information model on the left, and onboard system functions on the right.  Subsequent diagrams throughout this publication tease apart the details into more comprehensible units of exposition.  The important things to recognize in Figure 18 are the following:

- SOIS consists of application support, subnet, and device support services in an onboard computing platform.

- SOIS includes a model of that system, which enables a design-time tool chain to adapt services to the platform architecture chosen for a project.

The “Applications” function in Figure 18 includes any on-board functions of MOIMS.  The figure appears to arrange the on-board functions in four layers, which might be mistaken as directly relating to identified layers in the Open Systems Interconnect (OSI) model.  However, the application support functions are really a part of what is normally considered the application layer, and the subnet functions are more closely aligned with the ISO link layer. Due to the variety of protocol capabilities of on-board subnetworks, these may be integrated into the on-board computing platform, as will be explained in Section 7.4.

Figure 4-11: Summary of SOIS Architecture

Network protocols may be utilized on-board as well, in which case they will typically be layered, as is usual, above the subnet protocols and below any applications.

### SOIS Area Functions

The SOIS Green Book [4] describes services that facilitate communication between mission applications and onboard devices.  Figure 19 summarizes those services.

Figure 4-12: Functional Summary of SOIS

The mission applications that use SOIS services appear as a wild-card function at the top of Figure 19.  A set of device services, which are derived from SOIS EDSs, provide command and data handling functions for onboard devices.  The device services utilize SOIS subnet functions to communicate with devices connected via a variety of different onboard subnetwork technologies.

### Protocol Convergence Functions

One of the features of SOIS is the capability to compose a stack of “convergence” functions to provide uniform types of services across a variety of subnetworks that individually provide different levels of service.  The convergence functions are mentioned in the SOIS Green Book [1], but have not yet developed in relation to the SOIS model; for this reason, the diagram below shows the convergence functions in an arbitrary order.

Figure 4-13: SOIS Convergence Functions

### Management Functions

The “Management Services” shown in Figure 4-11 contains a collection of functions that are not directly in the path for data communications, but which define and control the path for data communications.

Figure 4-14: SOIS Management Services

The following additional management functions have been identified.

- Onboard device manifest

- Spacecraft Address/Subnet routing table

- Subnet/Subnet channel quality of service (QoS) table

A set of single-valued management parameters have also been identified.  These parameters can be specified in a mission-parametric SEDS.

- Out-of-sequence buffer size for SOIS sequence preservation convergence function

- Maximum frame size within a subnet

- Unacknowledged buffer size for SOIS retry convergence function

## Integrated Flight/Ground Functions (future territory, ???)

## Security Concepts for Functional View

<< Specific security functions, access control, encryption, authentication, key management.  Anything still in the process of “becoming” gets marked [Future]. >>

      - Neither MOIMS nor SOIS have defined an extensive suite of security features at a functional level.  The assumption is that terrestrial service interfaces will be secured, and that will be addressed in the Service viewpoint.  MOIMS does include an optional access control function that supports control over user access to some set of functions.

      - The Telemetry, Tracking and Commanding (TT&C) interfaces that MOIMS uses to communicate with spacecraft typically use access control at the function/service interfaces and often also utilize physical and network security mechanisms as well.  See the relevant sections of the SCCS-ADD for further descriptions of these security practices. The operational facilities, and the systems themselves, running the MOIMS MPS, MCS, NAVT, and OPD functions are also typically secured.

      - As for SOIS, many spacecraft are not directly secured in the sense of using any kind of overt access control or authentication mechanisms, and SOIS itself does not define any.  CCSDS, in the Systems Engineering Area (SEA), particularly the Security Working Group, in the SLS Space Data Link Security Working Group, and in the SIS Space Internetworking Area security protocols have been defined.  These are all available for use, terrestrially, over the space link, and on-board, but SOIS has not defined any particular uses of these functions.

# Information View (Information Objects)

## Overview

<< Information view covers the structure and contents (syntax & semantics) of the various information objects that are defined.  Anything still in the process of “becoming” gets marked [Future]. >>

## MOIMS Information Objects

This section addresses data or information objects within the scope of the CCSDS Mission Operations and Information Management Services (MOIMS) Area.  Information is only introduced at a relatively high-level, sufficient to identify the information exchanged between functions and any relationships between information objects exchanged across multiple interfaces.  For a full and detailed specification of the referenced information objects, the reader is directed to the relevant CCSDS standards.

Some information objects correspond to complex file-based schemas that are often relatively self-contained and self-documenting.  Other information oobects are simpler in data structure, but exposed at service-based interfaces where they have associated dynamic behavior: reporting status and being subject to discrete operations that may affect their state.

The remainder of the section is structured as follows:

- MOIMS Information Groups: top-level decomposition of MOIMS Area Information

- MO Common Object Model: introduction to the generic information model for Mission Operations services.

- Common Services Data:  applicable across multiple functional groups

- <Functional Group>: a set of sub-sections, one per MOIMS Area Functional Group, providing the second level decomposition into information objects.

Each subsection comprises an Information View diagram and a description of each of the information objects it contains.  Note that some smaller information groups are expanded within the top-level composition diagram.

### MOIMS Information Groups

Figure 5-1: MOIMS Information Groups

MOIMS Information is grouped into the same functional areas identified in the Functional View, but with an additional group relating to Mission Data Products that originate outside the MOIMS Area, but are exchanged with MOIMS functions:

- Mission Control (MCS)

- Navigation and Timing (NAVT), which is further decomposed into:

  - Navigation (NAV)

  - Timing (TIM)

- Mission Planning and Scheduling (MPS)

- Mission Data Products (MDP)

- Operations Preparation Data (OPD)

- Data Storage and Archiving (DSA), which comprises:

  - Data Archive Information (DAI)

  - File Transfer and Management (FTM), MO service for management of remote file stores

  - Common Archive (CAR), part of the MO Common Object Model (COM)

- Common Services Data (CSD)

The top-level information object associated with each of these groups is essentially an aggregation of lower-level information objects that are defined within specific CCSDS standards.

The Mission Operations (MO) service framework includes both a Common Object Model (COM) and Common Services, for which more detail is provided in the following two subsections.

Operations Preparation Data (OPD) relates to the configuration data required for other functional groups and defines the information objects that exist within those groups.  For this reason it has been fully expanded in the top-level diagram above to show how its information objects relate to those of other groups.  This applies primarily to Mission Control and Mission Planning, but could be extended to cover Navigation and other information groups.

Navigation and Timing (NAVT) comprises a set of defined message formats, listed in the diagram but detailed in the dedicated section below.

### MO Common Object Model

Figure 5-2: MO Common Object Model and Object Patterns

The Mission Operations (MO) service framework includes a Common Object Model (COM) from which the information objects associated with an MO-compliant service specification must be derived.

#### Domain

The majority of MOIMS Information (whether derived from the COM or not) may be scoped by the concept of domain, which provides a namespace context for the identity of an information object.  Domains may be hierarchical and correspond to a real-world entities such as a space mission, a spacecraft (or ground system) within that mission, or its component subsystems and equipment.

#### MO COM Object

At the core of the MO COM is a root abstract class, the COM Object (see Figure 18), from which MO service information objects may be derived.

COM Objects have:

- An Identity, including its domain

- A reference to a Source object that may be the originator or parent of the object.  This can be used to establish an audit trail of operations, for example from Planning Request > Planned Activity > Automated Procedure > M&C Activity [Command].

- A reference to a Related object, the meaning of which is specific to the derived information object.  For example, an instance of an object may use this to point to its definition.

Any information objects defined in terms of the COM can be packaged as Common Archive (CAR) data using the generic MO COM Archive service.

#### MO COM Event

The COM Event is itself derived from the COM Object, but additionally has a Time attribute that represents its occurrence at a specific point in time.  It is defined as a specific object representing ‘something that happens in the system at a given point in time’.  Any information objects derived from the COM Event can also make use of the generic COM Event service.  Each MO service can define the specific events that it supports.

#### MO COM Object Patterns

The information objects associated with MO compliant services are often implemented as a compound set of objects, each of which is derived from the MO COM Object.  These compound objects often follow one of a number of common object patterns.  Some of these patterns are themselves standardised within the MO Common Object Model, while others are inferred here from repeated patterns found in standard MO services.

Four such COM Object Patterns are illustrated in Figure 18:

- COM Activity

- COM State

- COM Static Item

- COM Defined Event

These share the following common elements:

- A unique Identity that may be used to reference all occurrences of the information object (compound object) throughout the mission lifetime.  This is a combination of the domain and a unique name within the domain.

- A Definition that comprises the statically declared information associated with the information object.  This may, for example, include a description, set of defined arguments or any other information that applies to all occurrences of the information object.  There may be multiple Definitions [versions] over the mission lifetime associated with the same Identity, each with its own unique Definition ID.  Definitions are typically contained in configuration databases that are maintained off-line under version control and deployed for use in the on-line environment.

The Related object reference of a Definition points to its Identity.

- An Instance that maintains the current status associated with the information object or a specific occurrence of it.  This may, for example, include a current value or set of values for defined arguments.  Each occurrence of the information object has a separate Instance, with its own unique Instance ID.

The Related object reference of an Instance points to its Definition.

- An Event (derived from COM Event) that represents an occurrence at a specific point in time.  These may be used to represent changes in state of an Instance, in which case the Source object reference of the Event points to the associated Instance.  Events may be used to disseminate changing status and to record the detailed status history of the information object.

For specific information objects, according to which pattern it follows, there will be a specialised object derived from the COM Object associated with each of these elements.

##### COM Activity Object Pattern

A COM Activity is a compound information object representing any type of operation that is repeatable and extends over a measurable period of time.  Examples include M&C Actions, M&C Procedures and MPS Activities.

COM Activity comprises all four of the elements described above: Identity, Definition, Instance and Event.  As a COM Activity may be invoked multiple times, there may be multiple Instance objects created from the same Definition object.

The COM Activity is formally specified within the MO Common Object Model, together with an associated generic COM Activity Tracking service that enables distribution of the evolving status of the COM Activity.

##### COM State Object Pattern

A COM State is a compound information object representing a status: it is a persistent object for which there is only one status value at any given time (although this may not be known).  Examples include M&C Parameters and MPS Planning Resources.

COM State is similar to COM Activity, comprising all four of the elements described above: Identity, Definition, Instance and Event.  As it is persistent rather than instantiated, there is notionally only a single Instance object created from each Definition object.

##### COM Static Item

A COM Static Item is a compound information object that only comprises statically declared information with no evolving status.  Examples are M&C Checks and Conversions.

COM Static Items therefore only comprise the Identity and Definition elements described above.

##### COM Defined Event

A COM Defined Event is a compound information object that represents an occurrence of a specific defined type at a specific point in time.  An example is the M&C Alert.

COM Defined Event comprises the Identity, Definition and Event elements described above.  There is no associated Instance object, instead the Related object reference of the Event points directly to its Definition.

### MO Common Services Data

MO Common Services Data is applicable across multiple functional groups as it relates to the MO Common Services (Directory, Login and Configuration) that can be used in conjunction with any other MO Service.  Configuration data is closely associated with the Operations Preparation functional group and detailed in §5.2.8 below.

The following information objects (shown in Figure 17) are specific to Common Services:

#### Service Directory (SDIR)

The Service Directory (SDIR) is a list of all MO Services available within a given system, together with the following information:

- Service ID and Version

- Description

- Service Scope (by domain)

- Supported Capability Sets/Operations

- Service Provider(s)

The Service Directory may be updated and interrogated using the MO Common Directory service.

#### Login and Authentication Credentials (LAC)

Login and Authentication Credentials (LAC) are the information associated with user access control and include:

- User ID

- Password

- Access Rights (by Service, Domain and Capability Set/Operation)

The information is used in conjunction with the MO Common Login service.

### Mission Control Data

Mission Control Data is closely associated with the Mission Control functional group and its sub-functions.  As the information model associated with the Monitoring & Control (M&C) function is relatively complex, this is addressed separately below.

#### Monitoring and Control Data

The following diagram shows the top-level information objects associated with the Monitoring & Control function and the CCSDS MO Monitoring & Control services.

Figure 5-3: Mission Control Data (Monitoring & Control)

The core information objects of Monitoring & Control are parameter, action and alert.  Parameters can also have associated monitoring checks, statistics and raw to engineering value conversions.  Sets of parameters can also be defined as parameter aggregations.

##### Action

An action is a single executable task of an M&C service provider, for example a telecommand, but it can also be a ground system command or any other discrete operation.

Actions follow the MO COM Activity object pattern, comprising separate identity, definition, instance and event objects.  A new Instance object is created each time the action is invoked.  The state of progress of an action is reflected in a series of event objects associated with each action instance.

Actions are typically defined with an associated set of arguments that can be used to parameterise the associated executable task. In this case, the argument definitions are contained within the action definition, and the argument values within the action instance.

##### Alert

An alert corresponds to any operationally significant event that is raised asynchronously by an M&C service provider.

Alerts follow the MO COM Defined Event object pattern, comprising identity, definition and event objects.  A new Event object is created each time the alert is raised.

Alerts may be defined with an associated set of arguments that can be used to provide more detailed information about the alert event.  In this case, the argument definitions are contained within the alert definition, and the argument values within the alert event.

##### Parameter

A parameter is a single unit of data reported by an M&C service provider, for example a telemetry parameter, but it can be any discrete item of monitoring data.  Parameters can be of any supported data type

Parameters follow the MO COM State object pattern, comprising identity, definition, Value instance and event objects.  There is a single Value Instance object associated with each parameter definition, which holds the current status of the parameter.

Parameters may have both raw and engineering values.  Raw values are unconverted, while engineering values have been calibrated to have a meaningful value in defined engineering units.  The parameter definition can reference a conditional list of conversions to be applied under different circumstances.

Parameters may also have an associated validity condition that indicates whether the value is meaningful or not (for example, a reported parameter may be invalid if the equipment generating it is not powered).

Parameters may also have associated conversions, checks and statistics (see below).

Sets of parameters may also be defined as aggregations.

##### Aggregation

An aggregation is a collection of parameters provided as a set by a service provider.

Aggregations follow the same MO COM State object pattern as parameters, comprising identity, definition, instance and event objects.

##### Check

A check may be defined and applied to parameters by an M&C service provider which then reports the check results.  Check types are extensible, but include:

- limit check: the parameter value lies within a specified range

- constant check: the parameter value is checked against a specified value or the value of another parameter

- delta check: the change in value is checked against a pair of thresholds

The structure of checks follows the MO COM Static item object pattern, comprising identity and definition objects (specific to check type). This is then extended by check link and check link definition objects to associate the check with one or more parameters.  Check transition events are generated by the M&C service provider to report changes in check result for a given check-parameter pair.

##### Conversion

A conversion may be defined an applied to parameters by an M&C service provider to convert raw parameter values to engineering values.

Conversions follow the MO COM Static item object pattern, comprising identity and definition objects (specific to conversion type).  The linkage to parameters is provided within the parameter definition object, the result of conversion [engineering value] stored in the parameter instance object and reported through parameter event objects.

##### Statistic

A statistic is a defined statistical evaluation (for example: min, max, mean, standard deviation) associated with parameters that is evaluated and reported by an M&C service provider.

Statistics have a bespoke structure of MO COM objects.  Supported statistical evaluations are defined as statistic function objects; the linkage to parameters is provided through statistic link and statistic link definition objects; the result of the statistical evaluation is stored in a statistic value instance object.

##### Group

A group is a collection of COM Objects of the same type. The MO M&C group service provides a mechanism for other services to reference sets of their own objects using a single group reference.

Groups follow the MO COM Static item object pattern, comprising identity and definition objects.

#### Other Mission Control Data

The following diagram shows the top-level information objects associated with the Automation and On-board Configuration Management functions and how they relate to the Monitoring and Control information objects described above.

Figure 5-4: Mission Control Data (Other Services)

##### Procedure

A procedure is a single executable task of an Automation service provider, which may have an extended duration.  A procedure may correspond to a simple predefined sequence of actions, a complex procedure script or a software function that is executed automatically by the service provider.

Procedures may be defined in terms of other Mission Control information objects:

- Referencing and/or setting parameter values

- Initiating [sending] and tracking actions

- Receiving and raising alerts

- Loading and/or Dumping OBSW images

- Loading and/or Dumping OBCP definitions

Procedures, like simple actions, follow the MO COM Activity object pattern, comprising separate identity, definition, instance and event objects.  A new Instance object is created each time the procedure is invoked.  The state of progress of a procedure is reflected in a series of event objects associated with each procedure instance.

Procedures are typically defined with an associated set of arguments that can be used to parameterise the associated executable task. In this case, the argument definitions are contained within the procedure definition, and the argument values within the procedure instance.

##### OSM: OBSW Image

On-board Software Management (OSM) data is associated with the on-board configuration management function and its management of On-Board SoftWare (OBSW).  This includes OBSW Images that may be transferred to or from a spacecraft.

##### OPM: OBCP Definition

On-board Procedure Management (OPM) data is associated with the on-board configuration management function and its management of On-Board Control Procedures (OBCP). This includes OBCP Definitions that may be transferred to or from a spacecraft.

### Navigation and Timing Data

The information objects associated with the Navigation and Timing functional group relate to one or more domains: spacecraft, ground stations and celestial bodies, as illustrated in the following diagram.  They fall into two main sub-groups: Navigation Data (NAV) and Timing Data (TIM).

Figure 5-5: Navigation and Timing Data

Navigation Events and Conjunctions contain the predicted timing of events that are significant to Mission Planning, and may be instantiated as Planning Events.

Similarly Planning Requests associated with the pointing of a spacecraft, instrument or antenna, may be elaborated by a Pointing Request Message.

#### NAV: Navigation Data

Navigation Data comprises a set of standardised messages relating to the Navigation functions.  All have currently been defined as XML schema and are typically exchanged as files.

##### ADM: Attitude Data Message

The Attitude Data Message (ADM) contains information that defines the attitude state of a spacecraft at one or more times.  The ADM supports two message formats:

- Attitude Parameter Message (APM)

- Attitude Ephemeris Message (AEM)

The APM consists of an instantaneous attitude state and optional additional information, including planned manoeuvres, that enable a consumer with attitude modelling capability to propagate the spacecraft attitude over time.

The AEM comprises a history or forecast of the spacecraft’s attitude as a series of attitude states at specific points in time.  A consumer can use interpolation techniques to determine the attitude states at arbitrary times within the span of the ephemeris.

Augmentation of the ADM is proposed to support the exchange of information on planned rotational manoeuvres.

##### CDM: Conjunction Data Message

The Conjunction Data Message (CDM) contains information that defines the relationship between the orbit states of different space objects at different times.

The CDM is the final product of Conjunction Assessment and can be used to provide spacecraft operators with the information they need to assess the risk of collision and plan collision avoidance manoeuvers if necessary.  The CDM notifies the spacecraft operator(s) of possible conjunctions with another space object and enables consistent warning by different organisations employing diverse CA techniques.  It comprises the identity of the affected objects, miss distance, probability of collision, and the relative position and velocities of the objects at the time of closest approach.

##### NEM: Navigation Event Message

The Navigation Event Message (NEM) contains the predicted timings of orbital events, such as ground station visibilities, sensor blindings, eclipses, etc.  The NEM is associated with a single object (usually a spacecraft) and contains all the predicted geometric events occurring within a specified time window.  For each event the following information is provided:

- Event Type

- Unique Event ID

- Predicted Time of the Event

- Related Object (e.g. celestial body or ground station ID) if relevant for the event type

- Duration of event if relevant

The NEM is an output of Orbit Propagation and is a key input to Mission Planning.

##### ODM: Orbit Data Message

The Orbit Data Message (ODM) contains information that defines the orbit state of a spacecraft at one or more times.  The ODM supports three message formats:

- Orbit Parameter Message (OPM)

- Orbit Mean-Elements Message (OMM)

- Orbit Ephemeris Message (OEM)

The OPM specifies the orbital state (single position and velocity in Cartesian coordinates) or osculating Keplerian elements of a spacecraft at an instant of time, while the OMM specifies the characteristics of the spacecraft orbit expressed in mean Keplerian elements at a specified epoch.  Neither the OPM nor OMM is designed for higher fidelity propagation.  However, the OPM allows the user to specify simple parameters related to finite and instantaneous manoeuvres, and provides simple parameters for the modelling of solar radiation pressure and atmospheric drag.

The OEM comprises a history or forecast (prediction) of the spacecraft’s orbit as a series of orbital state vectors at specific points in time and allows for the modelling of any number of gravitational and non-gravitational accelerations. The consumer can be use interpolation to obtain the spacecraft position and velocity state at times other than those explicitly contained in the message.

Augmentation of the ODM is proposed to support the exchange of information on planned translational manoeuvres.

##### PRM: Pointing Request Message

The Pointing Request Message (PRM) contains information on the desired attitude state of an object (spacecraft, instrument or antenna) at one or more times.  It provides a common and standardised format for the exchange of pointing requests between the requestor and spacecraft operators.

##### RDM: Re-entry Data Message

The Re-entry Data Message (RDM) contains information about a single re-entry event:

- information about the message itself (creation date, originator, etc.)

- identification of the re-entering object (name, id)

- basic re-entry information (mandatory): remaining orbital lifetime, whether the re-entry is controlled or not, and which celestial body the object is orbiting

- more complex re-entry information (optional): re-entry and impact windows, impact location and probabilities, state vector, object properties, the OD process, and observations used to predict the re-entry

The information is used by satellite operators, civil protection, or aviation authorities to assess the re-entry risk and plan any needed mitigation measures.

The RDM is not limited to man-made objects re-entering the Earth’s atmosphere. It could be used for any entry/impact event (e.g. a space probe landing on Venus, or an asteroid impacting Earth).

##### TDM: Tracking Data Message

The Tracking Data Message (TDM) contains information that can be used to determine the orbit state of a spacecraft.  It specifies a standard format for a single message type used in the exchange of spacecraft tracking data.  Currently the following tracking data types are supported:

- Ground-based Radiometric Tracking Data:

  - uplink and downlink frequencies

  - range, differenced range and range rate

  - Delta Differential One-way Ranging (Delta-DOR)

  - doppler (1-, 2-, 3- and 4-way) and differenced doppler

  - antenna angles (azimuth and elevation)

  - interferometric types

  - optical data (planned)

- Spacecraft-to-Spacecraft Doppler and Range

- Ancillary Information required to calculate measurement residuals: meteorological data (weather), media delays/correction, and clock bias/drift measurements

It is noted that the TDM does not currently support direct measurements of position, such as may be acquired using satellite navigation systems or image processing.

#### TIM: Timing Data

Three key information exchanges associated with timing have been identified, but CCSDS has not yet defined standards to support these, with the partial exception of Time Reception (see §5.2.5.2.2 below).

##### TRP: Time Report

A Time Report contains a full resolution time generated by a spacecraft on-board clock and transmitted to ground immediately, or with a known on-board processing delay.  The report is required, in conjunction with an associated Time Reception Message, to support correlation of the on-board clock to the mission time reference.

Other messages generated by the spacecraft may also contain a timestamp generated from the on-board clock, but these timestamps may not be at full resolution and may have variable on-board processing delays.

There is currently no CCSDS standard for the Time Report.

##### TRM: Time Reception Message

A Time Reception message is associated with a Time Report and provides an accurate timestamp (in terms of the mission time reference) of the ground reception time of that Time Report message.

While there is no current CCSDS standard for a dedicated Time Reception Message, the functionality is supported through the CCSDS CSS Space Link Extension Transfer Services, where Earth Receive Time is provided as an annotation parameter to the transfer data.  Assuming the Time Report itself is carried in a discrete frame or packet, the Earth Receive Time of the container frame or packet can be associated with it.

##### TCM: Time Correlation Message

The on-board clock correlation function takes Time Reports and associated Time Reception Messages, together with any required ancillary information such as:

- One-way light time between spacecraft and ground reception station, which can be derived from the spacecraft’s orbit vector and known location of the ground station.

- Statically defined processing delays: on-board the spacecraft in generating and transmitting the Time Report; and within the ground station in terms of generating the Time Reception Message

to derive on-board clock coefficients that specify the relationship between on-board clock and the mission reference time at a given point in time.  This enables conversions between on-board time and reference time to be performed.

The Time Correlation Message (TCM) contains one or a series of on-board clock correlation coefficients, together with their reference times.

There is currently no CCSDS standard for the TCM.

### Mission Planning Data

Figure 5-6: Mission Planning Data

The top-level information objects associated with Mission Planning and Scheduling functional group and how they relate to those of other functional areas are shown in Figure 22 above.

Planning requests and plans are essentially container structures used to represent the input and output respectively of the planning process.

Planning activities, planning events and planning resources are compound objects with associated definitions and instances that represent the items within a plan or planning request.

Planning constraints are rules or conditions applicable to the planning process which can be defined in advance and contained within the definitions of planning activities or planning resources or as self-standing constraints as part of the planning database.  They can also be defined at run-time in the context of a planning request or plan.

#### Plan

A plan is the output of the Planning Process.  It contains a set of selected planning activities associated to time, position or other planning event.  A plan may contain additional related information, including:

- Planning events

- Planning resource vectors

A plan also contains information relating to the plan itself, including:

- Source and Generation Time

- Time Period and domains it relates to

- Predecessor plan, if any – relevant if plans are iterative

In the context of the Mission Planning and Scheduling standardisation activity, there is no distinction between the terms “plan” and “schedule” and only the term plan is used.

#### Planning Activity

A planning activity is a meaningful unit of what can be planned: the building blocks from which plans are constructed.  They may be hierarchical:  planning activities may be composed of other planning activities, but the leaf nodes of the hierarchy must be executable (either automatically or manually) and will typically correlate to Monitoring & Control actions or procedures that are to be initiated when the plan is executed.

Planning activities are instantiated within a plan in response to:

- an explicit planning request

- the inclusion of an associated planning event in the plan

- the occurrence of an associated M&C alert

Planning activities follow the MO COM Activity object pattern, comprising separate identity, definition, instance and event objects.  A new Instance object is created each time the activity is invoked.  The state of progress of a activity is reflected in a series of event objects associated with each activity instance.

Planning activities are typically defined with an associated set of arguments that can be used to parameterise the associated executable task. In this case, the argument definitions are contained within the activity definition, and the argument values within the activity instance.

#### Planning Constraint

A planning constraint is something that limits or restricts the scheduling of planning activities.  A planning constraint could be based on planning resources, with their specific allocation and consumption, but other types of constraint exist, including: time constraints, sequencing constraints, position or other geometric constraints, and exclusion constraints that restrict which activities can be executed in parallel.

Planning constraints can be defined or occur in various parts of the Mission Planning information model:

- within a planning request

- within a plan if it was defined in the context of a planning request

- within the definition of a planning activity or planning resource

- as a self-standing planning constraint that applies globally to the planning process

#### Planning Event

A planning event marks when or where something of significance to Planning is predicted to occur (or exceptionally during plan execution, has occurred).  The execution of planning activities may be linked to planning events.

Predicted events may be internally generated by the Planning function, but typically originate from an external function, such as a Navigation function or the scheduling of TT&C contacts, and a planning event is created within the plan to reference the external event.

Planning events follow the MO COM Activity object pattern, comprising separate identity, definition, instance and event objects.  A new Instance object is created each time the planning event is invoked within a plan.  The state of progress of a planning event is reflected in a series of event objects associated with each planning event instance.

Planning events are typically defined with an associated set of arguments that can be used to parameterise the associated planning activity to be executed. In this case, the argument definitions are contained within the planning event definition, and the argument values within the planning event instance.

#### Planning Resource

A planning resource is an abstract status modelling the state of the system being planned. It may be necessary to model some aspects of system state in order to:

- trigger the execution of a planning activity

- constrain the execution of a planning activity

While the modelling of planning resources is internal to the planning system, planning resources may be referenced in constraints associated with a planning request.  It may also be necessary in a distributed planning system to coordinate the value of planning resources between planning functions, in which case the resource values may be contained within a plan.

Planning resources follow the MO COM State object pattern, comprising identity, definition, Value instance and event objects.  There is a single Value Instance object associated with each resource definition, which holds the current status of the resource.

#### Planning Request

Planning requests are the main input to the planning function and are containers for the information needed to be exchanged between the requester and the planner. It is envisaged that this will support the specification of different types of request:

- request to plan a planning activity or a set of activities

- request to achieve a goal

- request to use a plan as an input to the planning process

- request to modify the content of a plan.

The main characteristic of the Planning Request is that, being a container, it needs to hold references to, or instances of, the constituent information items that are required by the planner and agreed by the interacting parties for exchange at interface level.  It has one or more planning activities as the basis of the request, optionally referencing planning events.

Information about constraints on when a requested activity can or shall be planned may also be exchanged as part of the planning request, by referencing constraints on the time, on the position, on the state of planning resources, or other planning activities.

### Mission Data Products

Mission Data Products (MDP) are mission data sets of potentially large size and varied internal structure that are transferred between distributed elements of a space system.

The MDP concept abstracts the structure, content and format in which diverse space mission data products can be persisted, requested and provisioned. This allows the specification of a generic set of services for managing, requesting and provisioning space mission data products, without making assumptions about the implementations of the underlying mission data product distribution systems.

An MDP may contain bulk data relating to Mission Operations stored on-board a spacecraft or historical data retrieved from a Data Archive.  Examples of this include parameter value evolution in a given time period and actions history.

An MDP may also contain science or other mission data acquired on-board a spacecraft or generated by an external Mission Data Processing function that is stored on-board a spacecraft or in a Data Archive.  Although the content of this data is not itself meaningful to Mission Operations functions, they are responsible for its transfer and storage.

The MDP information comprises:

- Mission Data Product Catalogue

- Mission Data Product:

  - Product Type and associated properties that describe the product

  - Product Source (the stored data product)

  - Product Specification (optional – specifies the internal format of the product)

A limited set of standard product specifications are provided for product types that are typically involved in interoperable mission operation scenarios.  For other product types, it is possible to add customised product specifications.

### Operations Preparation Data

The information objects associated with the Operations Preparation functional area are shown in the top-level diagram of MOIMS Information Groups (see Figure 17) as they constitute the configuration data for other MOIMS functional areas, primarily Mission Control and Mission Planning.

In general these information objects have not yet been standardized by CCSDS, although they are typically exchanged between spacecraft and instrument manufacturers and spacecraft operators.  An exception is the XML Telemetric and Command Exchange (XTCE) standard.

They may however be transferred between functions as opaque data structures using standard services, including:

- MO Common: Configuration Service [any Operations Preparation data]

- MO On-board Procedure Management (OPM) Service [APD only]

- MO On-board Software Management (OSM) Service [OSW only]

#### SDB: Spacecraft Database

The Spacecraft Database (SDB) is the configuration data required by the Mission Control: Monitoring & Control function (see §5.2.4.1) and includes:

- Parameter definitions together with associated aggregation, conversion, check and statistics definitions.

- Action definitions

- Alert definitions

It may also include information on how the above are encoded within CCSDS TM/TC Packets or Frames, although this is not required if the standard MO M&C services are used.

The current XML Telemetric and Command Exchange (XTCE) standard provides a partial solution for the SDB, supporting the definition of Parameters and Commands [actions] and their encoding in CCSDS TM/TC Packets.

The Electronic Data Sheet (EDS) standard currently under development in the SOIS Area may in the future also be applicable for the exchange of the SDB.

#### APD: Automated Procedure Definition

Automated Procedure Definitions (APD) are required to configure the Mission Control: Automation function.  In the event that Automation is on-board a spacecraft this is also associated with the Mission Control: On-board Configuration Management [On-board Procedure Management (OPM)] function for the definition of On-board Control Procedures (OBCP).

APDs are not currently subject to standardisation by CCSDS, but may be transported by the On-board Procedure Management (OPM) service and referenced or contained in a planning request and associated planning activities.

#### OSW: On-board Software Image

On-board Software (OSW) is typically exchanged as On-board Software Images both between spacecraft or instrument manufacturers and the spacecraft operator; and between the spacecraft operator and the spacecraft.  The latter is supported by the Mission Control: On-board Configuration Management function [On-board Software Management (OSM)] function.

OSW is not currently subject to standardisation by CCSDS, but may be transported by the On-board Software Management (OSM) service and referenced or contained in a planning request and associated planning activities.

#### PDB: Mission Planning Database

The Mission Planning Database (PDB) is the configuration data required by the Mission Planning functional area and includes:

- Planning Activity definitions

- Planning Event definitions

- Planning Resource definitions

- Planning Constraint definitions

The PDB is not currently subject to standardisation by CCSDS.  However, the information model defined for Mission Planning and Scheduling Services (see §5.2.6) identifies the key elements required within it.

### Data Storage and Archiving Data

The Data Storage and Archiving functional group comprises three distinct functions with associated information objects:

- Data Archive Information (DAI)

- On-board File Store

- Operations Archive

#### DAI: Data Archive Information

Figure 5-7: Data Archive Information

Abstract information objects associated with the long-term archiving and retrieval of mission data have been defined in the context of the CCSDS Open Archival Information System (OAIS) standard [CCSDS 650.0-M-2].

This identifies the concept of an Information Package (IP) which has three derived information classes:

- Submission Information Package (SIP): generated by an archive producer

- Archival Information Package (AIP): stored in an archive

- Dissemination Information Package (DIP): disseminated to an archive consumer

There is not a one-to-one relationship between these.  Multiple SIPs may be constituted into a single AIP during archive ingestion; and AIPs may be decomposed into multiple DIPs during dissemination.

An IP has three principal elements:

- Package Description: the definition of the structure of an IP

- Information Package: the IP itself

- Packaging Information: information that actually or logically binds or relates the components of the package into an identifiable entity on specific media.

Package Descriptions may be expressed using the following CCSDS standard data description languages:

- Parameter Value Language (PVL) Specification [CCSDS 641.0-B-2]

- Data Description Language EAST Specification [CCSDS 644.0-B-3]

- Data Entity Dictionary Specification Language (DEDSL) [CCSDS 647.1-B-1 (Abstract); CCSDS 647.2-B-1 (PVL); CCSDS 647.3-B-1 (XML/DTD)]

Information Packages comprise:

- Content Information: comprising Content Objects together with Representation Information that supports their interpretation.

- Preservation Description Information (PDI):  information that supports trust in, access to and context of the Content Information over a long and indefinite period of time.  This includes:

  - Reference: unambiguously identifies the content

  - Provenance: documents the source and history of any changes to the content

  - Context: describes the relationship of the content to its environment

  - Fixity: data integrity checks or validation/verification keys to ensure content has not been altered in an undocumented manner

  - Access Rights: access restrictions on the content, including legal framework, licensing terms, and access control

#### On-board File Store

File Transfer and Management (FTM) concerns the management of remote (on-board) file stores.  While grouped within Data Storage and Archiving, the corresponding services are typically associated with Mission Control, while the files themselves can contain data relating to any of the information groups (Mission Control, Mission Planning, Navigation, Operations Preparation and Mission Data Products).  In addition to the file content, the FTM data includes the drive and directory structure of the file store and the messages associated with File Transfer and File Management operations, including:

- File Upload/Download

- File Create/Rename/Delete/Move/Copy

- Directory List/Create/Rename/Delete/Move

#### Operations Archive

Common Archive (CAR) data is a generic term for any information object defined in terms of the MO Common Object Model (COM) (see §5.2.2) that can make use of the generic COM Archive service.

## SOIS Information Views

This section addresses data or information objects within the scope of the CCSDS Spacecraft Onboard Information Services (SOIS) Area.  Information is only introduced at a relatively high-level, sufficient to identify the information exchanged between functions and any relationships between information objects exchanged across multiple interfaces.  For a full and detailed specification of the referenced information objects, the reader is directed to the relevant CCSDS standards.

The remainder of the section is structured as follows:

- SOIS Information Model: top-level decomposition of SOIS Area Information

- SOIS Electronic Data Sheet Model: summary of the generic information model for SOIS.

- SOIS Dictionary of Terms:  summary of the mechanism for consistent interpretation of terms in the SOIS information model

- SOIS Functional Interface Data: summary of information objects that pass through SOIS service interfaces

Each subsection comprises an Information View diagram and a description of each of the information objects it contains.

### SOIS Information Model

The SOIS information model consists of four parts.

Figure 5-8: SOIS Information Model

The SOIS electronic data sheets are schema-controlled artefacts that guide a chain of software tools at each agency to compose a vehicle.  The SOIS dictionary of terms is an ontological model of terms used in the SOIS information model to assure consistent interpretation by algorithms.  The tool chain is a set of software tools that assist in designing the composition of a spacecraft.  The subnetwork concepts describe the elements of data that flow between functions in the SOIS subnetwork service access point.

### SEDS Concepts

Figure 18 identifies the roles of SOIS EDSs in the information model, which are restated and explained here.

The base structure of an instance of SEDS appears in Figure 31.  The tree structure has for its trunk a DataSheet element or a PackageFile element.  The primary purpose of the DataSheet element is to hold in its branches the description of a device.  The purpose of a PackageFile is to describe a software service onboard the vehicle.  Either kind of SOIS EDS can contain metadata.  A PackageFile can consist entirely of metadata that is shared by other SEDS instances; for example, parameters of a computing platform, such as word size, can appear in a PackageFile and be referenced by other PackageFiles that describe services.

Figure 5-9: Base Structure of a SOIS EDS

There are two kinds of branches from a Datasheet element.  One kind of branch, a Package element, may appear in any quantity, including zero.  The purpose of a Package element is to describe types of data, data interfaces, and behaviours that are peculiar to the composable part that is the subject of the data sheet.  The elements in a Package that describe types of data make a model of the syntactic structure of a data type, decorated with semantic tags defined in the SOIS DoT.

Figure 5-10: Structure of Package Elements in SOIS EDS

The other kind of branch, a Device element, may be present as a single branch, or may be absent.  The purpose of a Device element is to provide metadata about the composable part described by the Package element.  The metadata includes configuration management information such as manufacturer’s model and serial number, and a model of operation.  The elements in a Metadata element may be decorated with semantic tags defined in the DoT, which serve to clarify the intent of categories and values.

Figure 5-11: Structure of a SOIS EDS Metadata Element

A PackageFile element may contain a Package element that describes a composable software part.  Additionally, a PackageFile element may contain a Metadata element that includes configuration management data about the composable software part.  A PackageFile element may contain only a Metadata element that contains mission or platform metadata values.

The EDS and DoT are the parts of the System Model in Figure 18 that have been defined formally.  The [Future] third part of the System Model is a Deployment Description.  All three parts appear in Figure 34, Figure 35, and Figure 36 to describe how to compose a vehicle from its parts.

The application services collection of SOIS appears in Figure 34, near the left side, in the context of its related concepts.  The onboard platform architecture has a collection of application services and a collection of subnet services.  The Application Services has Mission Applications, Application Support Services, and Device Services.  A SEDS Datasheet may describe a device, and a SEDS PackageFile may describe a software object.  For a device, the SEDS Datasheet also specifies the behaviour of the Device Service that stands as a proxy for the device in an onboard computer.

Figure 5-12: SOIS Application Support Concepts

The subnet services collection appears in Figure 35 near the left side, in the context of its relationships with other concepts.   The Subnet Services have Communications Services, Convergence Functions, External Protocols, and Management Services.  A Deployment Description describes the Subnet Services collection, including features such as topology and schedule.  Describing those features requires reference to the Device Datasheets that describe Devices attached to the subnetwork.  The Deployment Description also specifies the behaviour and interfaces of management services for the subnetwork.

Figure 5-13: SOIS Subnet Layer Concepts

The SOIS System Model appears in Figure 36 at the left side, in the context of its relations to other concepts.  The System Model has PackageFiles, Datasheets, Deployment Description, and a Dictionary of Terms.  The PackageFiles, Datasheets, and Deployment Description reference the Dictionary of Terms.  The Deployment Description refers to Datasheets that describe Devices and that specify Device Services.  The Deployment Description specifies Management Services and Describes the Subnetwork.  Some of the PackageFiles describe externally provided Mission Application software objects.  Some of the PackageFiles describe Application Support Services identified in the SOIS green book [4].

Figure 5-14: SOIS System Model Concepts

### Dictionary of Terms

The Dictionary of Terms (DoT) provides a set of terms that may be used in the SOIS System Model for interpretation by algorithms in tool chains that assemble the flight software.  Human readable descriptions of the terms provide connections to the actual conversational vocabulary used by engineers in various shops, and these descriptions can be formatted into glossaries for use in those shops.  The formal terms in the DoT must be used in data sheets and in deployment descriptions because those are interpreted by algorithms that aren’t equipped to handle natural language.

Figure 5-15: High-Level Summary of Dictionary of Terms

A summary of the initial content of the DoT appears in Figure 38.  Concepts like “QuantityKind” and “Unit” come from the QUDV ontology.  The “SemanticProperties” are provided by SOIS and based on terms used in a self-organizing spacecraft experiment in the United States Air Force Research Laboratory.  The ModelOfOperations includes terms for describing subnetwork properties, as well as terms for describing major features of the operation of a device.

### Data Exchanged across SOIS Subnetwork Functional Interfaces

The categories of data exchanged across the SOIS Subnetwork functional interfaces appear in Figure 5-21.

Figure 5-21: Data at Subnetwork SAP Interfaces

The subnetwork interfaces are service access points (SAP).  Each subnetwork function provides a its own data interface.  The middle column in Figure 5-21 corresponds to the interface data items in Figure 4-12.  The right column in Figure 5-21 identifies major categories of data that may appear in the interface data items.

A SAP Address is an obsolescent concept that is being subsumed by the Flow-Identifier concept as the subnetwork magenta books suffer their five-year reviews.  The process of subsumption is not yet complete, so both concepts appear in the diagram above.  The Data constitutes the payload information that crosses the interface.  The SAP Metadata is information about a particular request or indication that crosses the interface.

#### Subnetwork Addresses

The subnetwork magenta books that have not yet been reviewed use a variety of terms for the usage of subnetwork addresses in the various subnetwork functional interfaces.  These terms appear with their relationships in Figure 5-22.

Figure 5-22 SOIS Subnetwork Addresses

An address in a subnetwork is an address that is defined by the standard of the subnetwork technology.

- SAP address: Service Access Point address

- A destination address identifies a device that is target of data transfer.

- A source address identifies a device that is the origin of data transfer.

- MASAP: Memory Access SAP address

- DDSAP: Device Discovery SAP address

- SYNCSAP: Synchronization SAP address

- TSAP: Test SAP address

- A device address identifies a device discovered or lost.

- A test address identifies a device that is the subject of a test operation.

#### Flow Identifiers

With the five-year review of SOIS subnetwork magenta books, the concept of flow-id was introduced.  Instead of simply depicting a single endpoint of a data transfer as in SAP addresses in the preceding section, a flow-id depicts a pre-planned path between an origin endpoint and at least one destination endpoint.  This change more accurately represents the planning that goes into a network that must deliver some of its messages within a certain deadline.

Figure 5-23: The Composition of a SOIS Flow Identifier

The parts of a flow-id appear in Figure 5-23.  A subnetwork identifier denotes the technology of a subnetwork, and the subnetwork instance discriminates when there are more than one subnetworks of the same technology.  The destination and source addresses identify the endpoints of the flow; for broadcast, the destination would represent multiple addresses.  The protocol identifier tells the protocol for which the path was designated.  The payload identifier serves to identify packets or other data that do not carry clues to their own types.  The maximum transmission unit (MTU) limits the units of transfer along the path.  The channel identifier is a designation specific to the subnetwork that reserves resources for delivery of data.  The quality of service (QoS) contains additional qualifiers that are specific to the subnetwork that specify a level of service, such as priority.

#### Subnetwork Data Payloads

The payload of a transfer is the data that is the reason for the transfer.  Examples of these data appear in Figure 5-24.

Figure 5-24: Subnetwork Data Payloads

The test status provides the result of testing a device.  The memory id is a pointer to a region in the memory model of a memory-access device.  The remaining items characterize an event.

#### Subnetwork Metadata

The metadata exchanged across the subnetwork service access point consists of information about the particular transfer.  See Figure 5-25.

Figure 5-25: Subnetwork Metadata

The device metadata describes a device that joined or departed the subnetwork.  The failure metadata appears in an indication that a packet operation failed.  Result metadata describes a memory-access operation.  A transaction identifier enables an asynchronous result to be paired with the request that initiated it.  Authorization is a key to gain memory access.  The verification item indicates whether data should be verified before committing it to memory when writing to the memory of a device.  The length is the size of a packet, for packets that don’t contain clues to their own length.

## Security Concepts for Information View

<< Specific information / data security including for privacy purposes.  Anything still in the process of “becoming” gets marked [Future]. >>

# Service View

## Overview

<< Service view covers the nature, interfaces, and behaviors of the various Service objects that are defined.  Anything still in the process of “becoming” gets marked [Future]. >>

## MO Services

Mission Operations (MO) services have already been identified in the Functional View (§4) as the connecting lines between functions annotated with the information objects exchanged and identifying the service provider by a colour-coded circle at one end.

These correspond to both on-line interactions between functions (true services with associated behavior) and off-line exchange of information (data formats, typically contained in files, with no associated interface behaviour).

The Service View is represented as a set of tables listing the identified service interfaces or data formats for each functional group.  For each service, the table lists the functions interfaced, the information objects exchanged, operations/capabilities of the service/data exchange and references the CCSDS standards that relate to this. For a full definition of the table columns and colour coding used, refer to §3.3.3.

Several of these services have been, or are planned to be, defined in terms of the Mission Operations (MO) service framework.  This framework is a set of CCSDS standards that provide a way of specifying abstract Mission Operations services that can be deployed over various communications protocols and technologies.  The framework includes:

- Message Abstraction Layer (MAL)

- Common Object Model (COM)

- Common Services

- Bindings for various programming languages that define the Application Program Interface (API) for any MO compliant service.

- Bindings for various communications technologies for encoding and transport of service messages.

The remainder of the section is structured as follows:

- MO COM and Common Services: lists generic and common services defined as part of the MO service framework, either in the MO Common Object Model (MO) or MO Common Services standards.

- Mission Control Services: lists services associated with the Mission Control functional area.  These services are defined in terms of the MO service framework.

- Navigation and Timing Services: lists message formats and services associated with the Navigation and Timing functional areas.  For Navigation, the majority of standards are currently defined as message formats, although future Navigation services have been identified to be based on the MO service framework and existing Navigation message formats.

- Mission Planning and Scheduling Services: services supporting the Mission Planning and Scheduling functional area and based on the MO service framework are currently under development.

- Operations Preparation Services: corresponding to the Operations Preparation functional area, this section identifies potential services and/or data formats to support the exchanges of configuration data for Mission Operations functions.  Currently CCSDS only provides the XTCE standard for exchange of telemetry and command definitions.

- Data Archiving Services: services relating to the Mission Data Archive function and exchange of associated Data Archive Information (DAI).

- Other Mission Operations Services: lists additional services based on the MO service framework that are not associated with specific MO functional areas.  These include:

  - Mission Data Product (MDP) Distribution

  - File Transfer and Management

### MO COM and Common Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO COM | Archiving | Providers: Ops. Archive Consumers: <any function> | Store Update Query | <any COM obj> | Generic archive service for MO services defined in terms of the Common Object Model [COM]. | MO Common Object Model [CCSDS 521.1-B-1] |  |  |
|  |  | Activity Tracking | Providers: <any function> Consumers: <any function> | Publish/Subscribe | <any Activity> | Activities are COM objects that have a limited duration. The service provides a mechanism to report progress/status and uses the COM Event service. |  |  |  |
|  |  | Event | Providers: <any function> Consumers: <any function> | Publish/Subscribe | <any Event> | Events are COM objects that represent an occurrence at a point in time. Each service can define the Events it supports. |  |  |  |
|  | MO Common | Directory | Providers: Service Directory Consumers: <any function> | Publish Provider Withdraw Provider Lookup Provider Get Service XML | Service Descriptor | Allows Providers to publish information about the services they provide; and Consumers to query the Service Directory and retrieve Service XML descriptors. | MO Common Services [CCSDS 522.0-R-n] |  |  |
|  |  | Login | Providers: Login and Authentication Consumers: <any function> | Login Logout Report Available Roles Handover to other User | Authentication Credentials | Common login service for submission of authentication details to a deployment specific security system. Integrated with Access Control aspect of MO MAL. |  |  |  |
|  |  | Configuration | Providers: Configuration Management & Distribution Consumers: <any function> | Activate List Get Current Get XML Add Remove Store Current Store XML | Configurations XML Configurations <any Config Data> | Configurations can be hard-coded, use bespoke configuration data, or a standard COM service configuration. Service consumers can activate predefined configurations of a service provder; and list, get, add, remove and store current configurations. It also defines a standardised XML representation for configurations. |  |  |  |

### Mission Control Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO M&C | Action | Providers: Monitoring & Control Consumers: Automation On-board Configuration Management Navigation Interface Planning Plan Execution Operations Preparation Operations Archive User Support Mission Data Processing Spacecraft Development & Maintenance | Submit Action preCheck Action Manage Action Definitions | Action | Allows control directives (e.g. a spacecraft telecommand) to be invoked and their evolving status to be monitored. Uses COM Services for Action Tracking and Archiving. | MO Monitoring & Control Services [CCSDS 522.1-B-1] |  |  |
|  |  | Parameter |  | Monitor Value Get Value Set Value Enable Generation Manage Parameter Definitions | Parameter | Provides the capability to monitor and set parameter values. Uses COM Archiving service for Parameter Archiving. |  |  |  |
|  |  | Alert |  | Enable Generation Manage Alert Definitions | Alert | Provides a mechanism for asynchronous notification of operationally significant events or anomalies. Uses COM Event and Archiving services to publish/subscribe to Alerts and to archive them. |  |  |  |
|  |  | Check |  | Get Current Transition List Get Summary Report Enable Service Get Service Status Enable Check Trigger Check Manage Check Definitions | Parameter Check Check Link Check Transition Event | Provides on-line checking of Parameter values against defined checks [Limit, Constant, Delta] and notification of check violations. Uses COM Event service to publish/subscribe to check status transition events. |  |  |  |
|  |  | Statistics |  | Get Statistics Reset Evaluation Monitor Statistics Enable Service Get Service Status Enable Generation Add Parameter Evaluation Update Parameter Evaluation Remove Parameter Evaluation | Parameter Statistic Link Statistic Value | Provides on-line statistical evaluation of Parameter values. Uses COM Archive service. |  |  |  |
|  |  | Aggregation |  | Monitor Value Get Value Enable Generation Enable Filter Manage Aggregate Definitions | Aggregation of Parameters | Provides aggregation of separate Parameter values into coherent sets. Uses COM Archive service. |  |  |  |
|  |  | Conversion |  | none | Parameter Conversion | Provides conversion of raw Parameter values into engineering units. Uses COM Archive service |  |  |  |
|  |  | Group |  | none | Group of <any COM object>s | Provides the ability to define groupings of objects to simplify the operations of other services. Uses COM Archive service |  |  |  |
|  | MO AUT | Automation | Providers: Automation Consumers: Monitoring and Control Navigation Interface Plan Execution Operations Archive | Start Procedure Stop Procedure Suspend/Resume Procedure Manual Control Manage Procedure Definitions [TBD] | Procedure | Provides support for automation of mission operations. The service allows automated procedures or autonomous functions to be invoked, controlled, and their evolving status to be monitored. Uses COM Services for Procedure Tracking and Archiving. | MO Services Concept [CCSDS 520.0-G-3] Automation Service |  |  |
|  | MO OSM | Software Management | Providers: On-board Configuration Management Consumers: Automation Plan Execution | Load Software Image Dump Software Image Check Software Image | On-board Software Image | Supports the management of software loaded into the remote system [spacecraft]. | MO Services Concept [CCSDS 520.0-G-3] Software Management Service |  |  |
|  |  | Procedure Management | Providers: On-board Configuration Management Consumers: Automation Plan Execution | Load Procedure Definition Dump Procedure Definition Check Procedure Definition Manage Procedure Definitions [TBD] | On-board Procedure Definition | Supports the management of automated procedure definitions loaded into the remote system [spacecraft]. | Proposed extension to scope of MO OSM service. |  |  |

### Navigation and Timing Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TIM | Time | Providers: Time/Position Determination Consumers: Time Correlation | Report Time Set Time Configure rate of Time Report generation. | Time Report | Provides accurate reporting of on-board time. For an unsynchronised on-board clock this may require correlation with the system reference time | MO Services Concept [CCSDS 520.0-G-3] Time |  |  |
|  |  | Time Correlation | Providers: Time Correlation Consumers: Mission Control Mission Data Processing | Correlate Time | Time Correlation | Supports Time correlation between on-board clocks and the system reference time. | MO Services Concept [CCSDS 520.0-G-3] Time |  |  |
|  |  | Time Reception | Providers: TT&C Consumers: Time Correlation | Report Reception Time | Reception Time | Provides accurate ground reception time reporting that can be associated with a Time Report. This is currently supported by the CSS Space Link Extension Transfer Services, where Earth Receive Time is provided as an annotation parameter to the transfer data. An alternative service may be required where SLE is not used. | Space Link Extension – Return All Frames Service Specification [CCSDS 911.1-B-4] And related CSS SLE Transfer Services |  |  |
|  | NAV | Navigation Services | Providers: Navigation Functions Consumers: Mission Control Mission Planning & Scheduling Mission Data Processing Data Storage & Archiving User Support TT&C Satellite Dev. & Maintenance | Request Navigation Message Retrieve Navigation Message Subscribe to Navigation Message | Orbit Vector Attitude Tracking Data Predicted Orbital Events Conjunction Data Re-entry Data Pointing Request | Supports the provision of spacecraft positioning information such as: - Position reports (e.g., from on-board GPS) - Spacecraft ranging and range-rate measurements - Antenna tracking azimuth and elevation - Orbit vectors - Attitude vectors - Trajectory requests - Predicted orbital events (including ground station visibilities) The services will use the following data message formats defined by the CCSDS Navigation working group, but wrap these as service specifications based on the MO framework. | MO Services Concept [CCSDS 520.0-G-3] Navigation |  |  |
|  |  | Orbit Data Message [ODM] | Providers: Orbit Determination and Propagation Consumers: Manoeuvre Planning Attitude Determination Conjunction Assessment Mission Control Mission Planning & Scheduling Mission Data Processing TT&C User Support | - | Orbit Vector | The ODM contains information that defines the orbit state of a spacecraft at one or more times. | Orbit Data Messages [CCSDS 502.0-B-2] |  |  |
|  |  | Attitude Data Message [ADM] | Providers: Attitude Determination Consumers: Orbit Determination and Propagation Manoeuvre Planning Mission Control Mission Data Processing User Support | - | Attitude | The ADM contains information that defines the attitude state of a spacecraft at one or more times. | Attitude Data Messages [CCSDS 504.0-B-1] |  |  |
|  |  | Tracking Data Message [TDM] | Providers: Time/Position Determination TT&C Mission Data Processing Consumers: Orbit Determination and Propagation | - | Tracking Data | The TDM contains information that can be used to determine the orbit state of a spacecraft. | Tracking Data Messages [CCSDS 503.0-B-1] |  |  |
|  |  | Navigation Event Message [NEM] | Providers: Orbit Determination and Propagation Consumers: Attitude Determination Manoeuvre Planning Mission Planning & Scheduling Mission Control | - | Predicted Orbital Events | The NEM contains the predicted timings of orbital events, such as ground station visibilities, sensor blindings, eclipses, etc. | Navigation Event Message CCSDS Standard under development |  |  |
|  |  | Conjunction Data Message [CDM] | Providers: Conjunction Assessment Consumers: Manoeuvre Planning Mission Planning & Scheduling | - | Conjunction Data | The CDM contains information that defines the relationship between the orbit states of different space objects at different times. | Conjunction Data Messages [CCSDS 508.0-B-1] |  |  |
|  |  | Re-entry Data Message [RDM] | Providers: Re-entry Assessment Consumers: Manoeuvre Planning Mission Planning & Scheduling | - | Re-entry Data | The RDM contains information about a single re-entry event of a natural or man-made object entering the atmosphere of the Earth or another planet. | Re-entry Data Message CCSDS Standard under development |  |  |
|  |  | Pointing Request Message [PRM] | Providers: User Support Mission Data Processing Consumers: Attitude Determination Manoeuvre Planning Mission Control | - | Pointing Request | The PRM contains information on the pointing of a spacecraft or instrument desired by a mission user at one or more times. | Pointing Request Message [CCSDS 509.0-R-1] |  |  |

### Mission Planning and Scheduling Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO MPS | Planning Request [PRS] | Providers: Planning Plan Execution Consumers: User Support Operations Preparation Navigation & Timing Planning [Distributed] | Submit Request Update or cancel Requests Provide Request Status feedback Manage Request Definitions | Planning Request Plan Planning Activity Planning Event Planning Resource Planning Constraint | Asynchronous submission of planning requests, associated responses and their subsequent management and status feedback. A planning request may reference a Plan (output from an earlier planning process), in which case the provided feedback includes the status of the plan in terms of its contained activities and other items. | MO Services Concept [CCSDS 520.0-G-3] Planning Request Mission Planning and Scheduling [CCSDS 000.0-G-1] |  |  |
|  |  | Plan Distribution & Retrieval [PLS] | Providers: Planning Plan Execution Consumers: User Support Navigation & Timing Mission Data Processing Planning [Distributed] | Retrieve Plan or Plan Status Subscribe to Plan or Plan Status | Plan Planning Activity Planning Event Planning Resource Planning Constraint | Provides distribution and access to plans generated by the planning function. | MO Services Concept [CCSDS 520.0-G-3] Scheduling [Part] Mission Planning and Scheduling [CCSDS 000.0-G-1] |  |  |
|  |  | Planning Process Management [PMS] | Providers: Planning Consumers: Plan Execution Mission Control Navigation & Timing User Support Mission Data Processing Planning [Distributed] | Initiate, Monitor and Control Planning Processes Update Plan Status Manage Planning Definitions | Plan Planning Activity Planning Event Planning Resource Planning Constraint | Management of the planning process itself - initiation, status feedback and control. Also supports provision of plan status updates by a third party. | MO Services Concept [CCSDS 520.0-G-3] Scheduling [Part] Mission Planning and Scheduling [CCSDS 000.0-G-1] |  |  |
|  |  | Plan Execution Management [PES] | Providers: Plan Execution Consumers: Planning Mission Control | Initiate, Monitor and Control Plan execution Edit Plan content Update Planning Events and Resources. Manage Planning Definitions | Plan Planning Activity Planning Event Planning Resource Planning Constraint | Control and management of the execution of a plan, including actions to Start/Stop and Pause/Resume execution. Update [editing] of the executing plan at activity level. Update planning events/resources. | MO Services Concept [CCSDS 520.0-G-3] Scheduling [Part] Mission Planning and Scheduling [CCSDS 000.0-G-1] |  |  |

### Operations Preparation Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OPD | Satellite DB [SDB] | Providers: Satellite DB Definition Spacecraft Development & Maintenance Consumers: Mission Control Spacecraft Development & Maintenance | - | Satellite DB | Contains definition of Telemetry Data, Telecommands and Events present in the TM/TC interface with the spacecraft and represented within the Mission Control System. XTCE provides an exchange format for TM/TC data between systems. | XML Telemetric and Command Exchange (XTCE) [CCSDS 660.0-B-1] |  |  |
|  |  | Automated Procedure Definition [APD] | Providers: Automated Procedure Definition Spacecraft Development & Maintenance Consumers: Mission Control Spacecraft Development & Maintenance | - | Automated Procedure | Definition of an operational procedure that can be automatically executed within a space system (either on-board a spacecraft, or within the mission control system). | No CCSDS standard. ECSS PLUTO [ECSS E-70-32] defines a standard model for a procedure, but not a normative representation. |  |  |
|  |  | Planning Data Definitions [PDB] | Providers: Planning DB Definition Consumers: Mission Planning & Scheduling | - | Planning Data Definitions | Definitions of Planning Data (Activities, Events, Resources and Constraints) and potentially of Planning Rules. | No CCSDS standard Mission Planning and Scheduling Concept [Draft Green Book] introduces Planning Data Model. |  |  |
|  |  | On-board Software [OSW] | Providers: On-board Software Definition Spacecraft Development & Maintenance Consumers: Mission Control Spacecraft Development & Maintenance | - | On-board Software | On-board Software Image | No CCSDS standard |  |  |

### Data Archiving Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DAI | Producer Archive Interface [PAIS] | Providers: Data Archive Ingestion Consumers: Mission Data Processing | TBD | Submission Information Package (SIP) SIP Sequencing Constraint Transfer Object Collection Descriptor | The current PAIS standard provides the abstract syntax and an XML implementation of descriptions of data to be sent to an archive. It addresses how these data will be aggregated into packages for transmission and one concrete implementation for the packages based on the XML Formatted Data Unit (XFDU) standard. A service specification is proposed but has not yet been developed. | Reference Model for an Open Archival Information System (OAIS) [CCSDS 650.0-M-2] Producer-Archive Interface Specification (PAIS) [CCSDS 651.1-B-1] XML Formatted Data Unit (XFDU) Structure and Construction Rules [CCSDS 661.0-B-1] |  |  |
|  |  | Consumer Archive Interface [CAIS] | Providers: Data Archive Access Consumers: User Support | TBD | Dissemination Information Package (DIP) | Delivery of digital sources from the Archive. | Reference Model for an Open Archival Information System (OAIS) [CCSDS 650.0-M-2] |  |  |
|  |  | Archive Storage and Retrieval | Providers: Data Archive Storage Consumers: Data Archive Ingestion Data Archive Access | TBD | Archival Information Package (AIP) | Storage and Retrieval of standard Archival Information Packages (AIP). Abstract specification of an AIP may be expressed using PVL, EAST or DEDSL languages. | Reference Model for an Open Archival Information System (OAIS) [CCSDS 650.0-M-2] Parameter Value Language (PVL) Specification [CCSDS 641.0-B-2] The Data Description Language EAST Specification [CCSDS 644.0-B-3] Data Entity Dictionary Specification Language (DEDSL) [CCSDS 647.1-B-1] Abstract [CCSDS 647.2-B-1] PVL [CCSDS 647.3-B-1] XML/DTD |  |  |

### Other MO Services

| A | Group | Service | Functions | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MO MDP | Data Product Distribution | Providers: Operations Archive On-board File Store Consumers: Mission Data Processing User Support Spacecraft Development & Maintenance | Subscribe Online Subscribe Batch Retrieve [TBD] | Mission Data Product | Mission Data Product Distribution Service is used for the distribution of historical archived data and on-line 'live' data It provides two delivery modes, batch mode and stream mode. | MO Mission Data Product Distribution Services [CCSDS 522.2-R-1] |  |  |
|  | MO FTM | File Transfer & Management | Providers: On-board File Store Consumers: Mission Control Planning & Scheduling Navigation and Timing | List, Rename, Move, Copy and Delete Files and Directories Add Directory Get Drive Information Upload File Download File | Directory File Drive | Supports the management of a remote [on-board] file store and the initiation of transfers of files between local and remote file stores. | MO Services Concept [CCSDS 520.0-G-3] Remote Buffer Management Service |  |  |

## SOIS Services

The table of services for SOIS appears below.  The colors in the leftmost column correspond to the colors in Figure 18.  The colors in the rightmost two columns are defined in Section 3.3.3.

| A | Service | Operations | Data | Description | Standards | S | D |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Packet Service | PACKET_SEND, PACKET_RECEIVE, PACKET_FAILURE | Flow-Id, Data, Length, Transaction-Id, Failure Metadata | Provides means to read or to write packets from or to devices. | [2] |  |  |
|  | Memory Access Service | READ, WRITE, READ/MODIFY/WRITE, MEMORY_ACCESS_RESULT | MASAP Address, Destination Address, Transaction Id, Memory Id, Result Metadata | Provides means to read or to write data from or to memory of a device. | [3] |  |  |
|  | Device Discovery Service | DEVICE_DISCOVERY, DEVICE_DISCOVERY_LOSS | DDSAP Address, Device Address, Device Metadata | Detects devices attached to subnetworks; detects connection and disconnection of devices to and from subnetworks; notifies management functions. | [4] |  |  |
|  | Synchronization Service | TIME, EVENT | SYNCSAP Address, Time, Event Id, Event Time, Event Data, Start/Stop, | Provides means to maintain knowledge of time that is common across a single subnetwork. | [5] |  |  |
|  | Test Service | TEST | TSAP Address, Test Address, Test Status | Provides basic capability to request tests of devices and to receive the results of the tests. | [6] |  |  |
|  | Management Information Service |  |  | Provides management information for spacecraft onboard information services |  |  |  |
|  | Electronic Data Sheets | <Device services, application support services, and applications generated by tool chain using SEDS specifications> | <Data described in EDSs> | Defines interfaces and behavior of onboard devices and services | [7] |  |  |
|  | Dictionary of Terms |  |  | Defines standard terms to be used in interpreting electronic data sheets for matching and/or adapting interfaces. | [8] |  |  |
|  | Deployment Description |  |  | Defines topology, traffic flows, and schedules of subnetworks. |  |  |  |

## Integrated Flight/Ground Services (discussion)

## Dependence of ASL Services on SCCS Architecture

## Security Concepts for Service View

<< Secure service interfaces.  Anything still in the process of “becoming” gets marked [Future]. >>

# Communications View (Protocol stacks)

## Overview

<< Communications view covers the protocol stacks that are defined.  These will mostly, but not only, be associated with service interface bindings and should be thought of as “building blocks” for deploying services on nodes.  Anything still in the process of “becoming” gets marked [Future]. >>

## ISO Protocol Stack and Layer Definitions

<< Borrow from SCCS-ADD. >>

## Specific Protocols for MOIMS Service Interface Binding

MOIMS area standards concern end-to-end application level information exchange and are defined in terms of two principle information exchange paradigms:

- Message Based Interaction (bi-directional message exchange)

- File Transfer (uni-directional message transfer)

For message based interaction, Mission Operation (MO) Services are defined in a way that is abstracted from the underlying communications technology.  This allows them to be deployed in different communications contexts, providing the corresponding technology binding is defined.

In the case of file transfer, MOIMS standards include defined message formats that can be encoded in a file.  A specific means of effecting the file transfer is not imposed.  The application is expected to use an existing standard (or bespoke) protocol for achieving this.

The description of the specific protocols for MOIMS Service interface binding in the remainder of this section is structured as follows:

- Communications Deployment Contexts

- Generic Protocol Stack

- Space Link Context

- Ground Context

- On-board Context

- Context Bridging

### Communications Deployment Contexts

In terms of the underlying communications architecture, MOIMS Services and message formats may be deployed in three principle contexts (illustrated in Figure 24, Figure 25, and Figure 26 respectively):

- Space Link: across CCSDS SCCS ARD compliant communications protocol stacks.  The ARD describes the communications architecture for two primary cases:

  - ABA: a point-to-point space communications configuration that involves a single direct link from spacecraft to ground.  It should be noted that this includes the use of CCSDS CSS Space Link Extension (SLE) services to extend the space link from a terrestrial ground station to a Mission Operations Centre (MOC) or other ground facility.  The term ABA derives from the case of a spacecraft and MOC owned by Agency A using a TT&C ground station owned by Agency B, and hence there being interoperability between Agencies.  However, it should also be noted that this is only one example of several possible deployment architectures.

  - SSI: a Solar System Internetwork (SSI) protocols deployed over potentially multiple space links and terrestrial networks.  It is a loose confederation of independent space communications networks that all share a single network layer protocol that allows them to interoperate and exchange network layer messages [PDUs], each often owned and administered by a different space agency.

- Ground: across a Terrestrial Network, using industry standard communications protocol stacks

- On-board: within a Spacecraft, using a CCSDS SOIS compliant architecture

Initially, MO Services may only be deployed in the Ground context, but as a standard technology binding to CCSDS Space Packet Protocols has already been defined, there is also potential for deployment in Space Link  and On-board contexts.

Bridging between these separate communications contexts is performed at application level.  However, where the MO Message Abstraction Layer (MAL) is used, a generic MAL bridge can provide this as an application independent service.  This is discussed further in §7.3.6 below.

Figure 7-1: MOIMS Space Link Communications Context

Figure 7-2: MOIMS Terrestrial Link Communications Context

Figure 7-3: MOIMS On-board Link Communications Context

### Generic Protocol Stack

The following diagram shows the generic protocol stack applicable to all communications deployment contexts for both MO compliant services and MOIMS file exchange.

Figure 7-4: MOIMS Services Generic Protocol Stack

The blue layers correspond to those of the underlying communications architecture:

- Transfer (Application) Layer (e.g. FTP, HTTP or 0MQ) providing message exchange or file transfer.

- Transport and Network Layers combined (e.g. TCP/IP or UDP/IP).

- Physical Link Layer or “Network” (e.g. RF Space Link, WAN, LAN or WLAN).

The pink layers correspond to those of the MOIMS application level services themselves.  In the case of an MO compliant service, these comprise:

- The MO Service specification itself (one of many), which can be any service defined in terms of the underlying MO Service Framework.

- The MO Message Abstraction Layer (MAL) [CCSDS 521.0-B-2] that provides a syntax for the representation of MO service operations and messages that is abstracted from the underlying communications transfer layer.

- The MO MAL Technology Binding (one of several available) that defines how the MAL is mapped to the underlying communications transfer layer.  There are two aspects of this which may be separate or combined in a single standard:

  - Encoding: how the MAL service messages are encoded within the transfer message

  - Transport: how the MAL service operations are bound to those of the underlying transfer service.

An extensible set of standard MAL Technology Bindings is available, including:

  - MO MAL Space Packet Transport Binding and Binary Encoding [CCSDS 524.1-B-1]

  - MO MAL Binding to TCP/IP Transport and Split Binary Encoding [CCSDS 524.2-B-1]

  - MO MAL HTTP Transport Binding and XML Encoding (in preparation)

  - MO MAL ZeroMQ Transport Binding and CNES Binary Encoding (in preparation)

- The MO Language Binding (one of several available) which defines how the MAL and any MO Service expressed in terms of the MAL is presented to the application as a language specific API.  An extensible set of standard MO Language Bindings is available, including:

  - MO MAL – Java API [CCSDS 523.1-M-1]

  - MO MAL – C++ API (in preparation)

For MOIMS File Formats, where only the message format is defined, a specific file transfer protocol is not imposed.  The communicating applications are expected to make use of an existing standard (or bespoke) protocol to effect the file transfer.  Message formats may optionally be defined in terms of the MO MAL, which brings the benefit that the different message encodings available through the MO MAL Technology Bindings can be used.

The following diagram provides a generic deployment example for two MOIMS User Applications communicating using any MO compliant service.

Figure 7-5: MO Service Generic Service Deployment Example

Protocol stacks at both Communications and MOIMS Application levels must match between communicating User Applications.  The MO Language Binding can, however, differ between communicating User Applications.

The following diagram shows the same deployment case for a specific MO compliant service: the MO Monitoring & Control service.  Note that this is still a generic deployment diagram.  In an actual deployment a specific MAL Technology Binding (Encoding and Transport) must be selected and used by both communicating applications, but the MO Language Binding can be selected according to the programming language used for each application.

Figure 7-6: MOIMS Specific Service Deployment Example

The following three subsections elaborate the specific protocol stacks for each of the identified communications deployment contexts.

### Space Link Context

In the Space Link Context, the communications Transfer layer is assumed to be either CCSDS Space Packets, or the CCSDS CFDP File Transfer protocol overlaid on this.

Figure 7-7: MOIMS Services Space Link Protocol Stack

The full communications layer protocol stacks are defined in the CCSDS SCCS ARD for both the ABA and SSI communications architecture cases.

At the MOIMS Application Layer, the MAL Technology Binding is one or a combination of MAL to Space Packet [CCSDS 524.1-B-1] and/or MAL to CFDP (no CCSDS standard currently available, although existing encodings may be used to generate files).  The potential for using file transfer is identified for larger MO Service messages, or for the bulk transfer of many smaller MO Service messages (e.g. recorded on-board the spacecraft), but this has not yet been the subject of standardisation.

For MOIMS File Exchange, CFDP may be used across the Space Link, but initiation of the CFDP file transfer must be initiated directly by the application.

The following diagram shows an example deployment of any MOIMS Service across a Space Link that follows the defined ABA deployment case, comprising a Space User Node, TT&C Ground Station and Earth User Node.  In this case the MOIMS Service Provider is on-board the spacecraft, and the MOIMS Service Consumer is within the Earth User Node.  MO Service messages are encoded within Space Packets and transferred over the extended Space Link.  The application level software does not need to be aware of how the encoding or transfer in terms of Space Packets is achieved – at this level the exchange is in terms of MO Service messages.  The actual encoding and transfer is specified through the selected MAL Technology binding and underlying communications protocol stack.

Figure 7-8: MOIMS Services Deployment over Space Link (ABA Example)

This example extends the Space Data Link from the Space User Node to the Earth User Node through the use of CCSDS Cross Support Services (CSS), specifically the CSS Transfer Services (CSTS) sometimes referred to as Space Link Extension (SLE).  This enables bridging between the physical space link (Coding and Synchronisation; RF and Modulation layers) between the spacecraft and ground station, and TCP/IP and a terrestrial link between the Ground Station and the Earth User Node.  It is noted that CSTS offers multiple service options for both forward and return links.  A typical deployment case is shown using the Return All Frames (R-AF) and Forward CLTU (F-CLTU) services.  The link between Ground Station and Earth User Node is established according to a previously negotiated Service Instance (SI).

### Ground Context

In the Ground Context, the underlying communications layers are assumed to be based on TCP/IP, but a range of options are available for the transfer layer.  Figure 32 illustrates an example protocol stack using HTTP and FTP, but it is stressed that other transfer protocols may be used, providing the corresponding MAL technology binding is available.

Figure 7-9: Example MOIMS Services Terrestrial Link Protocol Stack

For message exchange, technology bindings are currently under development for both HTTP and ZeroMQ.  CCSDS may develop standard bindings to other messaging technologies in the future, as demand and resources permit.  This approach provides a degree of future-proofing for MAL-compliant applications.  Note that it is possible to use a proprietary or bespoke protocol, if a bespoke technology binding is developed and deployed on both sides of the interface.

As for the Space Link context, there is also the potential for use file transfer for bulk message exchange, but there is currently no standard MAL-FTP technology binding to support this.

For MOIMS file exchange, no specific file transfer protocol is imposed.  FTP, CFDP, other standards or bespoke protocols may be used by the communicating applications (or a 3rd party) to effect the file transfer.

Figure 7-10: Example MOIMS Services Deployment over Terrestrial Link

### On-board Context

In an On-board Context, MOIMS applications require an exposed File Access Service (FAS) to interact with on-board file systems, and a Messaging Service to support interaction with other on-board applications.

Figure 7-11: Example MOIMS Services On-board Link Protocol Stack

Although identifying Message Transfer, Packet Store and File Services, the CCSDS SOIS area currently does not provide specification of such standardised application support services.  It does provide a standard means to declare such services.  In practice, such services are specific to the on-board software architecture implemented for a given mission, which may be based on a manufacturer’s proprietary infrastructure.  A bespoke MAL Technology Binding is therefore required for each on-board infrastructure solution

CCSDS Asynchronous Messaging Service (AMS) may be used to provide a standardised messaging service to on-board applications, but other/bespoke services may also be used, requiring a dedicated MAL transport binding.  The AMS example is illustrated in Figure 35

If there is a distinct on-board Packet Store (rather than storing packets in files), then an additional binding may be required between the MAL and the Packet Store Service.  Note: this is associated with the Space Link from an application perspective.

For File Exchange, the transfer must be independently initiated at Communications Protocol level, either directly by the User Application or by a 3rd party.

Figure 7-12: Example MOIMS Services Deployment over On-board Link

### Context Bridging

Figure 7-13: MAL Bridging Concept

As MO Services are specified in terms of the Message Abstraction Layer (MAL), it is possible to implement a bridge between two separate communications contexts, that enables all MO Services to be carried across the boundary transparently to the communicating applications.  The MAL Bridge supports two technology bindings and translates between them.

This is illustrated in Figure 36 above, which shows an example case for two messaging technologies (Transfer Protocols A and B).  All MO Services defined in terms of the MAL can be transferred across the same generic MAL Bridge.

This approach can be used to link different communications contexts, such as Space-Ground and Ground-Ground, enabling end-to-end application level communication across a heterogeneous network.

## Specific Protocols for SOIS Service Interface Binding

SOIS area standards concern collection and distribution of information from and to onboard devices.

- For the onboard sensors and actuators, SOIS protocols are simple, and exploit the protocols provided by the subnetwork.

- When the onboard devices are gateways to other networks, SOIS may optionally have some transport and networking functions as needed to represent the endpoints in the other networks.

The description of the specific protocols for SOIS Services in the remainder of this section is structured as follows:

- Simple Subnetwork

- Software Message Bus

- Multiple Processors Separating Application and Instrument

- Multiple Processors Separating Applications

- Connection to External Network

### Simple Subnetwork

Figure 7-14 shows a connection between an onboard device and an application, which is one of the basic purposes of SOIS.  The application executes in an onboard computer, which is connected to an onboard subnetwork.  The device is connected to the same subnetwork.  The EDS for the device describes the interfaces presented by the Device Service on behalf of the device.  Conversion between D-PDU’s (in the syntax of the device) and A-PDU’s (in the syntax of the application) occurs in the device service, which consists of a Device-Specific Access Protocol (DACP) and an optional Device Abstraction Control Procedure (DACP).  The effect is as if a PDU flowed between the application and the device, with virtual translation.

Figure 7-14: Connecting a Device to an Application

NOTE: The device is shown communicating through the subnet packet service, but memory access could be used depending on the device.

### Software Message Bus

Figure 7-15 shows a connection between an application and an onboard device which offers some flexibility for reuse of both the device and the application in other contexts.  The software bus is shown here as a service in the application support layer, such as AMS.  The adapter for the device will be generated from the SEDS for the device.  Conversion between D-PDU’s (in the syntax of the device) and A-PDU’s (in the syntax of the application) occurs in the device service, which consists of a Device-Specific Access Protocol (DACP) and an optional Device Abstraction Control Procedure (DACP).  The adapter for the application can be generated from the SEDS for the application.  The effect is as if a PDU flowed between the application and the device, with virtual translation.

Figure 7-15: Connection between Onboard Device and Application through Software Bus

NOTE: The device is shown communicating through the subnet packet service, but memory access could be used depending on the device.

### Multiple Processors Separating Application and Instrument

Figure 54 shows a connection between an application and an onboard device in which the device connects to one processor, and the application is on a different processor connected through a subnetwork.  The processor containing the EDS-Derived Device Access Service might be any of the following.

- an adapter for the device

- a remote interface unit

- a time/space partition (The message bus could then be an inter-partition communication channel, and the internetworking layers would be unnecessary.)

- a processor peer of the processor containing the application

The “Internetworking Layers” in the diagram represent a protocol standard outside SOIS, which support addressing and end-to-end connections on the subnetwork that provides the hardware message bus.

Figure 7-16: Connection Between Onboard Device and an Application through a Subnetwork

NOTE: The device is shown communicating through the subnet packet service, but memory access could be used depending on the device.

### Multiple Processors Separating Applications

Figure 55 shows a connection between two applications on different onboard processors.  The applications could be MOIMS SM&C components, for example.  The processors containing the applications might be any of the following.

- time/space partitions (The message bus could then be an inter-partition communication channel, and the internetworking layers would be unnecessary.)

- peer processors

The “Internetworking Layers” in the diagram represent a protocol standard outside SOIS, which support addressing and end-to-end connections on the subnetwork that provides the hardware message bus.

Figure 7-17: Connection between Applications on Separate Onboard Processors through Subnetwork

### Connection to External Network

SOIS occupies one end of a space datalink, which is one A leg of an ABA configuration.  The following diagram shows how the vehicle’s antenna and radio connect through SOIS protocols to onboard applications.  The EDS-derived device service presents the interface for the TM/TC radio and antenna(s).

Figure 7-18: SOIS Configuration for Telemetry and Telecommands

[Future] Figure 57 summarizes the use of SOIS protocols to participate in an SSI configuration.  Connection to the body of a solar system internet is through one or more high-capacity channels, such as an antenna and radio or an optical emitter and detector, as shown below.  The EDS-derived device service presents the interface for the delay-tolerant networking device.

Figure 7-19: Solar System Internet

## Integrated Flight/Ground Protocols

Include SOIS diagram:

But include equivalent representation for two additional contexts to cover the 3 MO transitional phases: 1) ground only 2) space link – as above 3) on-board between applications.

## Remaining Challenges to Protocol Deployment

<<Multi-mission protocol interoperability for widely distributed heterogeneous systems>>

## Security Concepts for Protocol View

<< Secure protocols, link and network layer encryption are handled in SCCS-ADD, application service layer encryption, & authentication. Key management protocols.  Anything still in the process of “becoming” gets marked [Future]. >>

# Physical (Connectivity) View

## Overview

<< Physical view covers the types of nodes that must be defined in addition to those SCCS nodes (ESLT & other).  Much of this is likely to just be specialization of the User Nodes types (Space User Node & Earth User Node) from the SCCS ADD.  Anything still in the process of “becoming” gets marked [Future].

Make it very plain that the deployment cases are just examples selected to illustrate cases where interfaces may be exposed to an interoperability boundary>>

## MOIMS Physical Elements

MOIMS functions may be distributed over a wide network of Space and Earth User Nodes and as a result MO Services and MOIMS File Exchange may be deployed across the boundaries between these distributed User Nodes, supported by communications links that fall into one of three communications contexts as identified in the Communications View (Space Link, Ground and On-board).

There is no definitive set of standard User Nodes.  Each space mission system has its own architecture and identifies its own set of physical nodes, each of which may be owned or operated by a different agency or organisation.  The remainder of this section provides some illustrative but realistic examples of potential physical architectures that can be used to identify where interfaces between MOIMS functions could be exposed to an interoperability boundary.

It is structured as follows:

- Identification of Deployment Nodes

- Physical Deployment Architecture Examples

- Potential Functional Deployment

### Identification of Deployment Nodes

As indicated above, the SCCS ADD has identified only three classes of physical deployment node: Space User Node, Earth User Node and Earth-Space Link Terminal (ESLT).  For the purposes of modelling distributed networks with multiple deployment nodes in both the Space and Ground segments, a representative set of deployment nodes is identified below.  It is stressed that these deployment nodes are only examples, and actual space system physical architectures may identify other classes of deployment node.

- Space User Nodes:

  - Spacecraft (Orbiter/Relay, Lander/Rover)

  - Habitat (Station, Base or Suit)

  - Payload (or Instrument, hosted in a Spacecraft/Habitat)

- Earth User Nodes:

  - Mission Operations Centre [MOC]

  - Payload (or Rover) Operations Centre [POC/ROC]

  - Navigation Services Centre [NSC]

  - Data Processing Centre [DPC]

  - Data Archive Centre [DAC]

  - PI / User

  - Spacecraft Manufacturer [SCM]

There may be multiple Spacecraft nodes within a given space system.  This may be the case for interplanetary missions which include both an Orbiter that also acts as a communications relay, and a Lander or Rover deployed to the surface.  In the case of constellation missions, there may be a large number of spacecraft nodes.

Habitats are manned space system nodes, which may range from a space station, a lunar or mars base, or an individual astronaut’s suit.

Payload nodes are instruments or other equipment that is hosted on-board a Spacecraft (or Habitat), which is owned or operated by a different authority to the host.

Most space systems include a Mission Operations Centre [MOC] that is responsible for mission operations functions, including:  Mission Control, Mission Planning, Navigation and Operations Preparation.

Many missions also have a dedicated Payload Operations Centre [POC] for the operation of the payload instrument(s) that is more closely associated with its scientific objectives and interacts more closely with Principal Investigators (PIs) and the User community.  This is sometimes termed a Science Operations Centre [SOC].  In the case of a deployed surface rover (or other probe), there may also be a dedicated Rover Operations Centre [ROC].

Some Navigation functions may be centralised at a dedicated Navigation Services Centre [NSC], which provides specialised support to multiple missions.  This is commonly the case for functions such as Conjunction Assessment that provides collision warnings, but many low-cost cubesat missions also use centralised services to perform their Orbit Determination.

The processing of acquired mission data may be performed at a separate Data Processing Centre [DPC].  Although Mission Data Processing is currently outside the scope of MOIMS, Mission Operations functions frequently interact with such facilities.

In a similar way, the long-term archiving of mission or science data may be delegated to a dedicated Data Archive Centre [DAC] that stores both the raw data acquired and processed mission data products.

It is noted that MOC, POC, DPC and DAC may all be co-located at a single site or distributed in any combination across multiple sites operated by different agencies, but they represent functional groups found in many space system architectures.  Even where co-located they typically correspond to distinct systems.  There is also the potential for some of these (DPC and DAC) to be multi-mission facilities.

Principal Investigators (PI), or Users, correspond to external entities that are responsible for tasking and utilizing the space system.  They may be academic teams responsible for a science payload, astronomers using an observatory mission, or the users of an Earth observation system.  They are typically widely distributed and will interact with the space system primarily through Mission Planning and the distribution of mission data.

Spacecraft (or Payload) Manufacturers [SCM] have a responsibility to provide initial information required to support Operations Preparation for the configuration of Mission Operations.  This includes the provision of the spacecraft database (telemetry and telecommand definitions), Operations Procedures and On-board Software.  Manufacturers often also have an on-going responsibility to support anomaly resolution and performance monitoring of the spacecraft or payload.  This requires access to Mission Control data for investigative purposes.

Note other Earth User Nodes could include functions required that are outside the normal scope of Mission Operations, and therefore not discussed further here.  These include:

- Pre-launch Spacecraft Assembly, Integration and Verification (AIV) [also known as Checkout] facilities.

- Launch facilities

- Spacecraft Operations Simulators

- Training facilities

It is noted that such additional earth user nodes normally contain variations on the set of functions already identified in this document, potentially extended by specialised functions.

### Physical Deployment Architecture Examples

This section provides examples of typical physical deployment architectures for space systems. The examples have been chosen to illustrate deployment of the different types of communications contexts (as identified in the Communications View) for typical space systems.  There is no intention to standardise the deployment architectures themselves and many alternative physical deployment architectures are possible for actual space missions.

Each example shows a set of deployment nodes (based on the example set identified in the previous section) and the topology of the communications links between those nodes, using the notation defined in §3.3.5.  Communications links are colour coded to indicate the type of link in terms of the communications deployment context (Space Link, Ground or On-board).

The following example physical deployment architectures are provided:

- ABA Space Link

- Solar System Internetwork (SSI)

- Hosted Payload

Each example builds on its predecessor to introduce additional communications deployment contexts.  The following points are noted:

- At application level, the ABA Space Link case is effectively a subset of the SSI case

- The routing function is only indicated for SSI routing nodes.  In practice all ground deployment nodes will have a routing function for the terrestrial network.

- Habitats and deployed Rovers/Landers have similar network topology and have been represented as a single deployment node.

- There may be multiple instance of all types of deployment node within a single system.

#### ABA Space Link Example

Figure 8-1: Physical Deployment Architecture: ABA Space Link Example

This example shows a relatively simple mission comprising a single Spacecraft deployment node, an ESLT (ground station – although this could equally be a network of ground stations), and a full set of example ground segment deployment nodes.

CCSDS Cross Support Transfer Services (CSTS) are used to extend the Space Link from the ESLT to both Mission Operations and Data Processing Centres.  It is not unusual in Earth Observation missions to have separate communications channels for TT&C communications and mission data downlink.  In some cases, separate ESLT ground stations are used for TT&C and mission data acquisition.

Terrestrial networks are used to link the distributed physical deployment nodes of the ground segment.

A Navigation Services Centre (NSC) is similarly linked through terrestrial networks to the ESLT ground stations for the acquisition of satellite tracking data.

#### SSI Example

Figure 8-2: Physical Deployment Architecture: SSI Example

In this Solar System Internetwork (SSI) example, the space segment comprises three separate deployment nodes.  One Spacecraft is acting as a space routing node, with an inter-satellite link to a second Spacecraft node, and a proximity link to a surface Lander, Rover or Habitat.

In the case of a space system using the Solar System Internetwork (SSI), all deployment nodes forming part of the SSI have contain an SSI router function supporting data storage and forwarding.  It is noted that all deployment nodes on the terrestrial network will also include normal network routing capability, but this is omitted for clarity.

#### Hosted Payload Example

Figure 8-3: Physical Deployment Architecture: Hosted Payload Example

In this Hosted Payload example, the Spacecraft Platform and the hosted Payload are considered separate deployment nodes, each capable of hosting MOIMS functions.  Communications between them are via an on-board link.

In the ground segment, the SSI has been extended across the terrestrial network from the Mission Operations Centre (MOC) to the Payload Operations Centre (POC), enabling direct end-to-end communications between POC and the Payload it controls.

### Potential Functional Deployment

Figure 8-4: Potential Functional Deployment

The various physical deployment architectures presented in the previous section contain no information about the deployment of application level MOIMS functions.  It is only when Mission Operations functions are placed within the deployment nodes that it becomes possible to identify which MOIMS level information exchanges are exposed to the potentially interoperable boundaries between those nodes.  This is the primary purpose of the Deployment View contained in the next chapter.

Figure 40 gives a high-level summary of the potential deployment of functions, based on the purpose of the identified example classes of deployment node given in §8.2.1 above.  The coloured ellipses correspond to the top-level MOIMS functional groups identified in the Functional View.

All Space Segment nodes could potentially host a subset of the following Mission Operations functions:  Mission Control, Mission Planning, Navigation and Timing and On-board Data Storage.

ESLT nodes host the external TT&C function.

The MOC and POC may host all Mission Operations functions applicable to their respective domain of interest.

The DPC primarily hosts the external Mission Data Processing function, but may also host local Data Archiving and a subset of Mission Planning and Navigation functions.  The last of these is where spacecraft position data is generated as a by-product of image or other mission data processing.

The DAC hosts long-term Data Archiving functions.

A dedicated NSC hosts a sub-set of Navigation functions, potentially including Conjunction Assessment, Re-entry Assessment and Orbit Determination.

PI/User deployment nodes host the external User Support function.

SCM nodes host the external Spacecraft Development and Maintenance function, and may also contribute to the Operations Preparation function.

## SOIS Physical Elements

The present statement of scope is as follows:

“The basic context of SOIS services is that of a single spacecraft within a single mission.”

[Future] Recent discussion has asked whether the SOIS charter might be extended. A draft of this extension

- Begins with the current scope, in which SOIS subnet and SOIS app covers the communication onboard a spacecraft between application software and devices that provide the sensory and actuation capabilities of the vehicle,

- And generalizes the idea of “spacecraft” to include a variety of “vehicles” in which a local network connects sensors and actuators to processing elements onboard the vehicle for the management of the functions of the vehicle.

- This generalization might include the following possible kinds of “vehicles”:

  - Spacecraft (current scope), including earth observation, exploration, and relay missions

  - Human excursion vehicles, including spacesuits

  - Unmanned excursion vehicles, including probes and rovers

  - Habitats

- The communications between vehicles, and between vehicles and mission control centers, would remain in the domain of other CCSDS areas.

  - How to treat docked vehicles which may share the control network?

  - The SOIS wireless working group is one of the areas of communication between vehicles.

  - The boundary between SOIS and extra-vehicular communication devices consists of a SOIS side and an external side.

    - The SOIS side is described and managed like any other onboard device interface.

    - The external side is described by other CCSDS areas.

## Security Concepts for Physical View

<< Secure service sites (physical and logical).  Anything still in the process of “becoming” gets marked [Future]. >>

# [Functional] Deployment View

## Overview

<< End-to-end deployment views will just include a limited, but useful, set of examples.  They cannot be exhaustive because there are un-countable numbers of possible deployments.  The views will be end-to-end, possibly for interoperability and cross support, with a suitable set of nodes, protocol stacks, and application layer deployments.  Anything still in the process of “becoming” gets marked [Future]. >>

## MO Functional Deployment

In the Physical View it has been shown above that there are many possible physical deployment architectures for a space system comprising multiple deployment nodes.  In the Deployment View, the distribution of MOIMS functions across these deployment nodes is shown.  For any given physical architecture, there is a wide range of possible functional deployments.

An objective of the Deployment View is to show which CCSDS standard information exchanges and services are exposed to the potentially interoperable interfaces that may occur at the boundaries between physical deployment nodes.  It is these interfaces for which standardization is most appropriate.  Within the MOIMS Area Functional View, information exchanges and services are only fully resolved at the second level of decomposition of MOIMS functions.

With many possible physical deployment architectures, and many possible Level 2 functional deployments for each physical architecture, it is not possible to cover every possibility within this document.  For this reason, a realistic and representative set of deployment examples has been selected in order to demonstrate coverage of:

- All MOIMS functions [Functional View]

- All MOIMS information objects [Information View]

- All MOIMS services [Service View]

- All identified classes of communications context (Link type) [Communications View]

- All identified example MOIMS deployment nodes [Physical View]

- A range of Mission types

Representation of common and generic services is omitted from this viewpoint to reduce complexity, but it in principal can be applied across any of the identified boundaries.

Showing all MOIMS Level 2 functions within a single diagram is not practical.  The functional breakdown shown within each deployment node is therefore limited to MOIMS Level 1 functions.  If only a subset of Level 2 functions is deployed on a given node, then these are listed within the corresponding colour-coded function “bubble”.  If all Level 2 functions are present, then the Level 1 functional group name is used.

Only those interfaces exposed to a boundary between deployment nodes are shown.  Interfaces between functions within a deployment node are omitted for clarity.  The set of internal interfaces present can be derived from the Functional View.

The example Functional Deployments given, represent the following use cases or scenarios:

- Communications Satellite (ABA)

- Earth Observation Mission (ABA)

- Deep Space Mission with Orbiter/Relay and Rover (SSI)

- Constellation Mission (SSI)

- Hosted Payload (SSI)

- Manned Mission

It is stressed that selected use cases are only examples – other mission scenarios are also possible. The functional deployment illustrated for each scenario is also only an example, other physical architectures and functional deployments are also possible for real space missions.

The examples show existing CCSDS standards, those currently under-development and those identified on the future CCSDS roadmap.  It is noted that alternative standards and bespoke solutions may also be used to implement the identified interactions.

The remainder of this section presents each of these deployment scenarios in turn, preceded by a discussion of the representation of distributed functions.  The descriptions provided are incremental, describing differences and additions to the preceding examples.

### Distributed Functions

Figure 9-1: Distributed and Proxy Functions

The Functional View in general only shows application level interfaces (exchanges of information objects or services) between heterogeneous functions.  In a distributed system, however, it is common for individual functions themselves to be distributed, resulting in application level interfaces between homogeneous functions.

Figure 41 shows two ways in which a function may be distributed across multiple nodes.

Proxy Functions

Mission Operations (MO) functions may be distributed between space and ground deployment nodes, with the MOC or other nodes acting as ground proxies for the Spacecraft – e.g. Monitoring and Control.  In the example shown, Function A is deployed on-board the spacecraft, while Function A Proxy is deployed on ground.  Other ground based functions [Function B] then effectively interact with the spacecraft via the ground proxy. Where physical communication with the spacecraft is intermittent, this has the additional benefit of providing a permanent presence representing the last known state of the spacecraft function and queuing interactions with it.  Mission Control systems typically act as a proxy for the spacecraft in this way.  The proxy function may also augment the data provided by the on-board function.  For example deriving additional status information and statistics or performing checks.

Distributed Functions

MO functions may be distributed across multiple Nodes, requiring peer-to-peer application interfaces between functionally similar applications, as well as those between functionally dissimilar applications identified in the Functional View.  In the example shown, Function C is distributed between the spacecraft and two ground deployment nodes.  This is typically the case for functions such as Mission Planning and Scheduling: both POC and MOC may be involved in Mission Planning and deliver plans [schedules] to the spacecraft for execution.  In this case the same standard interface may be used between the distributed elements, although the content may differ across each interface.

### Communications Satellite (ABA) Example

Figure 9-2: Example Functional Deployment (ABA): Communications Satellite

This example functional deployment shown in Figure 42 represents a relatively simple traditional mission comprising a communications satellite, ground station [ESLT] and Mission Operations Centre (MOC).  The deployment architecture can also support cases where there are multiple satellites and ground stations, providing there is no interaction between the satellites.  The only additional ground user node is the Spacecraft Manufacturer.

The communications architecture follows the ABA pattern identified in the CCSDS SCCS ADD.

On-board functions:

- Mission Control: basic spacecraft Monitoring & Control; on-board procedures (Automation); On-Board Configuration Management (procedures and software).

- Mission Planning: on-board scheduler (Plan Execution)

- Navigation and Timing:  on-board GNSS receiver supporting Time and Position determination.

- On-Board File Store: used to store M&C history; OBCP definitions; OBSW images; Plans [Schedules]; Position and Timing data; Mission Data Products.

MOC functions:

- Mission Control (all functions)

- Mission Planning

- Navigation and Timing:  Orbit Determination and Propagation; Attitude Determination; Manoeuvre Planning; Time Correlation

- Data Storage and Archiving: Operations Archive

- Operations Preparation

Many Mission Operations interactions are exposed to the Spacecraft to MOC interface, but it is noted that in most existing missions non-CCSDS, proprietary or bespoke standards are currently used.  With the migration of increasingly complex functions on-board the spacecraft, there is increasing benefit to the use of standardized services across this interface.

The ground station (ESLT) supports TT&C functions, including spacecraft tracking and ranging, and may be dedicated to the mission.  There is widespread use in existing systems on the ESLT-MOC interface of CCSDS Cross Support Services and MOIMS Navigation message formats.

The Spacecraft Manufacturer provides initial TM/TC definitions [Spacecraft DB], On-board Software and OBCP definitions.  CCSDS currently only provides partial coverage of this interface with the XTCE standard for the exchange of telemetry and command definitions.  The Spacecraft Manufacturer also supports performance monitoring and anomaly investigation throughout the mission lifetime, requiring access to historical Mission Control and Navigation data.

It is noted that this example only addresses Mission Operations functions for the communications satellite, and that the associated ground communications infrastructure (Network Operations Centre (NOC), Gateway stations and User terminals), which is outside the scope considered.

### Earth Observation Satellite (ABA) Example

Figure 9-3: Example Functional Deployment (ABA): Earth Observation Satellite

This example builds upon the previous case and illustrates the more complex ground segment architecture that is typical of Earth Observation missions, while continuing to be based on the ABA space link communications architecture.

On-board functions are extended to include:

- Navigation and Timing: on-board Orbit and Attitude Determination

- Mission Planning: Replanning in response to events detected on-board

The migration of more complex functionality on-board the spacecraft, exposes additional and more complex Mission Operations interactions to the Spacecraft – Ground Segment interface:  Orbit and Attitude Data; higher-level Mission Planning interactions.

Instead of there being a single MOC, the mission ground segment also includes separate Payload Operations (POC), Data Processing (DPC) and Data Archive Centres (DAC):

- MOC functions are as before.

- POC functions include:

  - Payload Planning [Mission Planning]

  - Payload Control [Mission Control]

- DPC hosts Mission Data Processing

- DAC hosts a long-term mission Data Archive of both Mission Data Products and Operations History.  This archive may be shared by multiple missions.

A community of Principal Investigators (PIs) or Users is responsible for tasking the mission (Payload Planning) and receives the acquired mission data.  These primarily interact with the POC.

An external Navigation Services Centre (NSC) supports Conjunction Assessment and provides collision warnings.

It is common in Earth Observation Missions to use separate communications frequencies for TT&C and the higher data rate mission data downlink.  This often results in an architecture with separated ground segments for Mission Operations and Payload Data processing, utilizing separate ground stations (ESLT) for TT&C and Payload Data Acquisition (PDA), connected to MOC and DPC respectively.  Alternatively, a single Ground Station can route data to separate MOC and DPC systems.

This distributed ground segment architecture results in many more MOIMS application level interactions being exposed to the interfaces between ground segment deployment nodes:

- Navigation messages are exchanged between the MOC and POC, PIs/Users and NSC.  In some cases Mission Data Processing can also be the source of spacecraft position and attitude data derived as a by-product of image processing.

- Mission Control interactions between MOC and POC.

- Mission Planning interactions between MOC and POC, POC and PIs/Users and MOC and DPC

- Mission Data is effectively distributed at application level directly by the spacecraft to the DPC.  The DPC forwards raw and derived  Mission Data Products to the DAC where they can be accessed by PIs/Users.

### Deep Space Mission (SSI) Example

Figure 9-4: Example Functional Deployment (SSI): Deep Space Mission

The Deep Space Mission example shown in Figure 44 has been chosen to illustrate a more complex space segment comprising a planetary Orbiter and a surface deployed Rover.  It is based on the SSI communications architecture.

A simplified ground segment (compared with the Earth Observation example) comprises a MOC and separate Rover Operations Centre (ROC).  It is noted that such a mission may also include DPC, DAC and PIs/Users in a more distributed ground segment, but as this has already been covered in the previous example it has been omitted here.

There is significant interaction between MOC and ROC, including Mission Control, Mission Planning and Navigation & Timing information exchange.

The Orbiter acts as a communications relay for the Rover.  Orbiter, Rover, ESLT, MOC and ROC all support the SSI routing function that enables end-to-end communication using store and forward capability on-board the Orbiter.  This enables the ROC to have direct end-to-end interaction with the Rover.  Optionally the MOC could act as a Ground Router for all data to/from the Rover.

The Rover is assumed to have a high degree of autonomy, both in terms of automation and on-board planning.  GNSS is not available to support position determination, but other systems, such as landmark recognition and inertial navigation, may be used to provide position data.  This exposes more complex Mission Control and Mission Planning interactions to the space-ground interface.

While only a single Spacecraft Manufacturer (SCM) node is shown, it is noted that it is highly likely that there would be separate Spacecraft Manufacturers for the Orbiter and Rover.

### Constellation Mission (SSI) Example

Figure 9-5: Example Functional Deployment (SSI): Constellation Mission

The example shown is of an Earth-orbiting Constellation of Earth Observation satellites.  Like the Deep Space example, it is based on the SSI communications architecture.

It should be noted that there may be many Spacecraft nodes.  The underlying assumption is that there are multiple satellites in each of potentially multiple constellation planes.  Satellites within the same plane are connected via Inter-Satellite Links (ISL).  At least one satellite per plane is in direct contact with the ground via a Space-Ground Link (SGL).

Only two Satellite instances are shown in the diagram: a Spacecraft that is currently supporting an SGL, and another Spacecraft that is connected to the ground via an ISL to the first.  These are roles that the spacecraft are fulfilling at a particular point in time – the satellites themselves are interchangeable.

On-board functionality includes:

- On-board GNSS receivers and Orbit Determination

- On-board Event Detection and Collaborative Replanning between Spacecraft

The existence of ISLs allows direct communication between satellites.  This is likely to be at high-level and aimed at enabling autonomous operation of the satellite constellation.  Events may be detected on-board the satellite (examples are: cloud cover; forest fires or volcanic eruptions; oil slicks; etc.).  These can be used by the on-board replanning function to schedule observation of the detected event or to defer a planned observation.  Given a short period of target visibility, it may not be possible to respond to the detected event locally on the detecting satellite.  Instead the Event can be communicated to the following satellites, such they can replan to perform the observation.  Alternatively, a Planning Request can be forwarded to another satellite.

The fact that the mission plan can be updated locally on the satellites as a result of on-board re-planning requires more complex interaction between the on-board planner and ground-based mission planning to ensure visibility of the current mission plan and its execution status.

A simplified ground architecture is shown here, compared with the Earth Observation example given in §9.2.3.  This is to focus on the constellation aspects of the mission.  A single payload processing centre is shown, hosting DPC and DAC functions and supporting direct interaction with external Users.  This could be omitted altogether and integrated with the MOC, or separate DPC, POC and DAC nodes identified as for the previous Earth Observation example.

### Hosted Payload Mission (SSI) Example

Figure 9-6: Example Functional Deployment (SSI): Hosted Payload

The Hosted Payload example shown above is topologically similar to the Deep Space Mission example given previously, and is also based on the SSI communications architecture (although an ABA equivalent would also be possible).

The key difference is that the Hosted Payload is physically located on-board the Spacecraft and interfaces with it via a SOIS compatible on-board link.

As the Spacecraft is assumed to be in Low Earth Orbit, it has on-board GNSS providing Time and Position Determination, together with on-board Orbit Determination.

Both the Spacecraft and the Hosted Payload independently have on-board Automation (procedures) and on-board schedulers (Plan Execution).  The Hosted Payload performs Image Acquisition, storing acquired mission data in files in an On-board File Store.  Events detected on board the host Spacecraft are passed to the Hosted Payload to enable synchronisation of payload operations.

A simplified ground segment architecture has independent MOC (for Spacecraft) and POC (for Payload), the latter responsible for operation of the hosted payload and associated mission data processing.

The Hosted Payload has end-to-end communication with the POC, supporting interaction for Mission Planning, Payload Control, and direct transfer of acquired image data to the Mission Data Processing function.

In this example interaction between MOC and POC is restricted to transfer of Navigation Data from MOC to POC and Mission Planning interactions.  This could be extended to support Mission Control interaction between Mission Control and Payload Control functions.

Separate manufacturers are assumed for Spacecraft and Payload.

### Manned Mission Example

Figure 9-7: Example Functional Deployment (SSI): Manned Mission

Section TBD

Seems to be similar to deep space mission with Habitat instead Rover:

Relay Station may be an essential part of deployment but is omitted

There may be more individual but tiered spacecraft serving as proxy the same time

Habitat may be like an individual P/L but dependent from spacecraft

There may be more tiered habitats of different purpose / characteristics

## SOIS End-to-End Deployment Views

SOIS deployments for space to ground link, and for internet access appear in Section 7.4.1.

[Future] SOIS deployment for for extravehicular activity appears in Figure 68.  The EDS-derived device service presents the interface for the short-range radio.

Figure 9-8: Extra-Vehicular Activity Link

## Security Concepts For End-to-End Protocol View

# Implementation View

## Introduction

## MOIMS Implementation

Figure 10-1: MO Implementation View

Figure 10-2: NAV Message Implementation View

## SOIS Implementation

This section is structured as follows:

- SOIS EDS-Derived Device Services:  tool chain functions using SOIS EDS to generate device services

- Application Support Functions: tool chain functions using SOIS EDS to integrate application support services

- SOIS Tool Chain Concepts: summary of the use of the SOIS information model in an agency’s tool chain for composing a vehicle

- Assembly of SOIS Convergence Functions: description of selection of protocol convergence functions to normalize capabilities across subnetworks

### SOIS EDS-Derived Device Services

The SOIS publications include a magenta book for each of the subnetwork functions in Figure 19.  There is also a SEDS instance in SANA that describes the interfaces of the subnetwork functions.  In an implementation of SOIS that does not use SEDS, the subnetwork functions may be implemented as concrete application programming interfaces (API) defined by the magenta books.  In an implementation of SOIS that uses SEDS, the subnetwork functions are abstract service access points (SAP) defined by the SEDS subnetwork instance in SANA.  The SEDS for devices use the abstract interface descriptions in the SEDS subnetwork instance to describe interactions with a device through the subnetwork, and the tool chain builds concrete implementations of those interactions that work with the concrete subnetwork API of the agency that owns the tool chain.

The generation of device services occurs as shown in Figure 20.  An electronic data sheet describes the data interfaces of a device, and that description includes a specification of the device service in the style of UML.  A tool chain that is specific to the agency responsible for the vehicle generates the software for the device services before flight.  During flight, the device services communicate with the device through an onboard subnetwork.  Onboard applications use the device services for command and data handling.

Figure 10-3: Derivation of Device Services from EDS

The specification in an EDS has two parts that correspond to the SOIS concepts of Device Access Service (DAS) and Device Virtualization Service (DVS).  The DAS presents the raw device data interface, in which the data elements are often counts from analog-to-digital converters.  The DVS restates the data elements in engineering units.  The DVS is optional onboard, because many designers of spacecraft prefer to work with the raw counts.  Nevertheless, the DVS information is needed for presentation of engineering units in a mission control center.

### Application Support Functions

Application support functions can be integrated into a vehicle by means of their descriptions in EDSs, as shown in Figure 21.

Figure 10-4: Integration of Application Support Functions

As in the case of a device, an EDS describes the data interface of an application support service.  Mission- and platform-specific EDSs describe features and requirements that are relevant to a particular vehicle, such as word sizes, array sizes, and interface paradigm.  A tool chain uses that description to tailor the interface of the application support service.  Examples of interface paradigms are application programming interface or message bus.

The tailoring may take the form of generating a wrapper for a concrete interface, or it may alter parameters in a c header file (for example) before compilation.  The former is appropriate for a software object distributed as a library or script; the latter is appropriate for a software object distributed as source code.

### SOIS System Model

This section describes how the SOIS System Model can work with a tool chain to configure the flight software for a vehicle.

The general process to integrate a device into a vehicle appears in Figure 35.  An EDS describing the device is the primary input to the process; it contains both Device and Namespace sections.  There may be other EDSs that define shared syntax, such as CCSDS packet headers.  One or more EDSs shared across the mission provide metadata that represents design choices for platforms and for the mission as a whole.  The output is executable software artifacts in a particular programming language for execution on a particular platform.  Figure 35 shows c header and implementation files as an example.

Figure 5-17: A Tool Chain Integrates a Device into Flight Software

The general process to compose an entire vehicle appears in Figure 36.  The input to the process includes a collection of EDS’s describing the devices that compose the vehicle.  A deployment description describes topology and scheduling of the devices in onboard subnetworks.  The union of the sets of shared EDSs described in Figure 35 provide common syntax.  The same EDSs shared across the mission in Figure 35 provide metadata that represents design choices for platforms and for the mission as a whole.  The output is a number of artifacts for the vehicle, including a mission operations data base, network utilization schedules, flight software files, and an integration and test schedule.

Figure 5-18: A Tool Chain Composes a Vehicle

A more concrete example of tool chain activities to compose a software bus appears in Figure 37.  In this example, the software bus happens to be implemented in shared memory using queues provided by the operating system.  The packets on the bus use the format specified by CCSDS Space Packet Protocol.  The APID in each packet identifies its topic in a publish/subscribe design-pattern.  In this example, the tool chain is generating software suitable for the NASA core Flight Software bus (cFS), but the same collection of EDSs could be used by another agency’s tool chain that generates software suitable for a CCSDS Asynchronous Messaging Service, with some differences in the platform EDS.  For cFS, the deployment description generates the routing table that maps APIDs to destination applications, and the device EDSs populate the device manager database.

Figure 5-19: Composing a Software Bus

The example in Figure 37 extends in Figure 38 to describe the configuration of the management information for the SOIS Subnet Services collection.

Figure 5-20: Configuring Management Data for SOIS Subnet Services

Figure 39 is a business process diagram, which summarizes the usage of EDSs and tool chains to compose flight software for a mission.  It includes a feedback mechanism to maintain the content of the DoT.

Figure 5-16: Flow of Information in SOIS Tool Chain

### Assembly of SOIS Protocol Functions

The SOIS protocol convergence functions are intended to provide uniformity of protocol services across a variety of subnetwork protocols, which might lack some of the convergence functions.

Figure 10-5: Assembly of SOIS Convergence Functions

By describing convergence functions in SEDSs, a tool chain can match and shim interfaces as in Figure 10-5.  Time-Triggered Ethernet (TTE) puts some functions of UDP/IP into subnetwork chips in order to fragment and to reassemble packets in hardware.

In , some additional SOIS convergence functions have been selected to complete the stack above what is offered by the TTE chip.  Address translation in SOIS is between spacecraft network addresses and internet protocol (IP) addresses.  Address translation by TTE is between IP and media access control address (MAC).

## Security Concepts for Implementation View

ACRONYMS

| Acronym | Definition |
| --- | --- |
| ABA |  |
| ADM | Attitude Data Message |
| AEM | Attitude Ephemeris Message |
| AIP | Archival Information Packages |
| AIV | Assembly, Integration and Verification |
| APD | Automated Procedure Definitions |
| APM | Attitude Parameter Message |
| ASL | Application and Support Layer |
| CAIS | Consumer Archive Interface Specification |
| CAR |  |
| CDM | Conjunction Data Message |
| COM | Common Object Model |
| CSD | Common Services Data |
| CSS | Cross Support Services |
| CSTS | CSS Transfer Services |
| DAC | Data Archive Centre |
| DAI | Data Archive Ingestion |
| DAS | Device Access Service |
| DEDSL | Data Entity Dictionary Specification Language |
| DIP | Dissemination Information Packages |
| DPC | Data Processing Centre |
| DSA | Data Storage and Archiving |
| DVS | Device Virtualization Service |
| DoT | Dictionary of Terms |
| EAST |  |
| ESLT | Earth-Space Link Terminal |
| F-CLTU | Frames (R-AF) and Forward CLTU |
| FTM | File Transfer and Management |
| IP | Information Package |
| ISL | Inter-Satellite Links |
| LAC | Login and Authentication Credentials |
| M&C | Monitoring & Control |
| MAL | Message Abstraction Layer |
| MCS |  |
| MDP | Mission Data Products |
| MO | Mission Operations and Information Management Services |
| MOC | Mission Operations Centre |
| MOIMS | Mission Operations and Information Management Services |
| MOS | Mission Operations Services |
| MPS | Mission Planning and Scheduling |
| NAV |  |
| NAVT |  |
| NEM | Navigation Event Message |
| NOC | (Network Operations Centre |
| NSC | Navigation Services Centre |
| OAIS | Open Archival Information System |
| OBCP | On-Board Control Procedures |
| OBSW | On-Board SoftWare |
| ODM | Orbit Data Message |
| OEM | Orbit Ephemeris Message |
| OMM | Orbit Mean-Elements Message |
| OPD | Operations Preparation Data |
| OPM | Orbit Parameter Message |
| OSM | On-board Software Management |
| OSW |  |
| PAIS | Producer Archive Interface Specification |
| PDA | Payload Data Acquisition |
| PDB |  |
| PDI | Preservation Description Information |
| PI | Principal Investigators |
| POC | Payload (or Rover) Operations Centre |
| PRM | Pointing Request Message |
| PVL | Parameter Value Language |
| R-AF |  |
| RASDS | Reference Architecture for Space Data Systems |
| RDM | Re-entry Data Message |
| ROC | Rover) Operations Centre [POC/ |
| SCCS | Space Communications Cross Support |
| SCCS ADD | Space Communications Cross Support Architecture Description Document |
| SCM |  |
| SDB |  |
| SDIR |  |
| SEA | System Engineering Area |
| SGL | Space-Ground Link |
| SI | Space Internetworking Services |
| SIP | Submission Information Package |
| SIS | Space Internetworking Services |
| SLE | Space Link Extension |
| SLS | Space Link Services |
| SM | Service Management |
| SOC | Science Operations Centre |
| SOIS | Spacecraft On-board Interface Services |
| SSA | Space Situational Awareness |
| SSI | Space System Internet |
| SST | Space Surveillance and Tracking |
| TCM | Time Correlation Message |
| TDM | Tracking Data Message |
| TIM |  |
| TRM | [Time Reception Message |
| TS | Transfer Service |
| TT&C |  |
| XTCE | XML Telemetric and Command Exchange |

BACKGROUND