Draft Recommendation for
Space Data System Standards

Reference Architecture for Space Data Systems

Draft Recommended Practice

CCSDS 311.0-M-2

April 2024

AUTHORITY

|  | Issue: | Pink Book, Issue 2 |  |
| --- | --- | --- | --- |
|  | Date: | April 2024 |  |
|  | Location: | Not Applicable |  |

(WHEN THIS RECOMMENDED PRACTICE IS FINALIZED, IT WILL CONTAIN THE FOLLOWING STATEMENT OF AUTHORITY:)

This document has been approved for publication by the Management Council of the Consultative Committee for Space Data Systems (CCSDS) and represents the consensus technical agreement of the participating CCSDS Member Agencies.  The procedure for review and authorization of CCSDS documents is detailed in the Procedures Manual for the Consultative Committee for Space Data Systems, and the record of Agency participation in the authorization of this document can be obtained from the CCSDS Secretariat at the address below.

This document is published and maintained by:

CCSDS Secretariat

Office of Space Communication (Code M-3)

National Aeronautics and Space Administration

Washington, DC  20546, USA

STATEMENT OF INTENT

(WHEN THIS RECOMMENDED PRACTICE IS FINALIZED, IT WILL CONTAIN THE FOLLOWING STATEMENT OF INTENT:)

The Consultative Committee for Space Data Systems (CCSDS) is an organization officially established by the management of its members. The Committee meets periodically to address data systems problems that are common to all participants, and to formulate sound technical solutions to these problems. Inasmuch as participation in the CCSDS is completely voluntary, the results of Committee actions are termed Recommendations and are not in themselves considered binding on any Agency.

CCSDS Recommendations take two forms: Recommended Standards that are prescriptive and are the formal vehicles by which CCSDS Agencies create the standards that specify how elements of the space mission support infrastructure shall operate and interoperate with others; and Recommended Practices that are more descriptive in nature and are intended to provide guidance about how to approach a particular problem associated with space mission support. This Recommended Practice is issued by, and represents the consensus of, the CCSDS members.  Endorsement of this Recommended Practice is entirely voluntary and does not imply a commitment by any Agency or organization to implement its recommendations in a prescriptive sense.

No later than five years from its date of issuance, this Recommended Practice will be reviewed by the CCSDS to determine whether it should: (1) remain in effect without change; (2) be changed to reflect the impact of new technologies, new requirements, or new directions; or (3) be retired or canceled.

In those instances when a new version of a Recommended Practice is issued, existing CCSDS-related member Practices and implementations are not negated or deemed to be non-CCSDS compatible. It is the responsibility of each member to determine when such Practices or implementations are to be modified.  Each member is, however, strongly encouraged to direct planning for its new Practices and implementations towards the later version of the Recommended Practice.

FOREWORD

Through the process of normal evolution, it is expected that expansion, deletion, or modification of this document may occur.  This Recommended Practice is therefore subject to CCSDS document management and change control procedures, which are defined in the Procedures Manual for the Consultative Committee for Space Data Systems.  Current versions of CCSDS documents are maintained at the CCSDS Web site:

http://www.ccsds.org/

Questions relating to the contents or status of this document should be addressed to the CCSDS Secretariat at the address indicated on page i.

At time of publication, the active Member and Observer Agencies of the CCSDS were:

Member Agencies

- Agenzia Spaziale Italiana (ASI)/Italy.

- British National Space Centre (BNSC)/United Kingdom.

- Canadian Space Agency (CSA)/Canada.

- Centre National d’Etudes Spatiales (CNES)/France.

- Deutsches Zentrum für Luft- und Raumfahrt e.V.  (DLR)/Germany.

- European Space Agency (ESA)/Europe.

- Federal Space Agency (Roskosmos)/Russian Federation.

- Instituto Nacional de Pesquisas Espaciais (INPE)/Brazil.

- Japan Aerospace Exploration Agency (JAXA)/Japan.

- National Aeronautics and Space Administration (NASA)/USA.

Observer Agencies

- Austrian Space Agency (ASA)/Austria.

- Belgian Federal Science Policy Office (BFSPO)/Belgium.

- Central Research Institute of Machine Building (TsNIIMash)/Russian Federation.

- Centro Tecnico Aeroespacial (CTA)/Brazil.

- Chinese Academy of Space Technology (CAST)/China.

- Commonwealth Scientific and Industrial Research Organization (CSIRO)/Australia.

- Danish Space Research Institute (DSRI)/Denmark.

- European Organization for the Exploitation of Meteorological Satellites (EUMETSAT)/Europe.

- European Telecommunications Satellite Organization (EUTELSAT)/Europe.

- Hellenic National Space Committee (HNSC)/Greece.

- Indian Space Research Organization (ISRO)/India.

- Institute of Space Research (IKI)/Russian Federation.

- KFKI Research Institute for Particle & Nuclear Physics (KFKI)/Hungary.

- Korea Aerospace Research Institute (KARI)/Korea.

- MIKOMTEK:  CSIR (CSIR)/Republic of South Africa.

- Ministry of Communications (MOC)/Israel.

- National Institute of Information and Communications Technology (NICT)/Japan.

- National Oceanic & Atmospheric Administration (NOAA)/USA.

- National Space Organization (NSPO)/Taipei.

- Space and Upper Atmosphere Research Commission (SUPARCO)/Pakistan.

- Swedish Space Corporation (SSC)/Sweden.

- United States Geological Survey (USGS)/USA.

PREFACE

This document is a draft CCSDS Recommended Practice.  Its draft status indicates that the CCSDS believes the document to be technically mature and has released it for formal review by appropriate technical organizations.  As such, its technical contents are not stable, and several iterations of it may occur in response to comments received during the review process.

Implementers are cautioned not to fabricate any final equipment in accordance with this document’s technical content.

DOCUMENT CONTROL

| Document | Title | Date | Status/Remarks |
| --- | --- | --- | --- |
| CCSDS 311.0-R-1 | Reference Architecture for Space Data Systems, Draft Recommended Practice, Issue 1 | April 2008 | Current issue |
| CCSDS 311.0-R-1 | Reference Architecture for Space Data Systems, Draft Recommended Practice, Issue 2 | April 2024 | Draft Issue |

CONTENTS

Section	Page

1	Introduction	1-1

1.1	Scope	1-1

1.2	Purpose	1-1

1.3	Applicability	1-1

1.4	Rationale	1-2

1.5	Document Structure	1-3

1.6	REFERENCES	1-4

2	Overview	7

2.1	ViewPOINTs and viewpoint specifications	Error! Bookmark not defined.

2.2	Overview of viewpoints	8

2.3	Enterprise ViewPOINT	Error! Bookmark not defined.

2.4	Functional ViewPOINT	Error! Bookmark not defined.

2.5	Connectivity Viewpoint	Error! Bookmark not defined.

2.6	Communications ViewPOINT	Error! Bookmark not defined.

2.7	Information ViewPOINT	Error! Bookmark not defined.

2.8	Correspondences Between Viewpoints	Error! Bookmark not defined.

2.9	Other Views Derived From the basic ViewPOINTs	Error! Bookmark not defined.

3	Basic COncepts	3-1

3.1	General	3-1

3.2	Definitions	3-1

3.3	Graphical Representations	3-6

3.4	CHARACTERISTICS of Objects	3-7

4	Enterprise ViewPOINT	4-1

4.1	Overview	4-1

4.2	Concepts	4-1

4.3	Enterprise Objects	Error! Bookmark not defined.

4.4	characteristics of Enterprise Objects	4-2

4.5	Examples of Space Data Systems described 
with Enterprise ViewPOINT	4-7

4.6	Security Issues in the Enterprise ViewPOINT	4-10

CONTENTS (continued)

Section	Page

5	Functional viewPOINT	5-1

5.1	Overview	Error! Bookmark not defined.

5.2	Concepts	5-1

5.3	Functional Objects	Error! Bookmark not defined.

5.4	characteristics of Functional objects	5-2

5.5	Example of a Space Data System described from the Functional ViewPOINT	5-5

5.6	Example of Space Data System with Information Management Infrastructure	5-9

5.7	Security Issues in the Functional ViewPOINT	5-10

6	CONNECTIVITY VIEWPOINT	6-11

6.1	Overview	6-12

6.2	Concepts	7-1

6.3	characteristics of CONNECTIVITY objects	7-2

6.4	Nodes	Error! Bookmark not defined.

6.5	Links	Error! Bookmark not defined.

6.6	Examples of Space Data Systems described with Connectivity ViewS	7-10

6.7	Security Issues in the Connectivity Viewpoint	7-14

7	CommUnications viewPOINT	9-1

7.1	Overview	Error! Bookmark not defined.

7.2	Concepts	9-1

7.3	Characteristics of communications objects	9-2

7.4	PROTOCOL ENTITIES	9-7

7.5	Examples of Space Data Systems described with Communications ViewPOINT	9-8

7.6	Protocol representations in the Communications 
Viewpoint	Error! Bookmark not defined.

7.7	Security Issues in the Communications ViewPOINT	9-13

CONTENTS (continued)

Section	Page

8	Information ViewPOINT	10-1

8.1	Overview	10-1

8.2	Concepts	10-1

8.3	characteristics of information objects	10-2

8.4	Information Object Views	10-6

8.5	Example of Space Data System Functions with 
Information ViewPOINT	10-8

ANNEX A	NOTES ON USE OF RASDS	A-1

ANNEX B	FORMAL METHODS AND TOOLS	B-1

ANNEX C	RASDS AND DoDAF COMPARISON	C-4

ANNEX D	GLOSSARY AND ACRONYMS	E-1

Figure

2-1	Simple Example of Enterprise View	Error! Bookmark not defined.

2-2	Simple Example of a Functional View	Error! Bookmark not defined.

2-3	Simple Connectivity View Example	Error! Bookmark not defined.

2-4	Example Connectivity View Showing Implemented Functions	Error! Bookmark not defined.

2-5	Simple Example of Communications View	Error! Bookmark not defined.

2-6	Communications View with Protocol Entities and Engineering Objects	Error! Bookmark not defined.

2-7	Example of Information View Showing Basic Object Model	Error! Bookmark not defined.

2-8	RASDS Top Level Object Relationships	Error! Bookmark not defined.

2-9(a)	Enterprise View of a Cross Support Service	Error! Bookmark not defined.

2-9(b)	Connectivity View of a Cross Support Service	Error! Bookmark not defined.

2-9(c)	Communications View of a Cross Support Service	Error! Bookmark not defined.

2-10	Simple Example of Layered View	Error! Bookmark not defined.

2-11	Example of Functional View of Framing and Coding	Error! Bookmark not defined.

2-12	Example of Trade Study Mapping Functional to Connectivity View	Error! Bookmark not defined.

3-1	Icons Used in This Document	3-7

3-2	Representation of Objects	3-8

4-1	Attributes of Enterprise Objects	4-3

4-2	Example of Enterprise View (Mission A)	4-8

4-3	Example of Multi-agency Enterprise View (Mission Q)	4-9

5-1	Functional Object Interfaces	5-2

5-2	Example of Functional View (Functional Objects and Interactions)	5-8

5-3	Representative Functional Objects and Information Management 
Infrastructure Elements	5-9

CONTENTS (continued)

Figure	Page

6-1	Attributes of Nodes	7-3

6-2	Example of Connectivity View (Nodes for Mission A)	7-10

6-3	Example of Connectivity View (Nodes for Mission Q)	Error! Bookmark not defined.

6-4	Example of Connectivity View (Node Decomposition)	Error! Bookmark not defined.

6-5	Example of Connectivity View with Allocated Engineering Objects	Error! Bookmark not defined.

6-6(a)	Functional View of Image Compression	7-13

6-6(b)	Connectivity View of Software Compression Approach	7-13

6-6(c)	Connectivity View of Hardware Only Compression Approach	7-13

7-1	Attributes of Protocol Entities	9-3

7-2	Example of Communications View Showing Protocol Stack and Nodes	Error! Bookmark not defined.

7-3	PDU Example, Space Packet Protocol	9-12

7-4	Example State Machine Diagram—SLE RCF	9-12

8-1	Attributes of Information Objects	10-3

8-2	Information Object Representations	10-7

8-3	Example of Functional View with Representation of Information Objects	10-8

B-1	SysML Diagram Types (SysML Partners)	Error! Bookmark not defined.

C-1	DoDAF Elements and Relationships (Partial)	Error! Bookmark not defined.

Table

4-1	Enterprise Objects	Error! Bookmark not defined.

5-1	Example Functional Objects	Error! Bookmark not defined.

5-2	Typical Infrastructure Objects	Error! Bookmark not defined.

6-1	Typical Nodes	Error! Bookmark not defined.

6-2	Typical Links	7-10

7-1	Typical Protocol Entities	9-7

C-1	DoDAF Views and Products and RASDS Viewpoints	Error! Bookmark not defined.

# Introduction

## Scope

This document describes a Reference Architecture for Space Data Systems (RASDS) that is intended to support modeling of space data systems and operations.  The RASDS provides a standardized framework and methodology for modeling space system architectures and related high-level designs, which individual agencies, projects, and working groups may use within CCSDS, ISO TC20/SC13, or ISO TC20/SC14 or the space agencies adopting this recommendation.

The extended approach in this revision has been renamed RASDS++, it is specifically adapted for the space domain and is aligned with best current practices in the fields of system and software architecture and modeling. While this architecture modeling methodology is intended for use within CCSDS and ISO Space Systems it is also suitable for use by mission and project design teams, to describe system architectures and designs within the space domain.  It uses a document-based representation, and does not propose any specific formal modeling method or tool, but with creation of suitable profiles it can be used with more formalized representations such as UML or SysML.

## Purpose

Within CCSDS and ISO TC20 the RASDS++ will be used for the following purposes:

- to establish an overall CCSDS and ISO TC20 recommended methodology for developing and modeling domain-specific architectures;

- to define a common language, taxonomy, and set of representations so that challenges, requirements, and solutions in the area of space systems can be readily communicated;

- to provide a kit of architect’s tools that domain experts may use to describe different specific complex space system architectures;

- to facilitate development of CCSDS and ISO TC20 Recommended Standards in a consistent way so that any standard can be used with other appropriate standards in a space system;

- to provide a framework and guidelines for presenting the architectural aspects of Recommended Standards developed by CCSDS and ISO TC20 in a systematic way so that their functionality, applicability, interrelationships, and interoperability may be clearly understood.

## Applicability

The methodology described in this Recommended Practice may be used by agencies, projects, or individual working groups to create models of space system architectures in any relevant CCSDS and ISO TC20 documents.

It is important to keep in mind that not all Viewpoints are needed for every task.  Only those Viewpoints that are required for a given purpose need to be used. In many instances, only the Functional and Connectivity Viewpoints may be needed.  The methodology provided in this document may be used in describing the architectures of individual system elements, entire mission space systems—or system of systems.

New views that align with this framework may be created as needed, and alternative representations may be adopted where they improve alignment with current practices in specialized subdomains.

As a Recommended Practice, this document provides a tool for CCSDS or ISO TC20.  Its use is encouraged in all CCSDS and ISO TC20 space systems documents where system or reference architecture descriptions are provided, and where the analytical and descriptive methodologies provided in this document will be useful.

The representations that are adopted in this document use simple drawing tools, but the viewpoints, views, terminology, and associated methodology have been directly adapted for use with the UML and SysML representations that are provided by typical MBSE tools.

## Rationale

Several different standard methods that are currently available for the description of software-intensive systems architectures were analyzed.    These all share the concepts of developing a consistent set of terminology and modeling elements, and also a relevant set of viewpoints, views, and specifications, with which to describe systems and their architectures.

All these standard methods assume that the elements of these systems are fixed in place or move over or near the surface of the Earth and that they are typically in continuous, instantaneous, communication over what are nominally error-free communications channels that suffer only occasional disruptions.  The physical environment in which many systems operate is often given only broad consideration.

Systems operating in space tend to violate many of these basic assumptions for terrestrial systems, and have particular concerns associated with orbital/planetary distances and the effects of the space plasma environment, Space elements may only occasionally be in contact with one another, typically require use of very expensive and over-subscribed ground and space communications assets, are strongly affected by the physical environment in which they have to operate and usually cannot be repaired or replaced. These environmental issues affect what must be done to provide reliable communications between elements, how control interactions must be designed, and how these systems must be developed, launched, tested, and operated.

RASDS++ provides a methodology and an abstract model for the description of the functionality, physical deployments, communication, structures, and operations of space systems that accounts for the realities of operating in the space environment. This is a domain-specific architectural approach adapted to the requirements of space systems.

Several separate viewpoints may be required in any given architecture description, depending upon the specific needs of a project.  The viewpoints that the original CCSDS RASDS document were adapted for the architectural description of systems in space: Enterprise, Functional, Connectivity, Communications, and Information. This RASDS++ version adds three other viewpoints: Services, Operational, and Physical (a superset of Connectivity), to support the needs of ISO TC20/SC14.  Other viewpoints may be identified as needed and this core set may be extended.

This is intended to be a pragmatic and useful document, including a set of straightforward drawing conventions that have been chosen to ensure that the diagrams for any View can be unambiguously interpreted.  The Objects that appear in any viewpoint have concrete and clearly documented representation, syntax, and semantic elements.  This document defines and adopts a consistent approach that can form the basis for a more formal MBSE representation of space systems such as might be developed in a UML [6] or SysML [9] tool.  Such formal representations using SysML have been developed for various projects.  More discussion of this topic is provided in annex B.

RASDS++ is consistent with the current version of ISO 42010-2022 [15], Software, systems and enterprise — Architecture description, and earlier architecture references [1], [2]. Where these “meta-architecture” documents describe “core terms, definitions and relationships for the Architecture Description”, define the meaning of terms like Viewpoint and View, and identify relevant software architecture frameworks, they do not provide specific instances of such viewpoints.

The RASDS++ methodology leverages this meta-model and provides concrete descriptions and specific viewpoints suitable for a wide variety of system architecture descriptions in the space system domain.  For those who are interested the derivation of this model is documented in Annex C.

## Document Structure

Section 2 provides an overview of RASDS++ in a narrative style and briefly introduces the key architectural concepts and the set of Viewpoints that are used.

Section 3 introduces and describes basic concepts and defines specific terms, elements, Viewpoints, Objects, and representation styles that are used throughout this document.

In sections 4 through 11 RASDS++ documents in detail each of the Viewpoints introduced in section 2, and formalizes each object and its interfaces, concepts, representation, special terms, and practical examples of how to use it.

Section 12 presents and example of how to extend RASDS++, using the basic concepts, to describe new views on a system.

Annexes A-D provide additional information on use of RASDS++, its relationship to other methods and models, including UML and SysML, and a summary of the terminology and acronyms used in the document.

## References

The following documents are referenced in this document.  At the time of publication, the editions indicated were valid.  All documents are subject to revision, and users of this document are encouraged to investigate the possibility of applying the most recent editions of the documents indicated below.  The CCSDS Secretariat maintains a register of currently valid CCSDS Reports and Recommended Standards.

[1]	Information Technology—Open Distributed Processing—Reference Model: Architecture.  International Standard, ISO/IEC 10746-3:2009.  Geneva:  ISO, 2009.

[2]	IEEE Recommended Practice for Architectural Description of Software-Intensive Systems.  IEEE Std 1471-2000.  New York: IEEE, 2000.

[3]	IEEE Standard for Application and Management of the Systems Engineering Process.  IEEE Std  1220-2005.  New York: IEEE, 2005.

[4]	Security Architecture for Space Data Systems.  CCSDS 351.0-M-1.  Magenta Book  Issue 1.  Washington, D.C.: CCSDS, November 2012.

[5]	Reference Architecture for Space Information Management.  CCSDS 312.0-G-1.  Green Book.  Issue 1.  Washington, D.C.: CCSDS, March 2013.

[6]	Unified Modeling Language (UML).  Version 2.5.1, formal/2017-12-05.  Needham, Massachusetts: Object Management Group, December 2017.  <https://www.omg.org/spec/UML/2.5.1/PDF>

[7]	Unified Modeling Language: Infrastructure.  Version 2.4.1, formal/2011-08-05.  Needham, Massachusetts: Object Management Group, August 2011.  <https://www.omg.org/spec/UML/2.4.1/Infrastructure/PDF>

[8]	Meta Object Facility (MOF) Core Specification.  Version 2.5.1, formal/2019-10-01.  Needham, Massachusetts: Object Management Group, October 2019.  <https://www.omg.org/spec/MOF/2.5.1/PDF>

[9]	Systems Modeling Language (SysML) Specification.  Version 1.6.  OMG Document: formal/19-11-01.  Needham, Massachusetts: Object Management Group, November 2019.  <https://sysml.org/.res/docs/specs/OMGSysML-v1.6-19-11-01.pdf>

[10]	SysML Extension for Physical Interaction and Signal Flow Simulation.  OMG Document: formal/21-05-03.  Needham, Massachusetts: Object Management Group, June 2021.  < https://www.omg.org/spec/SysPhS/1.1/PDF>

[11]	“DoD Architecture Framework (DoDAF).”  Version 2.02.  Department of Defense (DoD).  August 2010 <https://dodcio.defense.gov/portals/0/documents/dodaf/dodaf_v2-02_web.pdf/>

[12]	The Open Group Architecture Framework (TOGAF).  Version 10.0 Enterprise Edition.  San Francisco: The Open Group, April 2022 <https://publications.opengroup.org/downloadable/customer/products/>

[13]	International Standard for Systems and Software Engineering -- Life Cycle Management -- Part 4: Systems Engineering Planning. International Standard, IEEE/ISO/IEC 24748-4-2016. New York: IEEE, 2016.

[14]	Information Technology—Open Systems Interconnection—Basic Reference Model: The Basic Model.  International Standard, ISO/IEC 7498-1:1994.  2nd ed.  Geneva:  ISO, 1994.

[15]	Software, systems and enterprise — Architecture description, ISO/IEC/IEEE 42010:2022, Second edition 2022-11.  Geneva: ISO November 2022.

[16]	Overview of Space Communications Protocols.  Report Concerning Space Data System Standards, CCSDS 130.0-G-4.  Green Book.  Issue 4.  Washington, D.C.: CCSDS, April 2023.

[17]	The Application of Security to CCSDS Protocols.  Report Concerning Space Data System Standards, CCSDS 350.0-G-3.  Green Book.  Issue 3.  Washington, D.C.: CCSDS, March 2019.

[18]	Security Threats against Space Missions.  Report Concerning Space Data System Standards, CCSDS 350.1-G-3.  Green Book.  Issue 3.  Washington, D.C.: CCSDS, February 2022.

[19]	Space Communications Cross Support Architecture Requirements Document (SCCS-ARD).  Report Concerning Space Data System Standards, CCSDS 901.1-M-1.  Magenta Book.  Issue 1.  Washington, D.C.: CCSDS, May 2015.

[20]	P. Shames and J. Skipper.  “Toward a Framework for Modeling Space Systems Architectures.”  In Proceedings of the Ninth International Conference on Space Operations (SpaceOps 2006) (Rome, Italy).  SpaceOps, 2006.  http://www.aiaa.org/spaceops2006/presentations/57418.ppt < https://arc.aiaa.org/doi/10.2514/6.2006-5581 >

[21]	M. Maier.  “Architecting Principles for Systems-of-Systems.”  Systems Engineering 1, no. 4 (February 1999): 267-284.

[22]	CCSDS Bundle Protocol Specification, Report Concerning Space Data System Standards, CCSDS 734.2-B-1, Blue Book.  Issue 1.  Washington, D.C.: CCSDS, Sept 2015.

[23]	Operations Concept for a Solar System Internetwork (SSI). IOAG.T.RC.001.V1. Washington, DC: IOAG, 15 October 2010.

[24]	Cross Support Transfer Services—Monitored Data Service (MD-CSTS), Report Concerning Space Data System Standards, CCSDS 922.1-B-2, Blue Book.  Issue 1.  Washington, D.C.: CCSDS, Sept 2022.

[25] “Service oriented architecture Modeling Language (SoaML) Specification”, OMG Document: formal/20120501.  Needham, Massachusetts: Object Management Group, May 2012 < https://www.omg.org/spec/SoaML/1.0.1/PDF>

[26]	Application and Support Layer Architecture (ASL).  Report Concerning Space Data System Standards, CCSDS 371.0-G-1.  Green Book.  Issue 1.  Washington, D.C.: CCSDS, November 2020.

[27]	P. Shames, S Freidenthal, M. Sarrel.  “A Representative Application of a Layered Interface Modeling Pattern.”  In 26th Annual INCOSE International Symposium (IS 2016) (Edinburgh, Scotland, UK).  INCOSE 2016, July 2016.  <http://www.omgsysml.org/INCOSE_IS_2016_paper_Application-of-a-Layered-Interface-Modeling-Pattern.pdf >

[28]	P. Shames, et al.  “NASA Integrated Network Monitor and Control Software Architecture .”  In Proceedings of the Twelfth International Conference on Space Operations (SpaceOps 2012) (Stockholm, Sweden).  SpaceOps, 2012. < https://ntrs.nasa.gov/citations/20130009082>

[29]	Charles Kohlhase, editor (artist Phil Gwinn). “The Voyager Neptune Travel Guide”, JPL Pub 89-24: June 1989. < https://ntrs.nasa.gov/citations/19900004096>

[30]	Richard Hilliard.  “Systems and software engineering — Architecture description”  ISO Architecture.org. < http://www.iso-architecture.org/42010/cm>

[31] “Unified Architecture Framework (UAF) Domain Metamodel”, OMG Document: formal/22-07-03.  Needham, Massachusetts: Object Management Group, August 2022 < https://www.omg.org/spec/UAF/1.2/DMM/PDF>

[32]	Mordecai, Yaniv. “Model-Based Protocol Specification.” Systems Engineering 22 (2). 2019. < https://www.researchgate.net/publication/331454717_Model-based_protocol_specification/figures>

[33]	NASA Systems Engineering Processes and Requirements, NASA NPR 7123.1D, July 2023. https://nodis3.gsfc.nasa.gov/npg_img/N_PR_7123_001D_/N_PR_7123_001D_.pdf

[34]  “The SQL Standard”, International Standard.  ISO/IEC 9075:2023 (ANSI X3.135).  Geneva:  ISO, 2023

[35]	SPARQL 1.1 Query Language, International Standard, SPARQL.  World Wide Web Consortium (W3C), March 2013.  <https://www.w3.org/TR/sparql11-query/>

[36]	Space Assigned Numbers Authority (SANA) – Role, Responsibilities, Policies, and Procedures, Report Concerning Space Data System Standards, CCSDS 313.0-Y-3.  Yellow Book.  Issue 3.  Washington, D.C.: CCSDS, October 2020.

[37]	CCSDS SANA Registry Management Policy (RMP), Report Concerning Space Data System Standards, CCSDS 313.1-Y-2.  Yellow Book.  Issue 2.  Washington, D.C.: CCSDS, October 2020.

[38]	Spacecraft Onboard Interface Services – Specification for Dictionary of Terms for Electronic Data Sheets.  Draft Recommended Practice, CCSDS 876.1-R-3. Draft Red Book. Issue 3. Washington, D.C.: CCSDS, July 2023.

[39]	OWL 2 Web Ontology Language Document Overview (Second Edition), International Standard, OWL2.  World Wide Web Consortium (W3C), December 2012.  <https://www.w3.org/TR/2012/REC-owl2-overview-20121211/>

[40]	Protege – ontology development environment (Protégé), Free, open source tool.  Stanford University.  <https://protege.stanford.edu/software.php>

[41]	Business Process Model and Notation (BPMN 2.0.2), OMG Document, formal/2013-12-09.  Object Management Group, December 2013. https://www.omg.org/spec/BPMN/2.0/PDF

[42]	Security architecture for Open Systems Interconnection for CCITT applications, Recommendation X.800, Geneva: ITU, March 1991

[43]	Baseline capabilities for enhanced global identity management and interoperability, Recommendation ITU-T X.1250, Geneva: ITU-T, September 2009

[44]	XML Specification for Electronic Data Sheets.  Recommended Standard, CCSDS 876.0-B-1.  Blue Book. Issue 1. Washington, D.C.: CCSDS, April 2019.

[45]	System life cycle processes, International Standard. ISO/IEC/IEEE 15288:2023, second edition. Geneva: ISO, May 2023.

[46]  J. Postel. Transmission Control Protocol (TCP). STD 7. Reston, Virginia: ISOC, September 1981.

[47] OMG Systems Modeling Language ™ (SysML®).  Version 2.0 Beta 1, Preliminary Revision 2024-01.  OMG Document.  Needham, Massachusetts: Object Management Group, February 2024.  < https://www.omg.org/spec/SysML/2.0/Language/ >

# Overview

This overview section introduces the basic RASDS++ modeling terminology and concepts in a narrative form.  Many of these terms will be familiar, but some have special meanings in RASDS++.  Please refer to the Glossary, Annex D, as needed for specific definitions of terms.

## Relating Architecture and systems engineering

It is generally understood that all systems (like all buildings) have an architecture, even if that may not always be well documented.    It also appears to be well understood that the more complex the system the more it is useful, if not essential, to document and analyze the architecture before starting detailed system design.  In fact, it is frequently the case that some sort of architecture views will be constructed during early design phases, even if there is not a formal system architecture created, reviewed, and approved as part of the process.

Interestingly, the classical “Systems Engineering Vee” is often shown without any reference to systems architecture.  See fig 2.1 which is borrowed from the NASA SE Handbook [33].  Note that this “Vee”, and variations on it, “Dual V”, “W”, appear in many related processes, including Agile development, and but that only some of these explicitly include systems architecture as a formal phase in the process.  The ISO Systems Engineering Lifecycle Processes, ISO/IEC 15288:2002, [45] provides a simplified Systems Architecture Design Process in sec 5.5.4.

Figure 2-1: Systems Engineering “Vee”

Copied from the NASA SE Process NPR 7123.1D

In the NASA Systems Engineering Vee systems architecture is referenced at essentially the phase called Logical Decomposition and Preliminary Design.  The Logical Decomposition step is defined as:

“The decomposition of the defined technical requirements by functions, time, and behaviors to determine the appropriate set of logical and data architecture models and related derived technical requirements. Models may include functional flow block diagrams, timelines, data control flow, states and modes, behavior diagrams, operator tasks, system data, metadata, data standards, taxonomy, and functional failure modes.“

So “functions and behaviors”, and “logical and data architecture models”, are an intended outcome of the logical decomposition process, but there is no stated process, features, nor conformance criteria for such a set of products.  By contrast, The Open Group Architecture Framework (TOGAF) [12], shows several stages of architecture development (Vision, Business, Information Systems, and Technology architectures, embedded in an Enterprise Metamodel) all feeding into a central Requirements Management process and then the design and development processes.

In the abstract, the relationship between systems architecting and systems engineering might be represented as shown in figure 2-2.

Figure 2-2: Systems Architecture vs Engineering

This discussion of the differences between architecting and engineering aside, what this document presents is an entirely practical methodology, one that is grounded in formalisms, but is suitable for direct application by working systems architects on their current projects.

## Overview of RASDS++ viewpoints

Each Viewpoint is a particular perspective on the specification of a complete system, established to model those aspects of a system relevant to the identified area of concern during the design of the system. The Viewpoints are intentionally independent to simplify reasoning about the complete specification. Mutual consistency among the Viewpoint Specifications is ensured by the architecture descriptions defined by RASDS++ and the use of an integrated object model that describes their elements and relationships.

Figure 2-3 provides an overview of all the defined viewpoints, the Concerns that they each address and the kinds of objects that they are used to represent.

Figure 2-3: 	RASDS++ Viewpoints, Concerns, and Objects

This figure introduces a specific set of color codes that will be referenced in the definitional object and ontology diagrams that are included in each Viewpoint.  Elsewhere in RASDS there is no special meaning assigned to colors, but the introductory diagrams in each viewpoint of this document use these colors to distinguish elements that have their “home” in a given viewpoint and those that are referenced, by Correspondence, from other viewpoints.  These concepts are introduced formally in the RASDS++ Conceptual Ontology, fig 2-4, and in related foundational ontology diagrams that occur in each Viewpoint section.

### RASDS++ Viewpoint Features

The RASDS++ framework provides seven specific and complementary viewpoints on the system and its environment, along with two derived viewpoints:

- The Enterprise Viewpoint focuses on the purpose, scope, and policies of a space system. It can be used to describe the organizational entities and relationships; their roles, requirements, goals, objectives, scenarios, constraints; and how to meet them.

- The Functional Viewpoint describes the functional decomposition of a space system into abstract objects that interact at interfaces. It describes the functionality provided by the space system, the behavior of the functional elements, and their functional decomposition.

- The Physical Viewpoint describes the engineered decomposition of a space system into physical components, their connections, and the external environment within which it operates. It describes the physical deployment and environmental aspects of space system elements, and the physical forces and behavior (motion, radiation, gravity) acting on the components. The connections may be manifestly material (electrical, weldments, bolts, mating surfaces, joints), or they may be more energetic (radiation, thermal, magnetic, gravitational). This Viewpoint “owns” the physical elements, but different aspects may be analyzed in derived Viewpoints, such as Connectivity and Structural.

- The Connectivity Viewpoint is derived from the Physical Viewpoint.  It specifically addresses the engineered decomposition of the space system into components (often referred to as nodes) that communicate across connectors (links). The Connectivity Viewpoint describes the communications aspects of the physical deployment of the space system.  The links may be manifestly material (network or data cables), or they may be more energetic (RF and optical signals).  The Connectivity Viewpoint may also be used to address the allocation of implemented functions (as engineered software or hardware objects) to these Nodes.

- The Structural Viewpoint is also derived from the Physical Viewpoint.  It specifically addresses the engineered decomposition of the space system into components that are physically connected one to another. The Structural Viewpoint describes the physical deployment and connection aspects of the space system, its physical decomposition, and its interactions with the rest of the physical environment within which it operates.  These structural connections are manifestly material: bolts, weldments, joints (fixed, flexural, or rotational), mating surfaces, etc.

- Other derived Physical Viewpoints: There may be other physical, or energetic, aspects of space system design that can be handled directly in the Physical Viewpoint or in other viewpoints derived from it.  Derived Viewpoints may be required for different energetic exchanges, such as thermal, gravitational, or electrostatic.

- The Communications Viewpoint focuses on the mechanisms and functions required to engineer, document, and implement the protocols, protocol stacks, and communications standards for a space system. This includes implementation choices and specifications, protocol stack choices, and allocation of this communications functionality to engineered components of the system.  This Viewpoint is treated separately in RASDS++ because it is essential for describing how end-to-end communications are designed and handled in space systems.

- The Information Viewpoint focuses on the kinds of information handled by the system, the structure & semantics of the information, and the interpretation of that information. It describes the information managed by the space system along with the structure, content, semantics, type, relationships, and constraints on the data used within the system.

- The Service Viewpoint is an aggregated view that focusses on the kinds of services provided by the system, the functions exposed at the system interfaces, the operations provided by the services, the kinds of information exchanged across these interfaces, and details of the interface bindings. It describes the exposed behavior at the space system interface.  The details of the information objects exchanged at the interface are defined by correspondence in a related Information View and the details of the protocols are defined by correspondence in a related Communications View.

- The Operations Viewpoint focuses on the kinds of operations supported by the system, the processes, procedures, activities, and the kinds of operational behaviors carried out by the system, whether carried out by people or systems elements.  The details of any information objects exchanged at operational interfaces are defined by correspondence in a related Information View. The details of the systems elements are described in a corresponding Functional or Connectivity View.

### Conceptual Model and Correspondences Between Viewpoints

Each of the RASDS++ Viewpoint Specifications is intended to be orthogonal to the others, and a description of a given system from any one Viewpoint should be self-consistent.  Additionally, many of the objects defined in their “home” RASDS++ Viewpoint will also have identifiable relationships or correspondences with objects defined in other Viewpoints.  This is an essential element of the methodology.  In figure 2-4 the relationships among the core set of objects defined in RASDS++ are shown in a conceptual model, using a simple form of graphical ontology.

In figure 2-4 only the top-level (or core) objects and attributes in RASDS++ are shown, those most central to understanding how the core objects from all the different Viewpoints are related.  Each of these core objects represents a class of objects, as will be described in the following sections.   These classes of objects may have subclasses and all have an associated set of attributes. The full set of attributes for each of these objects and their classes are shown in the later sections of this document.  Finally, figure 2-4 captures only the static relationships among objects; it does not capture any of the dynamic behavior of objects, in either a functional or physical sense.

The element named Meta-model in the upper right hand corner of figure 2-4 represents the RASDS++ Viewpoint Specifications, Views, and associated user concerns.  Each Viewpoint Specification can be thought of as a perspective on a system that defines the objects and rules for constructing views, and each permits only a subset of objects and representations relevant for a given concern to be analyzed.  Each of these top-level objects is defined in a ‘home’ Viewpoint in RASDS++, but many of them will have representations or correspondences in other Viewpoints.

Thus the ‘home’ Viewpoint for Information Objects is the Information Viewpoint, but representations of Information Objects will often appear in the Enterprise, Functional, and Connectivity Viewpoints.  Similarly, abstract Functional Objects defined in the Functional Viewpoint have correspondences with implemented Engineering Objects (hardware or software) in the Connectivity Viewpoint.  Communications Objects, in the form of protocol stacks, are often shown in correspondence with Connectivity Viewpoint objects (Nodes) and the Links that connect them.

Figure 2-4: RASDS++ Ontology – Concept Model, Objects and Relationships

Any set of RASDS-compliant system specifications that use different Viewpoint Specifications should not contain contradictory statements; i.e., the Views should be mutually consistent. Thus, a complete specification of a system will include descriptions of the correspondences relating elements in one View to elements in another View, and this helps to ensure that the model is consistent. The minimum requirement for consistency in a set of RASDS++ specifications for a system is that they adhere to the rules for the selected Viewpoints and exhibit within the set of specifications the relevant correspondences defined in this Reference Architecture.

In many of the examples used in this document color is used to distinguish different classes of objects.  In general, RASDS++ does not assign any particular meaning to the use of color and the user is free to employ color and other “decorations” wherever it assists in clarifying the representation of the architecture or bringing consistency to a set of diagrams.  There is, however, one specific use of color, and that is in the object definitions that are introduced starting in Fig 2-3 and the ontology in Fig 2-4.  These colors are treated as a “color key” and they are used in all of the foundational object definitions and individual object ontologies that appear in Sections 4 through 12.

## Selecting RASDS++ Viewpoints for a given use

To describe the architecture of a particular space system, RASDS++ defines seven primary Viewpoint Specifications, and two derived ones, as just described.  The user must decide which of these Viewpoints will be of value to describe a particular space system, and if that system can be characterized with fewer than all seven Viewpoints. Often only two or three Viewpoints are needed to define simple system architectures.  More complex and thorough architecture descriptions may require use of the full set.  Depending on the complexities of the architecture, multiple perspectives of the same Viewpoints may also be required, potentially with different levels of decomposition or different kinds of views.

Different users may find that one specific Viewpoint provides them with a most familiar or useful “point of entry”.  For instance, a data system application designer might favor the Functional Viewpoint, where a structural engineer might favor the Physical Viewpoint and the end-to-end information system engineer might favor the Communications Viewpoint as a starting point.  What RASDS++ provides is a methodology that helps relate all these different viewpoints to provide an integrated whole.

Sometimes a Viewpoint Specification will contain references to objects that are representations of related objects in another Viewpoint.   An example of this is the Physical Viewpoint that is newly introduced in RASDS++.  In RASDS++ the Physical Viewpoint “owns” all the physical, three-dimensional, objects in the system, the environment within which (and with which) they interact, and all their physical and environmental attributes.  However, different stakeholders will typically have different concerns and focus on different physical aspects: the structural engineer is concerned with the masses, centers of gravity, strength, and structural / flexural properties of connected components; but the communications engineer is concerned with the flows of data, whether using cables (which also have mass) or RF or optical transmissions, and the thermal engineer with the flow of heat.

The basic set of physical objects is the same, but the aspects that are considered and their attributes differ.  In RASDS++ the Physical Viewpoint has been identified as core viewpoint for physical objects, with the expectation that users will adopt one or more derived viewpoints: structural, communications, thermal, power, as needed for their specific purposes.  The core physical object attributes are intended to be referenced, via correspondence, in any of the derived viewpoints that are required for a given system model.

If it is impossible to capture all the important aspects of the system with the objects and Viewpoints described in these Viewpoint Specifications, the user may define a new Viewpoint Specification by leveraging the basic concepts and approaches described in section 3.

## Security Views in RASDS

The CCSDS publication guidelines mandate that Security topics shall be addressed in CCSDS Blue and Magenta Books and these are to include the following information: security background/introduction; statements of security concerns with respect to the CCSDS document; data privacy and integrity; authentication of communicating entities; control of access to resources; availability of resources; auditing of resource usage; potential threats and attack scenarios, consequences of not applying security to the technology (e.g., loss of life, loss of mission).

There is not a single “Security Viewpoint”, in RASDS++ largely because security is such a cross-cutting topic and it potentially touches on most of the defined viewpoints. Accordingly, this document provides guidance to users on how to describe security approaches within the context of system architecture, and each viewpoint provides the means to address the security topics that are relevant in that viewpoint. Security topics pertinent for each viewpoint are explicitly identified and briefly addressed in each section.  Detailed explanations of security topics and approaches are treated separately in the CCSDS Security Architecture [4] and in related security documents ([17] and [18]).

# RASDS++ Modelling Concepts

## General

This section defines in a more formal way the concepts and terms for systems architecture modeling that are used throughout this document. The RASDS++ is, at its core, a model-based systems engineering approach to specifying system architectures. Modeling using objects as part of developing a systems architecture provides a formalization that uses well-established abstraction and encapsulation design practices that are familiar from structured programming and that are adopted in most MBSE methods.

These abstractions specifically allow the descriptions of system functionality to be separated from the details of system implementation.

Encapsulation allows the hiding of the mechanisms of service provision from the service user, the hiding of design heterogeneity, the localization of interaction points, and the implementation of security.

The object modeling concepts cover:

- Basic model features, providing rigorous definitions for the core set of concepts (object, interface, action, and interaction) that form the basis for RASDS++ system descriptions and are applicable in all Viewpoints.

- Specification concepts that address notions such as object type and class that are necessary for reasoning about objects and the relationships among objects, providing general tools for design, and establishing clear viewpoint specification languages.

- Structuring concepts that build on the basic model features and the specification concepts to provide useful viewpoints from which to describe space system architectures, address recurrent structures that appear in distributed systems, and cover such concerns as role, behavior, capability, and communication.

## Definitions

### Overview

The following concepts and terms are used commonly in the Viewpoints presented in sections 4 through 11. In each of the sections that present Viewpoint Specifications, any definitions that are essential for that Viewpoint may be repeated, and definitions that are used only in that Viewpoint will be provided.

### Basic Elements

An Entity is any concrete or abstract thing of interest. For example, an entity may be a physical instrument, a computer, a piece of software, or a set of functions performed by a system.

Note: In general the word entity can be used to refer to anything, but in the context of modelling it is reserved to refer to things in the universe of discourse being modelled.

An Element is a constituent part of something; any thing that is one of the individual parts of which a composite entity is made up; an identifiable component, process, or entity of a system.

Abstraction is a mechanism and practice to reduce and factor out details so that one can focus on few related concepts at a time.  It is the process of extracting the underlying essence of a concept, removing any dependence on real world objects with which it might originally have been connected, and generalizing it so that it has wider applications.

An object is an abstract model of an entity in the real world. It contains information, has behavior, and may offer services. A system is composed of interacting objects. An object is characterized by that which makes it distinct from other objects.

### Properties of Elements

A type specifies the set of values allowed and the primitive operations that an object can provide. Types are grouped into classes, which share the same primitive operations.

An attribute is a characteristic of an object, i.e., a construct that system designers use to add additional information to system elements (e.g., objects, modules, types) to define their functionality.

An action is something that happens within an object, either with or without participation of another object.  An interaction is an action performed by an object with participation of another object or with its environment.

A behavior is a set of actions performed by an object for some purpose.

An activity is a specification of behavior described as a sequence of actions.

A constraint is a limitation or implied requirement that limits the design solution or implementation, is not changeable by the enterprise, and is generally nonallocable.

An interface is a set of interactions provided by an Element for participation with another object for some purpose, along with constraints on how they can occur.

Note: An interface represents a set of mechanical, electrical, signal, or other properties that describe some aspect of a Element’s connection to, or interaction with, another Element.

Configuration describes a collection of objects able to interact at interfaces. A configuration determines the set of objects involved in each interaction along with constraints on their interactions.

A service is a provision of an interface of an Element to support actions of another Element.

A service interface is a mechanism to enable access to a set of one or more functions of an Element, where the access is provided using a prescribed interface and is exercised consistent with constraints and policies as specified by the service description.

Relationship describes the way that two or more entities can be associated with each other.

Structure refers to the relationships among a set of elements that contribute to the properties of the whole and enable them to interact.

A role describes the way in which an entity participates in a relationship; an object’s set of behaviors and actions associated with the relationship of that object with other objects.

Syntax is the grammar defining the valid set of symbols and well-formed linguistic constructs of a language.

Semantics are the rules by which syntactic expressions and model elements are assigned meaning.

Interoperability refers to the technical capability of two or more systems or components to

exchange information and to use the information that has been exchanged.

### Logical & Physical Elements

An objective is something to be done or achieved.  Objectives tend to be precise, tangible, and concrete.

A goal is an aim or purpose toward which effort may be directed.  Goals tend to be broad or abstract and to state general intentions.

A function is the set of actions or activities performed by some element to achieve a goal.  The transformation of inputs to outputs may include the creation, modification, monitoring, or destruction of elements.

A Logical Object is an abstract entity that may be considered separately from any particular implementation or deployment.  It has no physical manifestation except as part of a model, but it may have associated behaviors and interfaces.  Enterprise, Functional, and Information objects are all logical objects.

A Logical Link is the locus of relations among Logical Objects.  It may be considered separately from any particular implementation or deployment and has no physical manifestation except as part of a model.

Note: A logical object interacts with other objects over a logical link.  A physical object (component) interacts with other objects using some physical link (connector).

An aggregation is several things grouped together or considered as a whole: aggregation is also the act of gathering things together.

Composition is a form of aggregation.  Composition may be recursive.

A Composite Object is an Object composed of two or more Objects via aggregation.  The behaviors of the composite object are determined by those of the Objects that it aggregates.

A Location is a point or extent in space.

The environment is a complex of external factors that acts on a system and determines its course and form of existence. The environment of some system or object consists of the substances, circumstances, objects, influences, or conditions by which it is surrounded or in which it occurs.

Note: An environment may be thought of as a superset, of which the given system is a subset. An environment may have one or more parameters, physical or otherwise.

A resource is anything available to a system that can support the achievement of objectives; any physical or virtual element that may be of limited availability within a system, such as hardware, software, programs, information, data, and other devices that are in use within or connected to a given system.

### Systems ArchitectURE Terms (from ISO 42010)

Architecture is the concepts and rules that define the structure, semantic behavior, and relationships among the parts of a system in its environment.  It includes the elements (models of entities) that compose the system, the relationships among the elements, the constraints that affect those relationships, a focus on the parts of the system, and a focus on the system as a whole.

Architecting is the process of conceiving, defining, documenting, maintaining, improving, and certifying an architecture throughout the lifecycle of a system. It is both a science and an art.

A model is an abstract formal specification of the structure and/or function of a system.

A system is a set of interacting or inter-related elements (people, hardware and software, facilities, equipment, material, and processes [automated as well as manual procedures]) that form a unified whole and whose behavior is intended to satisfy customer and/or operational needs.

Note: Every system has an architecture and includes a set of entities, even if the architecture is not clearly and accurately described.

An Architecture Description is a work product used to express an Architecture.  It may contain stakeholders, concerns, viewpoint specifications, viewpoints, views, correspondences, representations, and other elements.

A Stakeholder is an individual, team, or organization (or classes thereof) with interests in, or concerns relative to, a system.

Concerns are those interests, which pertain to the system’s development, its operation, or any other aspects that are critical or otherwise important to one or more stakeholders. Concerns include system considerations such as performance, reliability, security, distribution, and evolvability.

Perspective in systems architecture is the choice of a context or a reference (or the result of this choice) from which to describe, categorize, explain, or codify system design, typically for comparing with another.

Note: To choose a perspective is to choose a value system related to a set of stakeholder concerns. An Enterprise perspective relates to an organizational value system. A Functional perspective relates to a capability value system.

A Viewpoint is set of conventions, achieved using a selected set of architectural concepts and structuring rules, for the creation interpretation, and use of an architecture viewpoint to frame one or more particular concerns within a space system.

A Viewpoint Specification defines a pattern or template from which to construct individual views, and it establishes the rules, techniques and methods employed in constructing a view.

A View is a representation of a system from the perspective of a set of concerns. Views are themselves modular and well formed, and each view is intended to correspond to exactly one Viewpoint. A View may include representations or correspondences to elements defined in other Viewpoints.

Correspondence an identified relationship from an element in one viewpoint to a related element in another viewpoint.  May be used to express a wide range of relationships, such as equivalence, transformation, composition, refinement, consistency, constraint.

Aspects capture a set of characteristics or features of the entity of interest in its environment to address concerns within an Architecture Description.

A representation is some way of organizing, manipulating, presenting, and storing information; a visual or tangible rendering of something

A Requirement is a formal statement of: (1) An attribute to be possessed by the element or a function to be performed by the element. (2) the necessary performance for the attribute or function. (3) the measuring process to be used in verifying that the necessary performance has been met.

A specification is a set of requirements or other descriptive information for a system or classifier.

A policy is the set of guidelines and constraints on the behaviors and states exhibited by the objects in the system.

A standard is a formal specification that defines and governs functions and protocols at interfaces of a data system.  It can describe capabilities in detail and establishes the requirements to be met by interfacing subsystems to achieve compatibility.

## Graphical Representations

The icons shown in figure 3-1 are used throughout this document.  Some minor variants of these icons are introduced as needed in the body of the text.

The intent behind these representation choices is to provide a distinct set of icons associated with the different kinds of Objects used in each Viewpoint.  This is to avoid the frequently seen phenomenon of everything (system, team, software element) looking like the same rectangular box, and every interaction looking like the same solid line.  These same kinds of distinctions of object and connection types can be transferred into an MBSE tool as well, with no loss of meaning, by defining specific viewpoints and views and creating a profile with a suitable set of object stereotypes associated with each of these different kinds of objects.

Figure 3-1	:  Icons Used in This Document

## Characteristics of Objects

As introduced in section 2, each of the Viewpoints of RASDS are described by defining the key Objects and their interactions. The set of typical interfaces and attributes of Objects are shown in figure 3-2.  A specific set of representations for the objects and interfaces relevant to each viewpoint are provided in each Viewpoint section this document.

Any given Object may expose one or more Service Interfaces and provide one or more Core Functions.  Through its External Interfaces, it may call upon other Objects to provide services to it.  The Management Interfaces may be explicit (for instance, a Service Management call to a Protocol Entity) or they may be implicit and be represented by internal tables or configuration items.  The only Objects used in RASDS++ that do not exhibit all these interfaces are Information Objects.

Figure 3-2:  Unified Representation of Objects

The types of arrows used to indicate different kinds of interfaces in this, and other similar object overview diagrams, has no special significance in RASDS, but double headed arrows are typically used to represent input / output interfaces and single headed arrows are used to indicate directionality of the interfaces.  Other kinds of representations, see Fig 3-3, may use a specific set of arrow types to convey specific meanings.

RASDS does not normally assign any particular meaning to the use of color and the user is free to employ a color palette wherever it assists in clarifying the representation of the architecture or bringing consistency to a set of diagrams.  However, for clarity in understanding the relationships among objects defined in different viewpoints, and their correspondence relationships with objects defined in other viewpoints, each of the top level object and ontology diagrams in all of the following sections adopt the color codes for objects that are defined in Fig 2-4.

Each Viewpoint provides an ontology diagram describing the primary and secondary objects defined in each Viewpoint and the relationships among them.    These diagrams are themselves a form of Information View, and they use the kinds of arrows shown in Fig 3-3 to represent these relationships.  See the Information Viewpoint, Sec 10, for more discussion on various forms of information modeling.

Figure 3-3:  Relationship types among objects	 (derived from UML)

Correspondence is a relationship from an object defined in one viewpoint to related objects that are formally defined in other viewpoints.

Figure 3-4 provides an example of the kind of ontology diagram that is provided for each viewpoint.  It displays composition, directional associations, and correspondence (dashed line) relationships.

Figure 3-4:  Example of a RASDS Viewpoint Ontology (Functional)

All these primary object definition and ontology diagrams use the colors keys for viewpoint objects that are defined in Fig 2-4.

# Enterprise Viewpoint Overview

The Enterprise Viewpoint addresses the complex organizational relationships and roles involving various resources (spacecraft, instruments, ground systems) and personnel (scientists, staff, and contractors) that may be distributed among multiple organizations (space agencies, science institutes, companies, etc.).  The Enterprise Viewpoint also addresses other organizational aspects, such as policies, requirements, governance, assets, roles, and capabilities.

## Concerns

Concerns addressed by the Enterprise Viewpoint are:

- the organization responsible for the system;

- the capabilities provided by the system;

- the purpose, scope, and policies for the system;

- the objectives, concepts of operations, and scenarios for the system;

- the requirements and constraints on the system;

- roles played by the system elements.

## Concepts for Enterprise Viewpoint

The Enterprise Viewpoint of a space system focuses on the community, purpose, scope, roles, and policies for that system.  This Viewpoint includes organizations as well as other Enterprise Objects that have assigned roles, responsibilities, and interactions.

In the Enterprise Viewpoint, a space system is depicted as a set of Enterprise Objects and their relationships, interactions, and the roles that they perform. Enterprise Objects that have significant resources may appear in an Enterprise View as Facilities.

An Enterprise Object represents an entity that is governed by a single authority that has its own objectives and policies for operating the object.

An Enterprise Object may be a component of another larger Enterprise Object, which may in turn be a component of a third, even larger, Enterprise Object.  Enterprise Objects may participate wholly or in part in other Enterprise Objects.

A Facility is a physical infrastructure element that supports the use of services and other resources.

A Resource is anything available to a system that can support the achievement of objectives; such as hardware, software, programs, information, data, and other devices that are in use within or connected to a given system.  In this context a Resource is an Enterprise Object that has some role, offers services, and performs some action within a system. A resource may serve more than one activity.

An organizational Enterprise Object may own a facility or resource Enterprise Object.

Ownership means having administrative and fiscal responsibility for the owned element and the right to exclusively control and use that which is owned for one’s own purposes.   It is the state or fact of having exclusive possession or control of some object, facility, intellectual property, or some other kind of property.

Not every organizational object owns facilities or resources.  Some resources are owned by one organization and used by others.  The term Cross-Support is used to describe an  agreement between two or more organizations to exploit the technical capability of interoperability for mutual advantage, such as one organization offering support services to another in order to enhance or enable some aspect of a space mission.

## characteristics of Enterprise Objects

### General

The characteristics of Enterprise Objects are shown in figure 4-1. Enterprise Objects are characterized by their roles, objectives and resources, and their interactions involve Requirements, Agreements, contracts, and constraints such as policies and rules. They exchange information such as Requirements, Memoranda of Understanding, Service/Support Agreements, Interface Control Documents, and so on.

Interfaces among Enterprise Objects are often created because of shared science or exploration goals and may involve cross-support agreements, interoperability requirements, and agreements on data sharing and access. Various standards for information exchange or documenting procedures are often employed as the means for enabling these interfaces to work.

The Enterprise Viewpoint may also be used to represent Scenarios and Operations Concepts.  This is the primary system Viewpoint where personnel, operations issues, policies, and other organizational concerns are expressed.  Roles of Enterprise Objects may include terms like owner, operator, science user, service provider, contractor, developer, tester, manager, and data acquisition, data relay, orbiter, lander, or other descriptive names.

In collaborative (or commercial) joint enterprises attention will need to be paid to policies and agreements, and formal governance arrangements may need to be established and sustained.

Figure 4-1	:  Overview of Enterprise Objects

### Key Objects and Relationships

The following elements may appear in the Enterprise Viewpoint:

- Enterprise Objects:

- Types: Organizations, both formal and informal (missions, projects, communities, with their roles and responsibilities), and Assets (resources, people, and facilities or other elements having enterprise operational or service roles);

- Attributes: name, role, objectives, point of contact, location, members, assets, resources, provided services, types, interfaces and data, interaction modes, requirements, constraints;

- Domains (boundaries of responsibility or ownership);

- Relationships (ownership, membership, participation, roles, contractual);

- Information (defined instances of documents, agreements, contracts, policies, requirements, objectives, goals, scenarios, membership lists, interface specifications,  where formal specifications of the data to be exchanged are found in the Information Viewpoint).

#### Conceptual Object Model

Figure 4.3 provides a conceptual model of the key Enterprise objects and some of their relationships to the most closely related objects from other viewpoints, such as Resources and operations.

Figure 4-2	:  Ontology of Enterprise Objects

NOTE: Consistent with Fig 2-4, objects shown in green are defined within the Enterprise Viewpoint.  Other objects, shown in different colors, are referenced by correspondence from other viewpoints.

Figure 4-3	:  Representation of Enterprise Objects

#### Object Representation

Fig 4-3 shows the recommended, document-style, representation of Enterprise Objects using the adopted drawing method.  As with all the viewpoint representations, the intent is to adopt a drawing style that is both expressive and sufficiently unique to the viewpoint.

## Terms for the Enterprise Viewpoint

### Attributes for Enterprise Viewpoint

A Requirement specifies a condition or capability that must be met or possessed by a system or system element to satisfy an agreement, standard, specification, or other formally imposed document.

A Use Case is a list of actions or event steps typically defining the interactions between an actor and a system to achieve a goal.  A Use Case may describe a situation where a system may be used or a potential scenario in which a system receives an external request and responds to it.

A Capability is the ability to achieve a desired outcome under specified conditions using a combination of activities and resources to satisfy a stakeholder need.

A (Capability) Requirement is a type of requirement describing the capability that the organization or system must provide.

Risk Management is the program and supporting processes to manage information security risk to organizational operations (including mission, functions, image, reputation), organizational assets, individuals, other organizations, and the Nation, and includes: (i) establishing the context for risk-related activities; (ii) assessing risk; (iii) responding to risk once determined; and (iv) monitoring risk over time.

A contract is an agreement that specifies certain legally enforceable rights and obligations pertaining to two or more parties. A contract typically involves the transfer of goods, services, money, or a promise to transfer any of those at a future date

### Unique terms for Enterprise Viewpoint

An Organization is a formal group of people with one or more shared goals.

A Space Enterprise (e.g., NASA) is a top-level, autonomous entity that is dedicated to the exploration and/or exploitation of space. It has its own objectives, resources, and policies, and it is not a component of any other Space Enterprise.

Under a federated approach (see below) two or more space enterprises (e.g., NASA and ESA) may support common Enterprise objectives.

A Community (e.g., Earth Science) can exist within one Space Enterprise or across multiple Space Enterprises. It is distinguished by being bound by common objectives and relationships and offers a set of resources that can be shared within the Community and with other Communities.

A Domain (e.g., NASA Code Y) is a type of Community that is under single organizational, administrative, or technical control. A domain may have resources, policies, access control, and possibly constraints on quality of service.

A Domain may be subdivided into Subdomains. Multiple independent Domains may be organized into a Federation.

A Federation (e.g., CEOS or CCSDS) is a Community consisting of multiple Domains that come together to share resources while each domain retains its authority over its own resources. Federations are governed by negotiated agreements.

A Federation may include only some members of a Domain or Subdomain (e.g., a particular Earth Observing project).  Members of a Federation agree on rules for sharing resources and for joining and/or leaving the federation.

A scenario is a specific sequence of activities that describes system behaviors.  A scenario may be used to describe a set of interactions of system elements.  Scenarios may be used to derive use cases.

An operations concept is a verbal or graphic statement, in broad outline, of assumptions or intent regarding the operation of the system. The concept of operations frequently is embodied in observing plans and operations plans.  The concept is designed to give an overall picture of the operation of the system.

An Asset is a person, data, or other resource that is valued by an organization.

A Role is a set of assignments or job functions which may be assigned to a Person or other system entity.

## Example Enterprise Object TYPES

The following are examples of typical types of Enterprise Objects, each representing a class of organization.  These are listed in table Error! Reference source not found..

These example Objects, in one form or another, with whatever names are used for them, are involved in many space systems and some of their containment relationships. How any specific Space Enterprise is decomposed into component Enterprise Objects, and how they are named, depends on the organization.

Table 4-1	:  Example Enterprise Objects

| Enterprise Objects | Description |
| --- | --- |
| Mission | An Enterprise Object that is responsible for designing, building, and / or operating one or more spacecraft |
| Project | An Enterprise Object that is responsible for designing, building, and / or operating one or more space system components |
| Program | An Enterprise Object that is responsible for one or more Missions or Projects |
| Standards Organization | An Enterprise Object that defines relevant information system, communication protocol, data exchange, or other standards or specifications |

## Examples of Space systems described with Enterprise ViewPOINT

Systems in a RASDS++ model will typically be represented as a set of elements, e.g., people, engineered objects (hardware and software), facilities, equipment, material, and processes (automated as well as manual procedures) that are related and whose behavior satisfies customer/operational needs.  As such, a System is an abstract object that may be described in RASDS++ by a set of Enterprise, Functional, or Connectivity Views.  Only the organizations, facilities, and other resources that are part of a system are directly addressed in an Enterprise View.

Figure 4-4	:  Simple Example of a single mission Enterprise View

A simple example of an Enterprise View for a single mission is shown in figure 4-4, in which three Enterprise Objects (Mission A Spacecraft and Spacecraft Control Team, and a related Instrument Control Team) are shown as dotted boxes in relationship to an Agency B Ground Tracking Network (GTN).  For this purpose the GTN is being treated as inside the Agency A domain, when in reality it may belong to an entirely different Agency.  This diagram shows a very simplified view of a mission from the Enterprise Viewpoint, but such more complicated relationships may also be modeled as needed.

A simplified abstract view of the Enterprise Objects involved in the operation of the Mars Exploration Program Federation are shown in figure 4-5 together with the interfaces between them. MRO and MSL are missions of NASA, and MEx and ExoMars are missions of ESA.  A separate S/C contractor is involved in operating the MRO project and the Deep Space Network (DSN), which is an entirely separate organization from any of the missions, that provides deep space communications services.   Various formal agreements are in place to define the required bounds of cross-support.

Figure 4-5	:  Example of an Enterprise View (Mars Exploration Federation)

Organizational elements are shown as dashed 3-D boxes and the boundaries of the domains are shown as dashed rounded boxes.

Enterprise Objects involved in the operations of Mission Z are shown in figure 4-6 together with some of the interfaces between them. Mission Z is a joint mission between Agencies ABC and QRS, and therefore some Enterprise Objects belong to Agency ABC and some to Agency QRS.  Agency WXY provides the Launch Vehicle and support infrastructure.

Figure 4-6	:  Example of Multi-agency Enterprise Launch View (Mission Z)

Two primary kinds of elements are shown in this Enterprise Viewpoint, organizations, and their resources (teams or facilities).  Many of these multi-agency collaborating missions are based upon quid-pro-quo arrangements, involving some sort of agreement or contract, and they require a relationship of trust and interdependence between organizations.

## Enterprise Architecure and Systems Architecture

Enterprise Architecture is a somewhat separate discipline from Systems Architecture and there are various methods and tools available to support development of Enterprise Architectures.  One of the most prevalent methods at this point is TOGAF [12], and the Unified Architecture Framework (UAF) [31] is also now in use.  Particularly for planning and managing the growth of large enterprises these methods and tools can provide substantial benefits.  Figure 4-7, re-drawn from TOGAF in the RASDS Information Model style, represents the kinds of topics that are addressed by the TOGAF methodology.

Figure 4-7	:  Enterprise Architecture Ontology (adapted from TOGAF)

An Enterprise Architecture may describe capabilities and processes, and identify services, without specifying the technical architecture of the systems and the applications that provide those services.  RASDS++ provides the means to define the technical architecture as well, as shown in Fig 4-8.

Figure 4-8	:  Enterprise & Technical Architecture Ontology relationships

There are defined relationships between the Enterprise architecture and the Technical architecture, as described in the other viewpoints.

## Security Topics in the Enterprise ViewPOINT

Every viewpoint has its own set of specific security topics. In the Enterprise Viewpoint the security topics that may be addressed include governance, organizational roles, policies, rules, identities, trust relationships, domain boundaries (e.g. organizational or operational vs. science) and cross-support security agreements.  The implementation mechanisms to enforce these rules and agreements are detailed in other views.  Security responsibilities, analysis of threats, counter-measures, and issues are addressed in detail in a separate Security Architecture document [4] and in other related security documents within CCSDS, NIST, and ISO.

Examples of Specific Enterprise Security Topics:

- Risk Management

  - Policies

  - Plan (Authorization & Accreditation)

- Governance

  - Authorization to Operate (ATO)

- Requirements

- Rules

- Interface agreements

- Budgets

- Personnel

- Roles

- Resources

- Assets

# Functional viewPOINT

The Functional Viewpoint separates the analysis of abstract functional elements their logical interactions and abstracted data exchanges from the engineering concerns of how functions are implemented, where they are allocated, how they transfer information, which protocols are used, and what language is used to implement them.  Keeping the analysis of the functional behavior required of a system separate from the details of how (and where) to implement it provides a degree of freedom to separate functional design from the technical details that must be explored in doing implementation design trades.

## Concerns

The concerns addressed by the Functional Viewpoint are:

- the functional decomposition of the system into objects that interact at interfaces;

- the identification of the data objects that are exchanged;

- the abstract behavior of the system, its interactions, and constraints.

## Concepts for Functional Viewpoint

The Functional Viewpoint of a space system focuses on the behavior, structure and interaction of the functions performed by that system.  This Viewpoint addresses functional objects, their behavior, the logical connections between them, the information they exchange, and their logical interfaces and interactions.

The behavior of a Function is the set of actions performed by this element to achieve an objective. A Functional Object performs actions to achieve an objective of a space system or to support actions of another Functional Object, and this may involve data transformation, generation, or processing in performing those actions.

Functional Views define Functional Objects to control and manage system behavior, such as planning, scheduling, monitoring, and other active control elements that are part of describing the functional behavior of the system. They also describe processing functions and the logical flows of information among these Objects.

To describe the full behavior of a complex system, separate depictions of data flows, control flows, and management flows may be shown for a given set of Functional Objects. These flows may use the same or different interfaces on the same Functional Object.  Several separate views of the same Functional Objects, all of which obey the same rules, may be required to show all of the different aspects of the objects and interactions that compose the Functional Views of a system.

The information objects that appear in the Functional Viewpoint are references to the information objects that are fully described in the Information Viewpoint.  The details of how these information objects are defined, described, and controlled are covered in the Information Viewpoint (Section10).

A Functional view shows behavior and other attributes and the logical flow of information among objects.  In the engineering of any given system, implemented instances of these Functional Objects may be allocated to one or more Nodes as represented in the Connectivity or Structural Viewpoints. The physical means for providing communications connections among implemented functions are treated in the Connectivity Viewpoint (Section 6), as are the physical attributes of the connections and their behavior. The physical means for providing connection or articulation functions will be treated in a Structural Viewpoint.  These allocation processes are part of usual Systems Engineering practices, as described in ISO 15288 and elsewhere.

## characteristics of Functional objects

### General

The overview of Functional Objects are shown in figure 5-1.

Figure 5-1	:  Overview of Functional Object

Each Functional Object has three categories of interfaces: Service Interfaces, External Interfaces, and Management Interfaces. Every Functional Object has one or more interfaces through which the actions of the object are invoked.   These interfaces may be shown explicitly or just implied as the locus of the connection between one Functional Object and another.

### Key Objects and Relationships

The following elements may appear in the Functional Viewpoint:

- Functional Objects (abstract set of functions, their behaviors, interfaces and configurations);

- Types: data source, data sink, data transformation, control, planning, monitoring, analysis;

- Attributes: role, name, type, behavior, interface signature, data types handled, interaction modes, constraints, allocated requirements;

- Logical Links (connections between Functional Objects, connected to associated logical behavior and properties);

- Relationships (configuration, precedence, control and data flows, management flows, allocations);

- Information (representations of data that are exchanged among Functional Objects, where formal specifications for exchange are found in the Information Viewpoint).

#### Conceptual Object Model

Fig 5-3 shows the Ontology of Functional Objects in relationship with other RASDS++ objects.

Figure 5-2	:  Ontology of Functional Objects

Fig 5-3 shows the standard representation defined for Functional Objects.

Figure 5-3	:  Representation of Functional Objects

## Terms for Functional Viewpoint

### Attributes for Functional Viewpoint

The Identifier for a functional object is a unique name assigned within some context that allows the object to be uniquely identified.

The Type assigned to a function is the subset of all possible types that have meaning in a given context.  The user is free to create classes of types that are meaningful for their needs.

Behavior is the extrinsic aspects of what a function does, such as perform some algorithm or transform data in some way.

The Data attribute of a function is the means to define what classes of data it will process.

### Unique terms for Functional Viewpoint

Abstraction is a mechanism and practice to reduce and factor out details so that one can focus on few concepts at a time. In this context it is the separation of the description of system functionality from the details of system implementation.

Information Management Functional Objects are active functional elements that support the location, access, delivery, and management of Information Objects.

## Example Functional Object TYPES

Table 5-1 shows a set of typical high level functional objects used in space systems, which may occur on orbit, or on the surface of a planet, or both.   These are provided only as examples, and any given system may add other functions, decompose these differently, or use other names for the same functions.  For instance, Orbit Determination and Trajectory Design may be named separately in one system or aggregated and called Flight Dynamics in another.  These examples intentionally show only very high-level functions, which will typically be further decomposed during the design process.

Depending on the system, Functional Objects may be decomposed into subfunctions, each of which is performed by a component Functional Object of the parent Functional Object. How Functional Objects are decomposed into component Functional Objects depends heavily on the system design and local practice, and it is beyond the scope of this reference architecture to define specific decompositions of these Functional Objects.

Table 5-1	:  Example Functional Objects

| Functional Objects | Description |
| --- | --- |
| Experiment control | A function to control an experiment or observation (data acquisition, sample acquisition, etc.). |
| Data transport | A function to manage and control the execution of data transport functions supplied by Communications Objects. |
| Directive execution | A function to execute a set of directives (goals or a time-ordered set of directions). |
| Directive management | A function to manage remotely a set of directives (goals or a time-ordered set of directions). |
| Directive generation | A function to generate a set of directives (goals or a time-ordered set of directions) based on a mission plan. |
| Monitor and Control | A function to monitor the status of other functional objects and to request execution of necessary actions when a predefined anomaly or deviation occurs. |
| Mission planning | A function to generate a mission plan (time-ordered set of goals or sequence of activities). |
| Spacecraft analysis | A function to analyze the status of a spacecraft using data from a data store. |
| Mission analysis | A function to analyze the status of instruments and to assess the level of achievement of mission goals, using data from a data store. |
| Tracking | A function to steer an antenna to maintain communications links with a spacecraft or a ground station. |
| Radiometric data collection | A function to collect radiometric data (e.g., range and Doppler). |
| Orbit determination | A function to estimate the state vector of a spacecraft using radiometric data and possibly image or other position-sensitive data taken by the spacecraft. |
| Trajectory design | A function to design the trajectory of a spacecraft including plans for orbit change maneuvers. |
| Space Transport | A function to change a spacecraft orbit or location. |

Table 5-2 shows several typical infrastructure functional objects.  These are also Functional Objects, but they are distinguished because they typically provide supporting services for the more application-oriented Functional Objects shown in table 5-1.  Sometimes these infrastructure objects may be shown supported by a separate application stack layer, as described in the CCSDS Application and Support Layer Architecture document (ASL) [26].

Table 5-2	:  Typical Infrastructure Objects

| Functional Objects | Description |
| --- | --- |
| Information Management | A set of functions to store, locate, access, and deliver data; see the Information Viewpoint for more details on these elements. |
| System Management | A set of functions to monitor, manage, configure, and control other functions in a system, usually via their Management interfaces. |
| Messaging Middleware | A set of functions to provide services for naming, locating, accessing, and interfacing with elements of a distributed system. May also be a Communications Viewpoint set of objects. |

## Examples of a Space system described from the Functional ViewPOINT

A simple example of a Functional View is shown in figure 5-4, where a set of related Functional Objects are represented as ovals, and the functional interactions are represented by dashed lines.  These interactions take place at the interfaces of these objects.  In the top example only the flows are shown.  In the bottom example simple representations of Information Objects, described more fully in Sec 10, are shown as being exchanged across these logical links, and the provided interfaces of these Functional Objects are also identified.  Either representation may be used.

Figure 5-4	:  Simple Example of a Functional View

A Functional Object may be composed of other Functional Objects.  A formal or informal group of Functional Objects that provide some service in a space system, such as a related set of navigation or data processing services, may be modeled as a higher-level element in a Functional View. The decomposition of these elements may also be shown in related Views.  The Information Objects that are exchanged across the interfaces between Functional Objects are abstractions, the full specification for these Information Objects will be found, by correspondence, in a related Information View.

Figure 5-5 shows a representative set of Functional Objects used at an application layer in typical space systems together with the logical interactions that occur among them (shown with dotted lines).  The points of connection are at the interfaces, shown as bubbles on the edges of the functions in this view, denoting the provided interfaces. On these logical links flow various forms of information, which are shown in this particular view as labelled rectangles.  These should be referenced to Information Objects defined in an Information View.  See Sec 10 for more discussion of the Information Viewpoint where the means to describe these data objects are defined.

Figure 5-5	:  Example of Functional View (Functional Objects, provided interfaces, & data)

Other views of this same set of Functional Objects might be shown, including views showing the decomposition of each of these high-level functions into lower-level ones, or a view showing the control flows between directive generation, execution, and data acquisition.  All these views may be developed using the same Functional Viewpoint specification.

RASDS does not provide any specific recommendations for the representation of the internal behavior of Functional Objects.  Decomposition to lower levels of detail is one way of showing this.  Other formal means, such as use of state charts, activity diagrams or algorithmic specifications may also be employed as needed in a Functional View. For describing interfaces any suitable annotation may be adopted to represent and label information objects or to label interfaces, or the features of the Service Viewpoint may be employed if more formal Service Interfaces must be defined.

## Example of Space system with Information Management Infrastructure

A specific set of Information Management Functional Objects may be included in the Functional Viewpoint. These are the elements of an information infrastructure that support the location, access, delivery, and management of Information Objects.   These Information Management Functional Objects are Functional Objects, but they are often considered together with Information Objects because of their close relationship to them.

The upper part of Figure 5-6 shows a representative set of Functional Objects that might be used to carry out some information management activity.  The lower part of Figure 5-6 shows the set of Information Management Functional Objects (query service, registry service, repository service, product service) that provide an infrastructure for managing, accessing, locating, processing, and distributing the information exchanged by other Functional Objects.

Figure 5-6	:  Representative Functional Objects and Information Management Infrastructure Elements

In some systems, these infrastructure elements may be instantiated by simple files, tables, or even data stored in memory.  In other systems, these will be system-level functional elements, implemented as subsystems and using various commercial elements like data base management systems (DBMS) or distributed system frameworks.

These basic Information Management Functional Objects may be composed into a broad set of information management services to support mission operations functions as well as on-board data management. They may also be combined with other functions that do transaction management or data ingest to produce federated data systems and back-end archival systems.  A more complete description of these Information Management Functional Objects, their functions and interfaces is separately addressed in the Reference Architecture for Space Information Management [5].

These information management services may be remote from a system and customized for specific purposes or classes of data.  They may manage different kinds of data structures, such tables, lists, or knowledge graphs, and they may use standardized query languages like SQL [34] or specialized languages like SPARQL [35].  CCSDS itself manages a set of registries in the Space Assigned Numbers Authority (SANA) [36] that are accessible on the Web and contain specialized data relating to its standards and the uses that are made of them.  These are documented in the Registry Management Policy (RMP) [37] and companion documents.  Many CCSDS standards refer to these or create other new registries as needed.

## Security Topics in the Functional ViewPOINT

In the Functional Viewpoint, the functional objects that are used to implement security policies and approaches are defined.  These may include: access control interfaces on functions and specific functional elements such as authentication, source level encryption, and key management subsystems.  Some of these may be shown as Functional Objects in their own right (e.g., Public Key Infrastructure (PKI) management function), or just as attributes of other functional objects (e.g., access control on a management or control function).

Examples of Specific Functional Security Elements:

- Threat assessment of all functions

- Verification (Authentication)

- Confidentiality (Encryption)

- Risk assessment

- Threat assessment

- Access Monitoring / Intrusion Detection

- Identity management / ICAM (Identity Credential and Access Management)

- Key Management

- Application management

- Network management

- Perimeter management

- Vulnerability scanning and management

# Physical Viewpoint

## Overview

The Physical Viewpoint provides the core description of the physical elements that operate in space where connections between elements, physics of the elements, and interactions with forces in the external environment are considered.  More specific Viewpoints, using more specific terms in their domain of discourse, can be derived from this Physical Viewpoint.

The “derivation” of other viewpoints enables the derived viewpoints to implicitly carry the following aspects.

- A graphical model of composition in which the connectors and physical interfaces that connect physical objects appear in the physical viewpoint and in derived viewpoints.

- The graphical model of composition makes it possible to explain the deployment of separate instances of the same class of objects throughout space.

- The authors of derived viewpoints can use the connections to explain relationships and dynamics among the components.

- Some concerns of security that involve physical objects may be inherited by derived viewpoints.

- For architectures that do not need to describe different physical aspects, the core physical viewpoint can provide a place for physical specifications without using new viewpoints.  For architectures that describe many physical aspects, the derived viewpoints provide means to organize those specifications.

Two examples of derived viewpoints, the Connectivity Viewpoint and the Structural Viewpoint, appear in Sections 7 and 8.

For analysis of space systems in general, all the physical aspects of the system, including the propulsion, power, thermal, structural, etc., aspects associated with them, must be considered, and represented in this Physical Viewpoint, or in derived viewpoints.

The Physical Viewpoint and derived viewpoints include all aspects of space system design dealing with the composition of physical elements, their physical connections, and the allocation of functionality to these elements.  The physical elements include processors, instruments, storage devices, radios, bus structures, and other components as well as hardwired links, buses, and RF and optical links.  The Physical Viewpoint and derived viewpoints are where these engineering issues are handled, along with the issues associated with choosing the best strategy of how to implement the selected logical functionality in hardware and software components.

The derivation of physical viewpoints provides a means to balance the distribution of architectural information across an architectural description.  An architecture that leaves most physical description to be resolved by engineers who implement the architecture can package most of its physical description in a Physical Viewpoint.  Sometimes an architectural description must specify details in one or more viewpoints derived from the Physical Viewpoint.  The Connectivity Viewpoint is an example of this need in architectural descriptions of the communications aspects of space systems.

### Concerns

The concerns addressed in the Physical Viewpoint are:

- the physical decomposition of the system into objects that interact at interfaces;

- the concrete behavior of the system, its interactions and constraints;

- The interactions of the system with the physical world and energetic forces.

## Concepts for the Physical Viewpoint

The Physical Viewpoint of a space system focuses on the physical objects that compose the system.  This Viewpoint addresses physical objects, their behavior, the physical connections between them, the physical quantities they exchange, their physical interfaces and interactions within the system and with the outside environment and forces.

The behavior of a physical element is the set of roles or actions performed by the element to achieve an objective. A Physical Object performs actions to achieve an objective of a space system or it plays a role to connect other Physical Objects, and this may involve trajectories, locations, orientations, temperatures, or provision or consumption of resources.

Physical Views define Physical Objects to realize system behavior, such as travelling, stabilizing, articulating, conducting energy, and other roles and actions that are part of describing the physical behavior of the system. They also allocate instances of Functions among these Objects.

For describing the full behavior of a complex system, separate depictions of data flows, energy flows and articulations may be shown for a given set of Physical Objects. These different  kinds of flows may use the same or different interfaces on the same Physical Object.  Several separate views of the same Physical Objects, all of which obey the same rules, may be required to show data connectivity, structure and articulations, propulsion, attitude control, navigation, thermal control, radiation, and other different aspects of the objects and interactions that compose the Physical Views of a system.

A Functional  view shows behavior and other attributes and the logical flow of information among objects.  In the engineering of any given system, implemented instances of these Functional Objects may be allocated to one or more Physical Objects as represented in the Connectivity Viewpoint. The physical means for providing communications connections among functions distributed in space are treated in the Connectivity Viewpoint (section 7), as are the physical attributes of the connections and their behavior.

## Characteristics of Physical Objects

The Physical Objects may have any of all the interfaces shown in Figure 6-1.

Figure 6-1 Physical Object Overview

The Physical Objects may offer four categories of interfaces: Service Interfaces, External Interfaces, Environmental Interfaces, and Management Interfaces.  Every Physical Object has one or more interfaces through which it interacts with the physical environment, according to laws of motion and energetic exchanges.  External interfaces of Physical Objects provide the means to invoke function instances of other Physical Objects.  Service interfaces provide the means to invoke and manage functions implemented by the object.

Good practice identifies generalized sets of physical objects as an aid to re-use. Specialized sets are developed only as needed.  Current definitions of design patterns are examples of a similar approach now used in software development.

### Key Objects and Relationships

The following elements may appear in the Physical Viewpoint:

- Physical Objects that implement Functions (abstract set of functions, their behaviors, interfaces and configurations);

- Types: data source, data sink, data transformation, control, planning, monitoring, analysis;

- Attributes: role, name, type, behavior, interface signature, data types handled, interaction modes, constraints, allocated requirements;

- Physical Objects that provide energetic exchanges;

- Nature of energetic exchange, such as heat, electrical power, electromagnetic radiation, structural continuity, momentum, angular momentum;

- Logical Links (connections between Physical Objects, connected to associated logical behavior and properties);

- Relationships (configuration, precedence, control and data flows, management flows, allocations);

- Information (representations of data that are exchanged among Connectivity Objects, where formal specifications for exchange are found in the Information Viewpoint).

#### Ontology of Physical Objects

Fig 6-2 shows the Ontology of Physical Objects in relationship with other RASDS++ objects.

It also explicitly covers the relationship between Components and the Environment.

Figure 6-2: Ontology for Physical Viewpoint

#### Representation of Physical Objects

In RASDS++ diagrams Physical Objects are represented using the drawing style shown in Fig 6-3.  The focus here is on physical characterization, connections, and flows, which may involve energy, fluids, or data.

Figure 6-3:  Physical Object Representation

NOTE: It is recognized and acknowledged that these simple drawings of physical components and connectors do not capture all the details nor nuance that typical mechanical or electrical engineering CAD/CAM drawing tools are capable of, nor is it expected that this would replace such engineering tools.  What these views do permit, however, is to allow the functional, logical, physical, structural, and communications aspects of a system to be related in a coherent way.

## Terms for Physical Viewpoint

### Attributes for Physical Viewpoint

Mass Properties: Center of mass, inertia matrix

Connections: The types of connections and the sub-components that they connect.  Thermal conductivity, wiring harness, etc.

Physical Environment: Limits on environmental properties, such as pressure, ambient temperature, etc.

Configuration : DIP switch settings, jumpers, etc.

Constraints: Operational limits, such as mechanical stress, flexibility, angular momentum storage, electrical power storage, etc.

Hardware is the mechanical, magnetic, electrical, and electronic devices or components of a system used for producing, collecting, processing, storing, or transporting data.

Software or computer programs are the components of information systems that provide operating instructions for specific task based applications that run on computing hardware

Firmware is software that is contained in a read-only memory (ROM) device.  It is typically treated as software unless there is a reason for showing the hardware component itself.

An Engineering Object is an implementation or realization of some abstract function.  It may be implemented as hardware (Node) or as software (application or software component).

### Other Terms for Physical Viewpoint

A Component is a physical entity operating in a physical environment.  A Component is a configuration of engineering objects forming a single unit for the purpose of location in space, and embodying a set of functions.  A Component has some well-understood, possibly rapidly moving, location, and it may be composed of two or more (sub)components.

Each Component has one or more ports where connections to other Components are made. Any given Port on a Component may expose one or more provided or required Service Interfaces.

In some contexts, especially the derived Connectivity viewpoint, a Component may be called a Node.

A Connector is a thing which links two or more things together.  A Connector may be rigid, flexible, hinged, rotational, articulated or simply energetic. Connectors connect Components at a port.

In some contexts, especially the derived Connectivity viewpoint, a Connector may be called a communications Link.

Structural Objects are concrete elements that support the location  and orientation of instruments in a spacecraft in a Structural Viewpoint (See Section 8).   Structural objects usually have no data interfaces, but often have thermal interfaces that appear in a Thermal Viewpoint.

Spacecraft  includes spacecraft that travel in space, rovers, habitats, and other element in space or on a remote planetary surface.

## Typical Objects in the Physical Viewpoint

Table 6-1 shows typical physical objects used in space systems.   These are provided only as examples, and any given system may decompose these differently or use other names for the same objects.  These examples intentionally show only very high-level objects, which will typically be further decomposed during the design process.

Depending on the system, Physical Objects may be decomposed into component objects. How Physical Objects are decomposed into component objects depends heavily on the system design and local practice, and it is beyond the scope of this reference architecture to define specific decompositions of these Physical Objects.

Table 6-1	:  Example Physical Objects

| Physical Objects | Description |
| --- | --- |
| Spacecraft | A moveable device that may communicate with other spacecraft or with ground stations during a mission. spacecraft can appear in the Connectivity Viewpoint as Nodes. The Structural Viewpoint may describe the internal structure of a spacecraft. |
| Thruster | An actuator that provides a reaction force to a spacecraft in flight by ejecting mass. If the force passes through the center of mass of the spacecraft, then the thruster moves the spacecraft along a trajectory; otherwise, the thruster changes the attitude of the spacecraft. |
| Star Tracker | A sensor that recognizes arrangements of stars and reports its orientation in celestial coordinates. |
| Inertial Measurement Unit | A device that measures and reports changes in orientation and location during flight. |
| Torque Bar | An actuator that interacts with ambient magnetic fields to apply torque to the attitude of a spacecraft in flight. |
| Computer | A device that provides software implementations (often called instances) of Functional Objects. |
| Bulkhead | A part of the structure of a spacecraft that acts as a wall. |
| Antenna | A device that sends and receives electromagnetic signals. In the Connectivity View, antennas form the interfaces for communication Links between spacecraft and between spacecraft and ground stations |
| Sub Network | A device for electrical communication between components onboard a spacecraft in the Connectivity View. |

## Examples of a Space System described with PHYSICAL views

Figure 6-4 shows a simple set of Physical Objects used in a space system together with the interactions that occur among them.

Figure 6-4 represents a connection for on-orbit servicing using a structural conduit and showing fluid flow.  Other derived views of this same set of Physical Objects might be

shown, such as a Connectivity View showing pressure telemetry sent from the fluid pump to signal when to stop pumping.

Figure 6-4: Simple Example of Physical View Connector

A simple example of a Physical View is shown in Fig 6-5, where a few 3-D boxes represent Components attached to a flat panel (which is also a Component).  Fig 6-5 omits details that will be resolved by engineers who implement the architecture, such as coordinate systems for the science instrument and for the spacecraft.  The Connections between the boxes and panels might be welded or bolted mating surfaces, but these details are not shown in this view.

Figure 6-5 	Physical View Example Composition

This 3-D representation of Components was chosen is because this viewpoint deals with physical objects.  Components are physical Engineering Objects, the other primary elements that may appear in the Physical Viewpoint are explicit Connectors.  If more architectural details are wanted, it may be necessary to explicitly include both a structural viewpoint and communication viewpoint in the architecture description.

## Security Topics in the Physical Viewpoint

In the Physical Viewpoint, the physical objects that are used to implement security functions, policies, and approaches are defined.  Threats relating specifically to elements of the physical viewpoint should be considered [18]. These may include: access control interfaces on data systems such as authentication, source level encryption, and key management subsystems.  Some of these may be shown as Physical Objects in their own right (e.g., Public Key Infrastructure (PKI) management system), or just as attributes of other Physical objects (e.g., access control on a facility).  Passive security features may include warnings to integrators of devices that require special security, for example, using red and black wires for secure subnetworks and common subnetworks, respectively.

Examples of Specific Physical Security Elements

- Firewalls

- Routers

- Security modules

- Walls, doors, locks

- Badge readers

- VPN servers

- EMI Shielded spaces

- Identity devices

- Network / security devices (IDS, Key Management)

- spoofing signals etc

# Connectivity Viewpoint – Derived

The Connectivity Viewpoint is derived from the Physical Viewpoint, and it is used to represent the communication aspects of physical elements that operate either in space, where connections between elements, the physics of motion, and interactions with forces in the external environment are considered, or on the surface of the Earth or some other physical body.  The Connectivity Viewpoint deals with the composition of these physical elements and specifically their communication connections and interactions to provide for end-to-end communications paths and network connections.

For the description of space data systems, the Connectivity Viewpoint is where consideration is given to nodes (components), links (connectors), computational and data transport functions, external forces that affect communications (motion, interference), and other considerations related to the engineering of data system communications functionality and performance.

Frequently there are elements of space data systems that are in motion through space and consequently connectivity issues associated with pointing, scheduling, long round-trip light times, intermittent visibility, and low signal-to-noise ratios all must be considered. All these challenges must be dealt with using special protocols, functionality, and controls.  The Connectivity Viewpoint is used to address all these aspects of space data systems.

## Concerns

Concerns for the Connectivity Viewpoint are:

- the mechanisms and functions required to support distributed communications between objects in the system;

- the selected allocation of functions to the nodes of the system, including their implementation choices and constraints on implementation, connections, configuration, and operations imposed by the communication links and the environment;

- the behavior and performance of elements in the system, including their capabilities, physical motion, and their interactions with the physical environment

## Concepts for the Connectivity Viewpoint

The Connectivity Viewpoint is an engineering view on a space data system that shows Engineering Objects, which may be hardware or software. The Components and Connections of the Physical Viewpoint correspond directly to the terms Nodes and Links in the Connectivity Viewpoint.

The Connectivity Viewpoint is focused on a Node and Link view of a system, the composition of the Nodes, the physical connections among Nodes used for communications, their physical and environmental constraints, and their physical dynamics as it affects communications.

The Connectivity Viewpoint also describes how the abstract functional design described in the Functional Viewpoint may be implemented as software Engineering Objects, i.e., applications or software components, or hardware Engineering Objects, and how these are allocated to the major hardware Engineering Objects (Nodes) of the system. These Engineering Objects are the “to be implemented” versions of the Functional Objects which were described in detail in section 5.

In the Connectivity View, a space data system is depicted with Nodes and the physical communications connections among them (Links). This view is also used to describe how these Nodes move through space and the effects that the environment has upon their behaviors.

This view also includes description of certain aspects of physical behavior of the system, such as spacecraft trajectory, communication view periods, orbits, or the motion of the physical body on which the element is located.  The physical behavior is important for understanding the communications challenges from the physical environment in which the systems operate, particularly the motion, discontinuous or disrupted connectivity, and extremely distant and broad distribution of physical devices.  Specialized protocols and systems design are needed to provide reliable and secure communications to deal with many aspects of the space physical environment.

## Characteristics of CONNECTIVITY objects

The primary objects shown in the Connectivity Viewpoint are physical Nodes and the Links that connect them. The abstract Node and its interfaces are shown in figure 7-1.

Each of these interfaces is associated with one or more Links attached to the Node.  Physical Links attach to physical Ports on Nodes, and these links provide the means for nodes to communicate.  Each node typically implements one or more functions (defined in a functional view), either as software or hardware Engineering Objects. The allocation of the set of Functional Objects to Nodes and determination of their implementation choices is a primary activity associated with the development of the Connectivity Views for a system.

The services associated with a Node are determined by the Functional elements that the Node implements.  So the functional behavior of a Node is determined by the Functional elements implemented in the Node and the mechanisms that enable interactions with Functional elements in other Nodes. The functional elements allocated to a node have associated logical interfaces, and these become associated with the physical Ports on the Node.  One physical Port may support more than one logical interface, just as an Ethernet port may support multiple services like file transfer, web browsing, or database services.

Figure 7-1	:  Overview of Connectivity Object (Node)

From a computational point of view the physical behavior of a Node is determined by its performance characteristics, its processing speed, internal bandwidth, data paths, memory sizes, or other performance related attributes.  The performance of the engineered system, either in a local or an end-to-end sense, may be specified once the performance capabilities of the nodes and links have been specified, the performance requirements of the allocated functions have been determined, and the effects of the environment have been characterized.

When viewed at the coarsest level of granularity some Nodes of a system, such as a spacecraft, will exhibit physical behavior that is determined by the physical forces acting upon the Node.  These forces may be propulsive or gravitational, or they may be caused by other elements in the environment that determine the velocity, direction of motion, acceleration, or mobility of the spacecraft.  The physical location and behavior of the spacecraft (orbit, trajectory, path), the performance of some of its components (e.g., antenna aperture, transmitter power, receiver sensitivity), and the physical characteristics of the environment, all exert a strong influence on the performance of the communications systems and the behavior of the Links.

The protocols that implement the communication stacks are described in the Communications Viewpoint and they are selected to deal with these behavioral and environmental factors.

### Key Objects and Relationships

The following elements may appear in the Connectivity Viewpoint:

- Engineering Objects (Nodes, Links, Applications):

- Nodes (hardware Engineering Objects, provide processing and other computing and data resources, ports, performance, and other associated physical behavior):

- Types: hardware objects, composite hardware objects, ports;

- Attributes: name, type, location (place, trajectory, orbit), laws of motion, available resources, physical interfaces, capabilities (e.g., processor speed, throughput, bandwidth, storage capacity, aperture size, etc.), allocated functions, constraints;

- Links (connections between Nodes, associated physical behavior and properties);

- Types: space link (RF or optical), physical link (e.g., point-to-point, bus, network, copper, fiber, etc.);

- Attributes: name, type, end-points (port on node), physical interfaces, performance (e.g., throughput, bandwidth, frequency band, round trip light time (RTLT), pointing and view periods, signal attenuation, constraints, environmental effects, etc.), access and ownership;

- Applications (software Engineering Objects, behavior, and processing/resource requirements, may be layered):

- Types: software engineering objects, composite software engineering objects;

- Attributes: name, type, algorithms, implemented functions, allocation / deployment to nodes, required resources (e.g., memory, CPU, storage), implemented interfaces (provided and required), implementation language, operating system (OS) / framework dependencies, constraints;

- Relationships (composition, interfaces, constraints, configurations, allocation);

- Environment (physical environs, physical forces [gravity and others], physical interactions and effects);

- Information (defined representations of data that are exchanged among Engineering Objects, where formal specifications of data architecture are found in the Information Viewpoint).

NOTE	–	The Connectivity Viewpoint is focused upon space data systems and communications aspects.   All other physical aspects are addressed elsewhere in the Physical Viewpoint or other derived viewpoints in this document.  See Toward a Framework for Modeling Space Systems Architectures [20] for more information on one such extended approach.  The Connectivity Viewpoint in this document corresponds to the Navigation, Telecomm, and Data System Views in reference [20].

### Ontology of Connectivity Objects

Figure 7-2 shows the ontology of Connectivity objects.  These are derived from the Physical Viewpoint, but with specialization of the terms to address connectivity concerns.

Figure 7-2: Ontology for Connectivity Viewpoint

### Representation of Connectivity Objects

In RASDS++ diagrams Connectivity Objects are represented using the drawing style shown in Fig 7-3.  This diagram contains terms “Node” and “Link”, which are used in the Connectivity Viewpoint, and the focus here is on communications connections and data flow, not data content.

Figure 7-3	:  Representation of Connectivity Objects

Nodes will explicitly connect via some Link.  The mass, power, thermal, and other physical properties of Nodes are inherited from the Physical Viewpoint.

The Link represents some communications constraint. This may be dynamic or static, and it may be concretely physical (cable, copper, fiber) or energetic (RF, optical).

May show allocation of implemented Functions in hardware (a sub-component) tied by correspondence to the Functional View where they are defined.

May explicitly identify the data objects (defined in Information Views) exchanged by the functions using the link (defined in these views).

May explicitly identify the physical interface or connection type.

May explicitly describe the relationships between Nodes and the Environment.

## Terms for the Connectivity Viewpoint

### Attributes for the Connectivity Viewpoint

A Unique Identifier is a value used in specified fields of CCSDS-defined (or other) link layer data structures.  It provides a unique identifier for the Node.

A network address is an identifier for a node or host on a telecommunications network. Network addresses are designed to be unique identifiers across the network, although some networks allow for local, private addresses, or locally administered addresses that may not be unique. Special network addresses are allocated as broadcast or multicast addresses. These too are not unique.

System performance is the amount of useful work accomplished by a system.  Outside of specific contexts, performance is estimated in terms of accuracy, efficiency, and speed of executing computer program instructions or transferring data.

Space environment creates conditions in space that affect the design and operation of spacecraft. Effects on spacecraft can arise from temperature extremes, radiation, space debris and meteoroid impact, upper atmospheric drag, spacecraft electrostatic charging, and gravity.

The configuration of a system refers to the arrangement of each of its functional units, according to their nature, number, and chief characteristics. Often, configuration pertains to the choice of hardware, software, firmware, and documentation as well as the selection of control parameters.

A Constraint is a limitation or implied requirement that limits the design solution or implementation, is not changeable by the enterprise, and is generally non-allocable.

### Other Terms for the Connectivity Viewpoint

A Node is a physical hardware Engineering Object that is a run-time computational resource and generally has at least memory and often processing capability. Run-time software engineering objects reside on nodes.  A Node has some well-understood, possibly rapidly moving, location. A Node may be composed of two or more (sub)Nodes.

All nodes are hardware Engineering Objects, but not all hardware engineering objects are nodes.  We refer to the larger discrete items of hardware in a space system as nodes.  From the perspective of systems architecture descriptions, below some level of granularity it may not be useful to describe minor hardware elements as nodes, but rather components.

A Link is the locus of relations among Nodes.  It provides interconnections between Nodes for communication and coordination. It may be implemented by a wired connection or with some RF or optical communications media.  Links implement the primary function of transporting data.  Links connect to Nodes at a Port.

Links are Engineering Objects, but only some of them are hardware.  Some links, such as an Ethernet cable or a CPU backplane are implemented as hardware.   Some links, such as an RF or optical link, are a physical effect produced by hardware Engineering Objects. These links are not physical devices, but are physical manifestations that can be sensed, measured, and analyzed for their information content..

A port is the physical element of a Node where a Link is connected.  Nodes may have one or more Ports.  Each Port may connect to one or more physical Ports on (sub)Nodes that are contained within the Node.  Where needed a port may be represented as a simple small rectangle at the edge of a node.

A single physical Link between two Nodes may carry one or more logical connections between applications implemented on those Nodes.

An application consists of one or more pieces of software designed to perform some specific function; it is a configuration of interacting engineering objects.

The process of allocation is mapping between one set of model elements and another. The mapping is often performed as part of the design process to refine the design. Typical examples of allocation include allocation of functions to nodes, logical to physical components, logical to physical links, and software application instances to hardware.

Tracking Station is a node in a Connectivity Viewpoint that occupies a fixed location on a planet (including Earth) or asteroid.

## Typical Connectivity Objects

Table 7-1 shows typical Nodes that are used in space systems. Which Nodes are used in any given space system may differ from system to system, and the following list shows only typical Nodes used in many space systems.

Table 7-1	:  Typical Nodes

| Nodes | Description |
| --- | --- |
| Spacecraft | A spacecraft (or a lander, rover, balloon, etc.) used to achieve mission goals (e.g., observations or experiments). |
| Relay satellite | A spacecraft (or a lander) that relays data between spacecraft and a tracking network or between different sets of spacecraft. It may not exist as a physical object in all space systems. |
| Instrument | A component of a spacecraft used to achieve mission goals (e.g., observations or experiments). It may not exist as a physical object in all space systems. |
| Computer | Component of a spacecraft or ground system used to process data. |
| Data storage system | Subsystem used to store and manage data. It may not exist as a separate physical object in all spacecraft or ground systems. |
| Ground Tracking Network | Typically a multi-mission subsystem that may be composed of one or more Nodes with one or more tracking stations, and possibly a network control center. It is used for communicating with spacecraft and performing radiometric measurements against spacecraft. |
| Tracking Station | A subsystem of a ground tracking network with an aperture that is used to track spacecraft, transmit commands, receive telemetry, and optionally to produce radiometric and tracking data. |
| Spacecraft Control Center | A center used for controlling one or more spacecraft. |
| Spacecraft control facility | A facility that is part of a mission operations system that is used to plan, control, and monitor spacecraft operations. |
| Instrument control facility | A facility that is part of a mission operations system that is used to plan, control, and monitor instrument operations. It may not exist as a separate facility in all enterprises. |
| Orbit determination facility | A facility that is part of a mission operations system that is used to analyze radiometric tracking data and to determine the orbit and attitude of a spacecraft. It may not exist as a separate facility in all enterprises. |
| Trajectory design facility | A facility that is part of a mission operations system that is used to design a spacecraft trajectory and plan maneuvers. It may not exist as a separate facility in all enterprises. |
| Mission planning facility | A facility that is part of a mission operations system that is used to create, control, and monitor mission operational plans. This may include overall observation and mission scenario planning. It may not exist as a separate facility in all enterprises. |
| Science facility | A facility that requests activities of a spacecraft and analyzes data obtained from that spacecraft. It may not exist as an enterprise object in all space systems. |
| Data analysis facility | A facility that is part of a mission operations system that is used to process instrument data and to perform a variety of additional data analyses. It may not exist as a separate facility in all enterprises. |
| Data Archive Center | A facility that archives data obtained from spacecraft and delivers requested data to a science institute. It may not exist as an enterprise object in all space systems. |

Node functionality is implemented by creating a set of instantiated functions, in software, or possibly even in hardware or firmware such as an FPGA.  Good practice identifies generalized sets of implemented functional objects as an aid to re-use. Specialized sets are developed only as needed.  There are several different kinds of software interface design patterns that are now used in software development.  Note that even the cloud and data center approaches that are currently in vogue are still composed of nodes and links, just with different deployment and decomposition patterns.

Table 7-2 shows typical Links that are used in space systems. Which Links are used in any specific space system differs from system to system, and the following list only shows typical Links and attributes that are considered in many space systems.

Table 7-2	:  Typical Links

| Links | Description | Attributes |
| --- | --- | --- |
| Space Link | A Link between a Node in space (e.g., a spacecraft) and a Node on the ground (e.g., a ground station), or a Link between two Nodes in space (e.g., between two spacecraft). | - Continuous vs. episodic connectivity - Pointing and view periods - Frequency band (RF) or wavelength (optical) - Delay and signal attenuation - Single vs. multiple access - Bit rate, possibly variable |
| Ground Link or Network | A Link between two Nodes or a network among multiple Nodes on the ground. | - Wide area or local area - Dedicated or public - Single vs. multiple access - Bit rate |
| Onboard Link or Bus | A Link between two Nodes or a bus among multiple Nodes on the same spacecraft. | - Single vs. multiple access - Redundancy - Bit rate |

## Examples of Space systems described with Connectivity ViewS

Fig 7-4 shows the Nodes used to support a Mission.  In this figure just the major nodes are shown and named, along with the high level characteristics of the links used to connect them.  The three 3-D boxes represent Nodes and the Links between them are shown as solid lines.  This 3-D representation of Nodes is used  because this view deals with physical objects.  The other primary elements that appear in the Connectivity Viewpoint may be software and hardware Engineering Objects that implement (and correspondence to) to Functional Objects defined in a related Functional View.

Figure 7-4	:  Simple Connectivity View (Nodes for Mission A)

A more complex example of this same Connectivity View is shown in figure 7-5.  As part of the engineering process of designing the system, abstract functions are allocated to physical nodes and implementation choices are made about use of software or hardware. In the Connectivity Viewpoint, representations of Functional Objects are shown as Engineering Objects, either as physical hardware (Nodes or hardware Engineering Objects) or as software (software Engineering Objects) that are allocated to the Nodes of the system.

Figure 7-5	:  Connectivity View with Node Details

When this level of functional allocation must be described, the allocation of Software Engineering Objects (software or applications) may be shown by overlaying a representation of the implemented Functions on the Nodes shown in a Connectivity View.

Figure 7-6, below, shows one possible decomposition of the Nodes used for Mission A into example component Nodes.  For clarity the nodes could be colored like the corresponding nodes in Fig 7-5 but there is no requirement to do this nor is there anything special about these colors. Any given architecture document may adopt its own color and decomposition hierarchy and naming conventions.

In Fig 7-6 the nodes owned by two agencies are shown, along with an overlay of the functions deployed on the different Nodes.  In reality the “as implemented” functions might be allocated to different hardware nodes than those shown and no one to one mapping can be assumed. Data flows are labelled as to data types.

Of course, the nodes shown in Fig 7-6 may be further decomposed into lower-level nodes in separate Views, with their own internal links where this level of detail is required.  Many system-engineering disciplines provide a hierarchy of names to describe different levels of decomposition of Nodes within a system.  The IEEE/ISO/IEC Systems Engineering Planning document [3] defines the terms system, product, subsystem, assembly, component, and subcomponent, but other terms for system decomposition may be appropriate for different projects, and none is prescribed here.

Figure 7-6  Connectivity View with Allocated Engineering Objects

This same methodology supports definition of Systems of Systems views, but no specific recommendation is made other than to suggest starting at the highest level overview of the collection of systems and then using successive decomposition and clear specification of interfaces at each level, in as many views and expanded details as are required.  In Architecting Principles for Systems-of-Systems (SoS [21]) one of the key points of emphasis is that each system has its own purpose and that the leverage point for architecting the SoS is at the communications interfaces between these entities.  This is, of course, also true of all external and internal interfaces of any system elements.  A paper that uses RASDS methods and adopts SysML for modeling a system-of-systems architecture is NASA Integrated Network Monitor and Control Software Architecture [28].

As an example of how allocation works, figure 7-7 shows how the functionality defined by the set of example Functional Objects shown in figure 5-5 might be allocated to the high level Nodes that were just shown in figure 7-7. This Connectivity View diagram shows how the Functional elements, implemented as Engineering Objects, might be allocated to Nodes.  On a more detailed view the explicit choice of implementation options for implementing these functions in hardware or software engineering objects might be shown.

Once the combination of the performance support requirements of the implemented Engineering Objects and the performance capabilities of the Nodes and Links have been defined analysis of the end-to-end performance of the system may be determined.

Figure 7-6 could be redrawn to represent an autonomous spacecraft, with specialized copies of the Planning, Directive Generation, and Monitor and Control functions moved on-board.  Exploring the implications of these allocation options on system functionality, performance, and support requirements is possible once all the elements of the Functional View have been identified and the allocations of these within a Connectivity View have been specified.

Connectivity views have other uses during trade studies to select between hardware and software implementation options.  Consider the problem of implementing an image compression function for a high-performance telemetry system.  Two possible approaches might be to implement the compression function directly in software, perhaps on the Command and Data Handling (C&DH) processor, or to implement a hardware compressor that might be a board integrated into the flight data recorder.

Figure 7-7 	:  Functional View of Image Compression

Both the hardware and software options would implement the identical functional flow as shown in figure 7-7, but the connectivity views look significantly different, as shown in figure 7-8(a) and 7-8(b) and the performance characteristics of these two options would also be significantly different.

Figure 7-8(a) 	:  Connectivity View of Software Compression Approach

Figure 7-8(b) 	:  Connectivity View of Hardware Only Compression Approach

Of course, there will also be mass, power, implementation and evolvability issues associated with these choices that must also be factored into any final design decision.

## Security Topics in the Connectivity Viewpoint

In the Connectivity Viewpoint security topics are dealt with by the physical elements that are used to implement security policies and barriers.  These might include: secure routers and firewalls, hardware security modules, and possibly physical boundaries such as shielded rooms or air gaps.  At the time of publication specialized security components and approaches may be deployed, such as two-factor identity tokens, Continuous Diagnostics and Mitigation (CDM) & Security Information and Event Management (SIEM) tools, virtualization platforms, and cloud deployments. The kinds of protocol entities that may implement elements of security functionality, such as security protocols or routing filters, will be addressed in the Communications Viewpoint.

Examples of Specific Connectivity Security Elements

- Firewalls

- Routers

- Backbone & leaf networks

- Primary & backup network services

- WiFi servers access point

- VPN servers

- Compute servers / data centers

- User devices

- Identity devices

- Network / security devices (IDS, etc)

- Radio Frequency / Optical Connectivity

# Structural Viewpoint - Derived

The Structural Viewpoint is derived from the Physical Viewpoint and is used to describe the structural aspects of physical elements and the connectors that physically tie an assembly together, and that allow for articulation of parts.  Because the locations and orientations of instruments are important for interpreting their commands and telemetry, the structural viewpoint may also be used to address the mounting of sensors and actuators within in an assembly.

## Concerns

Concerns for the Structural Viewpoint are:

- a physical decomposition of the system into objects that form an assembly and that interact at interfaces;

- ability of structure to survive launch, the space environment, and landing;

- ability of structure to provide required agility and field of view for pointing instruments.

## Concepts for the Structural Viewpoint

The Structural Viewpoint of a space system focuses on the physical aspects of structures and articulation of the parts of an assembly.  This Viewpoint addresses structural objects, their behavior, the physical connections between them, and their physical interfaces and interactions.

The behavior of a structural object is the set of actions performed by this element to achieve an objective. A Structural Object performs actions to achieve an objective of a space system, and this may involve rigidity, mechanical movement, thermal conduction, or shielding.

Structural Views describe Structural Objects, how assemblies are held together, how instruments are held in a particular orientation, and how instruments may be moved into adaptive working positions.

- The orientations of instruments relative to the coordinate system of the assembly makes it possible to transform measurements, images, and forces to the coordinate system of the assembly.

- Flexibility of Structural Objects enables estimation of error bounds on coordinate transformations.

- Location, orientation, and articulation of structural objects determines the field of view of instruments mounted on the structure.

- Mass properties, including center of mass and inertial matrix, of structural objects and of onboard instruments determine the response of an assembly to forces such as propulsion and torques.

- Articulation of structural objects can be measured by sensors in any control loops, but must be estimated from commands in open control loops.

- Information objects that carry measurements from sensors and commands to actuators can appear in a view that shows the aggregation of structural objects in an assembly.  The information objects that appear in the Structural Viewpoint are representations of the information objects that are fully described in the Information Viewpoint.  The details of how these information objects are defined, described, and controlled are covered in the Information Viewpoint (section 10).

A Structural view shows stasis, control of articulation, and other attributes of structural objects.  In the engineering of any given system, allocation of instances of control functions to articulated Structural Objects may be represented. The physical means for providing communications among implemented functions are treated in the Connectivity Viewpoint (section 7), as are the physical attributes of the connections and their behavior.

## Characteristics of STRUCTURAL objects

The fundamental features of Structural Objects and their interfaces have already been shown in the Physical Object overview, Fig 6-1.  No additional features are needed for the Structural Viewpoint, only added specificity and a deeper focus on the mechanical & structural aspects of the system.

The interfaces of Structural Objects are classified into four categories: Service Interfaces, External Interfaces, Environmental Interfaces, and Management Interfaces.  Every Structural Object has one or more interfaces through which it interacts with the physical environment, according to laws of motion and energetic exchanges, such as propulsion and solar heating.

### Key Objects and Relationships

The following elements may appear in the Structural Viewpoint:

- Structural Objects that provide energetic exchanges;

- Nature of energetic exchange, such as heat, electrical power, electromagnetic radiation, structural continuity, momentum, angular momentum;

- Structural Objects that provide a chassis that ties an assembly together or that provides articulation for instruments;

- Instruments that implement Functions (abstract set of functions, their behaviors, interfaces and configurations);

- Types: data source, data sink, data transformation, control, planning, monitoring, analysis;

- Attributes: role, name, type, behavior, interface signature, data types handled, interaction modes, constraints, allocated requirements;

- Logical Links (connections between Structural Objects, connected to associated logical behavior and properties);

- Relationships (configuration, precedence, control and data flows, management flows, allocations);

- Information (representations of data that are exchanged among Structural Objects and control/acquisition functions, where formal specifications for exchange are found in the Information Viewpoint).

### Ontology of Structural Objects

The ontology of Structural Objects is shown in Fig 8-1.  It differs from the Physical Object ontology in the specialization of possible flows and the addition of interface standards and an Interface Control Document (ICD).  The terms “structural element” and “structural connector” replace the terms “component” and “connector”.

Figure 8-1	:  Ontology of Structural Objects

#### Representation of Structural Objects

Structural objects use the same representation as Physical objects.  See Section 6.3.1.2 and Fig 6-3.  All the features of Physical Object representation that are needed for the Structural Viewpoint are already present in the Physical Viewpoint representation.  The distinction is that the Structural Viewpoint is focused on the structural aspects of the physical elements and elements in Structural views may focus more closely on the structural details of physical connections.

## Terms for Structural Viewpoint

The Structural Viewpoint inherits all the terms from the Physical Viewpoint.  See Section 6.4.

### Attributes for Structural Viewpoint

Mass: The inertial property of a Structural object.

Center of mass: The location of the balance point of a Structural object in the coordinate system of the object.

Inertia matrix: The moments of inertia of a Structural object arranged in an array.

### Other Terms for Structural Viewpoint

Stress: A measure of forces (internal or external) acting over some cross-sectional area of an object.

Location: The coordinates of the origin of the coordinate space associated with a Structural object in the coordinate system of the assembly that contains the object.

Orientation:  The rotation of a Structural object from alignment of its coordinate system with the coordinate system of the assembly that contains the object.

Flows: may represent movement of data or of substance or energy. Flows of data are shown using named data objects.  The formal definitions will be found in Information views.  Flows of matter or energy may appear in the Structural Viewpoint

## Typical Structural Object Types

Table 8-1 provides examples of some common structural objects.

Table 8-1: Examples of Structural Objects

| Bolt | a fastener that joins two or more structural objects through aligned drilled holes in each object |
| --- | --- |
| Boom | a linear static arm that can be deployed to separate an instrument such as a magnetometer from unwanted influences such as magnetic torque bars in a spacecraft |
| Bulkhead | a wall where instruments may be mounted or radiation may be shielded |
| Heat Pipe | is a structural element with good heat conductivity for transferring heat from a hotter object, such as a computer, to a cooler object, such as a radiator |
| Instruments | concrete elements that support a mission by sensing and/or actuating. In the structural view, instruments obtain their locations and orientations |
| Strap | flat connector that joins two structural elements forming a corner or buttment |
| Weldment | a place where two structural objects make contact and have been melted to join rigidly at the point of contact |
| Wiring Harness | a collection of cables for distribution of power or data signals between instruments an assembly |

## Examples of Space Systems described with Structural View

As noted earlier, in Sec 6.3.1.2,  RASDS++ only provides a rather “cartoon style” of representation for these Structural diagrams.  This is not adequate to accurately describe engineering details, but it is useful to quickly explore architecture options and to do so in the context of all the other related views available in an architectural model.  The usual mechanical and electrical engineering tools are expected to be used once trade studies of the option space have settled on the best choice.

Figure 8-2 shows a representative set of Structural Objects used in a space system together with the interactions that occur among them.  The points of connection are at the mounting interfaces, and the hinge contact surfaces.  In this view these hinge connections provide articulation points and may also flow heat energy.

Figure 8-2: Example view of a hinge assembly

Other views of this same set of Physical Objects might be shown, such as a Connectivity View showing thermistor telemetry, hinge angle commands, and hinge angle telemetry information flowing through a communications subnetwork to/from a thermal control function.

As a related example, Figure 6-4 could be drawn with a cartoon hose, as shown in Figure 8-3, or Figure 6-5 could be drawn with a cartoon articulation for the antenna as shown in Figure 8-4.  These architectural explorations can result in a designer’s choice of implementation technology, and this must be explained in accompanying text.  For example, in Figure 8-4 a mechanical articulation is specified, ruling out other choices, such as a phased array.

Figure 8-3 A “cartoon” conduit hose added to Figure 6-4

Figure 8-4 A “cartoon” physical bus and articulated antenna added to Figure 6-5

## Security Topics in the Structural Viewpoint

In the Structural Viewpoint, the structural objects that are used to implement security policies and approaches are defined.   Passive security features may include warnings to integrators of devices that require special security, for example, using red and black wires for secure subnetworks and common subnetworks, respectively.  Radio-frequency shields may protect devices that could radiate sensitive information about their operation, or that could be affected by other radio-frequency generators, such as an antenna.

Examples of Specific Structural Security Elements:

- Gates

- Doors

- Air gaps

- RF Shields

# Communications viewpoint

The Communications Viewpoint defines the communications protocols and the layered sets of protocols (stacks) that are required to support communications among the software or hardware Engineering Objects in a space data system.  These protocols need to meet the requirements on performance and the constraints imposed by physical connectivity, environmental, and operational challenges.  The Communications Viewpoint is used to describe these layered communications protocols and their deployment and features, and to address these technical aspects of space data systems.

This is the Viewpoint where the lower five layers of the ISO seven-layer communications stack are typically addressed.  Application layer protocols, including specialized messaging  of web application protocols may also be addressed using this Viewpoint specification.  This Viewpoint is orthogonal to the other, upper-layer / application, Viewpoint, where multiple perspectives on the applications in a distributed system may be provided.  However, at any point where protocol designs for an interface must be described Views from this Viewpoint may be employed.

## Concerns

Concerns for the Communications Viewpoint are:

- the choice of communications and data transfer standards in the system;

- the end-to-end communications protocol functionality and reliability;

- implementation of the protocol specifications and the services they provide;

- the relevant interfaces and interactions;

- alignment of required and provided interfaces between two layers;

- interaction of protocol design with environmental constraints;

- support for design, evaluation of suitability, and integration into the rest of the system.

## Concepts for Communications Viewpoint

The Communications Viewpoint is a space data system engineering and technical view that focuses on the mechanisms and functions required to design and implement protocols and communications standards for a space data system, including implementation choices, and specifications and allocation of communications functionality to engineered components of the system.

This Viewpoint is used to provide details on layers one through five of the ISO seven-layer model. The first three RASDS Viewpoints are directly related to the top, or application layer, of the ISO Basic Reference Model (ISO-BRM [14]) and the Information Viewpoint is most closely related to the representational layer of the ISO-BRM model.

In the Communications Viewpoint, the communications aspects of a space data system are depicted with Protocol Objects, but these are called Protocol Entities for alignment with ISO-BRM terminology.  To understand their role in context these are often shown along with representations of the Nodes, Links, and software Engineering Objects that are defined separately in the Connectivity Viewpoint. The Communications Viewpoint describes in detail the protocols that are required for the software Engineering Objects to actually communicate with one another and supports descriptions of the end-to-end information system.

A Protocol Entity performs actions to exchange or transfer data in a space system (as distinguished from a Functional Object that generates or processes data).  Protocol Entities are used to support interactions between two Engineering Objects or among groups of Engineering Objects that are contained in separate Nodes.  Protocol Entities are often shown as two peer entities communicating with each other over a Link between connected Nodes or, using network layer protocols, within a network consisting of multiple Nodes.

Engineering Objects may implement protocols in hardware or software, and the Protocol Entities themselves may be implemented in hardware or software.  Some Nodes in a space data system may only have communications functions.  A Router (ISO layer 3) or Bridge (ISO Layer 2) is an example of a Node that contains only Protocol Entities (without other Functional elements).

While a full “typical” communications stack (application, transport, network, data link, physical) is often used in the terrestrial subdomain of a system, in many space subdomains only the lower data link and physical layers may be specified, with applications providing any upper layer functions that are required.  Newer space systems incorporate onboard networking and even networking among and between spacecraft using space qualified networking protocols.  Separate Management protocols may be employed for complex, networked, systems and various security protocols may also be employed where needed.

## Characteristics of communications objects

The interfaces of communications objects (Protocol Entities) are shown in figure 9-1.

The upper (provided) interface of a Protocol Entity to an Engineering Object or other Protocol Entity is called the Provided Interface and it is typically described in the protocol specification in terms of requests, indications, responses, and confirmations.  The services provided by a Protocol Entity are made available at its Service Interface, which is called a Service Access Point or SAP.

Protocol entities also receive services from a lower layer of the protocol stack.  The services required of the lower layer may be described in the protocol specification as the Required Interface.  These are not always fully described and may be treated as an implementation detail.

Figure 9-1	:  Communication Object Overview

Protocol Entities communicate with peer Protocol Entities, either directly or indirectly, through the stack of lower-layer Protocol Entities. The logical interactions between peer Protocol Entities, at the same ISO layer, are described by exchanges of Protocol Data Units (PDUs), and the behaviors that takes place within a Protocol Entity, in response to arriving PDUs, are most often described as a state machine or table of state transitions.  This state machine describes the actions that the protocol entity is to carry out upon arrival of any of several different PDUs or other events.  Activities within a Protocol Entity may also be triggered by events such as timers or by a management request from a peer or separate entity.

The management interface of a protocol entity may be defined by a relatively static set of configuration parameters defined in a Management Information Base (MIB), or it may be defined as some separate out of band interface or protocol, or by an “in band” protocol that is addressed to an interior or exterior management entity.  Specialized management protocols are most often found in higher level (ISO layer 3 and up) configurations.

## Key Objects and Relationships

The following elements may appear in the Communications Viewpoint:

- Protocol Entities (elements that implement specific protocols, with service access point (SAP) and peer interactions optionally shown, protocol stacks):

- Types: protocol purpose (e.g., coding, modulation, link, network, transport, middleware, application service);

- Attributes: name, type, capabilities (e.g., in order, once only, bandwidth efficient, error correcting, delay tolerant), applicability (e.g., deep space, near Earth, in situ), constraints, services (offered, required), interface signature (requests, indications, responses, confirmations), application programming interface (API, where appropriate), standard reference identifier, standards organization;

- Protocol design specification elements (PDU description, state machine or table description), reliability (acknowledged, unacknowledged, selectively acknowledged), other design views of the communications protocol or protocol stack;

- Nodes and Links (representations of physical elements from the Connectivity Viewpoint, for context);

- Software Engineering Objects (representations of implemented functions from the Connectivity Viewpoint, for context).

### Ontology of Protocol Entities

Figure 9-2 shows the Ontology of Communications Viewpoint Objects.  The normative specifications for protocol entities are usually specified by a standard, which is itself an Information object.  The stack of protocols that are defined at the boundary of a system element may provide a service interface that is called an Interface Binding in the Service viewpoint.

Figure 9-2 Ontology of Communications Viewpoint (Protocol) Objects

### Representation of Communication Objects

Figure 9-3 shows the defined representation of Communications Objects.  Two protocol entities are shown, Layer n and Layer n-1.  Each has a Service Access Point (SAP), the interface on top is the Service Provided Interface, offered for use by an upper layer protocol or application.  This gives access to the services provided by that protocol layer.  The interface below is the Service Required Interface, and it may (optionally) describe the kinds of services required from a lower layer.

If these Required and Provided interfaces between an adjacent pair of layers do not match then some protocol interface “shim” may be required.  This shim may be “thin”, requiring little to handle the impedance mismatch, or it may be “thick” enough to be identified, and documented, as a protocol layer or adapter in its own right.  If this shim is formalized in its own standard, it will typically be represented by a sub-layer in a protocol stack diagram, but sometimes it is just mentioned and left as an implementation detail.

While protocol PDUs logically flow “horizontally” between peer level protocol entities, in actuality they flow down (and then up) the stacks on either side.  At each layer the PDU for layer n becomes the Service Data Unit (SDU) for layer n-1.

Figure 9-3	:  Representation of Communication Objects

Often the stack of protocols, with their successive encapsulations of SDUs, shown as data inside lower level PDUs, may be presented in a diagram that reflects the actual assemblage of bits (or octets) that appear at the lowest, physical, layer in the stack, an “on the wire” view as it were.

## Terms for the Communications Viewpoint

### Attributes

Protocol Attributes are the key features of a specific protocol that describe the ISO layer at which it is intended to provide the defined protocol behavior.

A Standard is a formal specification that defines and governs functions and protocols at interfaces of a data system. It describes in detail the technical capabilities of, and establishes the requirements to be met by, interfacing subsystems to achieve compatibility and interoperability.

Functionality is the ability of a protocol to perform its intended tasks.

Applicability of a protocol is a statement of the fact or quality of applying to a certain situation or range of situations.  Many CCSDS (and other) protocols will contain an Applicability clause describing where and how it is intended to be used.

The Technology of a protocol is a description of how it is implemented.  The same protocol specification might be implemented in software, firmware, or hardware, depending upon performance or other requirements, such as a requirement to support future updates.

### Other Terms for the Communications Viewpoint

Most of the definitions in this section are drawn directly from the ISO-BRM [14].

An (N)-layer is any specific layer in a multi-layer protocol stack.  The layer above is called the (N+1)-layer, the layer below is called the (N-1)-layer. This notation is also used for other concepts in the model which are related to these layers, for example (N)-protocol, (N+1)-service.

At a given instant in time during the life of some object, State is a condition or situation that determines the set of all sequences of actions in which the object can take part.

A State Machine is a description of the discrete sequence of states that an object or interaction goes through during its life in response to events, together with its responses and actions.  A State Table is an alternative tabular representation of the same information.

A Protocol Entity is an active element within an (N)-communications-subsystem embodying a set of capabilities defined for the layer that corresponds to a specific (N)-entity-type (without any extra capabilities’ being used).  Protocol Entities implement protocol state machine behavior.

A Protocol Data Unit (PDU) is a unit of data specified in an (N)-protocol, consisting of (N)-protocol-control-information and possibly (N)-user-data.  PDUs are the actual data objects that are exchanged between peer protocol entities.

A protocol is the set of rules and formats (semantic and syntactic) used to determine the communication behavior of (N)-protocol-entities in the provision of (N)-services.  The behavior of the state machines that operate within a Protocol Entity and the PDUs that are exchanged between these entities specify a protocol.

A Service Access Point (SAP) is the point at which (N)-services are provided by an (N)-protocol-entity to an (N+l)-protocol-entity.

An application programming interface (API) is a set of definitions of the ways one piece of computer software communicates with another. It is a method of achieving abstraction, usually (but not necessarily) between lower-level and higher-level software.

## Typical Protocol Entities

Table 9-1 shows examples of typical Protocol Entities used in space data systems.   This table is representative, but does not include all of the available or applicable protocols for use in space data systems.  Not all combinations of these protocols are valid.  Protocols are normally associated with some layer in the ISO-BRM, but these layers are not identified here except by reference to protocol type.  See CCSDS Overview of Space Communication Protocols (OSCP [16]) for more information about which combinations of protocols are recommended from layer 4 down to the physical layer.  The CCSDS Space Communication Cross Support Architecture (SCCS-ARD [19]) addresses both the currently available CCSDS standards and their appropriate uses in much more depth, as well as their allocation to different components in a typical system.

Table 9-1	:  Typical Protocol Entities

| Protocol Entities | Type | Description |
| --- | --- | --- |
| Space Message Transfer (AMS) | Messaging | Provides message transfer services between functions |
| CCSDS File Delivery Protocol (CFDP) | File transfer protocol | Transfers files over one or multiple space links |
| File Transfer Protocol (FTP) | File transfer protocol | Transfers files over Internet protocols |
| Audio & Video | Application Layer protocols | Provides end-to-end audio & video communications |
| Transmission Control Protocol (TCP) | Transport protocol | Provides end-to-end communications in Internet |
| Space Packet Protocol | Network protocol | Provides a path identifier through a set of one or more space and terrestrial links |
| Delay Tolerant Network Layer (BPv7) | Network protocol | Provides routing through a delay (and disruption) tolerant network (DTN) involving a set of space links |
| Bundle Protocol Security (BPSec) | Network layer Security | Provides mechanisms for securing the data sent end-to-end across a DTN network |
| Internet Protocol | Network protocol | Provides routing through Internet |
| TM Space Data Link Protocol | Data link protocol | Provides communications from space to ground over a point-to-point space link |
| TC Space Data Link Protocol | Data link protocol | Provides communications from ground to space over a point-to-point space link |
| AOS Space Data Link Protocol | Data link protocol | Provides communications, space to ground, ground to space, or space to space, over a point-to-point space link |
| Unified Space Data Link Protocol (USLP) | Data link protocol | Provides communications, space to ground, ground to space, space to space, or orbit to planet surface, over a point-to-point space link |
| Space Data Link Security | Link layer security | Provides mechanisms for securing the data sent across a single link |
| TM Synchronization and Channel Coding | Channel coding | Provides mechanisms for data synchronization and error control |
| TC Synchronization and Channel Coding | Channel coding | Provides mechanisms for data synchronization and forward error correction (FEC) |
| Proximity-1 Space Link Protocol | Data link + physical protocol | Provides communications, orbit to planet surface, over a point-to-point space link |
| WiFi & 3GPP | Data link protocol | SOIS Layer 2 “on-board” protocols, also suitable for planet LAN / WAN use |
| 1553 & CAN bus |  | SOIS Layer 1 & 2 on board protocols |
| CCSDS RF and Modulation | Physical protocol | Define physical RF frequency bands and power or bandwidth efficient modulation (BWEM) to transmit and receive RF signals over a space link |
| CCSDS Optical and Modulation | Physical protocol | Define physical optical frequency bands and modulation to transmit and receive optical signals over a space link |

## Examples of Space systems described with Communications ViewPOINT

Descriptions of Protocol entities may just focus on the features of an individual protocol layer, including the PDUs, the interfaces, and the behavior of the entity described as a state machine.  Such descriptions may be used to just document a key part of a protocol stack that enables a certain kind of data transfer, such as across a space link.  Or the end-to-end protocol stack may be shown, describing how a sets of protocols provide capabilities, such as packet or file delivery, or delay and disruption tolerant space internetworking, are to be provided.

### Protocol Stack Diagrams

A simple example of a Communications View is shown in figure 9-4, in which three stacks of Protocol Entities are represented as groups of rectangles.  Each rectangle represents a specific Protocol Entity that implements services for each Layer in the protocol stack.  Each Protocol Entity offers services to the N+1 layer entity that is above it and uses the services of the N-1 layer that is below it.  Each N-layer Entity participates in an exchange of protocol data units with its peer N-layer Entity.

This figure is borrowed from the SCCS-ARD [19], which uses RASDS as a framework to describe the services, and possible deployments, of more than 70 different CCSDS layer 1-7 standards.

Figure 9-4	:  Simple Example of an End-to-End Communications View

Figure 9-4 represents the end-to-end flow of application data from a User Node on Earth to a User Node in space, using a ground station (referred to as an Earth-Space Link Terminal, ESLT) to provide the RF Ground to Space link.  In this case the Telecommand Protocol (TC) is used in the forward direction.  And, as you might notice, the protocol stacks on the terrestrial side and the space side of the ESLT are not fully symmetric. This is due to the different features implemented in the ESLT Space Link Extension (SLE) Forward CLTU (F-CLTU) service protocol, that transfers the TC PDUs (as encoded CLTUs) over the terrestrial WAN versus how they are transferred over an RF ground - space link.   The TC link layer protocol is carried end-to-end, but the underlying link layers (and the SLE “tunnel” are different).

Many of the protocols used in space exhibit these kinds of asymmetries, which is often due to the very constrained resources that are available on the typical spacecraft, where the size, weight, and power of the components is always a strong consideration.  Other drivers, such as SLE / CSTS terrestrial services to access TT&C space links, and the kinds of services that will become necessary in remote environments such as CisLunar, will also result in asymmetric configurations.

### End-to-End Protocol Diagrams

Frequently it is necessary to show not just the end-to-end protocol stacks, but to also show them in conjunction with the Physical nodes where the protocols are implemented and deployed.  The Physical nodes are referenced via correspondence.

Figure 9-5 shows an abstract view of an end-to-end protocol stack and overlays these protocol elements, by correspondence, on three nodes, two user nodes and a ground station, called an Earth Space Link Terminal. This figure is also borrowed from the SCCS-ARD [19] .

Figure 9-5	:  Example of Communications View Showing Abstract Protocol Stack and Allocation to Nodes

Figure 9-6 shows a more complicated set of link layer and network layer Protocol Entities that support the communications between an Earth User Node (on the right) and its Space User Node (on the far left).  These two user nodes are shown communicating using the Delay Tolerant Network (DTN) protocols [22] end-to-end.  This is an example of what is called a Solar System Internetwork (SSI) [23] deployment.

Intermediate nodes are also shown, the ESLT configured to support Solar System Internetwork (SSI), supports both “standard” link layer protocols and that also hosts the features that support SSI networking.  The other “support” nodes for SSI end-to-end communications that are included in this view are the two Space Routing Nodes that are placed out in a remote orbit around the Moon or Mars to relay data to the surface of the planet, and the Space Routing Node MOC that controls these relay orbiters.  End-to-end data traffic can flow through all these nodes, or, depending on how they are configured, DTN traffic may only flow through some subset of them.

Figure 9-6	:  Example of SSI End-to-End Communications View Showing Nodes

### Protocol PDU and Behavior Diagrams

A Communications View may just show Protocol Entities in a ‘black box’ view, with only the SAPs and some representation of peer protocol entities indicated.  But where required, more engineering details of the protocol specification may also be represented by showing the internal data flows, structure, and timing of PDUs, and even the internals of processing within the Protocol Entities.

In most CCSDS (and Internet) documents, a PDU is shown as a series of octets.  Several different presentation styles are in use, and a specific representation for PDUs is not defined here. However, figure 9-7 provides a useful example from the CCSDS Space Packet Protocol (SPP) document (133.0-B-2).  The IETF Requests for Comment (RFC) that document Internet protocols mandate a similar representation of PDUs, but they are expressed in an ASCII text form.  However they are shown, the important aspect of this is to depict the exact data structures, ordering, sizes, and types of fields, and to identify the control and data elements in the PDU.

Figure 9-7	:  PDU Example, Space Packet Protocol

In addition to describing the sets of PDU structures, it is usually necessary to specify the set of states and transitions that describe the actions taken within a Protocol Entity when a particular type of PDU arrives.  This may be shown diagrammatically using a state machine, as in Figure 9-8, which is a simple one taken from the CCSDS Space Link Extension (SLE) Return Channel Frames (RCF) document (911.2-B-4), or it may be described using a state table or even narrative text, as is done in several CCSDS and Internet documents.

RASDS does not require use of any specific form to describe protocol entity behavior, but a clear and concise state machine or state table specification is preferred, as these are typically more precise and more readily converted to code.

Figure 9-8	:  Example State Machine Diagram—SLE RCF

For interoperability, regardless of the representation used, the complete specification of a protocol must include both the PDU definitions (the data that are exchanged between peer Protocol Entities) and the behavior of the protocol state machine (action taken when a given PDU arrives).

The specification of the interface to a Protocol Entity, or SAP, may be required for application development, but agreement on a common API and language binding, while useful for portability of applications, is not essential for interoperability. Different protocol implementations, exposing different SAPs, with different APIs written in different languages, may be used in the two peer protocol entities with no effect on interoperability if the protocol behavior and PDUs are correctly implemented.

These examples all use the canonical RASDS drawing styles, which work well in document-based architectures.  A related paper, “Model-based Protocol Specification” [32] offers some additional formalized approaches for protocol modeling.  All these features can also be represented in SysML style drawings.  Refer to the paper ”A Representative Application of a Layered Interface Modeling Pattern.” [27] for a pragmatic approach, and worked examples, for representing all these system and protocol features using SysML.

## Security Topics in the Communications ViewPOINT

Certain functions for implementing data system security may be allocated to protocols and these will be addressed in the Communications Viewpoint.  These will typically include application-layer or network-layer security protocols, plus authentication, access control, identity, and key management, link-layer encryption and/or authentication, and spread-spectrum or related physical layer jamming avoidance approaches.

Secure link layer and network layer standards, for DTN and IP have been defined.  [Future] New protocols for DTN network management, key management, and identity management are currently under development. The details of where and when to apply these approaches are described in the CCSDS Security Architecture [4] and Threat Assessment [18] documents, and in the Security Architecture for Open Systems Interconnection for CCITT applications, X.800 [42]. The specific details of how to provide these capabilities are defined in the actual standards and specifications.

Figure 9-9 provides an example of an End-to-End Application and Network layer security deployment.  This Solar System Internetwork (SSI) example is also borrowed from the SCCS-ARD [19], which uses RASDS representations and viewpoints.

Security protocols are shown (in green) as being applied at two different layers, application, and network, in this diagram (the green protocol elements).  Application layer security achieves either information security (hiding) or authentication (you know who sent it), or both.  The important function of ensuring that you know who sent some communique is addressed in ITU-T X.1250, Baseline capabilities for enhanced global identity management and interoperability [43].  The data in the file stream are secured at the application layer, end-to-end, between the sender and the receiver.  BPSec network layer security may also be used to provide similar information security features from end-to-end within the network layer itself.

Figure 9-9	:  Example End-to-End Network (BPSec) and Application Layer (File Secure) Security Protocol Deployment

Examples of Specific Communications Viewpoint Security Protocols

- Peer entity authentication

- Data origin encryption

- Data integrity

- Access control protocols (e.g. SAML, OAuth, PKI)

- CCSDS Space Data Link Security (SDLS) and Key Management

- CCSDS Bundle Protocol Security (BPSec)

- Two Factor Authentication (2FA) protocols

- IPSec

- HTTPS

- Secure DNS

- Application layer data encryption & authentication

- Secure application access

- Secure routing updates

- Secure device monitoring

# Information Viewpoint

The Information Viewpoint provides the detailed descriptions of the data objects that are passed among the elements in a system.  These data objects may have different elements, structures, semantics, relationships, and policies.  The Information Viewpoint is used to address the data architecture and data definition aspects of space systems.  Representations of the Information Objects that fully defined in this Viewpoint appear in other Viewpoints by correspondence.  They are managed (that is, stored, located, accessed, and distributed) by information infrastructure elements and also shown as being passed among enterprise, functional, operations, and application entities. With the addition of carefully constructed relationships among information objects knowledge may be represented as well, in the form of ontologies or knowledge graphs.

## Concerns

Concerns are

- the structure and semantics of information and information management in a space system;

- the rules and constraints on information transformations and permanence in a space system;

- the relationships among information objects.

## Concepts for the Information Viewpoint

The Information Viewpoint specification of a space system focuses on the information used by that system.  This includes structural (syntactic) and semantic views of the information, the relationships among information elements, constraints on their use, rules for their management and transformation, and policies on access and persistence.

The Information Viewpoint looks at space systems from the perspective of the Information Objects and their relationships, separate from how they are implemented or used.

Information Objects are descriptions of data along with the necessary structure and syntax to allow interpretation and use of these Objects.  An Information Object may also have associated metadata, and information views may define the relationships among Data Objects, rules for their use and transformation, and policies on access.

Information is any communication or representation of knowledge such as facts, data, or opinions in any medium or form, including textual, numerical, graphic, cartographic, narrative, or audiovisual.

Knowledge is facts, information, and skills acquired by a person through experience or education; the theoretical or practical understanding of a subject.

Metadata is ‘data about data’, the information that describes content. It is information about the meaning of data, as well as the relationships among Data Objects, rules for their use and transformation, and policies on access.

An Information Package consists of a primary Information Object, with optional ancillary information, and any associated supporting information that is needed to use the Information Object.  The Information Package has associated Packaging Information used to delimit and identify the primary Information Object and Supporting Information.

A taxonomy (or taxonomical classification) is a hierarchical classification or categorization system in which all the terms belong to a single hierarchical structure and have parent/child or broader/narrower relationships to other terms.  Many taxonomies are hierarchies (and thus, have an intrinsic tree structure), but not all are.

An ontology encompasses a representation, formal naming, and definitions of the categories, properties, and relations between the concepts, data, or entities that pertain to one, many, or all domains of discourse. More simply, an ontology is a way of showing the properties of a subject area and how they are related, by defining a set of concepts and categories that represent the subject.

This document uses an informal style of ontological representation to document all the objects in each viewpoint, their functions, the relationships among those in each viewpoint, and their relationships, via correspondence, to objects in other viewpoints.

## characteristics of information objects

The characteristics of Information Objects are shown in figure 10-1. Unlike most other objects considered in this document, Information Objects are treated as static elements and are not represented with input or output interfaces. Information Objects will have some sort of schema that describes their structure, rules for use and transformation, and policies on access and permanence.

Information Objects may have simple relationships represented in a data structure or schema, or they may be represented in a hierarchical taxonomy along with related terms, or in an ontology where the relationships among the different terms may be highly expressive and complex.

Figure 10-1	:  Overview of Information Objects

### Key Objects and Relationships

The following elements may appear in the Information Viewpoint:

- Information Objects (abstract definitions of information elements, structures, semantics, schema):

- Types: data, metadata, information, package, schema, model, meta-model;

- Attributes: name, type, length, structure, syntax, semantics, permanence, provenance, realized by, rules, policies;

- relationships (information object aggregates, transformations);

- constraints (type checking rules, permanence, policies).

### Ontology of Information Objects

The ontology of Information objects is shown in Fig 10-2.

Figure 10-2	:  Information Object Ontology

### Representation of Information Objects

The representation of Information Objects recommended in this document is shown in Fig 10-3.  The representational style and relationships are based on UML [6] notation.

Figure 10-3	:  Representation of Information Objects

## Terms for the Information Viewpoint

### Attributes

Data is a representation of facts, concepts, or instructions in a manner suitable for communication, interpretation, or processing by humans or by automatic means.

Data element is a basic unit of information that has a unique meaning and subcategories (data items) of distinct value.  Examples of data elements include gender, race, and geographic location.

A data structure is a data organization, management, and storage format that is usually chosen for efficient access to data.

Data semantics is information that defines the meaning rather than the physical representation of data.  Semantics potentially cover a very large domain, from the simple domain, such as the units of one data entity, to a more complex one, such as the relationship between a data entity and another.

Relationship is the way in which two or more entities can be associated with one another.

### Other Terms

Artifact is any tangible thing made, modified, or used by people, or produced during system design, development, testing or operations.

Data Objects are the basic Information objects, either physical or digital.

A Data Model is the schema and structure definitions of information in a system.

A Meta-model is an explicit model of the constructs and rules needed to build specific models within a domain of interest.

Abstract Data Architecture Meta-Models are models for Specification and Standardization of Data Elements (e.g., ISO/IEC 11179, DEDSL).

Data Architecture is a model of the structure and relationships among the data elements used within a system.

A Knowledge Model is a representation of knowledge in a form that can be interpreted by both humans and machines and is used in knowledge-based systems.

A Schema is an information model defined in a document or a database. The universe of objects that can be described is defined in the schema. For each object class, the schema defines what attributes an instance of the class must have, what additional attributes it may have, and what object class can be a parent of the current object base.

Instantiation is the creation of an instance of some abstract element, achieved by an action of an object in the model. The element can be anything that can be instantiated, in particular objects and interfaces.  Data Models must be instantiated as real information objects to participate in system activities.

Realization is the act or the condition of becoming real.  Abstract data architecture elements must be realized as Data Models and stored in some sort of repository.

The Provenance of an information object documents its place of origin, proof of authenticity or record of previous processing. These are valuable pieces of information in the history of an object.

## Information Object Transformations

The Information Viewpoint includes descriptions of Information Objects (their structure and syntax), information about the meaning and use of these Objects (contents and semantics), the relationships among Information Objects (the data model), rules that define constraints on their use, transformation, and retention, and policies on access.

The basic semantics of Information Objects are shown in the figure 10-4.

Figure 10-4	:  Example of Information View Showing the Basic Object Semantics

Information Objects may be represented in a systems architecture in several different ways, ranging from very abstract views to quite concrete ones.  The Information Viewpoint primarily addresses the abstract specifications of data and provides a language for describing data transformations from the abstract to the concrete. The relationships among these different views are shown in figure 10-5.

Figure 10-5	:  Information Object Transformations

These views may include the data element definitions, the data schema, which specifies the set of data types and order contained within the object, and the relationships among different Information Objects that are defined within the system.  There will also be more concrete representations of Information Objects as they are implemented within the system.  These are shown in RASDS as correspondences in other Views, such as the Enterprise or Functional Views.

The most concrete representations of Information Objects are the Actual Data Objects, or the sets of bits or bytes of data, that are used to store information in memory or to exchange it across a communications link.  If an Information Object is ‘self descriptive’, it may contain within it both the semantic content and a description of the syntax.

Often a separate description of an Information Object may be required to interpret it (although there are also self-describing information objects).  This Data Model or metadata may be in the form of structure definitions within a program, schema definitions in a database or external document, or metadata stored in some other form.

A further level of abstraction that may be part of the Information Viewpoint is the Data Architecture, a design artifact that describes all of the different data elements and their relationships.  This may be stored in a machine-accessible format, or it may be defined in a document.

In a more generalized way relationships among Information Objects may also be defined with an Ontology, which describes in more detail the relationships among a broad set of Information Objects, i.e., is related to, is part of, or is used by.  Increasingly, formal information description mechanisms, such as an ontology, are being used to permit machine access to all these levels of abstraction.  Ontological representations, created using the rules from this viewpoint, are used throughout this document to describe the objects modeled in each Viewpoint.

The Information Viewpoint is primarily concerned with the abstract data architecture representation of information within a system. Representations of this abstract data architecture, in the form of instantiated schema and data models, and representations of concrete data objects, developed as the system is engineered, may appear in Functional or Connectivity Views.  Other representations of abstract data objects may appear in the Enterprise and Functional Views, and actual concrete data objects appear in other engineering views as the system enters detailed design.

## Example of Objects for the Information ViewPOINT

Figure 10-6 shows the relationships among some typical space system Functional Objects and the information that they exchange.  This example shows a mission planning flow, where the green objects are the Functional Objects and the blue ”narrow rounded rectangle” objects are the actual (fully realized) Information Objects they exchange.  This is a Functional View on a system showing representations of Information Objects, which would themselves be fully described in an Information View.  Another way to think of this is that the structure and meaning of the data are defined in an Information View, but how these data are used and transformed is represented in a Functional View.

The Reference Architecture for Space Information Management (RASIM [5]) has a much more complete treatment of the definition and use of information objects.  The interested reader is directed to that document.  It also provides a much more complete description of the information management functional objects introduced in Sec 5.7.

Figure 10-6	:  Example of Functional View with Representation of Information Objects

Figure 10-7 shows an example of an Information Object showing the use of various relationships.  The relationships are represented and labelled for clarity.  This figure is borrowed from the ASL document [26].

Figure 10-7	:  Example Information Model – Spacecraft Onboard Services

Figure 10-8 shows the Information model for the Spacecraft Onboard Dictionary of Terms (DoT) [38]. This model is rendered as an ontology in the Ontology Web Language (OWL) [39] using a tool called Protégé [40], but other tools can also create and manipulate OWL and related forms.  This looks a little like a taxonomy, with a hierarchical set of definitions, but the top-level elements in the ontology are classes and sub-classes, and the elements may be typed by their domain and range.  The domain is the class to which the subject defining. a given property belongs, and a range is the class of its object (value).

Figure 10-8	:  Information Object Formal Models

Formal models of information, and even more expressive knowledge models, are increasingly being used in many space information systems.

## Security Topics in the INFORMATION ViewPOINT

The techniques for ensuring confidentiality and integrity are very often applied to information objects as part of managing system security.  These techniques may be applied to data at rest or to data in motion.  Data may be encrypted, or digitally signed, or both when it is at rest.  And the same kinds of techniques may be applied, at different protocol stack or system layers, as data is manipulated or transported.

Examples of Specific Information Viewpoint Security Elements

- Security Keys

- Passwords

- Risk models

- Threat models

- User device database

- Identity management / Vetted Identities / ICAM registry

- Application database

- Network device database

- User certificates

- Encrypted data at rest / in transit

# Service Viewpoint

The Service Viewpoint is used to describe the exposed services offered by a space system, as formalized by functions, interfaces, and exchanged data objects.  It describes the service interfaces of systems entities (hardware, software, people, and/or procedures), how to request and provide services, the operations that the services provide, and the interface bindings used to access them.

The Service Viewpoint is a composite in that the technical details of service protocols, exchanged data, and the systems elements that offer services are all rendered by using the defined representations for each of those object classes. Furthermore, Enterprise services will often appear in the context of descriptions of Enterprise Capabilities, see Fig 4-7 and 4-8.

## Concerns

The Service Viewpoint addresses the following stakeholder Concerns.

Concerns

- The services provided by the system (H/W, SW, people);

- The interfaces, access points, and protocols for the system;

- The requirements and constraints on the services;

- The required and provided data for the services;

- Any contractual arrangements or agreements for services;

- And how the system provides for confidentiality, integrity, and availability.

## Concepts for the Services Viewpoint

Consistent with other Viewpoints, the Services Viewpoint is based on Service Objects with stated formal relationships among terms. Characteristic of the Services Viewpoint may vary in detail from architecture level to higher or lower layers of space system decomposition as they are realized in Service Views. Service descriptions are themselves dependent upon Function, Communication, and Information Views.

### Service Properties

A service has four properties according to one of many definitions of SOA (Definitions taken from https://en.wikipedia.org/wiki/Service-oriented_architecture):

- It logically represents a business activity with a specified outcome.

- It is self-contained .

- It is a black box for its consumers, meaning the consumer does not have to be aware of the service's inner workings.

- It may consist of other underlying services.

The RASDS++ definition of “Service” is consistent with these definitions.

## Characteristics of Service Objects

Types: Service types, both formal and informal (systems, people, software, etc), data types, and Resources (access to elements having service roles);

Attributes: service name, type, operations, point of contact, interfaces, interface binding signature, and data, interaction modes, policies, constraints;

Domains (boundaries of responsibility or ownership);

Relationships (ownership, membership, participation, roles, contractual);

Information (defined instances of services, functions, interface binding signature, and data specifications, along with documents, agreements, contracts, policies, requirements, objectives, goals, scenarios, membership lists, where formal specifications of all the data to be exchanged are found in Information Views).

In the context of Figure 11-1 the interfaces of Service Objects may involve an app (thick or thin client), or expose a formalized service protocol, or they may have their own user interface (UI), or they may be deployed in the cloud and be accessed using HTTP/REST and a web browser.  Service Objects may also be implemented by people, and accessed via phone, text, or Internet requests, or even, in increasingly rare cases, in person.

Figure 11-1: Service Viewpoint Overview

The external interfaces of Service Objects may be similarly varied.  The Service Object can have interfaces such as Message Bus, Web, Cloud, or AMS service types. ISO /IEC 7498 Basic Reference Model (BRM) treats all of this as “Application Layer”.  Server, web/cloud, or virtualized deployments all fit this same pattern.

### Key Objects and Relationships

#### Ontology of Service Objects

Figure 11-2 shows the Ontology of Service Objects.

Figure 11-2: Ontology of Service Viewpoint

In Figure 11-2 the interaction of service and system objects is formalized. System: A collection of interacting components organized to accomplish a specific function or set of functions.  A System is composed of Elements, which may be any of: Hardware, Software, Person, and Procedures.

#### Representation of Service Objects

Figure 11-3: Service Interface Representation

The Service protocol, which is the defined, exposed, interface, must have defined behavior, PDUs, and required / provided interfaces.  Behavior may be defined in a state machine, state table, or narrative form.  The specification usually only defines behavior of the sending side but may define both ends. A Service Protocol may be layered upon a standard protocol such as HTTP/REST, or use a message protocol like AMS, or be accessed via a defined API with some bespoke protocol.  The protocol stacks underlying the Service protocol follow normal Communication View rules.

Services may use externally supplied Authentication mechanisms.  They may also rely upon security mechanisms (encryption, authentication of data payloads) implemented within the Service layer or in underlying layers.

## Terms for Service Viewpoint

### Attributes

Service Type may be any one of: network service, link layer service, cross support service, mission operations service, web service, name service, or any of many other types of defined services.

Behavior of a service is a generic term for the kinds of behavior that a specific service exhibits.  There are many different kinds of behaviors.

Service Data is a generic term for the kinds of data provided by a specific service.  There are many different kinds of service data, which may be a discrete data object, a stream of data objects (that could be turned into audio or video), or other forms such as a file or a message.

### Other Terms

Service is the provision of an interface of an object to support actions of another object.

A service system is the set of Hardware and Software Components used to implement a Service in a real system. Service Systems may be implemented using one or more Hardware and Software Components.

A cross support service is a function provided by one space agency to support operations of a space mission of another space agency.

A service provider is the role played by a physical, functional, or organizational entity that provides a cross support (or other) service for a service user.

A service user is the role played by a physical, functional, or organizational entity that uses a cross support (or other) service provided by a service provider.

A web service is a software component or system designed to support interoperable machine- or application- oriented interaction over a network. A Web service has an interface described in a machine-processable format, specifically Web Services Description Language (WSDL).

A Space Link Extension (SLE) service is the set of services that extend one of the CCSDS Space Link Subnetwork services, providing access to the ground termination of that service from a remote ground-based system.  An SLE service supplies or consumes one or more channels of the same Space Data Channel type.

Mission Operations Service is a suite of end-to-end application-level services that constitute a Service Oriented Architecture for space mission operations.

## Typical Service Types

Table 11-1 shows examples of typical Service types that appear in space data systems.

Table 11-1	:  Examples of Typical Service Entities

| Service Entities | Type | Description |
| --- | --- | --- |
| Cross Support ground station | Space Link Extension (SLE) Service | A family of link layer cross support services delivering different kinds of spacecraft data between ground station and user. |
| Cross Support ground station | Cross Support Transfer Service (CSTS) | A family of cross support services delivering different kinds of spacecraft data between ground station and user. |
| Mission Operations Center | Mission Operations Service (MOS) | Provides a framework for mission operations services. |
| Mission Operations Center | Message Transfer Service (MTS) | Provides topic driven message transfer services among distributed elements in space and ground. |
| Mission Operations Center | File delivery service | Provides file transfer services among distributed elements in space and on the ground. |
| Cross Support ground station | Virtual Channel Service | Provides delivery services of link layer virtual channel data between distributed elements in space and on the ground. |
| Space relay service provider | Delay Tolerant Network (DTN) Service | Provides delivery services of DTN network data (bundles) from distributed elements in space to the ground. |
| Network service provider | Domain Name Service (DNS) | Provides a service that maps network entity names to Internet addresses. |
| Identity service provider | Identity Service | Provides the ability to assign and verify the identity of an entity using a secure token or other means. |
| Audio service provider (might be Mission Operations Center) | Audio service | Provides the ability to deliver audio data in a stream form (where RTLT permits) or as an audio recording. |
| SaaS provider (cloud services) | Software as a Service (SaaS) | Allows users to connect to and use cloud-based apps over the Internet. Common examples are email, calendaring, file storage, and office tools. |
| Communication Service Providers | Provide end-to-end data delivery (and networking) services | Commercial entities that offer end-to-end (from user MOC to user spacecraft) communications services, at link or network layer. |

## Examples of Space systems described with Services Views

Figure 11-4 provides a simple, abstract, service protocol borrowed from the Monitor Data – Cross Support Transfer Service (MD-CSTS) [24].  Standard network and transport protocols are used to provide connectivity and the details are abstracted away.  The focus in this diagram is on the layering within the service layer, which involves a user/provider interface pair and uses a separately defined CSTS Message layer.  The end-to-end service is between the service provider and the service user.

Figure 11-4: Abstract Example of a Cross Support Service: Monitor Data

This service protocol example is shown as being layered directly on TCP/IP via a technology binding.  Other kinds of bindings, such a web based services using HTTPS, or even cloud-based deployments using some vendor’s cloud service interfaces.  Regardless of the specific deployment patterns chosen, at bottom there will be kinds of layered protocol stacks between the user and the provider, wherever, and however, those components are deployed.

Figure 11-5 shows an example of a Service view that incorporates all four major aspects of a Service as defined within RASDS++.

Figure 11-5: Aspects of ASL-style Service: Mission Control

The four elements of a full service description, as shown in Fig 11-5, are:

- Named service and interface: Monitor & Control

- The Service View description, showing service names, functions, operations, and the kind of data handled

- Service Interface binding, describing the deployment mode and pointers to the protocol stack defined at the interface, including the pointers to the specifics of the service protocol

- The information objects that are exchanged, with pointers to the information view where they are defined

All services essentially follow this same pattern, what tends to differ are the characteristics of how and where the various components are deployed, the protocols that are used, and the granularity of the offered services.

### Relationship of Services to Enterprise and Operations

Enterprises may work with other Enterprises through provision of services. While distinct from service descriptions such as SOAML [25], Service Objects and the formalized relationships among terms are still fundamental to Service Views in Enterprise architecture. Functional, Physical and Operational Views in Enterprise descriptions depend on Services.

### Relationship of Services to Message Bus, Web or Cloud, or Message Protocols

From a protocol / interface binding perspective services may be deployed in many different ways, built upon different underlying protocol stacks: message bus, web services, “cloud”, or other messaging protocols.

- Message Bus: Typically a bespoke protocol implementation, possibly in an on-board context, or using something like a Pub/Sub message bus.  Often uses an API to hide implementation details which may not provide an interoperability spec, but may be layered on underlying transport/network layer like TCP/IP or a link layer.

- Web or Cloud: Typically use HTTP(S)/REST or related protocols on top of a network/transport layer. Server may be on local hardware, or at a data center, or in some “cloud” deployment.  The actual End-to-End protocol stack may involve intermediate caching systems for performance.

- Message protocols: May use an interoperable message protocol spec like AMS and layer upon TCP/IP or DTN network layer.

Services come in many different deployment “flavors”, including: fat client, thin client, browser “app”, virtualized, SaaS, PaaS, IaaS, micro-service, … All use one flavor or another of application and underlying comm protocol stacks with different deployment and granularity models.

## Security topics in the Services Viewpoint

Examples of Specific Services Security Topics:

- Confidentiality services (encryption)

- Integrity services (authentication, verification)

- Availability services (redundancy, resilience, DDOS protection)

- Access control services (Authentication & authorization)

- Procured services, supplier

- Locally built services, supply chain

- Service continuity and recovery

- Development services, supply chain and verification and validation

- Operations services, source, vetting

- Software as a service (SaaS), same as procured

- Network (e.g. ISP VPN), power, HVAC, and other services, integrity, reliability, access control

# Operations Viewpoint

The Operations viewpoint defines the objects, relationships, and rules necessary to describe various space mission operational scenarios.  Operational views address specific concerns and represent activities, processes and behaviors present in mission operations.

## Concerns for the Operations Viewpoint

The Operations Viewpoint addresses the following stakeholder concerns:

- The operations enabled by using the system

- The activities carried out using the system

- The effectiveness or efficiency of carrying out operations

- The requirements on operations and activities (from Enterprise Viewpoint)

- Responding to constraints (e.g. cost, performance, policies) on operations and activities

## Concepts for the Operations Viewpoint

Operations Objects are Tasks, treated like functions, abstract representations of behavior.  For this purpose they are shown within some system context (swim-lanes).  Operations Objects may themselves be decomposed into lower level Operations.

Operations views are intended to describe temporal flows among different systems elements as described earlier.  Such views may explicitly show timing and/or duration, and are suitable for representing instances, durative events, and sequences of events.  They may explicitly identify the information objects that are exchanged (tied by correspondence to the Information View where they are defined).

Consistent with other Viewpoints, the Operations Viewpoint is based on Operations Objects with a formal relationship among terms.  Operations Objects may be operational processes, activities, tasks, and relationships which are represented as using the Functions of Systems.  Operations objects are typically represented using UML Activity diagrams, but optionally Business Process Management Notation (BPMN, [41]) may be used as well.

The Operational Viewpoint may describe any of the following:

- Activity, Procedure and Task all may have stated temporal aspects

  - May have start time, stop time, or duration specified

  - May be used to represent both planning views and actual “as executed” views

- Action:

  - An Action may be instantaneous or have some finite duration

  - An Action may have both planning estimates and actual “as executed” observations

- Event:

  - Any observable system or natural occurrence.

  - The fundamental entity of observed physical reality represented by a point designated by three coordinates of place and one of time in the space-time continuum postulated by the theory of relativity.

- Sequence of Events (SOE):

  - a number of events or activities that come one after another in a particular order

  - the predicted order of events during spacecraft operations, and also the observed order of events during spacecraft operations

Operations processes and activities are often described in the context of some enterprise-defined Scenario.

A scenario is a sequential, narrative account of a hypothetical enterprise concern that provides the catalyst for the exercise and is intended to introduce situations that will inspire responses and thus allow demonstration of the exercise objectives.

A scenario may also be a postulated sequence or development of events, or a description of how things might happen in the future under certain circumstances.

## Characteristics of Operations Object

In the context of Figure 12-1 the service and external interfaces of Operations Objects may involve some formalized service protocol, or they may be initiated via email, or file transfer requests. Operations Objects may be implemented by people, and accessed via phone, text, or Internet requests.  They may also be triggered by events that occur inside the system.

Figure 12-1: Service Object

### Key Objects and Relationships

#### Ontology of Operations Objects

Figure 12-2 shows the Ontology of Operations Objects.

Figure 12-2: Ontology of Operations Viewpoint

In Figure 12-2 the relationships among operations objects, and those of systems and enterprises is formalized.  Operations Procedures may be decomposed into lower level activities and tasks.

#### Representation of Operations Objects

The preferred representation of Operations Objects is shown in Figure 12-3.  The representation formalism is borrowed from SysML Activity diagrams.

Figure 12-3: Operations Object Representation

Each swim-lane is typically associated with a single function group, system/sub-system, and organization and/or team.

## Terms for the Operations Viewpoint

### Attributes

There are many different Types of operations tasks.  They may involve mission lifecycle design, strategic and tactical planning, forecasting, commanding and monitoring spacecraft execution, data management and processing, analysis of events, and anomaly handling.

Flows of data are shown using named data objects.  The formal definitions will be found in Information views.

The may be different Interaction modes used among Operations elements.  These may include request & immediate response, or request with immediate acknowledgement (and later response), or even a request followed some time later with a response or acknowledgement.

A State is a condition or situation that determines the set of all sequences of actions in which the object can take part.

A Constraint is a limitation or implied requirement that limits the design solution or implementation, is not changeable by the enterprise, and is generally non-allocable.

### Operations Viewpoint Selected Terms

Operations objects may be any of the following types:

A Process is a set of interrelated or interacting activities which transforms inputs into outputs.

An Activity is a set of cohesive tasks of a process.

A Procedure is an ordered set of tasks for performing some action.

A Standard Operations Procedure (SOP) is a set of instructions used to describe a process or procedure that performs an explicit task or explicit reaction to a given event.

A Task is a specific defined piece of work that, combined with other identified Tasks, composes the work in a specific specialty area or work role.

An action is something that happens within an object, either with or without participation of another object. An interaction is an action performed by an object with participation of another object or with its environment.

A Product is the result of a process. Note: A system as a “product” is what is delivered by systems engineering.

Optionally the set of relationships described in Figure 12-4 may have a temporal aspect (i.e.: start & stop times, duration) which should be interpreted as an extension of the core model.

Figure 12-4: Operations Viewpoint Core Model with Temporal Aspects (“Schedule”)

Temporal aspects extend the core operations model by introducing the following constructs (see Figure 12-4):

- Plan: Captures the (acceptable) balance of risk vs. result in generating an operational product (e.g.: nominal or off-nominal plans).

- Schedule: Captures temporal context for Plan.  (e.g.: keeps track of SoE <anomaly vs. “as predicted/expected”>).

- Process: What needs to happen in the plan to generate an operational product.

## Examples of OPERATIONS VIEWs

The technical details of the system elements that implement Operations will typically be defined in separate views.  An activity that is part of a operations process may be implemented by people, or by software and hardware Components that are invoked or monitored by people. In its simplest form an Operations View may be described by an activity or sequence diagram showing operations and data within a single system element, or it may include flows among different system elements.

An Operations view may just be a simple ordering of activities, or it may be designed to orchestrate a carefully timed sequence of events, potentially tied to external or natural phenomena.  It may require specification of start and stop times, or of event times.  And it is often the case that the Operations views need to be able to describe the timing of planned activities and to support the comparison of what was planned to the actual timing that occurred.

Figure 12-5 	Simple Example of a Operations View

A simple example of a Operations View is shown in figure 12-5 in which the Activities involved in mission planning, command generation, and delivery are represented.

Fig 12-7: Telemetry & Mission Operations Functions & Service Interfaces

Swim-lanes, as shown in the more complex Figure 12-7, are typically associated with function groups, systems/sub-systems, and other organizations and/or teams. This includes identification of data objects that flow among activities.

Figure 12-7: Operations Viewpoint Mission System Tasks & Data Flows

### Relationship to Functional Viewpoint

The Functional Viewpoint documents functions (and associated Functional Groups) along with any defined interfaces.  It also captures any data being exchanged, which may be carefully defined in separate Information Views.  However, the Functional Viewpoint does not show any temporal aspects or recursion.

The Operations Viewpoint describes how to perform a process to accomplish tasks and shows tasks executed within System Elements (vertical or horizontal swim-lanes).

An Operations viewpoint may show temporal ordering of flows (starting upper left in this case) between Activities (or Tasks), including recursion.  It may also capture timing (start/stop, duration, or events) of activities (or tasks) and names the data objects that are exchanged.  As a byproduct of temporal representation, Operations Views may also represent both predicted and actual observed sequences of events.

### Relationship of Operations to Enterprise, Systems, and Information

Operations gets guidance from the Enterprises in the form of requirements, objectives, people and policies and it’s interfaces with actual Systems are described using Connectivity (Viewpoint).  Correspondence to Information are described by referencing defined instances of documents, agreements, contracts, policies, requirements, objectives, goals, and operational scenarios.

Boundaries of responsibility or ownership related to Enterprise are described using Domain definitions.

## Security topics in the Operations Viewpoint

Examples of Specific Operations Security Topics:

- Managing Access control to facilities

- Assigning Roles & Permissions

- Establishing Operations continuity and recovery processes

- Acquiring Operations services, sources, vetting

- Processes for handling Operational data privacy & authenticity

# Deriving other views From the basic ViewPOINTs

## General

RASDS++ provides a general set of viewpoints that may be used to describe many aspects of space system architectures.  However, depending on the set of concerns that need to be addressed during design of a particular space system, other Views may need to be constructed from the objects and relationships described in the nine basic Viewpoints.  This may be done either by defining correspondences to objects defined in two or three existing Viewpoints, as was done to create the Service Viewpoint, or by defining entirely new classes of objects or relationships using the RASDS object and ontology rules to create a new RASDS-style Viewpoint.

Any new viewpoint would ideally define the primary objects being addressed, their relationships to other object types, the concerns being addressed, the selected representation, and some discussion of how and where it might be used.

As has been demonstrated, providing adequate visibility into particular design concerns can often be accomplished by showing two related Views on one diagram and explicitly describing the correspondences between them.  In some cases it may be necessary to define new constructs, but this should be done with care, to avoid cluttering diagrams with new elements that then need to be defined and explained.

Figure 13-1: RASDS++ in wider social context

Figure 13-1 is an example of adopting large parts of the RASDS++ viewpoint structures, and adding two new dimensions that may occur in systems engineering projects.  This diagram makes explicit that RASDS++ may be applied at different scoping levels, from component, to city or regional scale, and out to space.  The other dimension explicitly identified in this figure relates to project lifecycle stages, from requirements, to architecture, and through development, to operations, and disposal.

There is nothing inherent in the RASDS++ framework that prevents it from being used at these different scales and lifecycle phases. The object types and relationships are clear, but generalized, so that they can be specialized as needed for different purposes.

NOTES ON USE OF RASDS++ to do systems architecture

INTRODUCTION

A detailed treatise on how to do system architecture is far too complex to attempt in these pages.  The interested reader who wishes to learn more is directed to any of the available texts that address this topic, Rechtin and Maier, is very well regarded (Mark W. Maier and Eberhardt Rechtin, The Art of Systems Architecting, 2nd edition, CRC Press, 2000).

The RASDS++ methodology may be applied in many different projects and scopes as part of a system architecture description or design process.  It has been applied to individual components and protocols, to systems, and to systems of systems.  Not all the Viewpoints of RASDS need to be used for all problems.  Regardless of the scale of the project, the first questions that are asked are usually:

- Who are the stakeholders?

- What are their concerns and which views address them?

- What is to be described with this set of architectural views?

- What is the right level of detail to expose during the process?

In many applications of RASDS++ only two or three Viewpoints may be needed, and only one or two Views may be needed to address different concerns in each Viewpoint.

Each Viewpoint Specification in RASDS++ defines the typical stakeholders and concerns, and defines the kinds of objects, their description, and relationships that may appear in any view on a system.  A set of recommended representations is also provided for each viewpoint.  Different representations may be selected instead of those offered here, but the recommendation is that whatever choices are made that they be clearly documented and consistently used.  The same is true of any color codes that might be adopted.  ISO 42010 provides a Legend element of an Architecture Description Framework (ADF) as a place to display such aspects.

During development of system views all these constructs should be treated as constraints on what may be represented in any given view.  For each element in any given viewpoint a set of attributes are specified in sections 4-12.  Not all attributes are needed in all views for any given viewpoint, and not all attributes are relevant for all objects in a view.  Choose the ones that are going to be most effective for the stakeholders that each view is intended to address.

Furthermore, RASDS++ does not provide any single method for capturing these attributes in each view.  In some cases they may be shown as notes, in others as tagged values associated with any element, or as implicit or explicit correspondence relationships.  In still others they may be shown in a separate table.  There are formal methods of capturing these views, as is discussed in the other annexes, that offer suitable means for capturing these object attributes.

The following subsections provide some brief examples of the sorts of heuristics that one can apply while using RASDS++ to produce a Functional View on a system.  Similar heuristics would be applied for each of the other Views that need to be generated, possibly at different levels of detail.  So a high-level Connectivity View showing the major Nodes and Links in a system may be accompanied by one or more detailed views that drill into the internal composition of those Nodes and the connectivity approaches adopted among all of the lower level Components.

Example Methodology for constructing Functional Views:

- For each stakeholder concern, the functional objects and interactions relevant to the concern are identified.

- For each functional object, the services provided to other functional objects are identified.

- For each functional object, the services used from other functional objects, if any, are identified.

- The cooperative actions performed by multiple functional objects, if any, are described.

- The resulting view is checked against the structuring rules.

- At least the abstract data types that are exchanged across each interface are identified.

- Constraints on interactions among Functional Objects are identified.

Example Structuring Rules for the Functional View

- Each functional object that is inside the system has at least one logical link.

- Each logical link is connected to at least one functional object inside the system and to at least one more functional object either inside or outside the system.

- Each functional object or logical link has a unique name.

- Each functional object provides at least one of: generation of data, transformation of data, initiation of action, or response to stimulus function.

- Each functional object has a defined set of interfaces.

- Each functional object has a defined set of behaviors and actions.

- Information is available:

- To explain the functional objects;

- To explain the logical links;

- To indicate whether a functional object is inside or outside the system.

- Taken in total, the functional objects, logical links, and attached notes completely address the concerns of the functional viewpoint at the level of detail appropriate for the audience.

Related Example Methodology for Connectivity View

A methodology similar to that of the Functional View is followed for the Connectivity View.  All the Engineering Objects that need to be represented to capture the breadth of the system implementation design, and enough of its required context, need to be identified.  Mappings should be made from identified Functional Objects to Engineering objects in the Connectivity View.

- Each functional object should be mapped to at least one engineering object in a connectivity view.

- For each logical link in a functional view it should be clear which physical links in a connectivity view support the actual communications.

- The performance envelope required by the assembled set of engineering objects should be described, and whether the capabilities provided by the nodes and links are adequate to meet requirements should be evaluated and documented.

- The interactions of the engineered objects with one another, and with the environment, should be documented.

- The ability of the engineered elements of the system to meet the performance requirements should be evaluated.

- The resulting views should be checked against the Connectivity View structuring rules and cross checked with the Functional View for completeness.

- The steps above may be iterated if necessary.

- Trade studies may be supported by creating more than one mapping of functional objects to engineering objects, and one of more approaches for creating those engineering objects may be evaluated.

FORMAL METHODS AND TOOLS

OVERVIEW

As noted in the Introduction to this document, one of the primary motivations for the RASDS++ is to provide a system architecture methodology that domain experts can use to describe and construct many different specific space system architectures for complex systems.  The RASDS++ methodology can be used very effectively in its current form to describe systems architectures, provide Viewpoints from which to examine them, related representations of architectures, guidelines for concerns to be addressed, and issues to consider at each viewpoint.   Even in the absence of a more formal notation, tools to support design, or formal models the consistent use of these concepts, methodology, and representational formalisms can help enormously in clarifying architectural descriptions and design.

This methodology has been successfully validated through use in describing real space systems. Several different missions and projects have used RASDS++ to describe their high-level architectures, and several CCSDS working groups have successfully used RASDS to document individual standards and complex reference architectures involving many tens of standards.  Feedback from all these activities has helped to refine the present document.

Although RASDS++ is useful even if it is only used to guide selection of useful views and their contents in a set of ‘design drawings’, for RASDS++ to be most useful for large scale systems design, tools are required that will permit the ready creation of system descriptions and that will automatically maintain the complex relationships between objects as seen from different viewpoints.  This requires both a mapping into some more formalized methodology and tools that implement it.

Architectures for several projects that use RASDS viewpoints and models have been developed using SysML formalisms.  These have been effective, and have shown the strengths and limitations of current modeling environments.  SysML, which is based upon UML 2.0, has been used because it already provides formalisms for requirements, verification, viewpoints, describing hardware and software objects, and handling of discrete and continuous data flows.

SysML provides a set of techniques and formalisms for modeling systems and software.  It brings a rich set of formalized modeling constructs that permit description of systems from several different aspects.  These aspects are represented by a set of diagram types, each of which maps to some underlying constructs in the model that the implementing tools maintain.  There are two very important concepts to keep clear about:

- In SysML the model is in the tool, the drawings are just external representations of the model.

- In order to adequately represent stakeholder viewpoints, diagrams must be constructed carefully, with defined objects and semantics used for each view.

The SysML methodology extends UML 2.0 by adding requirements, verification, and parametrics to the UML suite of diagram types.  The SysML Specification also supports modeling semantics for continuous as well as discrete behavior.  This provides good support in a general way for many front-end system engineering and architecting processes.

SysML has incorporated the concepts of Viewpoint and View, but it does not define any specific instances of these.  As part of the earlier CCSDS SAWG studies an analysis was done of the capabilities defined within SysML and how it would map to the kinds of constructs needed by RASDS.  The biggest realization was that many of these SysML diagram types could be used for more than one Viewpoint, and that what differentiated the different Viewpoint diagrams were the object types that were represented in any given viewpoint and the nature of the relationships that would be depicted.

The following description of the relationships between RASDS++ Viewpoints and SysML diagram types, while not complete, is intended to provide guidance to any group that wishes to use SysML to represent RASDS++ Viewpoints.  The SysML constructs suggested for Views in each Viewpoint are underlined:

- Enterprise Viewpoint:

- Organizational structure and behavior diagrams;

- Organizational use case, activity, and sequence diagrams;

- Requirements and constraints for rules, policies and agreements ;

- Functional Viewpoint:

- Logical structure, behavior and package diagrams;

- Functional activity, state chart, parametric, and sequence diagrams;

- Connectivity Viewpoint:

- Physical block definition, composition, behavior and class diagrams;

- Parametric diagram for performance and physical link characterization;

- UML deployment diagrams needed for allocation views;

- Information Viewpoint:

- Information block, package and parametric diagrams;

- Communication Viewpoint:

- Protocol structure and behavior diagrams;

- Protocol state machine, PDU sequence, and activity diagrams.

Clearly there is not just a one for one mapping between SysML diagram types and the kinds of constructs that we need for RASDS Viewpoints.  However, there is a sensible mapping for all the RASDS constructs into at least one of the SysML types once the stereotyping for the particular RASDS object and relationship classes has been made.  The fact that there is much more precision of expression possible with SysML models, instead of PPT drawings, means that additional information can be conveyed in any models developed within these frameworks.

The fact that model element attributes can be specified directly within the modeling environment, and that models can be checked for validity and completeness, makes this a particularly attractive approach for describing complex space system architectures.

See Annex C for some worked examples of using SysML tools to describe complex systems architectures.

RASDS and SysML Examples

INTRODUCTION

Since it was first published RASDS has been used within CCSDS to describe protocol standards, various frameworks, and systems architectures involving the use of many standards.  It has also been used a number of projects that have been written about and published.  Some of these have involved the use of SysML to provide accurate models of systems and to take advantage of the power and functionality of SysML modeling environments.

This Annex uses some of the materials that have been published elsewhere to provide some worked examples of RASDS models developed using SysML.  They can provide useful examples of patterns that might be adopted at various scales to model complex systems architectures.

CREATE A PROFILE

One of the first useful steps in modeling a complex system is to define a SysML profile and a decomposition hierarchy for the model.  This creates a sort of meta-framework for the model and defines the set of terms and relationships to be used in the model.

Figure C-1: Example SysML Profile

This and several of the following figures are borrowed from a SpaceOps 2012 paper titled “NASA Integrated Network Monitor and Control Software Architecture [28].  Figure C-1 defines a set of system decomposition stereotypes and also a set of color codes to be used this particular model.    These are used in the following figures, but there is nothing in this section that should be taken as gospel, they are just offered as a set of coherent, real world, examples.

Within a specific model these stereotypes and color codes should be defined to have meaning in the modeling context, which in this case was a systems of systems model.  The stereotypes that are selected may have as many levels as are needed, and will define the specific terms adopted for the system decomposition.  Note that in this figure the top three levels of decomposition are all different kinds of “systems” and that below that level the components are distinguished as either software or hardware.

# C3  Define the top level decomposition

The next step in the SysML model process may be to define the top level decomposition of the model, using the newly defined stereotypes.   Since this example reflects the decomposition of a space communications system it offers an example of a RASDS++  Connectivity Viewpoint, but one focused on the decomposition of the system into parts rather than just on the connections among them.  This is one of several possible Connectivity views that might be used.

Figure C-2: Example Connectivity View showing composition of Objects

Figure C-2 uses the Sterotypes and also the color codes defined in Fig C-1.  The top level object is one of the defined <<System>> objects and the other components are <<System Element>> and adopt the color codes used to distinguish the different kinds of elements  shown on the different, but related, diagrams.  This particular example also shows, within each <<System Element>> the parts that compose each one, which may be elaborated on lower level diagrams.

Now that these components have been defined they can be used in other Connectivity diagrams showing the communications connections among the different parts, and other details of the systems and communications structures, as needed.

# C4  DESCRIBE the DECOMPOSITION IN A CoNNECTIVITY VIEW

Each of the objects defined in the top level decomposition will have its own decomposition and internal structures.  Figure C-3 shows these features for the top level <<System>> named SNGS.

Figure C-3: Example Connectivity View showing interfaces and communication flows among Connectivity Objects

Figure C-3 uses the same defined decomposition hierarchy and color coding as defined in the Profile.  This is a Connectivity View showing the major <<System Elements>>, their interfaces, and data flows.  These interfaces and flows are named in this view, but the details are defined, by correspondence, in other views.

Several variants on these kinds of Views, as shown in Fig C-2 and C-3, may be developed as part of trade studies intended to determine the bast way to partition the functions in a system.

# C5  DESCRIBE the INTERNAL STRUCTURE IN A CoNNECTIVITY VIEW

Each of the objects defined at any level of decomposition may have their own diagram(s) showing their internal structures.  Of course, this process of successive decomposition is a familiar one, and it may be carried out, in these architecture diagrams, down to whatever level is useful.  Figure C-4 is an example Connectivity diagram showing the internal structures of a <<SW Subsystem>>, that is largely software, named Service Management.

Figure C-4: Example Connectivity View showing interfaces and communication flows among Connectivity Objects

Whereas in Fig C-3 the interfaces among <<System Elements>> were explicitly identified in this case they are not.  This detail is a representation choice and interfaces could be explicit, or left off, at either level.  Choosing to  show them on  Fig C-3 was an acknowledgement that interface control among major <<System Element>> is critical to being able to control the interactions among these elements.

This decomposition  process may be carried down out to whatever level of detail  is appropriate for the given project.  This entire modeling framework may be built upon, and further elaborated, as the development transitions from architecture into design.

# C6  DESCRIBE the SYSTEM DEPLOYMENT IN A CONNECTIVITY VIEW

Having defined the decomposition of some set of system elements it is often useful, in a different kind of Connectivity View, to describe how and where the different elements are to be deployed.  As noted in C4, more than one deployment diagram may be developed as part of trade studies to determine the best allocation of functions to different sites.

Figure C-5: Example Connectivity View showing allocation of System Elements to different sites

Fig C-5 is a Connectivity View that explicitly shows the allocation of different <<System>> components to a separate set of <<Site>>  objects.    Within the modeling tool the appearance of the Blocks are identical, but the use of such stereotypes allows different kinds of objects to be clearly identified.

Fig  C-5 also adopts another other useful feature, using correspondence to show the relationship between the separate <<System>>, called the Integrated Network Operations Center (INOC), which hosts the Service Management <<Subsystem>> and the  Network Control <<Subsystem>> allocated at each <<Site>>.

# C7  DESCRIBE the SYSTEM INTERFACES

Where such details are needed, various kinds of interfaces may also be modeling in SysML.  The following figures are borrowed from INCOSE 2016 paper titled “A Representative Application of a Layered Interface Modeling Pattern” [27] that provides a consistent set of examples for how to model the interfaces of systems components (both hardware and software), the protocols and their behavior, and the information objects that are passed from one system element to another.  This set of figures are different in style, and they do not explicitly define either a profile nor a set of color codes, but do name the different component types.

The value of these diagrams is that they describe how to model the details of the interfaces identified in Fig  C-3 and the data flows and data objects shown in Fig C-4, as well as protocol entity interfaces and behaviors.

Figure C-6: Example Connectivity View context for Interface Modeling

Figure C-6 is a top level RASDS Connectivity View showing a set of major <<System>> elements as well as <<External>> physical elements like a Thermal Source and the Atmosphere that RF signals radiate through.  This is the base model for the following set of diagrams, and it roughly corresponds to the top level Connectivity View shown Fig C-3.    The following subsections will dive down into the technical details of how to formally model these interfaces, protocols, and data objects.

# C8  DESCRIBE the COMPONENT INTERFACES USING A COMMUNICATIONS VIEW

In Fig C-7 a RASDS Communication View describes, in more detail, the protocol stacks and interfaces by which two of the  <<Subsystem>> elements introduced in Fig C-6 actually communicate.  This level of detail is not always needed in high level systems architectures, but for subsystem level systems architecture descriptions it is usually required.

Figure C-7: Example Communications View for Component Interface Modeling

What is shown in Fig C-7 are two of the <<Subsystem>> from Fig C-6, the Avionics Subsystem and the Telecom Subsystem.    Note that these components  are referenced by correspondence, and are included to provide context, because the real focus in this Communications View diagram are the protocol details  describing the interfaces of these two components.

Two levels of detail of these <<Component>> interfaces are shown, both the software based “Packet Port” on the two C&DH SW and Communications SW components and the lowest level hardware based “Ethernet Port” on the edge of each of the two <<Component>>.  These ports map to ISO BRM layers 7 (application packets), 4 (TCP), 3 (IP), 2 (1 Gb Ethernet), and 1 (RJ-45 plug).  The top three layers are software, the bottom two are hardware, and these hardware ports are literally shown at the edge of the Computer and Transceiver <<Component>>.

The <<Protocol Entity>> elements themselves each show the interfaces that are identified in RASDS Communication Views for Protocol elements, named as provided, required, and peer protocol interfaces.  Drilling down further, Figure C-8 shows an abstract representation of a <<Protocol Entity>>, showing all three interfaces of the <<Protocol Entity>> and an abstract view of the behavior of one of these entities that implements a specific protocol layer, the Transport Control Protocol (TCP [49]) in this case.

Figure C-8: Abstract Model of a Protocol Entity and Interfaces

Every Protocol Entity has similar features to those shown in Fig C-8.  A typical protocol standard will formally define the PDUs that get exchanged across the peer-to-peer interfaces and also the behavior of the protocol, which is abstractly shown inside the <<Protocol Entity>>.  In many protocol standards the Required and Provided interfaces will also be documented formally, but this is not always done.

A formal model of the behavior inside this Protocol Entity is shown in Fig C-9.  In this example the behavior of this Communications Viewpoint element is shown using a SysML State Chart.  This example shows the internal connection establishment behavior of the TCP protocol that makes up ISO layer 4 of the Component interface shown in Fig C-7.

Figure C-9: State Machine Model of TCP Protocol Connection Establishment Behavior

# C9  Describe the Data Objects in An Information View

All the Protocol Data Units (PDU) that are exchanged between Protocol Entities may be modelled as RASDS++ Information Objects.    These must be defined in a model as well, along with any other data objects that are exchanged at the application layer.  Fig C-10 shows a method for modeling these Information Objects.

Figure C-10: Model of Abstract and Realized Information Objects

Figure C-10 provides examples for modeling Information Objects from two different points of view, both abstract (bit, octet, and SPP Primary Header and Packet Data Field structures) and realized (Thermal Packet as a realized instance of a Space Packet).  Of course, once the abstract Packet structures are defined all sorts of models of other realized data packets may easily be defined.

# C10  Providing RASDS++ VIEWpoints IN SysML

The main focus in this Annex has been on providing examples of how SysML has been used to do formalized modeling of RASDS++ compliant systems architectures.  There are significant advantages to using formal modeling tools in that they provide automated means for creating and managing models, model elements, and model prototypes.  The tools will automatically maintain consistency where they can.  But, as should be clear by now, these tools do not provide all of the needed model elements, nor viewpoints and views, that are needed to do systems architecture modeling.  SysML defines the notions of Viewpoint and View, but does not provide a useful “starter set” of Viewpoints.

RASDS ++ provides a useful set of Viewpoints and Views, but work must be done to create the necessary conceptual framework within a SysML tool to support these kinds of models.  This section has provided some examples of how that has been done, but no pre-built “package” is available to provide a set of building blocks.  It is strongly recommended that any project that wishes to use SysML take some prior steps before starting to model a system:

- Identify the sets of Viewpoints and Views that will be needed for the model and establish the specific SysML diagram types to be used, along with stereotyped objects and formalized terms for each Viewpoint.

- Make the initial determinations of just what the scope is of the modeling effort and determine if Viewpoints will include physical as well as logical decomposition, if deployment locations are important as well as structures, and document how you are going to represent these distinctions.

- Create a Profile for the project, as in Fig C-1, that defines the formal names for the decomposition hierarchy and, where useful, the set of color codes that will help make the diagrams more meaningful.

- If you are working at a Systems-of-Systems level, or doing Trade Space studies, consider how you are going to partition these different aspects in the model and build a model decomposition framework that will support exploration of these different aspects.

As noted in Annex B there are a number of RASDS++ Viewpoints where existing SysML  drawing styles can be adapted and configured to provide suitable templates.  Activity diagrams are equally applicable to Operations Views and Functional Views.  State Charts may be applied to describe Functional behavior or Protocol behavior.  Various kinds of composition diagrams can be used to describe systems elements, whether they are software, or communications hardware, or structural parts.

All of the modeling and examples have been done in SysML V1 because that is version of SysML that is, at the time of publication, the most widely available   language with tool support. It is worth noting that OMG Systems Modeling Language ™ (SysML®) SysML V2  [47] is  now in development and that tools that support it are becoming available.  SysML V2 will support all of the features of V1, and much more.  As the current draft states:

“It provides the capability to create and visualize models that represent many different aspects of a system. This includes representing the requirements, structure, and behavior of the system, and the specification of analysis cases and verification cases used to analyze and verify the system. The language is intended to support multiple systems engineering methods and practices. The specific methods and practices may impose additional constraints on how the language is used. “

Some studies are now underway to re-create the interface modeling patterns [27] introduced in this section using SysML V2.  What is clear is that this newer modeling framework has all of the expressive power of SysML V1, and the advantage that it brings completely idempotent approaches for communicating the semantic, syntactic, and visual aspects of models, something that SysML V1 interchange formats were not capable of.

SysML V2 does support Viewpoint and View constructs, but, similar to SysML V1, does not provide anything like a standard set of these.  As a result, the same sorts of adaptations that were just described will also be required of a SysML V2 modeling tool and framework in order to support RASDS++ systems architecture modeling.  SysML V2 does include a concept documented as a “Domain Library”, which appears as though it will provide direct support for the Physical Viewpoint in the form of a “Geometry Domain Library“.  Other domain libraries can be defined, and this appears to be the “hook” upon which a set of RASDS++ Domain Libraries might be hung.

Architecture, Models, Meta-models, and Frameworks

Introduction

Most of the published systems engineering documents that have been developed by different agencies use the term “systems architecture” without ever really defining what one is nor what a good one might look like.  An example of this can be seen in the latest NASA Systems Engineering Handbook [33]:

“The system architecture can be seen as the strategic organization of the functional elements of the system, laid out to enable the roles, relationships, dependencies, and interfaces between elements to be clearly defined and understood. It is strategic in its focus on the overarching structure of the system and how its elements fit together to contribute to the whole, instead of on the particular workings of the elements themselves. It enables the elements to be developed separately from each other while ensuring that they work together effectively to achieve the top-level (or parent) requirements.

Much like the other elements of functional decomposition, the development of a good system-level architecture is a creative, recursive, collaborative, and iterative process that combines an excellent understanding of the project’s end objectives and constraints with an equally good knowledge of various potential technical means of delivering the end products. “

But at that point the reader is left hanging, because the document contains no well defined “system architecture” methodology nor modeling process.

Logical and data architecture models are an intended outcome of these SE processes, but there is no stated process or conformance criteria for such a set of products.  By contrast, The Open Group Architecture Framework (TOGAF) [12], shows several stages of architecture development (Vision, Business, Information Systems, and Technology architectures) all feeding into a central Requirements Management process and then the design and development processes.

This document provides both a systems architecture methodology and an abstract model that is grounded in best current practices in the field.  The fundamental motivation for such an approach is well stated in this quote:

"Modeling, in the broadest sense, is the cost-effective use of something in place of something else for some cognitive purpose. It allows us to use something that is simpler, safer, or cheaper than reality instead of reality for some purpose.

A model represents reality for the given purpose; the model is an abstraction of reality in the sense that it cannot represent all aspects of reality. This allows us to deal with the world in a simplified manner, avoiding the complexity, danger and irreversibility of reality."

Rothenberg, Jeff. “The Nature of Modeling”, RAND, https://www.rand.org/content/dam/rand/pubs/notes/2007/N3027.pdf

RASDS++ and ISO 42010

ISO has published an abstract systems architecture method, ISO 42010-2022 [15], Software, systems and enterprise — Architecture description, that is based on the earlier IEEE-P1471-2000, the Recommended Practice for Architectural Description of Software Intensive Systems [2]. ISO 42010 is a “meta-architecture” document that describes “core terms, definitions and relationships for the Architecture Description”, describes best practices for defining software architecture descriptions, and defines the meaning of Viewpoint and View.

RASDS++ utilizes concepts and terms defined in ISO 42010.  The fundamental concepts defined in ISO 42010 are captured in Fig C-1 [30].

Figure D-1: ISO 42010 Fundamental Concepts

Copyright 1998-2016 by Rich Hilliard, http://www.iso-architecture.org/42010/cm, used with permission

The objects highlighted in red and green are the ones that are focused on the most in RASDS++, which uses these meta-model concepts to create a Reference architecture with a specific set of Viewpoints and Views suitable for describing the architectures of space systems.  Figure C-2 provides a view of how these different “meta model” levels relate, and how they relate to a specific project model that uses this methodology.

Figure D-2: RASDS++ Related to Meta-Model Levels

Copyright 1998-2016 by Rich Hilliard, http://www.iso-architecture.org/ieee-1471/meta/, used with permission

As can now be understood, the RASDS++ meta-model (M2) defines a set of viewpoint specifications and a set of objects and representations.  The RASDS++ meta-model leverages the ISO 42010 (M3 & M2) definitions for all these fundamental viewpoint/view terms and provides specific guidance on viewpoint specifications suitable for the space system domain.

A user of RASDS++ defines those parts of the Architecture Description Framework (M1) that they are going to use to create their model.  The users of this adapted M1 model framework can then use the defined views to model their specific system (M0).  A recent paper, Toward Systems Engineering Meta-Methodology [32], explores in some depth the current work on modeling meta-methodology, of which this work is an example.

Viewpoints and viewpoint specifications

Space systems are complex entities that have many different aspects and it is usually impossible to depict these all of aspects with a single view or in a single framework.  Therefore, the architecture of a space system must often be described from multiple Viewpoints, each focusing on different concerns associated with the system. The RASDS++ reference architecture defines a carefully selected set of Viewpoint Specifications to describe and analyze architectures of space systems.

The notion of describing complex space systems from a variety of viewpoints is not a new one, as Fig C-3 from the Voyager Project [29] makes clear.  What has changed over time is the formalisms and methods, like ISO 42010 and RASDS++, that are available to describe these complex architectures and help inform and structure the architecting process.

Figure D-3: 	Different Viewpoints of a space system

From JPL Pub 89-24, “The Voyager Neptune Travel Guide”, Charles Kohlhase editor, June 1, 1989 (artist Phil Gwinn)

A Viewpoint Specification is an abstraction that uses a selected set of architectural concepts and structuring rules to focus on particular concerns within a space system. Each Viewpoint documents a different set of design concerns and issues, and each provides the means for reasoning about those aspects of the system.  Each Viewpoint is intended to be orthogonal to the others, but a specific modelling method has been defined to allow related elements appearing in different Viewpoints to be connected.  This is called correspondence, and it is the topic highlighted in green in Fig C-1.

Each Viewpoint Specification defines the methods for describing a space system as a set of Objects and the interactions among them.  An Object is an abstract model of an entity in the system.  Objects have type, behavior and state and are distinct from other types of objects.  Objects are defined in their primary Viewpoint, but may have corresponding objects that appear in other Viewpoints.  There are explicit Correspondence relationships that describe this. A Viewpoint Specification defines the rules for constructing Views of the system.

Viewpoint Specifications are described in terms of the objects that may appear in them, their attributes, and the relationships among them.  An object is a representation of an entity in the real world. It contains information, may exhibit behavior, and may offer services. A system is composed of interacting objects. An object is characterized by whatever distinguishes it from other objects and by specialization, encapsulation, abstraction, and behavior. Encapsulation is the property that the information contained in an object is accessible only through interactions at the interfaces supported by the object. Because internal objects are encapsulated, there are no hidden side effects of interactions. That is, an interaction with one object cannot affect the state of another object without some defined secondary interaction taking place with that object. Thus, any change in the state of an object can only occur as a result of an external request of an object, an internal action of the object, or an interaction of the object with its environment.

GLOSSARY AND ACRONYMS

Acronyms

2FA	Two (2) Factor Authentication

3GPP	3rd Generation Partnership Project

ADF	Architecture Description Framework

AMS	Asynchronous Message Service

ANSI	American National Standards Institute

AOS	Advanced Orbiting Systems

API	Application Program Interface

ASL	Application and Support Layer (architecture)

ATO	Authorization To Operate

BP	Bundle Protocol (DTN)

BPSec	Bundle Protocol Security

BWEM	Band Width Efficient Modulation

C&DH	Command and Data Handling

CCITT	Consultative Committee for International Telephony and Telegraphy

CCSDS	Consultative Committee for Space Data Systems

CDM	Continuous Diagnostics and Mitigation

CEOS	Committee on Earth Observation Satellites

CFDP	CCSDS File Delivery Protocol

CPU	Central Processing Unit

DBMS	Data Base Management System

DDOS	Distributed Denial of Service

DEDSL	Data Entity Dictionary Specification Language

DNS	Domain Name System

DoD	(U.S.) Department of Defense

DoDAF	DoD Architecture Framework

DoT	Dictionary of Terms

DSN	Deep Space Network

DTN	Delay (& Disruption) Tolerant Network

EMI	Electromagnetic Interference

ESLT	Earth - Space Link Terminal

F-CLTU	Forward Control Link Transmission Unit

FEC	Forward Error Correction

FPGA	Field Programmable Gate Array

FTP	File Transfer Protocol

GTN	Ground Tracking Network

H/W	Hardware

HTTP	HyperText Transport Protocol

HTTPS	HyperText Transport Protocol Secure

HVAC	Heating, Ventilation, and Air Conditioning

ICAM	Identity, Credential, and Access Management

ICD	Interface Control Document

IDS	Intrusion Detection System

IEC	International Electrotechnical Committee

IEEE	Institute of Electrical and Electronic Engineers

IPSec	IP Security (protocol)

ISO	International Standards Organization

ISO-BRM	ISO/IEC Basic Reference Model

ISP	Internet Service Provider

JTC	Joint Technical Committee (of ISO)

LAN	Local Area Network

MBSE	Model Based Systems Engineering

MDSD	Model Driven System Design

MIB	Management Information Base

MOF	Meta Object Facility

MOS	Mission Operations System

MRO	Mars Relay Orbiter

MTS	Message Transfer Service

OAUTH 	Open Authorization

ODP	Open Distributed Processing

OMG	Object Management Group

OS	Operating System

OSCP	Overview of Space Communication Protocols

OV	Operational View (of DoDAF)

OWL	Ontology Web Language

PDU	Protocol Data Unit

PKI	Public Key Infrastructure

RASDS	Reference Architecture for Space Data Systems

RASIM	Reference Architecture for Space Information Systems

RCF	Return (Virtual) Channel Frames

REST	Representational State Transfer

RF	Radio Frequency

RFC	Request for Comment (Internet standards)

RM-ODP	Reference Model-Open Distributed Processing

RMP	Registry Management Policy

ROM	Read Only Memory

RTLT	Round Trip Light Time

SaaS	Software as a Service

SAML	Security Assertion Markup Language

SANA	Space Assigned Numbers Authority

SAP	Service Access Point

SAWG	System Architecture Working Group

SBU	Sensitive But Unclassified

SC	Subcommittee (of ISO)

SCCS	Space Communication Cross Support

SCCS-ARD	Space Communication Cross Support – Architecture Requirements Document

SDLS	Space Data Link Security

SDU	Service Data Unit

SIEM	Security Information and Event Management

SLE	Space Link Extension

SOA	Service Oriented Architecture

SOE	Sequence of Events

SOP	Standard Operating Procedure

SOS	System of Systems

SPP	Space Packet Protocol

SSI	Solar System Internetwork

SV	System View (of DoDAF)

S/W	Software

SysML	System Modeling Language

TC	Telecommand

TCP	Transmission Control Protocol

TOGAF	The Open Group Architecture Framework

UAF	Unified Architecture Framework

UI	User Interface

UML	Unified Modeling Language

UPMS	UML Profile and Metamodel for Services

USLP	Unified Space Link Protocol

VPN	Virtual Private Network

W3C	World Wide Web Consortium

WAN	Wide Area Network

WG	Working Group

WiFi	wireless fidelity, trademarked by IEEE 802.11x

WSDL	Web Services Description Language

Terms

Abstraction—A mechanism and practice to reduce and factor out details so that one can focus on few concepts at a time.  It is the process of extracting the underlying essence of a concept, removing any dependence on real world objects with which it might originally have been connected, and generalizing it so that it has wider applications.  3.2.2, 5.3.2.2]

Abstract Data Architecture Meta-Models—Models for Specification and Standardization of Data Elements (e.g., ISO/IEC 11179, DEDSL). [10.4.2]

Action—Something that happens within an object, either with or without participation of another object.  An interaction is an action performed with participation of another object. [3.2.3, 12.2, 12.4.2]

Activity—A specification of behavior described as a sequence of actions. [3.2.3, 12.2, 12.2.4]

Aggregation—Several things grouped together or considered as a whole: the act of gathering things together. [3.2.4]

Allocation—A mapping between one set of model elements and another. The mapping is often performed as part of the design process to refine the design. Typical examples of allocation include allocation of functions to nodes, logical to physical components, logical to physical links, and software to hardware. [7.4.2]

Application—one or more pieces of software designed to perform some specific function; it is a configuration of interacting implemented software Engineering Objects. [7.4.2]

Application Programming Interface—A set of definitions of the ways one piece of computer software communicates with another. It is a method of achieving abstraction, usually (but not necessarily) between lower-level and higher-level software. [9.5.2]

Architecting—The process of defining, documenting, maintaining, improving, and certifying proper implementation of an architecture.  It is both a science and an art.  [3.2.5]

Architecture—The concepts and rules that define the structure, semantic behavior, and relationships among the parts of a system; a plan of something to be constructed.  It includes the elements (entities) that make up the thing, the relationships among the elements, the constraints that affect those relationships, a focus on the parts of the thing, and a focus on the thing as a whole. [3.2.5]

An Architecture Description is a work product used to express an Architecture.  It may contain stakeholders, concerns, viewpoint specifications, viewpoints, views, correspondences, representations, and other elements. [3.2.5]

Artifact—Any tangible objects made, modified, or used by people, or produced during system design, development, testing or operations. [10.4.2]

Aspects capture a set of characteristics or features of the entity of interest in its environment to address concerns within an Architecture Description. [3.2.5]

An Asset is a person, data, or other resource that is valued by an organization. [4.3.3.2]

Attribute—A characteristic of an object; a language construct that system designers use to add additional information to system elements (e.g., objects, modules, types) to define their functionality.  [3.2.3]

Behavior—A set of actions performed by an object for some purpose. [3.2.3, 11.4.1]

A Capability is the ability to achieve a desired outcome under specified conditions using a combination of activities and resources to satisfy a stakeholder need. [4.3.3.1]

Center of mass-- The location of the balance point of a Structural object in the coordinate system of the object. [8.3.3.1]

Communications Viewpoint—An engineering and technology view on a space system that focuses on the protocols and mechanisms of information transfer performed by that system. [7.2]

Community—An entity (e.g., Earth Science) that may exist within one Space Enterprise or across multiple Space Enterprises. It is distinguished by being bound by common objectives and relationships and offers a set of resources that are sharable within the Community and with other Communities. [4.3.3.2]

A Component is a physical entity operating in a physical environment.  A Component is a configuration of engineering objects forming a single unit for the purpose of location in space, and embodying a set of functions.  A Component has some well-understood, possibly rapidly moving, location, and it may be composed of two or more (sub)components. [6.3.2.2]

Composite Object—An Object composed of two or more Objects via aggregation.  The behaviors of the composite object are determined by those of the Objects that it aggregates. [3.2.4]

Composition—A form of aggregation.  Composition may be recursive. [3.2.4]

Concerns—Those interests which pertain to the system’s development, its operation, or any other aspects that are critical or otherwise important to one or more stakeholders. Concerns include system considerations such as performance, reliability, security, distribution, and evolvability. [3.2.5]

Configuration—A collection of objects able to interact at interfaces. A configuration determines the set of objects involved in each interaction along with constraints on their interactions. [3.2.3, 6.3.2.1, 7.4.1]

Connections-- The types of connections and the sub-components that they connect.  Thermal conductivity, wiring harness, etc. [6.3.2.1]

Connectivity Viewpoint—An engineering viewpoint on a space system that focuses on the Node and Link view of a system, the physical connections among Nodes, their physical and environmental constraints, physical dynamics, and (optionally) the allocation of implemented functions to Nodes. [7.2]

A Connector is a thing which links two or more things together.  A Connector may be rigid, flexible, hinged, rotational, articulated or simply energetic. Connectors connect Components at a port. [6.3.2.2]

Constraint—A limitation or implied requirement that limits the design solution or implementation, is not changeable by the enterprise, and is generally non-allocable.  [3.2.3, 7.4.1, 12.4.1,  6.3.2.1]

A contract is an agreement that specifies certain legally enforceable rights and obligations pertaining to two or more parties. A contract typically involves the transfer of goods, services, money, or a promise to transfer any of those at a future date. [4.3.3.1]

Correspondence—A function such that for elements in one viewpoint there is a related element in another viewpoint; the condition of being in conformity from elements in one viewpoint to elements in another. [3.2.5]

Cross-Support —An agreement between two or more organizations to exploit the technical capability of interoperability for mutual advantage, such as one organization offering support services to another in order to enhance or enable some aspect of a space mission.  [4.2]

Data is a representation of facts, concepts, or instructions in a manner suitable for communication, interpretation, or processing by humans or by automatic means. [10.4.1]

Data Architecture—Models of the structure and relationships among the data elements used within a system. [10.4.2]

Data element is a basic unit of information that has a unique meaning and subcategories (data items) of distinct value.  Examples of data elements include gender, race, and geographic location. [10.4.1]

Data Model—Schema and Structure Definitions. [10.4.2]

Data Objects—Information objects, either physical or digital. [10.4.2]

A data structure is a data organization, management, and storage format that is usually chosen for efficient access to data. [10.4.1]

Data semantics is information that defines the meaning rather than the physical representation of data.  Semantics potentially cover a very large domain, from the simple domain, such as the units of one data entity, to a more complex one, such as the relationship between a data entity and another.[10.4.1]

Domain—A Community that is under single organizational, administrative, or technical control (e.g., NASA Space Operations Mission Directorate). A domain may have resources, policies, access control, and possibly quality of service constraints. A Domain may be subdivided into Subdomains. Multiple Domains may be collected into a Federation. [4.3.3.2]

Element— An Element is a constituent part of something; any thing that is one of the individual parts of which a composite entity is made up; an identifiable component, process, or entity of a system. [3.2.2]

Engineering Object—An implementation or realization of some abstract function.  It may be implemented as hardware (Node) or as software (application or software component). [6.3.2.1]

Enterprise Object—An organizational entity that is governed by a single authority that has its own objectives and policies to operate the object.  An Enterprise Object may be a component of another larger Enterprise Object.  Enterprise Objects may participate wholly or in part in other Enterprise Objects.  [4.2]

Enterprise Viewpoint—A view of a space system that focuses on the community, purpose, scope, and policies for that system.  This viewpoint includes organizations as well as the Enterprise Objects that have assigned roles, responsibilities, and interactions. [4.2]

Entity—Any concrete or abstract thing of interest. For example, an entity may be a physical instrument, a computer, a piece of software, or a set of functions performed by a system. While in general the word entity can be used to refer to anything, in the context of modelling it is reserved to refer to things in the universe of discourse being modelled.  [3.2.2]

Environment—A complex of external factors that acts on a system and determines its course and form of existence. An environment may be thought of as a superset, of which the given system is a subset. An environment may have one or more parameters, physical or otherwise.  The environment of some system or object consists of the substances, circumstances, objects, or conditions by which it is surrounded or in which it occurs. [3.2.4]

Event--:Any observable system or natural occurrence.  The fundamental entity of observed physical reality represented by a point designated by three coordinates of place and one of time in the space-time continuum postulated by the theory of relativity. [12.4.2]

Facility—A physical infrastructure element that supports the use of services and other resources. [4.2]

Federation—A Community consisting of multiple Domains (e.g., CEOS or CCSDS) that come together to share resources while retaining their autonomy over those resources. Federations are bound by negotiated agreements. A Federation may include only some members of a Domain or Subdomain (e.g., a particular Earth Observing project).  Members of a Federation agree to rules for sharing resources and for joining and/or leaving the Federation. [4.3.3.2]

Firmware—software that is contained in a read-only memory (ROM) device.  We typically treat it as software unless there is a reason for showing the hardware component itself. [6.3.2.1]

Flows—may represent movement of data or of substance or energy. Flows of data are shown using named data objects.  The formal definitions will be found in Information views.  Flows of matter or energy may appear in the Structural Viewpoint. [8.3.3.2, 12.4.1]

Function—The set of actions or activities performed by some object to achieve a goal.  The transformation of inputs to outputs that may include the creation, modification, monitoring, or destruction of elements. [3.2.4]

Functional Object—An object that performs Functions to achieve a goal of a space system, or to support actions of another Functional Object and to transfer, generate, or process data in performing those actions.  [5.2]

Functional Viewpoint—A view on a space system that focuses on the structure of the functions performed by that system and their behavior and on the interactions among the functions.  This includes functional objects, the logical connections between them, their interactions, and logical interfaces. [2.2.1, 5.2]

Goal—An aim or purpose; the end toward which effort is directed.  Goals tend to be broad or abstract and to state general intentions. [3.2.4]

Hardware—the mechanical, magnetic, electrical and electronic devices or components of a system used for producing, collecting, processing, storing or transporting data.  [6.3.2.1]

Inertia matrix-- The moments of inertia of a Structural object arranged in an array. [8.3.3.1]

Information is any communication or representation of knowledge such as facts, data, or opinions in any medium or form, including textual, numerical, graphic, cartographic, narrative, or audiovisual. [10.2]

Information Management Functional Objects—Active functional elements that support the location, access, delivery, and management of passive Information Objects.  These Information Management Functional Objects are a class of Functional Objects. [5.3.2.2]

Information Object—Data, along with the necessary structure and syntax to allow interpretation and use of that data; may also have associated metadata, including the relationships among Data Objects, rules for their use and transformation, and policies on access.  [10.2]

Information Package—A primary Information Object, optional ancillary information,  and associated supporting information that is needed to use the Information Object.  The Information Package has associated Packaging Information used to delimit and identify the primary Information Object and Supporting Information. [10.2]

Information Viewpoint—A view of a space system that focuses on the information used by that system.  This includes structural (syntactic) and semantic views of the information, the relationships among information elements, and rules for their management and transformation. [10.2]

Instantiation—Creation of an instance of some abstract element, achieved by an action of an object in the model. Elements can be anything that can be instantiated, in particular objects and interfaces.  Data Models must be instantiated as real information objects in order to participate in system activities. [10.4.2]

Interaction—An action performed by an object with participation of another object or with its environment. [3.2.3, 12.2]

Interaction modes --used among Operations elements.  These may include request & immediate response, or request with immediate acknowledgement (and later response), or even a request followed some time after with a response or acknowledgement. [12.4.1]

Interface—A set of interactions performed by an object for participation with another object for some purpose, along with constraints on how they can occur. An interface is therefore where the behavior of an object is exposed.  Objects may have one or more interfaces. [3.2.3]

Interoperability —The technical capability of two or more systems or components to exchange information and to use the information that has been exchanged. Multiple degrees of interoperability are possible, ranging from basic Physical layer (e.g., frequency, modulation, and coding) compatibility up to full Application layer information exchange. [3.2.3]

Knowledge --is facts, information, and skills acquired by a person through experience or education; the theoretical or practical understanding of a subject. [10.2]

Knowledge Model --is a representation of knowledge in a form that can be interpreted by both humans and machines and is used in knowledge-based systems. [10.4.2]

Link—The locus of relations among Nodes. It provides interconnections between Nodes for communication and coordination.  It may be implemented by a wired connection or with some RF or optical communications media.  It may periodically become inactive because of the motion of a Node or the lack of availability of communications resources, for example.  Links connect Nodes at a Port. [7.4.2]

Location—A point or extent in space. [3.2.4, 8.3.3.2]

Logical Link—The locus of relations among Logical Objects.  It may be considered separately from any particular implementation or deployment and has no physical manifestation except as part of a model. [3.2.4]

Logical Object—An abstract entity that may be considered separately from any particular implementation or deployment.  It has no physical manifestation except as part of a model, but it may have associated behaviors and interfaces. [3.2.4]

Mass-- The inertial property of a Structural object. [8.3.3.1]

Mass Properties-- Center of mass, inertia matrix [6.3.2.1]

Metadata—‘data about data’; the information that describes content. It is information about the meaning of data, as well as the relationships among Data Objects, rules for their use and transformation, and policies on access. [10.2]

Meta-model—An explicit model of the constructs and rules needed to build specific models within a domain of interest. [10.4.2]

Mission Operations Service --is a suite of end-to-end application-level services that constitute a Service Oriented Architecture for space mission operations. [11.4.2]

Model—A formal specification of the structure and/or function of a system.  All models are abstractions; abstraction is the suppression of irrelevant detail.  [3.2.5]

(N)-layer—Any specific layer in a multi-layer protocol stack.  The layer above is called the (N+1)-layer, the layer below is called the (N-1)-layer. This notation is also used for other concepts in the model which are related to these layers, for example (N)-protocol, (N+1)-service. [9.5.2]

Network address --is an identifier for a node or host on a telecommunications network. Network addresses are designed to be unique identifiers across the network, although some networks allow for local, private addresses, or locally administered addresses that may not be unique. Special network addresses are allocated as broadcast or multicast addresses. These too are not unique. [7.4.1]

Node—A model of a space system physical entity operating in a physical environment.  A Node is a configuration of engineering objects forming a single unit for the purpose of location in space, and embodying a set of processing, storage, and communication functions. A Node has some well-understood, possibly rapidly moving, location, and it may be composed of two or more (sub)Nodes. [7.4.1]

Object—An abstract model of an entity in the real world, containing information, having behavior, and offering services. A system is composed of interacting objects. An object is characterized by that which makes it distinct from other objects. [3.2.2]

Objective—Something to be done or achieved.  Objectives tend to be precise, tangible, and concrete. [3.2.4]

Ontology --encompasses a representation, formal naming, and definitions of the categories, properties, and relations between the concepts, data, or entities that pertain to one, many, or all domains of discourse. More simply, an ontology is a way of showing the properties of a subject area and how they are related, by defining a set of concepts and categories that represent the subject. [10.2]

Operations Concept—a verbal or graphic statement, in broad outline, of assumptions or intent regarding the operation of the system. The concept of operations frequently is embodied in observing plans and operations plans.  The concept is designed to give an overall picture of the operation of the system. [4.3.3.2]

Organization—A formal group of people with one or more shared goals. [4.3.3.2]

Orientation--  The rotation of a Structural object from alignment of its coordinate system with the coordinate system of the assembly that contains the object. [8.3.3.2]

Ownership—Having administrative and fiscal responsibility for the owned element and the right to exclusively control and use that which is owned for one’s own purposes.   The state or fact of having exclusive possession or control of some object, facility, intellectual property, or some other kind of property.  [4.2]

Perspective—In systems architecture, the choice of a context or a reference (or the result of this choice) from which to describe, categorize, explain, or codify system design, typically for comparing with another.  [3.2.5]

Physical Environment-- Limits on environmental properties, such as pressure, ambient temperature, etc. [6.3.2.1]

Plan-- Captures the (acceptable) balance of risk vs. result in generating an operational product (e.g.: nominal or off-nominal plans). [12.4.2]

Policy—A set of guidelines and constraints on the behaviors exhibited by the objects in the system. [3.2.5]

Port—The physical element of a Node where a Link is connected.  Nodes may have one or more Ports.  [7.4.1]

Process-- What needs to happen in the plan to generate an operational product. [12.4.2]

Protocol—A set of rules and formats (semantic and syntactic) used to determine the communication behavior of (N-layer)-protocol-entities in the performance of (N-layer)-functions; the description of the state machines within a Protocol Entity and the PDUs that are exchanged between these entities. [9.5.2]

Protocol Data Unit (PDU)—A unit of data specified in an (N-layer)-protocol, consisting of (N-layer)-protocol-control-information and possibly (N-layer)-user-data; the actual data objects that are exchanged between peer protocol entities. [9.5.2]

Protocol Entity—An active element within an (N-layer)-communications-subsystem embodying a set of capabilities defined for the (N-layer)-layer that corresponds to a specific (N-layer)-entity-type (without any extra capabilities being used).  Protocol Entities implement protocols. [9.5.2]

Provenance—Documentation of the place of origin, proof of authenticity or record of previous processing. These are valuable pieces of information in the history of an object. [10.4.2]

Realization—The act or the condition of becoming real.  Abstract data architecture elements must be realized as Data Models and stored in some sort of repository. [10.4.2]

Relationship—The way in which two or more entities can be associated with one another. [3.2.3, 10.4.1]

Representation—Some way of organizing, manipulating, presenting, and storing information; a visual or tangible rendering of something. [3.2.5]

Requirement—A formal statement of: (1) An attribute to be possessed by the element or a function to be performed by the element. (2) the performance standard for the attribute or function. (3) the measuring process to be used in verifying that the standard has been met. [3.2.5, 4.3.3.1]

Resource— Anything available to a system that can support the achievement of objectives; any physical or virtual element that may be of limited availability within a system. A resource may be shared by more than one activity.  In the Enterprise Viewpoint a resource is an entity that has some role, offers services, and performs some action within a system. [3.2.4, 4.2]

Risk Management-- is the program and supporting processes to manage information security risk to organizational operations (including mission, functions, image, reputation), organizational assets, individuals, other organizations, and the Nation, and includes: (i) establishing the context for risk-related activities; (ii) assessing risk; (iii) responding to risk once determined; and (iv) monitoring risk over time. [4.3.3.1]

Role—The way in which an entity participates in a relationship; an object’s set of behaviors and actions associated with the relationship of that object with other objects. [3.2.3]

Scenario—A specific sequence of activities illustrating behaviors.  A scenario may be used to illustrate an interaction or an operations concept instance. [4.3.3.2]

Schedule-- Captures temporal context for Plan.  (e.g.: keeps track of SoE <anomaly vs. “as predicted/expected”>). [12.4.2]

Schema—An information model defined in a document or a database. The universe of objects that can be described is defined in the schema. For each object class, the schema defines what attributes an instance of the class must have, what additional attributes it may have, and what object class can be a parent of the current object base. [10.4.2]

Semantics—Rules by which syntactic expressions are assigned meaning. [3.2.3]

Sequence of Events (SOE)--  a number of events or activities that come one after another in a particular order.  the predicted order of events during spacecraft operations, and also the observed order of events during spacecraft operations. [12.4.2]

Service—A provision of an interface of an object to support actions of another object. [3.2.3, 11.4.2]

Service Access Point—The point at which (N-layer)-services are provided by an (N-layer)-protocol-entity to an (N+l-layer)-protocol-entity.  [9.5.2]

Service Data --is a generic term for the kinds of data provided by a specific service.  There are many different kinds of service data, which may be a discrete data object, a stream of data objects (that could be turned into audio or video), or other forms such as a file or a message. [11.4.1]

Service Interface --is a mechanism to enable access to a set of one or more functions of an Element, where the access is provided using a prescribed interface and is exercised consistent with constraints and policies as specified by the service description. [3.2.3]

Service Provider --is the role played by a physical, functional, or organizational entity that provides a cross support (or other) service for a service user. [11.4.2]

Service System --is the set of Hardware and Software Components used to implement a Service in a real system. Service Systems may be implemented using one or more Hardware and Software Components. [11.4.2]

Service Type --may be any one of: network service, link layer service, cross support service, mission operations service, web service, name service, or any of many other types of defined services. [11.4.1]

Service User --is the role played by a physical, functional, or organizational entity that uses a cross support (or other) service provided by a service provider. [11.4.2]

Software or computer programs—the components of information systems that provide operating instructions for specific task based applications that run on computing hardware.  [6.3.2.1]

Space Enterprise—A top-level autonomous entity (e.g., NASA) that is dedicated to the exploration and/or exploitation of space. It has its own objectives, resources, and policies, and it is not a component of any other Space Enterprise. [4.3.3.2]

Space Environment --creates conditions in space that affect the design and operation of spacecraft. Effects on spacecraft can arise from temperature extremes, radiation, space debris and meteoroid impact, upper atmospheric drag, spacecraft electrostatic charging, and gravity. [7.4.1]

Space Link Extension (SLE) Service-- is the set of services that extend one of the CCSDS Space Link Subnetwork services, providing access to the ground termination of that service from a remote ground-based system.  An SLE service supplies or consumes one or more channels of the same Space Data Channel type. [11.4.2]

Spacecraft--  includes spacecraft that travel in space, rovers, habitats, and other element in space or on a remote planetary surface. [6.3.2.2]

Specification— A set of requirements or other descriptive information for a system or classifier.  [3.2.5]

Stakeholder—An individual, team, or organization (or classes thereof) with interests in, or concerns relative to, a system. [3.2.5]

Standard—A formal specification that defines and governs functions and protocols at interfaces of a data system.  It describes in detail the technical capabilities of, and establishes the requirements to be met by, interfacing subsystems to achieve compatibility and interoperability. [3.2.5]

State—A condition or situation during the life of some object; at a given instant in time, the condition of an object that determines the set of all sequences of actions in which the object can take part.  [9.5.2, 12.4.1]

State Machine—The description of the discrete sequence of states that an object or interaction goes through during its life in response to events, together with its responses and actions. [9.5.2]

State Table --is an alternative tabular representation of the same information. [9.5.2]

Stress-- A measure of forces (internal or external) acting over some cross sectional area of an object. [8.3.3.2]

Structure—The relationship between a set of elements, contributing to the properties of the whole and enabling them to interact. [3.2.3]

Syntax—The grammar defining the valid set of symbols and well-formed linguistic constructs of a language. [3.2.3]

System—A set of elements (people, products [hardware and software], facilities, equipment, material, and processes [automated as well as manual procedures]) that are related and whose behavior satisfies customer and/or operational needs. [3.2.5]

System Performance --is the amount of useful work accomplished by a system.  Outside of specific contexts, performance is estimated in terms of accuracy, efficiency, and speed of executing computer program instructions or transferring data. [7.4.1]

Taxonomy-- (or taxonomical classification) is a hierarchical classification or categorization system in which all the terms belong to a single hierarchical structure and have parent/child or broader/narrower relationships to other terms.  Many taxonomies are hierarchies (and thus, have an intrinsic tree structure), but not all are.  [10.2]

Tracking Station-- is a node in a Connectivity Viewpoint that occupies a fixed location on a planet (including Earth) or asteroid. [7.4.2]

Type—Specifies the set of values allowed and the primitive operations, which an object can provide. Types are grouped into classes, which share the same primitive operations.  [3.2.3]

Unique Identifier --is a value used in specified fields of CCSDS-defined (or other) link layer data structures.  It provides a unique identifier for the Node. [7.4.1]

Use Case --is a list of actions or event steps typically defining the interactions between an actor and a system to achieve a goal.  A Use Case may describe a situation where a system may be used or a potential scenario in which a system receives an external request and responds to it. [4.3.3.1]

View—A representation of a whole system from the perspective of a set of concerns.  Views are themselves modular and well formed, and each view is intended to correspond to exactly one Viewpoint. A View may include representations or correspondences to elements defined in other Viewpoints. [3.2.5]

Viewpoint-- is set of conventions, achieved using a selected set of architectural concepts and structuring rules, for the creation interpretation, and use of an architecture viewpoint to frame one or more particular concerns within a space system. [3.2.5]

Viewpoint Specification—A form of abstraction achieved using a selected set of architectural concepts and structuring rules in order to focus on particular concerns within a space system. A Viewpoint Specification defines a p5.tern or template from which to construct individual views, and it establishes the rules, techniques and methods employed in constructing a view. [3.2.5]

Web Service --is a software component or system designed to support interoperable machine- or application- oriented interaction over a network. A Web service has an interface described in a machine-processable format, specifically Web Services Description Language (WSDL). [11.4.2]

These include the ISO Reference Model of Open Distributed Systems (RM-ODP, [1]), the IEEE Recommended Practice for Architectural Descriptions of Software-Intensive Systems (IEEE 1471-2000, [2]), Software, Systems, and Enterprise - Architecture Description (IEEE/ISO/IEC 42010-2022, [15]), the Standard for Application and Management of the System Engineering Process (IEEE 1220-2005, [3]), OMG Unified Modeling Language (UML, [6], [7], and [8]), Systems Modeling Language (SysML, [9] and [10]), DoD Architecture Framework (DoDAF, [11]), the Open Group Architecture Framework (TOGAF, [12]), the Unified Architecture Framework (UAF) [31], the foundational ISO Basic Reference Model (ISO-BRM, [14]), and others.

The definitions in this section, and others, have been derived from ISO 42010 [15], RM-ODP [1], TOGAF Enterprise Architecture [12], and other well recognized sources, but integrated into a self-consistent whole.  In common with MBSE & RM-ODP all the Viewpoints in RASDS are defined in terms of objects, but this terminology is not strictly the same as “object oriented” design terminology.  In this key instance, and in many others, the reader is cautioned to attend to the specific definitions provided for terms used in RASDS.

Multiple degrees of interoperability are possible, ranging from basic Physical layer (e.g., frequency, modulation and coding) compatibility up to full Application layer information exchange .

The Enterprise Viewpoint is based on the enterprise viewpoint of RM-ODP and Enterprise Architecture methods such as TOGAF.  Some modifications have been made to better describe the concerns of space systems and commonly adopted enterprise object names, such as Space Enterprise, mission, program, and project are adopted.

System management, lifecycle views on systems, scenario specifications, and other aspects that are relevant to the Enterprise Viewpoint may also be addressed in. this viewpoint.

See Information and Operations Viewpoints.

The Enterprise Viewpoint does not explicitly define use of an overview diagram such as the DoDAF OV-1 or AV-1.  However, it is often useful to provide such an overview of an Enterprise to provide easily accessible context for understanding the major objects and roles in the system.

Any graphical icons (spacecraft, antenna, etc) used in this and some other diagrams have no special significance in RASDS++.  Such icons may be employed where they provide additional information as to what is being represented.

The Functional Viewpoint corresponds to the computational viewpoint of RM-ODP. The computational viewpoint of RM-ODP describes the structure of application processes in a distributed processing system.

Objects shown in magenta are defined within the Functional Viewpoint.  Other items, shown in different colors, are referenced by correspondence from other viewpoints.

Figure 5-5 borrowed from CCSDS 371.0-G-1, Fig. 4-2

The Connectivity Viewpoint is one aspect of the Engineering Viewpoint of RM-ODP.  It is called out separately in RASDS because it exposes broad communications and related physical issues and constraints in the design of space data systems, which are distinct from those encountered in typical terrestrial distributed systems.

Ports, and their characteristics, may be described formally using an Electronic Data Sheet (EDS) [44]

This Viewpoint is related to both the Engineering (implemented functionality) and Technical (standards) Viewpoints of RM-ODP.  It is addressed separately in RASDS because of the need for specialized protocols to deal with the physical challenges affecting the design of systems communicating in space.

The Information Viewpoint corresponds directly to the information viewpoint of RM-ODP, but without reference to the static and dynamic views of data and its transformations. This abstract view on the system is refined during implementation by developing concrete specifications that are bound to some particular language or framework.

Detailed descriptions of Information Objects, and the means for managing and operating on them, may be found in a separate document, the Reference Architecture for Space Information Management [5].

RM-ODP describes static, dynamic, and snapshot aspects in their Information Viewpoint.  In RASDS only the static view of information structures and descriptions is treated. Any dynamic aspects of information transformation are to be handled by the corresponding representations of information objects that appear in the Functional and Connectivity Viewpoints.