Report Concerning Space Data System Standards

TM Synchronization and Channel Coding

Recommended Standard

CCSDS 131.0-B-4?

Blue Book

May 2021

# Introduction

## Purpose

The purpose of this Recommended Standard is to specify synchronization and channel coding schemes used with the TM Space Data Link Protocol (reference [1]), the AOS Space Data Link Protocol (reference [2]), or the Unified Space Data Link Protocol (USLP, reference [6]). These schemes are to be used over space-to-ground, space-to-space, or ground-to-space communications links by space missions.

## Document structure

This document is divided into thirteen numbered sections and seven annexes:

a)  section 1 presents the purpose, scope, applicability and rationale of this Recommended Standard and lists the conventions, definitions, and references used throughout the document;

b)  section 2 provides an overview of synchronization and channel coding;

c)  section 3 specifies convolutional coding;

d)  section 4 specifies Reed-Solomon coding;

e)  section 5 concatenated coding;

f)  section 6 specifies Turbo coding;

g)  section 7 specifies low-density parity-check coding of a Transfer Frame;

h)  section 8 specifies low-density parity-check coding of a stream of Sync-Marked Transfer Frames (SMTFs);

i) section 9 specifies use of these codes for forward links (ground to space or space to space);

j)  section 10 specifies the frame synchronization scheme;

k)  section 11 specifies the Pseudo-Randomizer;

l)  section 12 specifies the allowed lengths of Transfer Frames;

m)  section 13 lists the managed parameters associated with synchronization and channel coding;

n)  annex A defines the service provided to the users;

o)  annex B discusses security issues related to TM Channel Coding;

p)  annex C provides the generator matrix circulant table applicable to rate-223/255 LDPC coding (see 7.3);

q)  annex D lists acronyms and terms used within this document;

r)  annex E provides a list of informative references;

s)  annex F provides information on transformation between the Berlekamp (dual basis) and Conventional representations;

t)  annex G provides information on Reed-Solomon coefficients.

## References

[6]  Unified Space Data Link Protocol. Issue 1. Recommendation for Space Data System Standards (Blue Book), CCSDS 732.1-B-1. Washington, D.C.: CCSDS, October 2018.

# Overview

## Architecture

Figure 2-1 illustrates the relationship of this Recommended Standard to the Open Systems Interconnection reference model (reference [3]). Two sublayers of the Data Link Layer are defined for CCSDS space link protocols. The TM, AOS, and USLP Space Data Link Protocols specified in references [1], [2], and [6], respectively, correspond to the Data Link Protocol Sublayer, and provide functions for transferring data using the protocol data unit called the Transfer Frame. The Synchronization and Channel Coding Sublayer provides additional functions necessary for transferring Transfer Frames over a space link. These functions are error-control coding/decoding, Transfer Frame delimiting/synchronizing, and bit transition generation/removal.

Figure 2-1: Relationship with OSI Layers

## summary of functions

### general

The Synchronization and Channel Coding Sublayer provides the following three functions for transferring Transfer Frames over a space link:

a)  error-control coding, including frame validation;

b)  synchronization; and

c) pseudo-randomizing.

### Error-control coding

This Recommended Standard specifies the following four types of error-control coding:

a)  convolutional coding (section 3);

b)  Reed-Solomon coding (section 4);

c)  Turbo coding (section 5);

d)  Low-Density Parity-Check (LDPC) coding (sections 7 and 8).

One of the convolutional codes described in section 3 alone may be satisfactory depending on performance requirements.

For Physical Channels, which are bandwidth-constrained and cannot tolerate the increase in bandwidth required by the basic convolutional code specified in 3.3, the punctured convolutional codes specified in 3.4 have the advantage of smaller bandwidth expansion.

Alternatively, the Reed-Solomon codes and the high rate LDPC code specified in sections 4, 7, and 8 also have the advantage of smaller bandwidth expansion and have the capability to indicate the presence of uncorrectable errors. Where a greater coding gain is needed than can be provided by a convolutional code or Reed-Solomon code alone, a concatenation of a convolutional code as the inner code with a Reed-Solomon code as the outer code may be used for improved performance.

The Turbo codes specified in section 5 or the LDPC codes specified in sections 7 and 8 may be used to obtain even greater coding gain where the environment permits.

The AOS and USLP data link layer protocols are symmetrical and may be used in return, cross-link, or forward directions.

NOTES

- In this Recommended Standard, the characteristics of the codes are specified only to the extent necessary to ensure interoperability and cross-support. The specification does not attempt to quantify the relative coding gain or the merits of each approach discussed, nor does it specify the design requirements for encoders or decoders.

- The domains of applicability for the codes specified in this document are delineated in Mission Profiles for TM Synchronization and Channel Coding (reference [E5]).

# Convolutional coding

## general

### frame validation

When TM, AOS, or USLP Transfer Frames are used, the Frame Error Control Field (FECF) specified in references [1], [2], or [6] shall be used to validate the Transfer Frame, unless the convolutional code is concatenated with an outer Reed-Solomon code (see section 4).

NOTE   – 	If the decoder’s correction capability is exceeded, undetected bursts of errors may appear in the output.

# reed-solomon coding

## general

### frame validation

The FECF specified in references [1], [2], or [6] is optional. The system designer may choose to use it for additional codeblock validation, particularly with the E=8 code.

NOTE   – 	The Reed-Solomon code with E=16 has an extremely low undetected error rate, and that with E=8 has an undetected error rate low enough for some applications. Therefore the R-S decoder may be used alone to validate the codeblock, and consequently the contained TM Transfer Frame (reference [1]), AOS Transfer Frame (reference [2]), or USLP Transfer Frame (reference [6]).

## Specification

### Reed-solomon codeblock partitioning and virtual fill

4.3.8.1  Parts of the partitioned Reed-Solomon codeblock (see figure 4-1) are defined as follows:

a)  	The Reed-Solomon Check Symbols shall consist of the trailing 2EI symbols (2EIJ bits) of the codeblock.

NOTES

1	As an example, when E = 16 and k = 223, for I=5 this is always 1280 bits.

2	The Transfer Frame is defined by the TM Space Data Link Protocol (reference [1]), the AOS Space Data Link Protocol (reference [2]), or the Unified Space Data Link Protocol (reference [6]). For constraints on the length of the Transfer Frame, see section 12.

b)   The Attached Sync Marker used with R-S code

  - shall be the 32-bit pattern specified in section 10;

  - shall precede the transmitted codeblock.

NOTE   – 	Frame synchronizers should therefore be set to expect a marker at every transmitted codeblock + 32 bits.

c) 	The transmitted codeblock shall consist of the Transfer Frame (without the 32-bit sync marker) and R-S check symbols.

# turbo coding

## general

### frame validation

When Turbo codes are used with TM, AOS, or USLP Transfer Frames, the FECF specified in references [1], [2], or [6], respectively, shall be used to validate the Transfer Frame.

NOTE   – 	The Reed-Solomon code with E=16 has an extremely low undetected error rate, and that with E=8 has an undetected error rate low enough for some applications. Therefore the R-S decoder may be used alone to validate the codeblock, and consequently the contained TM Transfer Frame (reference [1]), AOS Transfer Frame (reference [2]), or USLP Transfer Frame (reference [6]).

# low-density parity-check coding of a transfer frame

## general

### frame validation

#### The LDPC decoder may be used alone to validate the codeword, and consequently the contained TM Transfer Frame (reference 1), AOS Transfer Frame (reference [2]), or USLP Transfer Frame (reference [6]).

#### The FECF specified in references [1], [2], and [6] is optional, and the system designer may choose to use it for additional frame validation.

NOTE   – 	The undetected frame and bit error rates of these LDPC codes lie several orders of magnitude below the corresponding detected error rates for any given operating signal-to-noise ratio.

# low-density parity-check coding of a stream of sync-marked transfer frames

## General

### Synchronization

[Note to editor: the previous two headings appear to have been confused in the currently published book]

### CSM bit patterns

### frame validation

The LDPC decoder may be used alone to validate the codeword, and consequently the contained TM Transfer Frame(s) (reference [1]), AOS Transfer Frame(s) (reference [2]), or USLP Transfer Frame(s) (reference [6]). Whenever an LDPC codeword fails decoding, the Quality Indicator (see annex A) of all the Transfer Frames affected by that decoding shall be set to show that there is an uncorrectable error in received Transfer Frame(s).

NOTE   – 	The FECF specified in references [1], [2], and [6] is optional, and the system designer may choose to use it for additional checks.

# use of telecommunications channel codes for forward links (ground to space or space to space)

## overview

The error control codes specified in this document are designed for use with fixed length space data link Transfer Frames as defined in the TM Space Data Link Protocol (reference [1]), AOS Space Data Link Protocol (reference [2]), or Unified Space Data Link Protocol (reference [6]).  The AOS and USLP protocols are defined for Telemetry (downlink) use, as is TM, but AOS and USLP are also designed for use for ground-to-space and space-to-space communications.  This use will typically be adopted for high rate missions, missions where the forward and return links are symmetric, or for missions that are adopting upper layer networking protocols like DTN or IP.

## Forward Link turbo codes

Turbo codes are best suited to power-constrained links, where the signal to noise ratio, Eb/N0, is a dominant concern. Their code rates of r ≤1/2 provide greater coding gain than LDPC codes, at a cost of greater bandwidth expansion.  They are best suited to links beyond low-Earth orbit.

9.3.1	For AOS or USLP forward links, any of the turbo codes in section 6 shall be selected.  They offer code rates of r =1/2, 1/3, 1/4, and 1/6, and block lengths of 1784, 3568, 7136, and 8920 information bits.

NOTE	–	When a low-rate turbo code (particularly 1/4 or 1/6) is used near its decoding threshold, the symbol SNR may be below -5 dB.  The radio receiver’s symbol tracking loop may require an uncommonly narrow loop bandwidth, and an external means for Doppler compensation.

## Forward Link Low-Density Parity-Check codes

LDPC codes are best suited to high data-rate links, due to their code rates of r ≥1/2, and the potentially parallel implementation architecture for the decoder.  They are best suited to links where bandwidth is limited, onboard computational resources  are available to support an iterative decoder, and with a physical-layer modulation that supports at least two code symbols per modulation symbol (QPSK/OQPSK and above).

9.4.1	For AOS or USLP forward links, any of the LDPC codes in section 7 shall be selected.  They offer code rates of r=1/2, 2/3, 4/5, and approximately 7/8.  Block lengths of 1024, 4096, and 16384 information bits are available with the first three code rates; and 7136 information bits in the last case.

## Forward Link coding of a stream of sync-marked transfer frames

In some cases, it is most practical to use a Transfer Frame length that need not match the information block length of the error correcting code.  To support this application, synchronization markers may be prepended to the Transfer Frames, the resulting Synchronization Marked Transfer Frames (SMTFs) concatenated into a stream, and then “sliced” according to the information block length of the code.

9.5.1	When a stream of Synchronization Marked Transfer Frames is chosen for an AOS or USLP forward link, the encoding procedure defined in Section 8 shall be selected.

# Transfer frame lengths

## Overview

Neither the TM Space Data Link Protocol (reference [1]) nor the AOS Space Data Link Protocol (reference [2]) specifies the length of Transfer Frames because there are constraints on the Transfer Frame length depending on the selected coding options.

The Unified Space Data Link Protocol (reference [6]) contains a frame length field that is specified by the sender for both fixed or variable length transfer frames.

The constraints on Transfer Frame lengths specified in this section apply to TM Transfer Frames, AOS Transfer Frames, and USLP Transfer Frames (when fixed length transfer frames are used).

Service

(Normative)

  - Overview of the service

The TM Synchronization and Channel Coding provides unidirectional (one way) transfer of a sequence of fixed-length TM, AOS, or USLP Transfer Frames at a constant frame rate over a Physical Channel across a space link, with optional error detection/correction.

Only one user can use this service on a Physical Channel.

  - service parameters

## A3.1    FRAME

A3.1.1   The Frame parameter is the service data unit of this service and shall be either a TM Transfer Frame defined in reference [1], an AOS Transfer Frame defined in reference [2], or a USLP Transfer Frame defined in reference [6].

Acronyms and Terms

(Informative)

  - introduction

This annex lists key acronyms and terms that are used throughout this Recommended Standard to describe synchronization and channel coding.

  - Acronyms

USLP	Unified Space Link Protocol