REVIEW ITEM DISPOSITION (RID):

RED BOOK RID INITIATION FORM

AGENCY RID NUMBER: GSFC 9/15/2005 16:00:00 Z

SUBMITTING ORGANIZATION (Agency, Center): GSFC

------------------------------------------------------------------

REVIEWER'S NAME:	Victor J. Sank

CODE:			567

E-MAIL ADDRESS:		victor.sank@gsfc.nasa.gov

TELEPHONE:		301 286-2645

------------------------------------------------------------------

DOCUMENT NUMBER:	CCSDS 211.1-P-2.1	Pink Book, Issue 2

DOCUMENT NAME:	Proximity-1 Space Link Protocol Physical Layer

DATE ISSUED:		December 2004

PAGE NUMBER:		3-9	PARAGRAPH NUMBER:  3.4.7.2

RID SHORT TITLE:   	Reduce Jitter Restriction

------------------------------------------------------------------

DESCRIPTION OF REQUESTED CHANGE:  (Use From: "..." To "..." format)

From:     Each symbol period, as measured a the output of the transmitter, shall differ by no more than 1% from the symbol period corresponding to the Proximity-1 data rate in use.

To: :     Each symbol period, as measured a the output of the transmitter, shall differ by no more than 3% from the symbol period corresponding to the Proximity-1 data rate in use.

------------------------------------------------------------------

CATEGORY OF REQUESTED CHANGE:  Recommended

Technical Fact ___    Recommended ___    Editorial ___

NOTES:

TECHNICAL FACT:  Major technical change of sufficient magnitude as to

render the Recommendation inaccurate and unacceptable if not

corrected.  (Supporting analysis/rationale is essential.)

RECOMMENDED:  Change of a nature that would, if incorporated, produce

a marked improvement in document quality and acceptance.

EDITORIAL:  Typographical or other factual error needing correction.

(This type of change will be made without feedback to submitter.)

------------------------------------------------------------------

SUPPORTING ANALYSIS:

Original 1% was too restrictive and unnecessary.  With square unfiltered bits expressed as dB the original is 1.01 = 0.043 dB which loosely would affect the power efficiency of the receiver by about 0.043 dB in terms of Eb/No performance.  1.03 = 0.128 dB will only cause a small additional loss of .085 dB which with spectrum efficient modulation will be even less.  Although this is not a rigorous evaluation, it should give some indication that 3% jitter will not cause undue loss and will allow more flexible design.

------------------------------------------------------------------

DISPOSITION: